Copyright and Trademark Information

ADSCSI.CPX and this documentation are copyright 1991 by ICD,

Inc.  All rights reserved.  These files are authorized for

distribution only to owners of ICD's STHA, Advantage, and

AdSCSI host adapters directly on disk from ICD, Inc. or

electronically from our BBS, GEnie, or CompuServe.  No other

distribution is authorized.  AdSCSI is a trademark of ICD,

Inc.  Other product names mentioned may be registered

trademarks or trademarks of their holders.

Introduction

Atari's new extensible control panel XCONTROL offers a new

level of flexibility to ST owners.  It is no longer

necessary to have a separate desk accessory for each

hardware or software feature.  With ADSCSI.CPX, ICD adds

several host adapter control and information features to

XCONTROL.

ADSCSI.CPX and ADSPEED.CPX (for ICD's AdSpeed ST

accelerator) are the first, to our knowledge, third party

extensions to Atari's new XCONTROL extensible control panel.

At this time, this control panel is included with Mega STe and

TT computers and available on GEnie and CompuServe.  The new

control panel does not require the new versions of TOS in these

computers to operate and will work fine on existing ST systems.

Installation

Simply copy the file ADSCSI.CPX to the directory containing

the rest of your *.CPX files.  For details on XCONTROL and

CPX installation, consult Atari's documentation.

Use

To access ADSCSI.CPX, open your control panel and scroll if

necessary until you see the AdSCSI Options entry.  Double

click on this entry to bring up the ADSCSI.CPX main window.

The main window for ADSCSI.CPX contains four sections:

Options, Information, Sense Codes, and Relog HDs.

The Options section allows you to enable or disable the

AdSCSI Hard Disk Handler write cache and enable and disable

write verify.  The buttons beside these options display the

current status (On or Off).  To change the status of either

of these features, click on the button, then click on your

choice.  Any changes made will not take effect until you

close the window or click on OK.

Clicking on OK or the window's close button or hitting the

RETURN key will put into effect any changes made in the

Options section.  Clicking on Cancel will quit ADSCSI.CPX

without making any changes.

Clicking on the Information box will bring up another window

giving information about the hard disk driver.  There is

more information available than will fit in the window.  To

see more, use the slider bar or click on the up or down

arrows.

The following information is available in the Information

window:

ICDBOOT Version.  This is the version of the ICD Hard Disk

Handler currently running in the system.

Folders Allocated.  This is the number of extra folders

allocated by the hard disk handler.  This number is set with

HDUTIL.PRG.

Max Sector Size.  This is the maximum sector size the driver

is configured for.  This is either the value saved in the

hard disk driver by HDUTIL.PRG or the largest sector size of

any partition in the system at boot, whichever is greater.

Cache Read Hits.  This number is the percentage of sector

reads attempted by the operating system that were read from

the cache rather than from the drive.  This number will

always be less than 100%, since some reads must come from

the drive.

Cache Write Hits.  This number is the percentage of sector

writes attempted by the operating system where the sector

was in the cache.  Unlike Cache Read Hits, this number is

not a clear indication of write cache effectiveness.

Further improvement of system performance is provided by the

write cache saving writes until periods of disk inactivity.

But any number seems to be better than none.

To exit the Information display, click on OK or the window's

close button.

The Sense Codes section shows a list of common sense codes

and their definitions.  Since there is no sense code

standard, and these numbers do come from the drive's

controller, it is possible that these definitions will not

hold for your system.  As with the information display,

there is more information available than will fit in the

window.  Click on the up and down arrows or use the scroll

bar to see the whole display.

Clicking on Relog HDs will cause the driver to check all

enabled IDs for hard drives.  This button is useful if you

have a removable media drive that was not ready (no

cartridge installed) when the computer booted.  Just insert

a cartridge, wait until the drive is ready, then click on

Relog HDs.