---------------------------------------------------------------------------

CyReL SUNRISE M16-1280 MONITOR LIST

Version: 1.6b            Date: 2/92

Version: 1.7b            Date: 9/92

---------------------------------------------------------------------------

CONTENTS

A. Introduction

B. Requesting Your Assistance

C. Disclaimer

D. MONITOR LIST

---------------------------------------------------------------------------

[A] Introduction

---------------------------------------------------------------------------

This list contains informations about 300+ color monitors from

Aamazing Technologies

ADI

Amax Engeneering

Amdec

American Mitac

Amstrad

AOC

ARX

AST

Atari

Conrac

Cordata

CTX International

Deawoo

Dell

Eizo

Epson

Falco

Fora

Goldstar

Hantarex

Highscreen

Hitachi

IBM

Idek

Iiama

Intec

IOcomm

JVC

KFK-Computertek

Laser

Loop Technologies

Mag Innovision

Magnavox

Microvitec

Miro

Mitsuba

Mitsubishi

Nanao

NEC

Nokia

Optiquest

Packard Bell

Panasonic

PC Craft

Princeton

Quimax

Qume

Radius

Relisys

Safe Technologies

Sakata

Sampo

Samsung

Samtron

Santec

Sceptre Technologies

Seiko

Sony

Tandy

Tatung

Taxan

Toshiba

TRL

TVM

ViewSonic

Voltron

Wyse Technology

ZDS

Zenith

The list has been compiled from a number of sources including product

specifications, monitor manuals and informations provided by various

manufacturers.

We would like to thank those companies for their kind and prompt support.

Use this monitor list to verify if your (or any other) monitor is

capable of operating in the predefined or intended modes.

---------------------------------------------------------------------------

[B] Requesting Your Assistance

---------------------------------------------------------------------------

Should you find that the information provided is incorrect or missing, please

fill in the appropriate values. Should you have information about monitors

that are not listed yet, we would appreciate it if you could add the

specifications to the existing list and send a copy of your revised file to

Cybercube. We will make all new and improved versions available to every

registered developer/user requesting an update. Therefore everyone will

benefit, even from small contributions. Please do not hesitate to send your

information regardless of extend and completeness. Your contributions will help

to establish a satisfied M16-1280 user-base. PLEASE CONSIDER YOUR COOPERATION.

THANK YOU.

---------------------------------------------------------------------------

[C] Disclaimer

---------------------------------------------------------------------------

THE INFORMATION FURNISHED BY CYBERCUBE RESEARCH LIMITED IN THIS FILE IS

BELIEVED TO BE ACCURATE; HOWEVER, NO RESPONSIBILITY IS ASSUMED FOR ITS USE.

---------------------------------------------------------------------------

[D] Monitor List

---------------------------------------------------------------------------

#MN: Aamazing Techn. CM-1726K

#MB: 48 MHz

#mH: 48 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Aamazing Techn. CM-8428SX

#MB: 40 MHz

#mH: 31.5 kHz

#MH: 35.5 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Aamazing Techn. CM8486X

#MB: 80 MHz

#mH: 31 kHz

#MH: 64 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: Acer 7013A

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 50 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: AcerView 25 LR

#MB: 80 MHz

#mH: 31.5 kHz

#MH: 60 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: AcerView 33 DL

#MB: 45 MHz

#mH: 31.5 kHz

#MH: 35 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: AcerView 34 TL

#MB: 65 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: ADI MicroScan 3E-LR

#MB: 75 MHz

#mH: 30 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 100 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: ADI Systems MicroScan 4A

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: ADI Systems MicroScan 4E

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: AE 12 C LCD-Monitor

#MB: 32 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 60 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: AmaxEng. ImpressionCM1448M

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Amdek AM/432LC (Mono)

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: Analog

#MN: Amdek AM/732LC

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Amdek AM/732+

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Amdek AM/738N

#MB: 45 MHz

#mH: 31.5 kHz

#MH: 38.0 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Amdek AM/738+

#MB: 45 MHz

#mH: 31.5 kHz

#MH: 38.0 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Amdek AM/817

#MB: 125 MHz

#mH: 30.0 kHz

#MH: 76.0 kHz

#mV: 40 Hz

#MV: 120 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: Amdek AM1817

#MB: 125 MHz

#mH: 30 kHz

#MH: 76 kHz

#mV: 40 Hz

#MV: 120 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: American Mitac M1458/N-1

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: American Mitac M1558 N-I

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: American Mitac M1740 N-I

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Amstrad PC 10 MD

#MB: 28 MHz

#mH: 31.5 kHz

#MH: 35.5 kHz

#mV: 70 Hz

#MV: 70 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Amstrad PC 10 HRCD

#MB: 28 MHz

#mH: 31.5 kHz

#MH: 35.5 kHz

#mV: 70 Hz

#MV: 70 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Amstrad PC 14 CD

#MB: 28 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Amstrad PC 14 FMD

#MB: 28 kHz

#mH: 31.5 kHz

#MH: 35.5 kHz

#mV: 70 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Amstrad PC 14 HRCD

#MB: 28 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Amstrad PC 14 HRCDR

#MB: 28 MHz

#mH: 31.5 kHz

#MH: 35.4 kHz

#mV: 70 Hz

#MV: 70 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Amstrad PC 14 M 28

#MB: 45 MHz

#mH: 31.5 kHz

#MH: 38 kHz

#mV: 55 Hz

#MV: 70 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Amstrad PC 14 M 39

#MB: 45 MHz

#mH: 31.5 kHz

#MH: 38 kHz

#mV: 55 Hz

#MV: 70 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: AOC CM-333

#MB: 32 MHz

#mH: 31.46 kHz

#MH: 31.46 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: AOC CM-335

#MB: 40 MHz

#mH: 30 kHz

#MH: 40 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: AOC CM336 (PT: CMLB 336)

#MB: 65 MHz

#mH: 30 kHz

#MH: 50 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: AOC CM337 (PT: CMLB 337)

#MB: 65 MHz

#mH: 30 kHz

#MH: 56 kHz

#mV: 56 Hz

#MV: 80 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: AOC CM-518

#MB: 40 MHz

#mH: 30 kHz

#MH: 65 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: AOC CM-734

#MB: 40 MHz

#mH: 30 kHz

#MH: 64 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: AOC CM-735

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: AOC CM-736

#MB: 48 MHz

#mH: 30 kHz

#MH: 76 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: ARX CM 1448 VA (Prototype)

#MB: 65 MHz

#mH: 30 kHz

#MH: 50 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: ARX DCM 1468 (Prototype)

#MB: 40 MHz

#mH: 30 kHz

#MH: 60 kHz

#mV: 50 Hz

#MV: 100 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: ARX DCM 1568 (Prototype)

#MB: 40 MHz

#mH: 30 kHz

#MH: 65 kHz

#mV: 50 Hz

#MV: 100 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Asem 4992 LR

#MB: 44 MHz

#mH: 31.5 kHz

#MH: 35.5 kHz

#mV: 50 Hz

#MV: 87 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: AST ASTCVGA

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: ATARI PTC1426

#MB: 40 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 60 Hz

#MX: 800

#MY: 512

#UC: RGB Analog

#MN: ATARI SM 124 (Mono)

#MB: 32 MHz

#mH: 35.7 kHz

#MH: 35.7 kHz

#mV: 71.2 Hz

#MV: 71.2 Hz

#MX: 640

#MY: 400

#UC: Digital

#MN: ATARI SC 1224

#MB: 16 MHz

#mH: 15.625 kHz

#MH: 15.75 kHz

#mV: 50 Hz

#MV: 60 Hz

#MX: 640

#MY: 200

#UC: RGB Analog

#MN: ATARI SC 1425

#MB: 32 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 60 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Bermon BER-CM-1028

#MB: 45 MHz

#mH: 30 kHz

#MH: 38 kHz

#mV: 47 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Commodore 1407

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Commodore 1935-II

#MB: 45 MHz

#mH: 31.5 kHz

#MH: 35.5 kHz

#mV: 50 Hz

#MV: 70 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Commodore 1936

#MB: 45 MHz

#mH: 31.5 kHz

#MH: 35.5 kHz

#mV: 56 Hz

#MV: 70 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Commodore 1944

#MB: 45 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Commodore 1944 S

#MB: 45 MHz

#mH: 31.5 kHz

#MH: 35.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Commodore 1960 Tri-Sync

#MB: 45 MHz

#mH: 15.6 kHz

#MH: 35.5 kHz

#mV: 50 Hz

#MV: 70 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: CONRAC 7164

#MB: 40 MHz

#mH: 15 kHz

#MH: 22 kHz

#mV: 50 Hz

#MV: 60 Hz

#MX: 640

#MY: 350

#UC: RGB TTL

#MN: CONRAC 7111

#MB: 25 MHz

#mH: 15 kHz

#MH: 25 kHz

#mV: 50 Hz

#MV: 60 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: CONRAC 7119

#MB: 40 MHz

#mH: 15 kHz

#MH: 29 kHz

#mV: 50 Hz

#MV: 60 Hz

#MX: 1280

#MY: 950

#UC: RGB Analog

#MN: CONRAC 7120

#MB: 40 MHz

#mH: 15 kHz

#MH: 29 kHz

#mV: 50 Hz

#MV: 60 Hz

#MX: 1280

#MY: 950

#UC: RGB Analog

#MN: CONRAC 7121

#MB: 40 MHz

#mH: 29 kHz

#MH: 38 kHz

#mV: 50 Hz

#MV: 60 Hz

#MX: 1280

#MY: 1024

#UC: Analog

#MN: CONRAC 7122

#MB: 40 MHz

#mH: 29 kHz

#MH: 38 kHz

#mV: 50 Hz

#MV: 60 Hz

#MX: 1280

#MY: 1024

#UC: Analog

#MN: CONRAC 7211

#MB: 40 MHz

#mH: 15 kHz

#MH: 37 kHz

#mV: 50 Hz

#MV: 60 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: CONRAC 7211

#MB: 40 MHz

#mH: 15 kHz

#MH: 37 kHz

#mV: 50 Hz

#MV: 60 Hz

#MX: 1280

#MY: 960

#UC: RGB Analog

#MN: CONRAC 7250

#MB: 40 MHz

#mH: 15.75 kHz

#MH: 37 kHz

#mV: 47 Hz

#MV: 80 Hz

#MX: 1024

#MY: 1024

#UC: TTL, Analog

#MN: CONRAC 7311

#MB: 70 MHz

#mH: 45 kHz

#MH: 50 kHz

#mV: 47 Hz

#MV: 63 Hz

#MX: 1024

#MY: 800

#UC: RGB, Composite RGB

#MN: CONRAC 7351

#MB: 110 MHz

#mH: 62.5 kHz

#MH: 67 kHz

#mV: 47 Hz

#MV: 63 Hz

#MX: 1280

#MY: 1024

#UC: RGB, Composite RGB

#MN: CONRAC 7550

#MB: 150 MHz

#mH: 67 kHz

#MH: 80 kHz

#mV: 40 Hz

#MV: 100 Hz

#MX: 1600

#MY: 1280

#UC: RGB Analog

#MN: Copam LR 1400 C

#MB: 45 MHz

#mH: 31.5 kHz

#MH: 35.5 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Cordata Techn. CMC-1414BA

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Cordata Techn. CMC-1500BA

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 60 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Cordata Techn. CMC-1700M

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: CTX Int'l 1760-F

#MB: 80 MHz

#mH: 30 kHz

#MH: 65 kHz

#mV: 50 Hz

#MV: 100 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: CTX Int'l CVG-5432

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: CTX Int'l 5468-NI

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Deawoo CM-140F (Mono)

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 50 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: Analog

#MN: Deawoo CM-1402SW (Mono)

#MB: 40 MHz

#mH: 31.5 kHz

#MH: 35.6 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: Analog

#MN: Deawoo CM-1700 (Mono)

#MB: 110 MHz

#mH: 31.5 kHz

#MH: 64 kHz

#mV: 50 Hz

#MV: 60 Hz

#MX: 1280

#MY: 1024

#UC: Analog

#MN: Deawoo CMC-1412A

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 34 kHz

#mV: 50 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Deawoo CMC-1412AD

#MB: 40 MHz

#mH: 31.5 kHz

#MH: 35.6 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Deawoo CMC-1412D

#MB: 22 MHz

#mH: 15.7 kHz

#MH: 21.8 kHz

#mV: 50 Hz

#MV: 60 Hz

#MX: 640

#MY: 350

#UC: TTL (64 color)

#MN: Deawoo CMC-1412T

#MB: 15 MHz

#mH: 15.7 kHz

#MH: 15.7 kHz

#mV: 50 Hz

#MV: 60 Hz

#MX: 640

#MY: 200

#UC: TTL (16 color)

#MN: Deawoo CMC-1414AD

#MB: 40 MHz

#mH: 31.5 kHz

#MH: 35.6 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Deawoo CMC-1414A

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 35.2 kHz

#mV: 50 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Deawoo CMC-1414BA

#MB: 70 MHz

#mH: 31.5 kHz

#MH: 50 kHz

#mV: 40 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Deawoo CMC-1500BA

#MB: 70 MHz

#mH: 30 kHz

#MH: 57 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Deawoo CMC-1700M

#MB: 110 MHz

#mH: 30 kHz

#MH: 65 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: Deawoo CMC-1700A

#MB: 110 MHz

#mH: 31.5 kHz

#MH: 64 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1280

#MY: 768

#UC: RGB Analog

#MN: Deawoo CMC-2100M

#MB: 110 MHz

#mH: 30 kHz

#MH: 65 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: Deawoo CMC-2100H

#MB: 100 MHz

#mH: 59 kHz

#MH: 65 kHz

#mV: 50 Hz

#MV: 85 Hz

#MX: 1280

#MY: 768

#UC: RGB Analog

#MN: Dell GPD 16C

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Dell UltraScan 14C

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Digiscan MPMAC

#MB: 40 MHz

#mH: 15 kHz

#MH: 36 kHz

#mV: 45 Hz

#MV: 120 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: EIZO F550i

#MB: 80 MHz

#mH: 30 kHz

#MH: 65 kHz

#mV: 55 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: EIZO 9065S

#MB: 40 MHz

#mH: 30 kHz

#MH: 49 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: EIZO Flexscan 9080i

#MB: 60 MHz

#mH: 30 kHz

#MH: 64 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: Analog

#MN: EIZO Flexscan T560i

#MB: 120 MHz

#mH: 30 kHz

#MH: 78 kHz

#mV: 55 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1024

#UC: Analog

#MN: Epson A804031

#MB: 28.5 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Epson America  PS Monitor

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Facit 17 33 03

#MB: 32 MHz

#mH: 31.5 kHz

#MH: 35.5 kHz

#mV: 50 Hz

#MV: 87 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Facit M2103

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 57 Hz

#MV: 73 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Falco Data Products FV15

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Fora Addonics MON-7C8B

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Fora Addonics MON-C171

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Goldstar 1490

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Hantarex CT 900 (1 SR 26)

#MB: 15 MHz

#mH: 15.6 kHz

#MH: 15.6 kHz

#mV: 50 Hz

#MV: 60 Hz

#MX: 580

#MY: 312

#UC: FBAS, RGB

#MN: Hantarex CT 9000 (1 MR 14)

#MB: 15 MHz

#mH: 15.6 kHz

#MH: 15.6 kHz

#mV: 50 Hz

#MV: 60 Hz

#MX: 390

#MY: 312

#UC: FBAS, RGB

#MN: Highscreen MS 800

#MB: 25 MHz

#mH: 15.5 kHz

#MH: 35 kHz

#mV: 45 Hz

#MV: 80 Hz

#MX: 800

#MY: 560

#UC: RGB Analog

#MN: Highscreen MS 1280

#MB: 110 MHz

#mH: 30 kHz

#MH: 64 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1024

#UC: Analog

#MN: Hitachi SuperScan15 CM1584MU

#MB: 65 MHz

#mH: 30 kHz

#MH: 58 kHz

#mV: 50 Hz

#MV: 100 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Hitachi15MVX(CM1584ME-301)

#MB: 75 MHz

#mH: 30 kHz

#MH: 58 kHz

#mV: 50 Hz

#MV: 100 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Hitachi CM2086A1

#MB: 32 MHz

#mH: 47.5 kHz

#MH: 52 kHz

#mV: 55 Hz

#MV: 65 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Hitachi CM1686A1

#MB: 32 MHz

#mH: 47.5 kHz

#MH: 52 kHz

#mV: 55 Hz

#MV: 65 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Hitachi CM2086A3

#MB: 45 MHz

#mH: 58 kHz

#MH: 64 kHz

#mV: 55 Hz

#MV: 65 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: Hitachi CM1686A3

#MB: 45 MHz

#mH: 58 kHz

#MH: 64 kHz

#mV: 55 Hz

#MV: 65 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: Hitachi CM 1588 AE

#MB: 70 Mz

#mH: 31.5 kHz

#MH: 48.8 kHz

#mV: 50 Hz

#MV: 60 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Hitachi CM 2073 AE

#MB: 90 MHz

#mH: 31.5 kHz

#MH: 48.8 kHz

#mV: 50 Hz

#MV: 60 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: Hitachi CM 2073 APR

#MB: 90 MHz

#mH: 31.5 kHz

#MH: 48.8 kHz

#mV: 50 Hz

#MV: 60 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: IBM 8512

#MB: 31.5 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: IBM 8513

#MB: 31.5 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Idek MF-5014

#MB: 32 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 60 Hz

#MX: 1024

#MY: 768

#UC: ??

#MN: Idek MF-5015/A

#MB: 30 MHz

#mH: 15 kHz

#MH: 38.5 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: Analog, TTL

#MN: Idek MF-5115

#MB: 50 MHz

#mH: 21.8 kHz

#MH: 50 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog, TTL

#MN: Idek MF-5117

#MB: 55 MHz

#mH: 20 kHz

#MH: 50 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: Analog, TTL

#MN: Idek MF-5217

#MB: 55 MHz

#mH: 30 kHz

#MH: 57 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: Analog

#MN: Idek MF-5317

#MB: 120 MHz

#mH: 30 kHz

#MH: 80 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1280

#UC: Analog

#MN: Idek MF-5021

#MB: 65 MHz

#mH: 15.5 kHz

#MH: 38.5 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: Analog, TTL

#MN: Idek MF-5121

#MB: 65 MHz

#mH: 21.8 kHz

#MH: 50 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: Analog, TTL

#MN: Idek MF-5221

#MB: 120 MHz

#mH: 30 kHz

#MH: 80 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1280

#UC: Analog

#MN: Idek MF-5321

#MB: 120 MHz

#mH: 30 kHz

#MH: 80 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1280

#UC: Analog

#MN: Idek MF-5421

#MB: 120 MHz

#mH: 30 kHz

#MH: 80 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1600

#MY: 1280

#UC: Analog

#MN: Idek SR-7114

#MB: 24 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: Analog

#MN: Iiyama MF-5115

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Iiyama MF-5117

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Iiyama MR-5314

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Imtec 1430V

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Iocomm ThinkSync 4E

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Iocomm ThinkSync 5

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 60 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Iocomm ThinkSync 7M

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 60 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: JVC GD-H6116SFW

#MB: 110 MHz

#mH: 31.5 kHz

#MH: 63.3 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 1260

#MY: 1024

#UC: Analog

#MN: KFC-Computek CT-14

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Laser 6448

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 50 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Lion CN-1470

#MB: 65 MHz

#mH: 29 kHz

#MH: 57 kHz

#mV: 47 Hz

#MV: 100 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Lion CN-1470A

#MB: 65 MHz

#mH: 29 kHz

#MH: 48 kHz

#mV: 47 Hz

#MV: 100 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Lion CT-1448

#MB: 30 MHz

#mH: 31 kHz

#MH: 35 kHz

#mV: 60 Hz

#MV: 80 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Lion CT-1469

#MB: 45 MHz

#mH: 29 kHz

#MH: 38 kHz

#mV: 47 Hz

#MV: 100 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Lion CT-1469 II

#MB: 40 MHz

#mH: 31 kHz

#MH: 35 kHz

#mV: 60 Hz

#MV: 87 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Lion CX-1469

#MB: 45 MHz

#mH: 29 kHz

#MH: 38 kHz

#mV: 47 Hz

#MV: 100 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Loop Technology CN1764

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Loop Technology CT1458N

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: MAG Innovision PMV1448

#MB: 65 MHz

#mH: 30 kHz

#MH: 49 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: MAG Innovision PMV14VC+

#MB: 45 MHz

#mH: 30 kHz

#MH: 37 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: MAG Innovision MX14H

#MB: 100 MHz

#mH: 30 kHz

#MH: 64 kHz

#mV: 50 Hz

#MV: 120 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: MAG Innovision MX14S

#MB: 100 MHz

#mH: 30 kHz

#MH: 64 kHz

#mV: 50 Hz

#MV: 120 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: MAG Innovison MX15F

#MB: 100 MHz

#mH: 30 kHz

#MH: 68 kHz

#mV: 50 Hz

#MV: 120 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: MAG Innovision MX17F

#MB: 120 MHz

#mH: 40 kHz

#MH: 68 kHz

#mV: 50 Hz

#MV: 120 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: MAG Innovision MX17S

#MB: 120 MHz

#mH: 30 kHz

#MH: 68 kHz

#mV: 50 Hz

#MV: 120 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: MAG Magic View II PMV1448L

#MB: 65 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: MAG MagicView14 Dig.(MX14)

#MB: 100 MHz

#mH: 30 kHz

#MH: 64 kHz

#mV: 50 Hz

#MV: 120 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: MAG Magic View 15H (MX15H)

#MB: 100 MHz

#mH: 30 kHz

#MH: 68 kHz

#mV: 50 Hz

#MV: 120 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: MAG MX17H

#MB: 110 MHz

#mH: 30 kHz

#MH: 68 kHz

#mV: 50 Hz

#MV: 120 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: Magnavox 7CM320

#MB: 32 MHz

#mH: 31.5

#MH: 35.2 kHz

#mV: 56 Hz

#MV: 70 Hz

#MX: 800

#MY: 600

#UC: Analog

#MN: Magnavox Pro 9CM082

#MB: 18 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Magnavox CM9032

#MB: 18 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Microvitec PRO

#MB: 32 MHz

#mH: 15 kHz

#MH: 36 kHz

#mV: 40 Hz

#MV: 100 Hz

#MX: 800

#MY: 600

#UC: Analog, RGB TTL

#MN: Microvitec PRO-SCAN 17 FST

#MB: 100 MHz

#mH: 30 kHz

#MH: 65 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1024

#UC: Analog

#MN: Microvitec Proscan 1550FST

#MB: 65 MHz

#mH: 30 kHz

#MH: 50 kHz

#mV: 45 Hz

#MV: 120 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Microvitec VGASCAN 14

#MB: 22 MHz

#mH: 30 kHz

#MH: 50 kHz

#mV: 45 Hz

#MV: 100 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Microvitec VGASCAN 14 Plus

#MB: 40 MHz

#mH: 30 kHz

#MH: 50 kHz

#mV: 45 Hz

#MV: 100 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: MicrovitecVGASCAN14PlusSSI

#MB: 40 MHz

#mH: 30 kHz

#MH: 50 kHz

#mV: 45 Hz

#MV: 100 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: MicrovitecVGASCAN15 FST

#MB: 40 MHz

#mH: 30 kHz

#MH: 50 kHz

#mV: 45 Hz

#MV: 100 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Miro C 1766

#MB: 110 MHz

#mH: 30 kHz

#MH: 66 kHz

#mV: 50 Hz

#MV: 140 Hz

#MX: 1280

#MY: 1024

#UC: Analog

#MN: Mitsuba 710A / 710B (Mono)

#MB: 20 MHz

#mH: 18.43 kHz

#MH: 18.43 kHz

#mV: 50 Hz

#MV: 50 Hz

#MX: 720

#MY: 348

#UC: Analog

#MN: Mitsuba 710MH

#MB: 45 MHz

#mH: 15 kHz

#MH: 38 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Mitsuba 710V

#MB: 20 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Mitsuba 710VE

#MB: 30 MHz

#mH: 31.46 kHz

#MH: 31.46 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Mitsuba 710VH

#MB: 45 MHz

#mH: 31.468 kHz

#MH: 35.52 kHz

#mV: 56 Hz

#MV: 87 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Mitsuba 710VW (Mono)

#MB: 30 MHz

#mH: 31.468 kHz

#MH: 35.2 kHz

#mV: 50 Hz

#MV: 70 Hz

#MX: 720

#MY: 480

#UC: Analog

#MN: Mitsuba M710H

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Mitsuba M710VR

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Mitsubishi Diamond Pro VGA

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Mitsubishi Diamond Pro 14

#MB: 70 MHz

#mH: 30 kHz

#MH: 58 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Mitsubishi Diamond Pro 20

#MB: 130 MHz

#mH: 30 kHz

#MH: 78 kHz

#mV: 50 Hz

#MV: 130 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: Mitsubishi Diamond Pro 26H

#MB: 110 MHz

#mH: 45 kHz

#MH: 72 kHz

#mV: 50 Hz

#MV: 80 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: Mitsubishi Diamond Pro 26M

#MB: 50 MHz

#mH: 15 kHz

#MH: 38 kHz

#mV: 45 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Mitsubishi DiamondPro37 DV

#MB: 80 MHz

#mH: 24 kHz

#MH: 64 kHz

#mV: 40 Hz

#MV: 120 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: Mitsubishi Diamond Scan 14

#MB: 30 MHz

#mH: 30 kHz

#MH: 38.5 kHz

#mV: 40 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Mitsubishi Diamond Scan14D

#MB: 50 MHz

#mH: 15 kHz

#MH: 36 kHz

#mV: 45 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Mitsubishi Diamond Scan 16

#MB: 100 MHz

#mH: 30 kHz

#MH: 64 kHz

#mV: 50 Hz

#MV: 130 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: Mitsubishi Diamond Scan 20

#MB: 110 MHz

#mH: 30 kHz

#MH: 64 kHz

#mV: 50 Hz

#MV: 130 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: Mitsubishi Diamond Scan20M

#MB: 50 MHz

#mH: 15 kHz

#MH: 38 kHz

#mV: 45 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Mitsubishi DiamondScan20LP

#MB: 50 MHz

#mH: 15 kHz

#MH: 38 kHz

#mV: 45 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Mitsubishi DiamondScan21FS

#MB: 110 MHz

#mH: 30 kHz

#MH: 64 kHz

#mV: 50 Hz

#MV: 120 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: Mitsubishi DiamondScan33DV

#MB: 35 MHz

#mH: 15 kHz

#MH: 38 kHz

#mV: 40 Hz

#MV: 120 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Mitsubishi DiamondScan37DV

#MB: 35 MHz

#mH: 15 kHz

#MH: 36 kHz

#mV: 40 Hz

#MV: 120 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Mitsubishi FL 6615 ETKV

#MB: 110 MHz

#mH: 30 kHz

#MH: 64 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1024

#UC: Analog

#MN: Mitsubishi XC-1404C

#MB: 22 MHz

#mH: 15.6 kHz

#MH: 15.6 kHz

#mV: 50 Hz

#MV: 50 Hz

#MX: 640

#MY: 200

#UC: RGB TTL

#MN: Mitsubishi XC-1419C

#MB: 22 MHz

#mH: 15.6 kHz

#MH: 15.6 kHz

#mV: 50 Hz

#MV: 50 Hz

#MX: 640

#MY: 200

#UC: RGB TTL

#MN: Mitsubishi XC-1410C

#MB: 12 MHz

#mH: 15.75 kHz

#MH: 22 kHz

#mV: 60 Hz

#MV: 60 Hz

#MX: 640

#MY: 350

#UC: RGB TTL

#MN: Mitsubishi XC-1429C

#MB: 12 MHz

#mH: 15.75 kHz

#MH: 22 kHz

#mV: 60 Hz

#MV: 60 Hz

#MX: 640

#MY: 350

#UC: RGB TTL

#MN: Mitsubishi XC-1430C

#MB: 12 MHz

#mH: 15.75 kHz

#MH: 22 kHz

#mV: 60 Hz

#MV: 60 Hz

#MX: 640

#MY: 350

#UC: RGB TTL

#MN: Mitsubishi XC-1440C

#MB: 12 MHz

#mH: 15.75 kHz

#MH: 22 kHz

#mV: 60 Hz

#MV: 60 Hz

#MX: 640

#MY: 350

#UC: RGB TTL

#MN: Mitsubishi XC-1429CH

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Mitsubishi EUM-1471A

#MB: 30 MHz

#mH: 15.6 kHz

#MH: 35 kHz

#mV: 45 Hz

#MV: 75 Hz

#MX: 800

#MY: 560

#UC: CVideo,RGB TTL,RGB Analog

#MN: Mitsubishi AUM1381ATS

#MB: 30 MHz

#mH: 15.7 kHz

#MH: 36 kHz

#mV: 45 Hz

#MV: 90 Hz

#MX: 800

#MY: 600

#UC: Analog, DB25

#MN: Mitsubishi HL6605ATK

#MB: 50 MHz

#mH: 30 kHz

#MH: 64 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1280

#MY: 800

#UC: Analog

#MN: Mitsubishi HC3905ATK

#MB: 40 MHz

#mH: 15.7 kHz

#MH: 36 kHz

#mV: 45

#MV: 80 Hz

#MX: 1024

#MY: 800

#UC: Analog DB9,Digital DB15

#MN: Mitsubishi HL6905ATK

#MB: 64 MHz

#mH: 30 kHz

#MH: 64 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1024

#UC: Analog

#MN: Nanao Flexscan 9065S

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Nanao Flexscan 9080i

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Nanao Flexscan F550i

#MB: 80 MHz

#mH: 30 kHz

#MH: 65 kHz

#mV: 55 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: Nanao T-560i CPD1604S

#MB: 120 MHz

#mH: 30 kHz

#MH: 78 kHz

#mV: 55 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: NEC Multisync plus

#MB: 55 MHz

#mH: 21.8 kHz

#MH: 45 kHz

#mV: 56 Hz

#MV: 80 Hz

#MX: 960

#MY: 720

#UC: Analog, TTL

#MN: NEC Multisync 2A

#MB: 38 MHz

#mH: 31.5 kHz

#MH: 35 kHz

#mV: 56 Hz

#MV: 70 Hz

#MX: 800

#MY: 600

#UC: Analog

#MN: NEC Multisync GS2A

#MB: 38 MHz

#mH: 31.5 kHz

#MH: 35 kHz

#mV: 56 Hz

#MV: 70 Hz

#MX: 800

#MY: 600

#UC: Analog (Gray scale)

#MN: NEC MultiSync 3D

#MB: 45 MHz

#mH: 15.75 kHz

#MH: 38 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: TTL 64, Analog

#MN: NEC Multisync 4D

#MB: 75 MHz

#mH: 30 kHz

#MH: 57 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: TTL, Analog

#MN: NEC Multisync 4DS

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: NEC Multisync 5D

#MB: 110 MHz

#mH: 30 kHz

#MH: 66 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1024

#UC: TTL,Analog

#MN: NEC Multisync 3FGx

#MB: 65 MHz

#mH: 31 kHz

#MH: 49 kHz

#mV: 55 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: Analog

#MN: NEC Multisync 4FG

#MB: 75 MHz

#mH: 27 kHz

#MH: 57 kHz

#mV: 55 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: Analog

#MN: NEC Multisync5FG JC174IUMA

#MB: 135 MHz

#mH: 27 kHz

#MH: 79 kHz

#mV: 55 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1024

#UC: Analog

#MN: NEC Multisync 6FG

#MB: 135 MHz

#mH: 27 kHz

#MH: 79 kHz

#mV: 55 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1024

#UC: Analog

#MN: NEC MultiSync XL

#MB: 65 MHz

#mH: 21.8 kHz

#MH: 50 kHz

#mV: 56 Hz

#MV: 80 Hz

#MX: 1024

#MY: 768

#UC: TTL, Analog

#MN: NOKIA Salora CED 4

#MB: 70 MHz

#mH: 31.5 kHz

#MH: 50 kHz

#mV: 50 Hz

#MV: 100 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Optiquest 1000

#MB: 65 MHz

#mH: 30 kHz

#MH: 48 kHz

#mV: 47 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Optiquest 1000D

#MB: 75 MHz

#mH: 30 kHz

#MH: 50 kHz

#mV: 47 Hz

#MV: 100 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Optiquest 2000

#MB: 65 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 87 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Optiquest 2000D

#MB: 75 MHz

#mH: 30 kHz

#MH: 58 kHz

#mV: 47 Hz

#MV: 100 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Optiquest 4000D

#MB: 125 MHz

#mH: 30 kHz

#MH: 76 kHz

#mV: 40 Hz

#MV: 120 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: Packard Bell PB8508VG

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 50 Hz

#MV: 70 Hz

#MX: 720

#MY: 480

#UC: RGB Analog

#MN: Packard Bell PB8509VG

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 50 Hz

#MV: 60 Hz

#MX: 720

#MY: 480

#UC: RGB Analog

#MN: Packard Bell PB8528SVG

#MB: 45 MHz

#mH: 31.5 kHz

#MH: 38.0 kHz

#mV: 55 Hz

#MV: 87 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Packard Bell PB8529VG

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 50 Hz

#MV: 70 Hz

#MX: 720

#MY: 480

#UC: RGB Analog

#MN: Packard Bell PB8531VG

#MB: 28 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 50 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Packard Bell PB8538VG

#MB: 45 MHz

#mH: 31.5 kHz

#MH: 38.0 kHz

#mV: 55 Hz

#MV: 87 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Packard Bell PB8539V

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 50 Hz

#MV: 70 Hz

#MX: 720

#MY: 480

#UC: RGB Analog

#MN: Packard Bell PB8551VG

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 720

#MY: 480

#UC: RGB Analog

#MN: Packard Bell PB8552VG

#MB: 28 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Panasonic PanaSync C1381

#MB: 30 MHz

#mH: 30 kHz

#MH: 37 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Panasonic PanaSync C1381i

#MB: 45 MHz

#mH: 30 kHz

#MH: 38 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Panasonic PanaSync C1391

#MB: 30 MHz

#mH: 15.5 kHz

#MH: 36 kHz

#mV: 40 Hz

#MV: 80 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Panasonic PanaSync C1395

#MB: 65 MHz

#mH: 30 kHz

#MH: 57 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Panasonic PanaSync C1395E

#MB: 65 MHz

#mH: 30 kHz

#MH: 57 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Panasonic PanaSync C1991

#MB: 110 MHz

#mH: 30 kHz

#MH: 68 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: Panasonic TX-1703MA

#MB: 110 MHz

#mH: 30 kHz

#MH: 64 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: Analog

#MN: PC Craft Power II NVGAM/4A

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Princeton PSC-28

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 50 Hz

#MV: 70 Hz

#MX: 770

#MY: 570

#UC: RGB Analog

#MN: Princeton Ultra 1200

#MB: 30 MHz

#mH: 15 kHz

#MH: 36.5 kHz

#mV: 45 Hz

#MV: 120 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Princeton Ultra 1400

#MB: 45 MHz

#mH: 15 kHz

#MH: 36.5 kHz

#mV: 45 Hz

#MV: 120 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Princeton Ultra 1600

#MB: 45 MHz

#mH: 15 kHz

#MH: 36.5 kHz

#mV: 45 Hz

#MV: 120 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Princeton Ultra 2000

#MB: 100 MHz

#mH: 45 kHz

#MH: 68 kHz

#mV: 50 Hz

#MV: 150 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: Profex 1401 SSI

#MB: 40 MHz

#mH: 31.5 kHz

#MH: 35.5 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Profex SWM 1440 SSI

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 720

#MY: 480

#UC: RGB Analog

#MN: Profex VCM 1401/II

#MB: 45 MHz

#mH: 31.5 kHz

#MH: 43.5 kHz

#mV: 50 Hz

#MV: 87 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Profex VCM 1428 SSI

#MB: 80 MHz

#mH: 30 kHz

#MH: 60 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: Profex VCM 1441

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 35.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Quimax DM-3114

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 50 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Qume QM835

#MB: 45 MHz

#mH: 30 kHz

#MH: 36.5 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Qume QM848

#MB: 64 MHz

#mH: 30 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Qume QM870

#MB: 75 MHz

#mH: 30 kHz

#MH: 76 kHz

#mV: 40 Hz

#MV: 120 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: Radius Full Page Pivot

#MB: 40 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 60 Hz

#MV: 107 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Radius PreciColorDisplay19

#MB: 100 MHz

#mH: 30 kHz

#MH: 68 kHz

#mV: 50 Hz

#MV: 75 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Radius PreciColorDisplay20

#MB: 100 MHz

#mH: 30 kHz

#MH: 71 kHz

#mV: 50 Hz

#MV: 107 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: Radius 2Page Display(Mono)

#MB: 40 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 65 Hz

#MV: 73 Hz

#MX: 1280

#MY: 960

#UC: Analog 1V Standard

#MN: Relisys RE1422

#MB: 65 MHz

#mH: 30 kHz

#MH: 50 kHz

#mV: 47 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Relisys RE1458

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Relisys RE9513

#MB: 35 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 50 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Safe Techn. Ultra-Low 7000

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Sakata SC 200 E

#MB: 12 MHz

#mH: 15.7 kHz

#MH: 15.7 kHz

#mV: 59.9 Hz

#MV: 59.9 Hz

#MX: 640

#MY: 200

#UC: FBAS, RGB

#MN: SAMPO KDM-1466D

#MB: 80 MHz

#mH: 30 kHz

#MH: 60 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: SAMPO TRISYNC

#MB: 45 MHz

#mH: 31.5 kHz

#MH: 35.5 kHz

#mV: 60 Hz

#MV: 96 Hz

#MX: 1024

#MY: 768

#UC: TTL, Analog RGB

#MN: SAMPO Prismview

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 50 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB, Analog, S+

#MN: SAMPO Alphascan

#MB: 30 MHz

#mH: 15 kHz

#MH: 35 kHz

#mV: 50 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB, TTL, S+

#MN: SAMPO AlphaScan LS

#MB: 100 MHz

#mH: 30 kHz

#MH: 67 kHz

#mV: 40 Hz

#MV: 120 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: SAMPO AlphaScan Plus

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: SAMPO AlphaScanPlusKDM1466

#MB: 80 MHz

#mH: 30 kHz

#MH: 60 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: SAMPO Officepro II (Mono)

#MB: 100 MHz

#mH: 31.5 kHz

#MH: 63.94 kHz

#mV: 50 Hz

#MV: 64.02 Hz

#MX: 1280

#MY: 1024

#UC: ECL, TTL

#MN: Samsung CJ4681

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: SamsungCSC1677/Syncmaster6

#MB: 200 MHz

#mH: 45 kHz

#MH: 90 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1600

#MY: 1280

#UC: RGB Analog

#MN: SamsungCSB9511/Syncmaster5

#MB: 100 MHz

#mH: 30 kHz

#MH: 65 kHz

#mV: 50 Hz

#MV: 100 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: SamsungCCB7577/Syncmastr5C

#MB: 100 MHz

#mH: 30 kHz

#MH: 65 kHz

#mV: 50 Hz

#MV: 100 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: SamsungCCB7571/Syncmastr4+

#MB: 100 MHz

#mH: 30 kHz

#MH: 65 kHz

#mV: 50 Hz

#MV: 100 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: SamsungCSA7571/Syncmaster4

#MB: 50 MHz

#mH: 21 kHz

#MH: 50 kHz

#mV: 53 Hz

#MV: 86 Hz

#MX: 1024

#MY: 768

#UC: Analog, TTL

#MN: SamsungCSD5577/Syncmastr4C

#MB: 50 MHz

#mH: 30 kHz

#MH: 59 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Samsung SyncMaster 15

#MB: 75 MHz

#mH: 30 kHz

#MH: 59 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Samsung CVB 4587

#MB: 45 MHz

#mH: 31.5 kHz

#MH: 35.5 kHz

#mV: 50 Hz

#MV: 86.8 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Samsung CVM 4967 LR

#MB: 45 MHz

#mH: 35.5 kHz

#MH: 35.5 kHz

#mV: 56 Hz

#MV: 86 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Samsung CVM 4961

#MB: 45 MHz

#mH: 31.5 kHz

#MH: 35.5 kHz

#mV: 50 Hz

#MV: 86.8 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Samsung CVL 4951

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Samsung CVL 4955

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Samsung MDA 1971 (Mono)

#MB: 187 MHz

#mH: 65 kHz

#MH: 85 kHz

#mV: 63 Hz

#MV: 72 Hz

#MX: 1664

#MY: 1200

#UC: Analog

#MN: Samsung MU 9511 (Mono)

#MB: 100 MHz

#mH: 63 kHz

#MH: 64 kHz

#mV: 60 Hz

#MV: 69 Hz

#MX: 1280

#MY: 1024

#UC: ECL

#MN: Samsung MP 5671 (Mono)

#MB: 70 MHz

#mH: 62 kHz

#MH: 75 kHz

#mV: 59 Hz

#MV: 75 Hz

#MX: 736

#MY: 1008

#UC: TTL

#MN: Samsung ML 4571 (Mono)

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: Analog

#MN: Samsung ML 4671LR

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 70 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Samsung MZ 4571

#MB: 25 MHz

#mH: 15.75 kHz

#MH: 18.43 kHz

#mV: 50 Hz

#MV: 60 Hz

#MX: 720

#MY: 350

#UC: TTL

#MN: Samsung MZ 4575

#MB: 25 MHz

#mH: 15.75 kHz

#MH: 18.43 kHz

#mV: 50 Hz

#MV: 60 Hz

#MX: 720

#MY: 350

#UC: TTL

#MN: Samtron SC-431V

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: Analog

#MN: Samtron SC-431E

#MB: 20 MHz

#mH: 15.75 kHz

#MH: 21.85 kHz

#mV: 60 Hz

#MV: 60 Hz

#MX: 640

#MY: 350

#UC: RGBI TTL, RR'GG'BB' TTL

#MN: Samtron SC-452C

#MB: 15 MHz

#mH: 15.75 kHz

#MH: 15.75 kHz

#mV: 60 Hz

#MV: 60 Hz

#MX: 640

#MY: 200

#UC: RGBI TTL

#MN: Samtron SM-430 (Mono)

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: Analog

#MN: Samtron SM 440/200

#MB: 18 MHz

#mH: 18.432 kHz

#MH: 18.432 kHz

#mV: 50 Hz

#MV: 50 Hz

#MX: 720

#MY: 350

#UC: TTL

#MN: Santec MF-5117

#MB: 55 MHz

#mH: 21.8 kHz

#MH: 50 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: Analog, TTL

#MN: SceptreTechnologies CL-615

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Sceptre Technologies CM-6N

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Seiko CM-1140

#MB: 25 MHz

#mH: 31 kHz

#MH: 40 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: VGA

#MN: Seiko CM-1445

#MB: 25 MHz

#mH: 31 kHz

#MH: 40 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 640

#MY: 480

#UC: VGA

#MN: Seiko CM-1445-RQ

#MB: 50 MHz

#mH: 31 kHz

#MH: 50 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 640

#MY: 480

#UC: VGA

#MN: Seiko CM-1450 14

#MB: 50 MHz

#mH: 31 kHz

#MH: 50 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: VGA

#MN: Seiko CM1760LR 17

#MB: 100 MHz

#mH: 31 kHz

#MH: 64 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: Seiko CM-2050/CBL

#MB: 40 MHz

#mH: 31 kHz

#MH: 50 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: VGA

#MN: SONY CPD-1304

#MB: 40 MHz

#mH: 28 kHz

#MH: 50 kHz

#mV: 50 Hz

#MV: 87 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: SONY CPD-1404E

#MB: 40 MHz

#mH: 28 kHz

#MH: 50 kHz

#mV: 50 Hz

#MV: 87 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: SONY CPD-1320

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: SONY CPD-1420E

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: SONY GDM-1601

#MB: 100 MHz

#mH: 36.34 kHz

#MH: 36.34 kHz

#mV: 60 Hz

#MV: 60 Hz

#MX: 1280

#MY: 1024

#UC: RGB, Composite

#MN: SONY GDM-1950

#MB: 100 MHz

#mH: 63.34 kHz

#MH: 63.34 kHz

#mV: 60 Hz

#MV: 60 Hz

#MX: 1280

#MY: 1024

#UC: RGB, Composite

#MN: SONY CPD-1302

#MB: 30 MHz

#mH: 15 kHz

#MH: 34 kHz

#mV: 50 Hz

#MV: 100 Hz

#MX: 900

#MY: 560

#UC: TTL, Analog

#MN: SONY CPD-1402E

#MB: 30 MHz

#mH: 15 kHz

#MH: 34 kHz

#mV: 50 Hz

#MV: 100 Hz

#MX: 1024

#MY: 768

#UC: TTL, Analog

#MN: SONY KX 14 CP 1

#MB: 22 MHz

#mH: 15.625

#MH: 15.75 kHz

#mV: 50 Hz

#MV: 60 Hz

#MX: 640

#MY: 200

#UC: FBAS, RGB, Analog

#MN: Sony Multiscan HG

#MB: 60 MHz

#mH: 28 kHz

#MH: 57 kHz

#mV: 50 Hz

#MV: 87 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Supercom SV 1485

#MB: 50 MHz

#mH: 31 kHz

#MH: 38 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Tandy VGM 200

#MB: 28 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Tandy VGM 300

#MB: 28 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Tatung CM-1296

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 50 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Tatung CM-1496

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 50 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Tatung CM14UAS

#MB: 65 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: Tatung CM17MBD

#MB: 100 MHz

#mH: 29 kHz

#MH: 65 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: TAXAN KX117 (Mono)

#MB: 20 MHz

#mH: 15.20 kHz

#MH: 16.30 kHz

#mV: 47 Hz

#MV: 63 Hz

#MX: 640

#MY: 256

#UC: Composite Video

#MN: TAXAN KX118 (Mono)

#MB: 20 MHz

#mH: 15.20 kHz

#MH: 16.30 kHz

#mV: 47 Hz

#MV: 63 Hz

#MX: 640

#MY: 256

#UC: Composite Video

#MN: TAXAN KX119 (Mono)

#MB: 20 MHz

#mH: 15.20 kHz

#MH: 16.30 kHz

#mV: 47 Hz

#MV: 63 Hz

#MX: 640

#MY: 256

#UC: Composite Video

#MN: TAXAN KX123 (Mono)

#MB: 20 MHz

#mH: 18.432 kHz

#MH: 18.432 kHz

#mV: 50 Hz

#MV: 50 Hz

#MX: 720

#MY: 350

#UC: TTL

#MN: TAXAN KX124 ( Mono)

#MB: 20 MHz

#mH: 18.432 kHz

#MH: 18.432 kHz

#mV: 50 Hz

#MV: 50 Hz

#MX: 720

#MY: 350

#UC: TTL

#MN: TAXAN KX143 (Mono)

#MB: 20 MHz

#mH: 18.432 kHz

#MH: 18.432 kHz

#mV: 50 Hz

#MV: 50 Hz

#MX: 720

#MY: 350

#UC: TTL

#MN: TAXAN KX144 (Mono)

#MB: 20 MHz

#mH: 18.432 kHz

#MH: 18.432 kHz

#mV: 50 Hz

#MV: 50 Hz

#MX: 720

#MY: 350

#UC: TTL

#MN: TAXANSupervsn620(K12SV620)

#MB: 18 MHz

#mH: 15.32 kHz

#MH: 15.92 kHz

#mV: 49 Hz

#MV: 61 Hz

#MX: 640

#MY: 200

#UC: RGB, RGBI TTL, S+

#MN: TAXANSupervsn625(K12SV625)

#MB: 18 MHz

#mH: 15.32 kHz

#MH: 15.92 kHz

#mV: 49 Hz

#MV: 61 Hz

#MX: 640

#MY: 200

#UC: RGB, RGBI TTL, S+

#MN: TAXANSupervsn760(K14SV760)

#MB: 25 MHz

#mH: 21.85 kHz

#MH: 21.85 kHz

#mV: 60 Hz

#MV: 60 Hz

#MX: 640

#MY: 350

#UC: RGBI, RGBrgb TTL, S+

#MN: TAXANSupervsn765(K14SV765)

#MB: 20 MHz

#mH: 15.75 kHz

#MH: 15.75 kHz

#mV: 60 Hz

#MV: 60 Hz

#MX: 640

#MY: 350

#UC: RGBI, RGBrgb TTL, S+

#MN: TAXANSupervsn780(K14SV780)

#MB: 30 MHz

#mH: 31.47 kHz

#MH: 31.47 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Taxan MultiVision 550

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: TAXANMultiv.770(K14MV770+)

#MB: 30 MHz

#mH: 15 kHz

#MH: 35 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 800

#MY: 600

#UC: RGB(I)TTL,RGBrgbTTL,Analog

#MN: Taxan MultiVision 788

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Taxan MultiVision 795

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: TAXAN MULTIVISION 875

#MB: 60 MHz

#mH: 30 kHz

#MH: 57 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: Analog

#MN: TAXAN MULTIVISION 870

#MB: 30 MHz

#mH: 15 kHz

#MH: 37 kHz

#mV: 49 Hz

#MV: 90 Hz

#MX: 800

#MY: 600

#UC: RGB(I)TTL,RGBrgbTTL,Analog

#MN: TAXAN Multivision MV-795EV

#MB: 50 MHz

#mH: 30 kHz

#MH: 57 kHz

#mV: 50 Hz

#MV: 100 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: TAXANSuperv.980(K20SV980+)

#MB: 70 MHz

#mH: 48 kHz

#MH: 52 kHz

#mV: 55 Hz

#MV: 65 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: TAXANSuperv.990(K20SV990+)

#MB: 100 MHz

#mH: 61 kHz

#MH: 65 kHz

#mV: 55 Hz

#MV: 65 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: TAXAN Ultra1000(K20UV1000)

#MB: 100 MHz

#mH: 30 kHz

#MH: 72 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: Toshiba FS Multi Frequency

#MB: 100 MHz

#mH: 30 kHz

#MH: 65 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: Toshiba P17CM01

#MB: 100 MHz

#mH: 30 kHz

#MH: 65 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: Toshiba P21CM00

#MB: 100 MHz

#mH: 30 kHz

#MH: 65 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: TRL T-1958

#MB: 100 MHz

#mH: 15 kHz

#MH: 51 kHz

#mV: 47 Hz

#MV: 74 Hz

#MX: 1024

#MY: 768

#UC: Analog, TTL, ECL

#MN: TRL CT-1958

#MB: 100 MHz

#mH: 15 kHz

#MH: 51 kHz

#mV: 47 Hz

#MV: 74 Hz

#MX: 1024

#MY: 768

#UC: Analog, TTL, ECL

#MN: TRL T-1968

#MB: 100 MHz

#mH: 51 kHz

#MH: 85 kHz

#mV: 47 Hz

#MV: 74 Hz

#MX: 1280

#MY: 1024

#UC: Analog, TTL, ECL

#MN: TRL CT-1968

#MB: 100 MHz

#mH: 51 kHz

#MH: 85 kHz

#mV: 47 Hz

#MV: 74 Hz

#MX: 1280

#MY: 1024

#UC: Analog, TTL, ECL

#MN: TRL T-1918 (Mono)

#MB: 100 MHz

#mH: 15 kHz

#MH: 51 kHz

#mV: 47 Hz

#MV: 74 Hz

#MX: 1024

#MY: 768

#UC: Analog, TTL, ECL

#MN: TRL T-1928 (Mono)

#MB: 100 MHz

#mH: 51 kHz

#MH: 85 kHz

#mV: 47 Hz

#MV: 74 Hz

#MX: 1280

#MY: 1024

#UC: Analog, TTL, ECL

#MN: TRL T-1418 (Mono)

#MB: 20 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 47 Hz

#MV: 64 Hz

#MX: 800

#MY: 600

#UC: TTL

#MN: TRL T-1428 (Mono)

#MB: 20 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 47 Hz

#MV: 64 Hz

#MX: 800

#MY: 600

#UC: TTL

#MN: TRL T-1429 (Mono)

#MB: 30 MHz

#mH: 31.5 kHz

#MH: 35 kHz

#mV: 60 Hz

#MV: 72 Hz

#MX: 1024

#MY: 768

#UC: TTL, RGB

#MN: TRL L-1438 (Mono)

#MB: 35 MHz

#mH: 31.5 kHz

#MH: 35 kHz

#mV: 47 Hz

#MV: 100 Hz

#MX: 1000

#MY: 900

#UC: TTL / Analog

#MN: TRL CT1456

#MB: 25 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 47 Hz

#MV: 100 Hz

#MX: 720

#MY: 350

#UC: TTL, RGB / TTL, Analog

#MN: TRL CT1458

#MB: 35 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 47 Hz

#MV: 100 Hz

#MX: 800

#MY: 600

#UC: TTL, RGB / TTL, Analog

#MN: TVM Low Radiation 5A+

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 60 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: TVM MediaScan 4A+

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: TVM SuperSync 2A

#MB: 30 MHz

#mH: 29.1 kHz

#MH: 32.8 kHz

#mV: 53 Hz

#MV: 70 Hz

#MX: 800

#MY: 600

#UC: Analog

#MN: TVM SuperSync 2A+

#MB: 38 MHz

#mH: 31.5 kHz

#MH: 35.2 kHz

#mV: 56 Hz

#MV: 70 Hz

#MX: 800

#MY: 600

#UC: Analog

#MN: TVM SuperSync 3A/3+

#MB: 45 MHz

#mH: 31.468 kHz

#MH: 38 kHz

#mV: 43.48 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: RGBI,RR'GG'BB' TTL+,Analog

#MN: TVM SuperSync 4A

#MB: 45 MHz

#mH: 31.5 kHz

#MH: 37.87 kHz

#mV: 43.48 Hz

#MV: 70 Hz

#MX: 1024

#MY: 768

#UC: Analog

#MN: TVM SuperSync 5A

#MB: 60 MHz

#mH: 31 kHz

#MH: 65 kHz

#mV: 40 Hz

#MV: 100 Hz

#MX: 1280

#MY: 1024

#UC: Analog

#MN: TVM MD-14 I+

#MB: 45 MHz

#mH: 31.5 kHz

#MH: 35.5 kHz

#mV: 45 Hz

#MV: 100 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: TVM MD-11 III+

#MB: 45 MHz

#mH: 15 kHz

#MH: 38 kHz

#mV: 46 Hz

#MV: 100 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: TVM MD-14 iV+

#MB: 65 MHz

#mH: 30 kHz

#MH: 48 kHz

#mV: 40 Hz

#MV: 100 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: TVM MD-15

#MB: 65 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 40 Hz

#MV: 100 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: TVM MD-15+

#MB: 75 MHz

#mH: 28 kHz

#MH: 58 kHz

#mV: 45 Hz

#MV: 100 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: TVM MD-17

#MB: 120 MHz

#mH: 30 kHz

#MH: 76 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1280

#MY: 1024

#UC: RGB Analog

#MN: TVM MD-300

#MB: 20 MHz

#mH: 15.75 kHz

#MH: 15.75 kHz

#mV: 60 Hz

#MV: 60 Hz

#MX: 640

#MY: 200

#UC: --

#MN: TVM MD-600

#MB: 20 MHz

#mH: 15.75 kHz

#MH: 15.75 kHz

#mV: 60 Hz

#MV: 60 Hz

#MX: 640

#MY: 200

#UC: --

#MN: TVM MD-700

#MB: 22 MHz

#mH: 15.75 kHz

#MH: 21.85 kHz

#mV: 60 Hz

#MV: 60 Hz

#MX: 640

#MY: 350

#UC: --

#MN: TVM MG-11 (Mono)

#MB: 40 MHz

#mH: 15 kHz

#MH: 38.5 kHz

#mV: 45 Hz

#MV: 75 Hz

#MX: 1024

#MY: 768

#UC: --

#MN: TVM MG-11 III (Mono)

#MB: 40 MHz

#mH: 15 kHz

#MH: 38.5 kHz

#mV: 45 Hz

#MV: 75 Hz

#MX: 1024

#MY: 768

#UC: --

#MN: TVM MG-12 (Mono)

#MB: 32 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 45 Hz

#MV: 75 Hz

#MX: 640

#MY: 480

#UC: --

#MN: TVM MG-14SuperSync1A(Mono)

#MB: 32 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 45 Hz

#MV: 75 Hz

#MX: 640

#MY: 480

#UC: --

#MN: TVM MG-14IIISuperS3A(Mono)

#MB: 40 MHz

#mH: 31.5 kHz

#MH: 35.5 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: --

#MN: TVM MG-15P (Mono)

#MB: 32 MHz

#mH: 31.5 kHz

#MH: 74 kHz

#mV: 50 Hz

#MV: 60 Hz

#MX: 1008

#MY: 736

#UC: --

#MN: Viewsonic 5+

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: ViewSonic 6L

#MB: 70 MHz

#mH: 30 kHz

#MH: 50 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: Analog

#MN: ViewSonic 6LR

#MB: 70 MHz

#mH: 30 kHz

#MH: 50 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: Analog

#MN: ViewSonic 6 TX14H30ETK

#MB: 65 MHz

#mH: 31.5 kHz

#MH: 49 kHz

#mV: 56.5 Hz

#MV: 86 Hz

#MX: 1024

#MY: 768

#UC: RGB Analog

#MN: ViewSonic 7

#MB: 110 MHz

#mH: 30 kHz

#MH: 64 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: Analog

#MN: ViewSonic 8

#MB: 110 MHz

#mH: 30 kHz

#MH: 64 kHz

#mV: 50 Hz

#MV: 90 Hz

#MX: 1024

#MY: 768

#UC: Analog

#MN: Voltron GM-1489 (Mono)

#MB: 26 MHz

#mH: 15.75 kHz

#MH: 29.5 kHz

#mV: 47 Hz

#MV: 63 Hz

#MX: 640

#MY: 400

#UC: TTL

#MN: Voltron TM-5156

#MB: 20 MHz

#mH: 15.75 kHz

#MH: 31.5 kHz

#mV: 45 Hz

#MV: 75 Hz

#MX: 640

#MY: 480

#UC: RGB, TTL

#MN: Voltron 55 Color Monitors

#MB: 26 MHz

#mH: 15.75 kHz

#MH: 24.83 kHz

#mV: 50 Hz

#MV: 60 Hz

#MX: 640

#MY: 400

#UC: TTL

#MN: Voltron TM-5157

#MB: 30 MHz

#mH: 15 kHz

#MH: 39 kHz

#mV: 45 Hz

#MV: 75 Hz

#MX: 1024

#MY: 768

#UC: TTL, Analog

#MN: Voltron TM-1958

#MB: 100 MHz

#mH: 51 kHz

#MH: 85 kHz

#mV: 47 Hz

#MV: 74 Hz

#MX: 1280

#MY: 1024

#UC: Analog, TTL, ECL

#MN: Voltron TM-1968

#MB: 100 MHz

#mH: 51 kHz

#MH: 85 kHz

#mV: 47 Hz

#MV: 74 Hz

#MX: 1280

#MY: 1024

#UC: Analog, TTL, ECL

#MN: Wyse Technology

#MB: 18 MHz

#mH: 15.75 kHz

#MH: 15.75 kHz

#mV: 60 Hz

#MV: 60 Hz

#MX: 640

#MY: 200

#UC: RGB TTL

#MN: Wyse WY650

#MB: 25 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 50 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: ZDS ZCM-1650

#MB: 48 MHz

#mH: 31.5 kHz

#MH: 48 kHz

#mV: 50 Hz

#MV: 72 Hz

#MX: 800

#MY: 600

#UC: RGB Analog

#MN: Zenith ZCM-1390

#MB: 25 MHz

#mH: 31.5 kHz

#MH: 31.5 kHz

#mV: 60 Hz

#MV: 70 Hz

#MX: 640

#MY: 480

#UC: RGB Analog

#MN: Zenith ZVM-1330-E

#MB: 20 MHz

#mH: 15.75 kHz

#MH: 15.75 kHz

#mV: 50 Hz

#MV: 50 Hz

#MX: 640

#MY: 480

#UC: RGB

#END

#MN:

#MB:

#mH:

#MH:

#mV:

#MV:

#MX:

#MY:

#UC:

---------------------------------------------------------------------------

<EOF>