/

/  CyReL CaTTamaran Installation Instructions

/  for  motherboard  revisions  Rev.F  and up

/

/////////////////////////////////////////////

The   following   text   will  describe  all

required steps to install a CyReL CaTTamaran

TT030 Accelerator in your system.

W A R N I N G

In  no event will Cybercube Research Limited

be   liable   for  any   damages  whatsoever

(including  without  limitation, damages for

loss    of   business   profits,    business

interruption,  loss of business information,

or  the  like)  arising  out  of the use of,

interruption  in the use of, or inability to

use  the CyReL CaTTamaran Accelerator Module

or   the   associated   software,  even   if

Cybercube  Research Limited has been advised

of  any  possibility  or  likelihood of such

damages.

Opening  your  Atari computer and installing

the  CyReL  CaTTamaran may void the warranty

of  your  computer  system.  Please refer to

your  manual before attempting the following

installation procedures.

= PREPARATION STEP 1 =

Please  make  sure  that you switch off your

system  and remove the power cord before you

begin.

Failure  to  do  so might damage your system

and or the CyReL CaTTamaran.

= PREPARATION STEP 2 =

Please  remove  the  keyboard, mouse and all

other cables from your Atari TT030.

Disconnect and remove any external drives.

= PREPARATION STEP 3 =

Disconnect and remove the monitor.

= PREPARATION STEP 4 =

Turn the unit upside down and remove all the

screws from the underside.

Please  note  that  this   might  void  your

warranty.  Consult  your  Atari TT030 manual

first  in  case  you  are not sure about the

warranty regulations.

= PREPARATION STEP 5 =

Turn  the  TT030 back up and gently lift the

cover.  Disconnect the power LED, the floppy

disk cable, and the floppy disk power cable.

If  you have an internal harddisk, remove it

too.

Remove  the  metal  shielding  (if  any)  to

expose the TT030 motherboard.

= PREPARATION STEP 6 =

Cautiously  remove  the  VME cage. Bend open

the  two  metal tabs that secure the cage to

the  VME  daughterboard,  remove  the screws

from  the  back,  disconnect the speaker and

then pull out the cage.

IMPORTANT:

Don't  forget  to take out any installed VME

cards before you remove the VME cage itself.

You  should  now  have full access to the TT

motherboard. Refer to the close-up.

The  close-up  shows a schematic overview of

the   TT   motherboard.   Only  a   few  key

components have been highlighted to help you

locate  the  necessary components and places

on the board.

= INSTALLATION STEP 1 =

Check  the  motherboard  revision  of  your

system:

You can usually find the revision printed on

a  white  label  in  the lower right hand or

left hand corner of the board. If your board

does  not  have one of these stickers, check

the  REV ___ field as shown. If the revision

field  is  blank, check if the CPU (MC68030)

is  situated on, the right side of the board

(location (2)).

Please  consult  the online documentation in

case  you  have  a board revision lower than

'F'  or if the CPU in your system is located

on the LEFT side of the board.

= INSTALLATION STEP 2 =

Remove  the  ST-RAM  board.  If  there  is a

ST-RAM  card  installed in your system, skip

to the next instruction.

If  you  do  NOT  have  such a board, please

insert  the supplied double row interconnect

strip  horizontally  into  the  lower   left

corner of J502 (see location (4) ), starting

at  pin 1 (see marks on the TT motherboard).

The  CyReL  CaTTamaran will then be inserted

into the interconnect strip

= INSTALLATION STEP 2b =

Locate the resistor R888. Unfortunately, you

can't   read   the  printed  label   on  the

motherboard  most  of  the  time  since  the

resistor obscures the label.

Resistors  employ  color   coded  rings  for

identification.  R888  has 4 rings: a brown,

black,  black  and  golden   ring.  Use  the

close-up  as  a guide when searching for the

resistor.  If  you  found it, gently bend it

aside  and  verify  if you located the right

component.

Cut  the  left end of the resistor (some may

have a wire connected to that end).

IMPORTANT:

Cut  the  indicated  pin  of the resistor as

close  to  the  BOARD (or WIRE) as possible.

You  will have to attach one of the clips to

the  metal  end  of the resistor in the next

step.

= INSTALLATION STEP 2c =

Take  the  R  E  D  clip.  Attach  it to the

resistor as follows:

Bend  the  resistor a little to the side and

attach  the R E D clip to the free metal end

of  the  resistor  R888  (at   the  position

indicated by the small arrow).

Make  sure  there  is  no  short between the

metal  end  with the clip and the cut end in

the board or the old wire.

Place the clip as flat (horizontally) to the

board as possible so that it will not hinder

you  later  when  you try to re-install your

ST-RAM board.

= INSTALLATION STEP 3 =

Locate  the  chip  U112.  Use  the  close-up

display  as  a guide when searching for this

IC.

TIP:

The  IC  is  located  very close to the R888

resistor.  It  may  be partially hidden from

view  by  the VME daughterboard. Most of the

time  it  can  also be identified by a white

label (reads 'C301837-001' or similar).

= INSTALLATION STEP 3b =

Take  the G R E E N clip. Attach it to pin 8

of the U112 chip as follows:

Attach  the  G  R  E  E N clip to pin 8 (see

arrow!) of the U112 chip. Pin 8 is the third

pin from the right in the bottom row.

Position the clip in such a way that it does

not  interfere  with  any  VME  cards or the

ST-RAM board.

Make  sure  that  you  do not short two pins

when  you  add  the  clip!  Check for proper

connection.

= INSTALLATION STEP 4 =

Reinstall  the  ST-RAM  board. If you do not

own   such   a   board,  skip  to  the  next

instruction.

Make  sure  to properly insert and push down

the ST RAM board. Check if either the RED or

GREEN  clip has to be re-arranged to allow a

proper fit.

= INSTALLATION STEP 4b =

Now,  take  the  CyReL  CaTTamaran board and

attach   it   either  to   the  interconnect

strip(if  you  do not own a ST-RAM board) or

to  the  pins  of  the  J2  connector of the

ST-RAM  board   (see  close-up).  The  CyReL

CaTTamaran  must  be  attached  to rows B&C,

starting  at  pin  1. CyReL CaTTamaran chips

facing down!

Use  the  large mounting hole (left side) in

the  CaTTamaran  as  a  guide.  Line  it  up

exactly with the inner hole on the left side

in the ST-RAM board.

= INSTALLATION STEP 4c =

Check  if  you attached the CyReL CaTTamaran

correctly  either  to the interconnect strip

or  your ST-RAM board. The lower left pin of

the  black  connector on the CaTTamaran must

be connected to pin 1 of row C of the ST-RAM

board connector J2.

Firmly  press  down  the CyReL CaTTamaran if

you  are  sure  that  you  placed the module

correctly.

= INSTALLATION STEP 5 =

Locate  the connector J601 (see arrow). J601

consists  of  two rows of two pins each. Use

the  close-up  as a guide when searching for

J601.

It  should  be  located  near the top of the

board,  under  the  VME   cage  area,  right

besides the two connectors for the SERIAL #1

and SERIAL #2 ports.

= INSTALLATION STEP 5b =

Take  the   B L A C K  clip and attach it to

the  upper right pin (see arrow) of the J601

connector.

Make  sure the  clip does not interfere with

the  VME  slot. To avoid any problems, place

this  clip  as  flat  (horizontally)  to the

board as possible so that it will not hinder

you later.

= INSTALLATION STEP 6 =

Locate  the  W101  connector. Again, use the

close-up  as  a guide. It should be situated

right in the middle of the TT030 motherboard

near the VME bus daughterboard.

Remove  the  jumper  (plastic clip) from the

W101 connector.

= INSTALLATION STEP 6b =

There  is  now only one more connector to be

attached  to  the TT030 motherboard. It is a

small black connector with three wires. Note

that one side bears a white mark.

Plug  this  connector  onto W101 so that the

white  mark  aligns  with the printed '1' on

the motherboard.

= INSTALLATION STEP 7 =

To  complete the hardware installation, look

for   the  DIP   switches  depicted  in  the

close-up section.

= INSTALLATION STEP 7b =

Set DIP switch 1 to ON (as shown).

This will tell the system that you installed

a  CyReL  CaTTamaran  48 MHz upgrade in your

TT030.

Finally, re-assemble your TT030 again.

Congratualtions!   You  just  completed  the

CyReL CaTTamaran installation.