---------------------------------------------------------------------------

CyReL CaTTamaran Additional Installation Tips, Hints

and Troubleshooting Guide.

Version: 1.4b                             Date: 7/94

---------------------------------------------------------------------------

CONTENTS

A.   OVERVIEW

B.   Last Minute Changes

C.   Hints and Tips

D.   Troubleshooting

E.   CyReL Return Policy

F.   NOTES

---------------------------------------------------------------------------

[A] OVERVIEW

---------------------------------------------------------------------------

The  following  informations  are  intended  as  a supplemental to the

existing CyReL CaTTamaran documentation and installation tutorial.

Section  [B]  contains  some  last  minute  changes. Please check this

section for any additional installation instructions.

Refer  to  section  [C]  for  any  tips  and  hints  to facilitate the

installation  process  or  the  usage  of the CyReL CaTTamaran and the

associated software drivers.

If  you  run into problems when operating the CyReL CaTTamaran, please

skip  to  section  [D]  for technical help. If you cannot resolve your

difficulties,  please  check  section [E] for the technical assistance

programme and our 10-days return policy.

---------------------------------------------------------------------------

[B] Last Minute Changes

---------------------------------------------------------------------------

/// Change in the shape of the CaTTamaran Rev.B boards:

-------------------------------------------------------

The  CyreL  CaTTamaran Rev.B boards have changed slightly (see figures

below).  Please keep this in mind whenever the documentation refers to

the shape of the boards.

__________________________________

| o              ............... o |

|                ...............   |

|__________________    __          |

\  |  | _   __  |  CaTTamaran Rev.A boards

\ |  || | |  | |  (underside)

||__||_| |__| |

|_____________|

___________________

| ............... o |

| ...............   |

|___    __          |

\  |  | _   __  |  CaTTamaran Rev.B boards

\ |  || | |  | |  (underside)

||__||_| |__| |

|_____________|

/// No J601 connector on older motherboard revisions:

-----------------------------------------------------

Look for a 4 pin connector with the 'J602' marking instead.

---------------------------------------------------------------------------

[C] Hints and Tips

---------------------------------------------------------------------------

/// Adding the 3 wire CaTTamaran connector to W101:

---------------------------------------------------

Push  through  the  3  wire  CaTTamaran  connector  underneath the VME

daughterboard,  then  plug  it  onto  the  W101  3  pin  header on the

motherboard for easier fit.

/// Permanently disabling the 48 MHz mode,

or forcing 32 MHz mode operation:

------------------------------------------

Should you (for any reason whatsoever) have to disable the 48 MHz mode

of the CyReL CaTTamaran, connect the BLACK clip to the lower right pin

of connector J601 as shown here:

___  1

|o o|

|o o|  <-- attach the BLACK clip to this pin

---

This  can also be used during testing to force the CyReL CaTTamaran to

boot in 32 MHz mode instead of 48 MHz.

/// What do I have to do in case I do not own a ST-RAM board ?

--------------------------------------------------------------

Take the free black interconnect strip (two rows of 15 pins),

|||||||||||||||   <-- short ends

view from the side ->     ===============

|||||||||||||||   <-- long ends

and plug it into the lower ST-RAM board connector J502 (shown here):

---__________________________________---

|<>| ................................ |<>|  J502

|   |::::::::::::::::::::::::::::::::|   |

--- -------------------------------- ---

1

To do this, take the interconnect strip and plug it with the long ends

into J502 at the position indicated by the X's:

---__________________________________---

|<>| ................................ |<>|  J502

|   |XXXXXXXXXXXXXXX:::::::::::::::::|   |

--- -------------------------------- ---

1

The  interconnect  strip should now be attached to the lower two rows,

starting at pin 1. After installing the interconnect strip, attach the

CyReL  CaTTamaran  to  the  still  free short pins of the interconnect

strip.

/// What if I can't add the CyReL CaTTamaran to the ST-RAM board J2

connector due to too short leads or clogged leads?

-------------------------------------------------------------------

Cybercube  also  offers  a limited quantity of CaTTamaran modules that

can  be  soldered  directly  onto the pins of the J2 connector. Please

contact Cybercube if the protruding ends of the ST-RAM board  pins are

too short to guarantee a proper  attachment  of  the CyReL CaTTamaran.

These  cases  are  very rare, so please double-check the length of the

pins.

If  your  pins  show signs of soldering flux residue or show excessive

solder  (pins  are  too  thick  to fit into the CaTTamaran connector),

clean  the  J2  connector  pins  before  attempting to install a CyReL

CaTTamaran.  CAUTION:  You  can  use  a  very  sharp  knife  to remove

excessive  solder  in case you do not have access to a soldering iron.

Exercise great care when cleaning the pins with a knife!

/// What do I have to do in case I have an older motherboard revision ?

-----------------------------------------------------------------------

Please   decompress  the  REV_A_II.LZH  or REV_B_II.LZH archives. They

contain  detailed instructions on how to adapt the CyReL CaTTamaran to

the  earlier  motherboard versions. The archives contain a 150 dpi GEM

*.IMG file. For ease of use, print out this file before attempting the

installation.    Older   motherboard   revisions   will  require  some

soldering in order to install a CyReL CaTTamaran.

/// How can I get the optimum performance level out of my system?

-----------------------------------------------------------------

There are several steps to achieve this:

1.  Check  your  \AUTO\  folder and *.ACC & *.CPX configuration. These

handy  litte  utilities and boot-up gadgets can cause 30% or more drop

of overall system speed.

2.  Use  a  ROM  relocator  like  GEMRAM (part of the WINX package) or

Alexander  Herzlinger's ROMRAM. These programs relocate the ROM to the

FastRAM, thus achieving higher system speeds.

3.  Use  a  software  screen  accelerator like WARP9 or NVDI whereever

possible.  These  drivers  will have an dramatic effect on your system

performance.

4.  Consider   using  a  OS  enhancer  like  GENEVA  or  Mag!x.  These

applications   offer  superior execution speed for most of the regular

OS functions.

/// What should I do with the cut wire from R888 ?

--------------------------------------------------

Either   wrap  a  tape  around  the  blank  metal  end  of the wire or

remove  it  altogether  since it is not longer necessary to have it in

your  system.  In  either case, make sure that the wire will not cause

any shorts.

/// I cannot identify the motherboard revision, what can I do?

--------------------------------------------------------------

There  are  three  major  groups  of TT motherboards. Please check the

following  points  to  identify  to  which  group your motherboard may

belong:

Look  for  the CPU (check for a chip with the 'MC68030' labelling). Is

it on the left hand side or right hand side of the board?

a) CPU on left hand side:

-------------------------

Is there a CPU daughterboard?

1)  CPU on daughterboard:

-------------------------

You  have   one   of  the older motherboards. Please check

the REV_A_II.LZH file.

2)  CPU in socket:

------------------

You  have one of the intermediate boards (we also refer to

them  as  Rev.B  motherboards,  despite  the fact that the

printing  on  the  underside  of  the board underneath the

power  supply  refers to them as Rev. A boards). If you do

have such a board, please check the REV_B_II.LZH archive.

b) CPU on right hand side, surface mounted:

-------------------------------------------

You  have  one of the newer boards. Please refer to the printed

installation instructions or the RUN-ME-FIRST program tutorial.

---------------------------------------------------------------------------

[D] Troubleshooting

---------------------------------------------------------------------------

/// FAULT1: Screen stays black:

------------------------------

Please  retrace  your  installation  steps. Make sure you attached all

connectors  properly  and that there are no shorts being caused by the

three clips.

Also  check  if  the  clips  firmly  grip  to their designated contact

points.

There  might  be  some  missing connection. See if there are still any

clips  or  connectors  that  have  not  been  attached to their proper

places.

Check if there may be any switched wires or connections.

The  connections  may be disrupted if anything is pressing down on the

pushbutton end of the clips.

Make  sure  you  inserted the three wire CaTTamaran connector properly

into  W101  and  that you removed the jumper from W101 before attacing

the three wire connector.

Verify  if the CyReL CaTTamaran has been firmly seated onto either the

interconnect strip or the protruding pins of J2 of the ST-RAM board.

Check  the  revision level of the ST-RAM board RAM controller (the big

black square plastic IC on the ST-RAM board). The CyReL CaTTamaran may

not work correctly with chip revisions 160704-0H, it does however work

with 160704-01 and up.

If  everything  fails,  remove  the  CyReL  CaTTamaran  completely and

attempt  the  installation  again,  paying  close  attention  to  the

installation instructions.

/// FAULT2: Computer does not boot:

-----------------------------------

Check all FAULT1 tips.

If you are sure you installed the CyReL CaTTamaran correctly, check if

you  re-assembled  your  TT in the proper fashion. Check if the floppy

disk drive and the harddisk have been connected properly.

Try  to force the CyReL CaTTamaran to boot in 32 MHz mode. See section

[C]  for details. If your system still does not boot, repeat the CyReL

CaTTamaran installation.

/// FAULT3: Computer hangs while testing the ST-RAM:

----------------------------------------------------

Check  the  revision level of the ST-RAM board RAM controller (the big

black square plastic IC on the ST-RAM board). The CyReL CaTTamaran may

not work correctly with chip revisions 160704-0H, it does however work

with 160704-01 and up. If possible, remove the ST-RAM board and attach

the  CyReL  CaTTamaran  directly  to  the  J502  connector  (using the

interconnect strip) and try the boot process again.

/// FAULT4: Computer hangs while testing the TT-RAM:

----------------------------------------------------

Remove  the  TT-RAM  board and try again. If it boots this time, force

your  CaTTamaran  to  boot  in  32 MHz mode (see section [c]) with the

TT-RAM  board  installed. If it also boots, try to reduce the capacity

of  your  TT-RAM  and  try again in 48 MHz mode. If it works with less

RAM,  try  to  get a more powerful power supply for your TT. The power

supply  of  your  TT  might  be  a  bit  strained  by  all  the  power

requirements  of  your  specific  setup.  Some PC dealers may sell the

older  models of PC power supplies (which can be used if they have the

same  type of connector) for reasonable prices. In any case check with

your Atari dealer or a knowledgeable technician.

/// FAULT5: Repetitive system crashes:

--------------------------------------

Please try to establish which software program, application, accessory

or  utility might be causing the fault. Please report your findings to

Cybercube.

We have made every effort to design the CyReL CaTTamaran as compatible

and  as  functional as possible. Should you find that it does not work

with any of your software programs, send us a note and we will try our

best to rectify this situation.

/// FAULT6: Occasional system crashes:

--------------------------------------

If your system crashes or hangs due to spurios or undetectable causes,

thoroughly  check  your  CaTTamaran  installation. Also, use the CyReL

Dashboard  as  an  indicator  for  heat  problems.  If  you experience

frequent  crashes while the heat indicator is in the MAX range, either

switch  back  to 32 MHz mode or switch off your system and let it cool

off.  A  five  minute  off-period  should  be  enough to cool down the

system.

Also,  make  sure  the  fan  in your power supply is working properly.

Extreme cases might be remedied by adding a second fan to your TT. The

motherboard and the case of the TT have already been equipped for such

an upgrade.

NOTE:  normal  operation  of the CyReL CaTTamaran does NOT require the

installation of an additional fan.

/// FAULT7: Peak performance levels are not being reached:

----------------------------------------------------------

Peak  performance  levels differ from motherboard revisions, component

variations  and  thermic  influences.  Please note that the benchmarks

released  by  Cybercube  have   been  obtained  under  the  conditions

described in the accompanying documentation.

/// FAULT8: RESET does not work properly:

-----------------------------------------

Some  systems  may be very sensitive towards the handling of the reset

procedure.  If  you  experience  problems  while resetting your system

(screen  stays  blank  or  black  or  scrolling lines), please try the

following modification to remedy the situation:

The  CaTTamaran receives the RESET signal via the 2 x 15 pin connector

on   the underside of the CaTT. Turn the CaTTamaran upside down (chips

facing up). Place the CaTTamaran in front of you like this:

__________________________________

| o              ............... o |

|                .*.............   |

|__________________    __          |

\  |  | _   __  |  CaTTamaran Rev.A boards

\ |  || | |X | |

||__||_| |__| |

|____^________|

___________________

| ............... o |

| .*.............   |

|___    __          |

\  |  | _   __  |  CaTTamaran Rev.B boards

\ |  || | |X | |

||__||_| |__| |

|____^________|

The  reset signal originates from the pin marked with the '*' and goes

to  the  chip marked with the 'X'. Disconnecting the reset signal from

the  chip   (GAL16V8B)  and patching it so that it will be pulled high

permanently will avoid the problems. Here is how:

Look  very  closely  at  the board surrounding the GAL16V8. One of the

pins  is  marked  with  a  small white arrow. That's pin #1. Now start

counting  counter  clockwise  until you reach pin #8! That's where the

RESET signal is fed into the chip.

Follow  the  trace  back  to  pin  #14  of  the  black  connector  for

verification.  Find a good spot to cut that trace (but do it in such a

way  that  you might be able to patch it again later should this patch

fail to work properly).

Use  a  continuity  tester  to  check  whether  you cut off the signal

completely (again by checking between pin #14 of the connector and pin

#8  of  the GAL16V8). After disconnecting the old reset signal, take a

small  wire  and  connect  pin #8 of the GAL16V8 to pin #14 of the F00

chip  (marked with the '^' in the figure above, lowest pin on the left

side of the F00 chip).

/// FAULT9: TT-High display not steady:

---------------------------------------

Hook up the mono monitor to the video port, switch on your TT030. Lift

the    CaTTamaran   wire going to the BLACK clip up (for at least 3 cm

(1"))  without  disconnecting  the  black  clip from the J601 (J602 on

older  boards) header. If the screen display returns to normal, try to

place  the wire in such a way that it does not interfere with the high

resolution   monochrome   circuitry   on  the  TT  motherboard.   This

circuitry  can  be  easily  identified by the round red component with

the  copper  windings  to   the   right   of   the power supply. Avoid

placing  the  black  wire  near   all  components  inside  the  square

marked  by  the  traces on the motherboard. We would like to thank Jim

Rinner for helping us to locate this 'fault'.

---------------------------------------------------------------------------

[E] CyReL 10-DAYS Return Policy

---------------------------------------------------------------------------

Cybercube  products  are  being designed and manufactured under strict

quality  control procedures. Should you find that the CyReL CaTTamaran

unit  does  not work properly in your system, and that all attempts to

rectify  this situation failed, you may return the unit within 10 days

of the purchase date.

DATE OF PURCHASE DEFINITION:

The  10  days  period  starts with day of shipment or date of invoice,

whichever  is  later.  Different arragements are available and must be

discussed and agreed upon by both parties prior to the purchase of the

unit.

RETURN CONDITIONS:

The  unit  must  be  returned in its original packaging, undamaged, in

working  and  re-sellable  condition.  All  copies  made  of the CyReL

CaTTamaran  software  package  must  be  destroyed.  Provide  proof of

purchase date.

REFUNDS:

If  the above conditions are met, you will be eligible for a refund of

your  money.  However,  there  is  a  15%  restocking and handling fee

involved  with  any  returned  unit.  Return  the unit to the point of

purchase.

Money  will  be  refunded  at  the  discretion  of  Cybercube,  or the

distributor/dealer  or  other  party  from  which  the  unit  has been

purchased.

---------------------------------------------------------------------------

[F] NOTES

---------------------------------------------------------------------------

Contact addresses for technical support:

Mail:       Cybercube Research Limited

126 Grenadier Cres

Thornhill, Ontario, L4J 7V7

Canada

Telephone:  (905) 882 0294, Mon-Fri, 10 AM to 4 PM, EST (Toronto)

Fax:        (905) 886 3261, 24h

BBS:        (905) 882 5895, 1,200-14.4kb, 24h

GEnie:      CYBERCUBE

CRS-Online: Cybercube Research

InterNet/

UseNet:     cybercube.research@canrem.com     or

cybercube@genie.geis.com

Please report all discrepancies to our CyReL Coordinator. Thank you.

---------------------------------------------------------------------------

<EOF>