---------------------------------------------------------------------------

CyReL SUNRISE M16-1280 XBIOS Emulator Documentation

Version: 1.0a                           Date: 11/92

Version: 1.1a                           Date: 11/92

---------------------------------------------------------------------------

CONTENTS

A.   OVERVIEW

B.   XBIOS Function List

C.   TT XBIOS Functions

D.   NOTES

---------------------------------------------------------------------------

[A] OVERVIEW

---------------------------------------------------------------------------

The following informations are intended as a supplemental to Atari's

XBios documentation and highlights only the changes in the implementation.

It is strongly recommended that the reader makes himself familiar with

the original XBios documentation prior to reading this document.

Wherever possible we have used the same designations for function calls

and parameter names. Declarations in the TT XBios section are based on

the 'Atari TT030 TOS Release Notes - 6 September 1991'.

The current implementation emulates only those functions listed below.

Any other calls are unaffected. The emulation can be enabled or disabled

at any time. Please refer to the VDICONF.ACC documentation for more

details.

---------------------------------------------------------------------------

[B] XBIOS Function List

---------------------------------------------------------------------------

XBIOS2  Physbase()--------------------------------------------------------

Returns the value presently stored in the system variable _v_bas_ad located at

address 0x0000044e. DO NOT MODIFY THIS VARIABLE.

XBIOS3  Logbase()---------------------------------------------------------

Returns the value presently stored in the system variable _v_bas_ad located at

address 0x0000044e. DO NOT MODIFY THIS VARIABLE.

XBIOS4 Getrez()------------------------------------------------------------

This function returns the current graphics mode.

The result depends on the user defined 'resolution_return_mode' setting:

resolution_return_mode:

0               XBIOS4 returns a user defined value.

This can be any value, but should be restricted

to those normally expected to be returned by

the XBIOS4 function.

If there is no user defined value available,

the call enters resolution_return_mode=1

1               XBIOS4 returns the value set by the last

Setscreen() call. If there is no such value

available, the call enters resolution_return_mode=2

2               XBIOS4 returns the current and closest matching

CyReL M16-1280 operating mode value. Examples:

CyReL M16-1280 Mode     XBIOS4 returns

- monochrome -           2 or 6  ST High,TT High

-   4 colors -           1       ST Mid

-  16 colors -           0 or 4  ST Low, TT Mid

- 256 colors -           7       TT Low

-    other   -           2 or 6  ST High,TT High

XBIOS5 Setscreen()---------------------------------------------------------

This function ignores any values regarding the start of the display buffer.

It is therefore NOT possible to relocate the display buffer base address.

The emulator stores the resolution mode value set by the calling application

and resets the VT52 emulator if it detects a change in the graphics mode.

If the mode value is negative, the VT52 is not initialized and the old mode

value is not altered.

XBIOS6 Setpalette()--------------------------------------------------------

This function redefines the first sixteen colors of the current color palette.

The values passed by the calling application are converted to the appropriate

CyReL M16-1280 format. The colors will be updated during the next CyReL M16-1280

frame flyback.

XBIOS7 Setcolor()----------------------------------------------------------

This function changes one color register of the first sixteen colors of

the current palette. The value passed by the calling application will be

converted to the appropriate CyReL M16-1280 format and will be updated

during the next CyReL M16-1280 frame flyback. Always returns the old

color setting. If the color value was negative, the color palette is not

altered. Color value is returned in Atari format.

XBIOS21 Cursconf()---------------------------------------------------------

The CyReL M16-1280 VT52 cursor will be configured according to the parameters

passed by the calling application. DO NOT USE THE LINE-A VARIABLES TO CHANGE

THE APPEARANCE OF THE CURSOR.

XBIOS37 Vsync()------------------------------------------------------------

This call will synchronize at the next CyReL M16-1280 frame flyback.

---------------------------------------------------------------------------

[C] TT XBIOS Functions

---------------------------------------------------------------------------

The following new TT XBios calls won't be affected unless the user enables

their emulation.

XBIOS80 EsetShift()--------------------------------------------------------

Stores the TT shifter mode passed by the calling application. The TT shifter

register is not altered.

XBIOS81 EgetShift()--------------------------------------------------------

Returns the last TT shifter mode set by EsetShift(). If there has been no

prior call to EsetShift() the function returns the TT shifter mode active

during the installation of the M16-XBios Emulator.

XBIOS82 EsetBank()---------------------------------------------------------

Causes a set of sixteen subsequent colors (also referred to as a bank) to

be mapped in as colors 0-15 (Bank #0, the only bank affected by such calls

as Setcolor() or Setpalette() ).

Effect: Color( 0 to 15 )=Color( bank*16 to bank*16+15)

Valid bank parameter range: 0-15.  The original color palette state will

be restored if the parameter equals zero. Always returns the old bank

mapping. If the value of the bank parameter is negative, the palette is

not altered and the currently mapped in bank number is returned instead.

XBIOS83 EsetColor()--------------------------------------------------------

This function changes one color register out of the entire color palette.

Valid color register range: 0-255. The color value passed by the calling

application will be converted to the appropriate CyReL M16-1280 format and

will be updated during the next CyReL M16-1280 frame flyback. Always returns

the old color setting. If the color value was negative, the color palette

is not altered. Color value is returned in Atari format.

XBIOS84 EsetPalette()------------------------------------------------------

This function redefines any portion of the current color palette. A contiguous

number of registers (up to 255) can be set beginning at the register indexed

by the colorNum parameter. The values passed by the calling application are

converted to the appropriate CyReL M16-1280 format. The colors will be updated

during the next CyReL M16-1280 frame flyback.

XBIOS85 EgetPalette()------------------------------------------------------

This function returns the color values of any portion of the current color

palette. The values of any contiguous number of registers (up to 255) can

be requested. Starting from the register indexed by the colorNum parameter,

the present color values are converted into the appropriate Atari format

and then copied into a buffer pointed to by palettePtr.

XBIOS86 EgetGray()---------------------------------------------------------

Sets the manner in which the color palette entries are interpreted by

the XBios Emulator. A switch parameter value of 1 (set to gray) triggers

the following  events:

a) exit if the color palette is already in 'gray' mode.

b) else copy the current palette settings,

c) and convert all color register entries into their

corresponding gray levels.

A switch value of zero (set to color) triggers these events:

a) exit if the color palette is already in 'color' mode

or if there is no valid copy of the color palette available.

b) restore the previously stored color palette settings.

If the switch parameter is negative, the current palette mode

is returned and the palette remains unaffected.

---------------------------------------------------------------------------

[D] NOTES

---------------------------------------------------------------------------

Please report all discrepancies to the M16-Product Coordinator. Thank you.

---------------------------------------------------------------------------

<EOF>