---------------------------------------------------------------------------

CyReL SUNRISE M16-1280

CM16_VIP  M16 Versatile Installation Program

(C) 1991,92,93,94 by Cybercube Research Ltd, Canada

HELP FILE

Version: 1.8b                                            Date:  1/94

---------------------------------------------------------------------------

Contents

A.   CyReL SUNRISE M16-1280 Software Installation

B.   De-install the CM16_VIP ?

C.   How to Skip the M16-1280 Installation

D.   Error Messages and Diagnostic Codes

E.   Notes

Skip to Section [D] in case you need help with an CM16_VIP error message!

---------------------------------------------------------------------------

[A] CyReL SUNRISE M16-1280 Software Installation

---------------------------------------------------------------------------

/// HOW TO INSTALL THE CYREL M16-1280

Simply use the provided installation program or copy the CM16_VIP.PRG file into

the AUTO folder of your boot drive. Make sure it will be executed during the

boot-up phase before any other program attempts to access a M16-1280 card.

These programs must be copied into your AUTO folder. Ordering of the files is

important, so please make sure you will have them in the same order:

CM16_VIP.PRG

SERMOUSE.PRG

M16VDI8B.PRG

M16VDITC.PRG

This can be achieved by either using a AUTO folder sort (such as the PD program

AUTOSORT.PRG) program or by renaming the AUTO folder, then creating a new

folder named AUTO and copying the programs into the new folder in the right

sequence. After copying all files, delete the old (and now renamed) AUTO

folder.

SUMMARY:

1. rename the existing AUTO folder to e.g. AUTOX

2. create a new folder called AUTO

3. copy each file seperately in the indicated order

from the AUTOX to the AUTO folder

4. delete the old AUTOX folder

/// ARE ALL THE CYREL M16-1280 ACCESSORIES NECESSARY ?

No. The included accessories are designed to provide a more comfortable working

environment. You do not necessarily need them to use the CyReL M16-1280 in its

basic modes. We recommend however, that you install at least the M16_CONF.ACC

to allow you to change various card related parameters at all times. To install

it, simply copy this accessory into the root directory of your boot drive.

---------------------------------------------------------------------------

[B] De-install the CM16_VIP ?

---------------------------------------------------------------------------

/// WHEN DO YOU HAVE TO REMOVE THE CM16_VIP FROM THE AUTO FOLDER ?

Leave the program in the AUTO folder even if you sometimes have to de-install

or remove the M16-1280 cards. The program will report that it has not found any

boards and abort execution. No memory will be occupied or reserved due to an

aborted installation process.

/// WHAT ELSE DO YOU HAVE TO LOOK OUT FOR ?

Note that the CM16_VIP program must be the first M16 Program in the AUTO

folder. The VDI cannot be installed if the the programs have been placed into

the AUTO folder in the wrong order. Do not rename the CM16_VIP program since

future updates will search for this file and replace it if necessary.

---------------------------------------------------------------------------

[C] How to Skip the M16-1280 Installation

---------------------------------------------------------------------------

/// WHAT DO YOU HAVE TO DO TO AVOID THE INSTALLATION ?

To avoid the installation of the CyReL M16-1280 cards during boot-up, simply

press and hold both SHIFT keys. Press 'N' as soon as the programs inquires if

you wish to proceed with the installation process.

/// HOW DO YOU ACTIVATE THE BASIC CONFIGURATION OF THE M16-1280 ?

Reset your system, then press and hold BOTH SHIFT keys during the boot-up

procedure. As soon as the CM16_VIP starts, the program will ask whether or not

you wish to proceed with the installation. Press 'Y'. Next the program will ask

you if you wish to use the current EEPROM configuration. Press 'N'. This will

activate the basic configuration of the cards. A standard 640x480 VGA

compatible resolution setting will be used to configure the boards regardless

of the contents of the configuration EEPROM on the CyReL M16-1280.

---------------------------------------------------------------------------

[D] Error Messages and Diagnostic Codes

---------------------------------------------------------------------------

Below you will find a detailed description of all the CM16_VIP error messages.

Please refer to the appropriate paragraphs.

Note: No board will be installed if the program reports an error condition.

This reduces the access attempts in case of any VME bus problems that

could lead to possible damage or destruction.

------------------------------------------------------------------------

ERROR:   No boards installed/found.

------------------------------------------------------------------------

Please make sure you followed the installation guides in your manual. Verify if

the board has been properly inserted and secured. Check if there might be a

short between the traces/component pins of the M16 board and any other metallic

object or surface.

Try to re-boot your system. If the error persists, browse through all the hints

and tips as outlined below or remove the card again, check the jumper settings

and re-install it according to the instructions in the manual.

In case you still get the same error message every time you boot your system,

try to install the card in another TT030 or Mega STE. Contact your nearest

dealer or Cybercube if you are unable to properly install the card.

If this error message appears only from time to time, you might want to install

a small hardware patch consisting of one 4.5 cm (1.77") wire. Please follow the

instructions listed below:

IMPORTANT: Install and use at your own risk. Warranty will be voided by

any changes or alterations of the product.

CAUTION:   Please do not touch the video connector pins or any other exposed

conductive parts of the CyReL M16-1280 without grounding yourself

first. Even after grounding, please reduce contact with such areas

to a minimum. Please make sure you are working in a static free

environment when attempting the following changes.

ELECTRO STATIC DISCHARGES CAN CAUSE FAULTS

AND EVEN LEAD TO PERMANENT DAMAGE OF THE COMPONENTS.

1. De-install the card and place the M16-1280 in front of you,

orienting it as shown in this diagram:

____

___\__/_____________[###]_____

|                      |

| !          ! ======= |

| !          ! ======= |

| !          ! ======= |

| !          ! ======= |

| !          ! ======= |

| !          ! ======= |

*

2. Each CyReL M16-1280 is equipped with two 32 pin expansion

connectors. Locate the expansion connector marked with an

'*' (left expansion connector)

3. Strip both ends of the 4.5 cm (1.77") insulated copper wire.

Remove approx. 5 mm (1/5") of the insulation to expose the

copper.

4. Begin counting from pin hole 1 (indicated by the '1' printed

on the board) up to pin hole 16.

5. Insert one end of the wire into the 16th pin hole of the

expansion connector on the left.

6. Insert the other end into the 31st pin hole (second from the

top since this is a 32 pin connector).

7. Now, please verify if you bridged the indicated pins in exactly

the same manner as indicated in the diagram below!

____

___\__/_____________[###]_____

|                      |

| !_.Pin 31  ! ======= |

| ! | to     ! ======= |

| ! |        ! ======= |

| !_!Pin 16  ! ======= |

| !          ! ======= |

| !          ! ======= |

8. Re-install the card according to the instructions in the manual.

Notes:

Do not forget to remove the bridge if you install any expansion

modules. The above listed procedure will cause the CyReL M16-1280

to be installed every time during the boot-up phase, provided you

have activated the CM16_VIP.PRG. Use at your own risk!

------------------------------------------------------------------------

ERROR:   M16 Structure Locked.

------------------------------------------------------------------------

The M16-1280 boards support device sharing and can be operate in a

multi-tasking environment. A  protocol has been defined, that allows only one

program at a time to access a special M16 information table, the M16 STRUCTURE.

In some cases however, it might happen that the program cannot perform as

expected and may be prevented to release the M16 Structure in the proper way

after accessing the M16 information tables.

This rare case might be triggered by an prematurely terminated or aborted

program. The M16 Structure may still be locked, preventing other programs from

accessing it.

To avoid such a dead-lock, simply double click the CM16_VIP icon again. The

installation program will examine the state of the M16 Structure and request a

confirmation to release the structure in case it found an invalid lock-up

condition. Press 'Y' to unlock the M16 Structure if you are sure that no other

program is evaluating the M16 information tables.

If you installed the VDI Configuration Accessory (VDI_CONF.ACC), you might

instead click on the checked 'Lock Board' box. The check mark should disappear

and the M16 Structure will be un-locked as soon as you leave the accessory.

------------------------------------------------------------------------

ERROR1:  Installation program found a CM16 cookie that is not being used

by a Cybercube product.

------------------------------------------------------------------------

There are three possible causes for this problem:

a)You might be using a third party product or program that uses the 'CM16' ID

as a cookie name. This will prohibit the use of all custom designed M16-1280

software packages. Try to change your boot configuration to avoid the

problem. Please report all name conflicts to Cybercube.

b)The installation program might be corrupted. Use the original M16 disks to

re-install the CM16_VIP program. Re-boot your system.

c)There might be a corrupted cookie jar and/or M16_STRUCT in the memory of your

computer system. To test this, switch off the computer system, wait for a

short while, then try to boot again. If the CM16_VIP program still reports

the same problem, refer to cause 'b'.

------------------------------------------------------------------------

ERROR2: Unknown M16 Structure Version.

------------------------------------------------------------------------

An already installed M16 Structure has been found that differs from the  format

normally  handled  by your CM16_VIP program.  Try to  cold-boot  your  computer

system.  Check the installed CM16_VIP program. There might be several different

versions of the program trying to initialize the hardware.  Only one program is

necessary  to setup your M16 boards.  Always keep the program with the  highest

version number.

------------------------------------------------------------------------

ERROR3: No _MCH cookie found.

------------------------------------------------------------------------

The Atari TT030 and Mega STE TOS versions install a cookie known as the '_MCH'

cookie to identify the computer type. Try to re-boot the system. If the error

persists, re-install the CM16_VIP program.

Any attempt to use the program on computers other that the TT030 or Mega STE

will cause an error condition. Contact Cybercube or your nearest dealer in case

you need an CM16_VIP update.

------------------------------------------------------------------------

ERROR4: Machine type not recognized.

------------------------------------------------------------------------

Any attempt to install the CM16_VIP program on computer systems other that the

TT030 or Mega STE will cause this error condition. Contact Cybercube or your

nearest dealer in case you need an CM16_VIP update.

------------------------------------------------------------------------

ERROR5: CPU type not recognized.

------------------------------------------------------------------------

The Atari TT030 and Mega STE TOS versions install a cookie known as the '_CPU'

cookie to identify the build-in central processing unit (CPU). The CM16_VIP is

designed to work with 68000, 68010, 68020, 68030 and 68040 CPUs. Try to re-boot

the system. If the error persists, re-install the CM16_VIP program. In case you

replaced the CPU by a custom CPU module, verify the installation guides given

in the appropriate manuals. Contact Cybercube or your nearest dealer in case

you need assistance due to an incompatible CPU module.

------------------------------------------------------------------------

ERROR6: Spurious BUS ERROR Exception.

------------------------------------------------------------------------

This error can be caused by several problems:

a) There might be an addressing conflict on the VME bus. Please verify if you

followed the installation guidelines in the M16 User's Manual or M16

Developer's Manual. Try to remove all VME bus cards except the M16 board,

then re-boot. If the error persists, check the jumper settings (see manual).

Otherwise refer to the manuals of the ALL other VME bus cards you are using.

Contact your nearest dealer or Cybercube in case you get the same error

message even if the M16 card is the only card installed.

b) There might be a general problem with the VME bus interface or your

computer. Please check this (if possible) by connecting other cards or

boards to your system.  Contact your nearest dealer or Cybercube in case you

get the error message every time you try to boot the system with an

installed M16 card.

c) The same problem may arise in case you are using a VME bus expander or VME

bus speeder. Try to use the card without any other attachments to the VME

bus. Refer also to paragraph 'a'.

d) If you are adding more than one M16-1280 card to the VME bus: Try to boot

with only one M16-1280 card connected. If it now boots correctly, switch off

your system, remove the card and repeat this process with all other M16-1280

boards you intend to install. If you get an error message during this

procedure refer to causes 'a'-'c'. Now install one of the 'good' M16-1280

boards and repeat this process by adding a second M16-1280 card and so on.

------------------------------------------------------------------------

ERROR7: Set jumpers to TT030 mode.

------------------------------------------------------------------------

The jumper settings might be set to the Mega STE addressing mode. Please check

the jumper configuration with those listed in the M16 User's Manual or M16

Developer's Manual and change the jumpers accordingly. There might be a problem

with the card or your VME bus if the jumper settings are set to the  right

decoding method. See also ERROR5.

------------------------------------------------------------------------

ERROR8: Unable to install CM16 cookie.

------------------------------------------------------------------------

There are 32 or more cookies in the cookie jar. Check your boot configuration

or programs that you frequently use to see if they install any cookies and are

the cause for this problem. The CM16_VIP program cannot relocate jars with more

than 31 cookies. Most of the time this error will be caused by a corrupted

cookie list. Try to re-boot the machine. Contact Cybercube with a list of all

installed programs and a description of your boot configuration if you get the

same error every time you attempt to use the M16-1280 card(s). The M16_VIP

program installs only one CM16 cookie even if you try to install more than one

M16-1280 card in your system.

------------------------------------------------------------------------

ERROR9: EEPROM checksum error.

------------------------------------------------------------------------

Every M16-1280 card features an configuration EEPROM. This device is being used

to store informations about the graphics mode, colors, setup informations and

initialization patterns of the M16-1280 hardware. The M16_VIP program always

checks the validity of the EEPROM data by calculating a checksum. If the

checksum does not match with the value read from the EEPROM we suggest you use

one of the supplied M16 utilities or any other program that features a M16

EEPROM setup function to re-initialize the EEPROM on your card. Please be aware

that you will loose all of your current EEPROM settings in such case.

If the error persists, reset your machine, and press BOTH SHIFT keys during the

boot-up procedure. The CM16_VIP program will inquire whether or not you wish to

proceed with the installation. Press 'Y'. Next the program will ask you if you

wish to use the current EEPROM configuration. Press 'N'. A standard 640x480 VGA

compatible resolution setting will be used to configure the card regardless of

the contents of the EEPROM. This should bypass the EEPROM error message. Next,

try to re-configure the EEPROM.

------------------------------------------------------------------------

ERROR10: VRAM read/write error.

------------------------------------------------------------------------

This error might be cause by the following conditions:

a) There is a faulty video RAM chip on one of the M16-1280 cards. Please

contact Cybercube or your nearest dealer.

b) There might be an addressing conflict on the VME bus. Refer to ERROR5 &

ERROR6.

------------------------------------------------------------------------

ERROR11: Variables buffer setup error.

------------------------------------------------------------------------

An bus error occurred during the attempt to initialize the runtime variables.

Please check the configuration of the program memory space in your computer

system. Enable or disable the appropriate program load flags in the header of

the M16_VIP file to check if there is i.e. a general problem0with your TT Fast

Ram Module or other memory expansion cards. In case you are using a virtual

memory management system, try to run the M16_VIP program without it. Please

report such incompatibilities to Cybercube. Also, try to re-boot. If the error

persists,  re-install the program using the original installation disk or your

backup copy of the installation disk.

------------------------------------------------------------------------

ERROR12: LAN/CR access timeout.

------------------------------------------------------------------------

A timeout or bus error occurred during the attempt to access the M16 control

registers or the M16 Local Area Network section.

In case you connected some cables to the LAN ports: Check if the same error

occurs if you remove the cables. Verify if the cable length does not exceed the

given maximum distances between two M16-1280 LAN ports. Make sure the cables

meet the required wiring diagram and quality standard as listed in your manual.

------------------------------------------------------------------------

ERROR13: G300 access timeout.

------------------------------------------------------------------------

A timeout or bus error occurred during the attempt to access the G300 Color

Video Controller.

Verify if the cable connected to the video connector complies with the pinout

and requirements as listed in your manual. Do not overload the video output

section of the M16-1280 board. Use appropriate video amplifiers where

necessary.

Check if the program that you are using selected the external clock input as a

source for the G300 VTG master clock AND no external clock source is available,

or does not meet the minimum/ maximum frequency requirements. The external

clock source will be selected when the frequency select pattern $1F is written

to the appropriate bits of the clock control register (CR1). This may lead to a

lock-up situation that can only be reversed by resetting the G300 VTG and

enabling the on-board clock sources. If you do not know how to reset the G300

or you do not have the right program to do it, simply reset your machine or

re-boot. Such actions will erase all data present in the VRAM buffer area.

How to correctly use the external frequency input:

Please make sure that the intended program really uses the external video

clock input mechanism and that there is an external source that can provide a

valid clock frequency. This source must conform to the frequency limits listed

below:

source               min. frequency      max. frequency

-------------------------------------------------------

internal                 5 MHz              128 MHz

external                 5 MHz               30 MHz

M16 Expansion Module     5 MHz               30 MHz

If the error persists, reset your machine, and press BOTH SHIFT keys during the

boot-up procedure. The program will inquire whether or not you wish to proceed

with the installation. Press 'Y'. Next the program will ask you if you wish to

use the current EEPROM configuration. Press 'N'. A standard 640x480 VGA

compatible resolution setting will be used to configure the card regardless of

the contents of the EEPROM.

WARNING: Do not touch the video connector pins or any parts and components of

the M16-1280 boards without grounding yourself first or while working in a non

static-free environment.

EXPOSURE TO ELECTROSTATIC FIELDS OR DISCHARGES MAY LEAD TO MALFUNCTIONS

OR EVEN DAMAGE THE CARD.

------------------------------------------------------------------------

ERROR14: VRAM access timeout.

------------------------------------------------------------------------

A timeout or bus error occurred during the access attempts to probe the

configuration and size of the video RAM buffer area.

Refer also to ERROR13.

------------------------------------------------------------------------

ERROR15: I2C CTRL access timeout.

------------------------------------------------------------------------

A timeout or bus error occurred during the attempt to access the I2C bus

controller.  Note: The I2C bus controller messages will only be displayed on

boards with a revision number of 2E and earlier.

------------------------------------------------------------------------

ERROR16: VRAM controller timeout.

------------------------------------------------------------------------

A timeout or bus error occurred during the attempt to initialize the video DRAM

controller. Reset your machine and try again. If the error persists, contact

Cybercube.

------------------------------------------------------------------------

ERROR17: Configuration EEPROM timeout.

------------------------------------------------------------------------

A timeout or bus error occurred during the attempt to access the configuration

EEPROM. Reset your machine, and press BOTH SHIFT keys during the boot-up

procedure. The program will inquire whether or not you wish to proceed with the

installation. Press 'Y'. Next the program will ask you if you wish to use the

current EEPROM configuration. Press 'N'. A standard 640x480 VGA compatible

resolution setting will be used to configure the card regardless of the

contents of the EEPROM.

------------------------------------------------------------------------

ERROR18: Expansion Module access timeout.

------------------------------------------------------------------------

A timeout or bus error occurred during the attempt to access the identification

registers of the expansion module. Please check if the expansion module has

been correctly inserted into (and is properly aligned with) the expansion

connectors. Further make sure to avoid shortages or contacts between the

expansion module and any other conductive parts. Refer to the proper

installation guidelines. If you discover a badly inserted board or any

shorts, *IMMEDIATELY* switch off your computer . Do *NOT* attempt to run

the system while the error persists. Remove the power cord, remove the graphics

card, remove the expansion board and follow the installation guidelines given

in the appropriate manuals. If the malfunction still occurs, please contact

your dealer or the manufacturer of the expansion module.

In case you are not able to resolve the installation problems outlined above,

or any problem not mentioned in this documentation has occurred, please contact

your nearest dealer or Cybercube's Service Department.

------------------------------------------------------------------------

<EOF>