The Naughty Unpacker

Version 2.2 documentation

August 13th, 1992

****************  DISCLAIMER  *******************

USE THIS PROGRAM ENTIRELY AT YOUR OWN RISK!

I ACCEPT NO RESPONSIBILITY FOR ANY DAMAGE CAUSED

BY THE USE (OR MISUSE) OF THIS PROGRAM!

1. Introduction

---------------

Like many other users, I like trading off a little time against

more disk space, i.e. I use packers to compress many programs on

my hard disk. Unfortunately, not all packers have the option to

unpack a packed program, so this has to be done "by hand" whenever

necessary. With this in mind, I started to write TUU, "The Universal

Unpacker" back in 1990, and soon I could unpack most of the files

I had packed with popular packers like "Pack-Ice" or the "Jek Packers".

TUU could also unpack pack several types of packed data files. To

achieve this, it was necessary to include the unpacking routines of

all supported packer types, and obviously this wasn't very flexible -

even newer versions of the same packer weren't recognized when they

were released. Thus, I had to think of something else, and what I

came up with is this program. It uses a quite clever trick (IMHO)

to spot the unpacking routine in the packed programs, executes them,

and returns just before the program is relocated. Then, the unpacked

program can be saved. To keep the program flexible, I decided not to

include any checks for specific packers, so that it's up to you to

make sure the program you choose is packed. Hence the name of this

program ("Naughty") - it's likely to crash if you insist on trying

it on a "normal" (not packed) program. Of course, it will check for

certain "signs" of an unpacking routine, and it will warn you if

the program contains relocation information - most packed programs

are PC-relative (when they are packed), and the few packers that

need to add relocation bits to the packed program are recognized

automatically.

2. History

----------

In the meantime, there are quite a few different versions of

Naughty floating around. I can't remember myself how many, but

it must be more than half a dozen. The first versions (1.X) used

to have a crude, .TOS-style user interface, and they were programmed

in 100% assembly language. Some TOS bugs (or perhaps my bad programming)

were responsible for problems with two TOS versions: 1.2 and 1.6.

These problems all disappeared (thanks to Richard Clarkson BTW, who

first reported those problems) when I rewrote the user-interface in

C, with the main routines still being in assembly language. Anyway,

these are the major versions, and what had changed in each of these:

(Please forgive me for keeping the information is brief and technical!)

[In brackets are later comments]

Version 1.0, 2 Feb 1992:

------------------------

The first version, tested with Pack-Ice, Pack-Fire, and many other

packers. No checks being made at all.

Version 1.1, 3 Feb 1992:

------------------------

- Fixed incompatibility with Pompey Packer v2.6

- Added some Alert-boxes for easier use.

- Added checks for executability, relocation, symbol table.

- Wrote a documentation (fingers still hurting :)

- Fixed a bug which caused the program to crash after trying

3 files...

Version 1.2, 12 Feb 1992:

-------------------------

- Relocatable programs can now be unpacked, if they have been

packed with the Happy Packer or Pompey Packers (I hope!)

- First version of Packers list (PACKERS.LST). Thanks to Richard

Clarkson for the idea!

Version 1.3, 16 Feb 1992:

-------------------------

- I finally managed to test the program with PP v1.9a packed files,

and fixed it so that they are recognized (they are relocated too).

- (Potential) unpacking routines which used the following sequence:

move #2/#3,-(sp)

trap #14

addq.w/.l #2,sp

to find out the screen address and copy the unpacking routine

below that address are now forced to use a safer address...

- The JAM Packer versions 3.0 pre-release and release 1.1 are now

supported (pre-release only LZH compression), due to the above

change!

Version 1.4, not released.

--------------------------

- some bugs fixed.

Version 1.5, 29. March 1992 (ST NEWS version [never released!])

---------------------------------------------

- Programs that had a symbol table can now be unpacked.

- Unpacking routines which copy themselves into the space

after screen memory on 512K computers ($7fd00) are forced to

use a safe area.

- Unpacking on exactly the same memory location is now allowed.

- The sequence move.l (a0)+,d0; add.l d1,(a2,d0.l) is now

recognized as relocation preparation (used at least by one

packed file I have).

- If the header of the file has larger values for TEXT-length and

DATA-length than the file size, it is assumed to be a copy of

the header of the unpacked file (the file is assumed to packed).

At least one I know of uses this.

- Added Atomik v3.5 compatibility

- From now on, Naughty MUST NOT have the 'fastload' flag set. This

is not a new problem, but I can't protect unused memory from ALL

unpacking routines...

[Since 2.X this 'fastload' is OK!]

Version 1.6, 28th April 1992

----------------------------

- Some bugs fixed again, I hope it works on all TOS versions now.

- From now on you may choose whether to continue if file with

relocation information has been chosen! This is very risky,

and not recommended... It could be the only way though if you

have a very exotic file format. Better only use it if you are

100% sure this is a packed file!

[ some versions missing here ... extensive beta-testing by other

people, mainly W. Alan B. Evans! Thanks a lot! ]

Version 2.0, June 1992

-----------------------

- First version with GEM user interface! Added some nice things

like "stay-on" dialogs to show what's happening. There seem

to be no more TOS incompatibilities!

- The resources look a bit weird on colour monitors...

Version 2.1, July 1992

-----------------------

- This is the first single-file version after 2.0, the resource-

file is now included in the program file.

Version 2.2, August 13th, 1992

------------------------------

- Added support for the BAPACK program. Thanks Michael for the

packer! This version was made for you!

- Fixed the .RSC file - all objects are character-aligned now,

and should look the same in all resolutions...

3. Usage

--------

I don't think there's much to say at this point, basically you'll

have to click one of the buttons on the main dialog, which has the

thing you want to do on top of it! :) I think the whole thing is

rather self-explanatory... If something doesn't work, check if the

program you tried to unpack was packed (or let someone who knows

what he's doing have a look at it), and if it was, please let me

know! I can be reached by e-mail or by snail-mail, my address is

somewhere in this file!

4. Copying

----------

This program may be copied freely, as long as both files

(NAUGHTY2.PRG and NAUGHTY2.DOC) are copied, and they are not

modified in any way. If you find hacked versions of this program

anywhere, please let me know! Feel free to copy this program to

your friends, and let me know if you don't like something about

it! Perhaps you can just let me know if it's any use to you -

just send me an e-mail if it doesn't cost you anything (and

if you have e-mail access!). I might send you a new version in

return (if there is one) or some other things (PD)...

5. List Of Packers (*drum rolls*)

---------------------------------

This is a list of all packers, which have been reported to

work with the latest version of Naughty, plus some technical

information about them:

information:

Packer:                       date:  supported:  |R|M|S|Other

-------------------------------------------------|-|-|-|-----------

ATOMIC 3.1                     ???      yes      |?| |?|(atomic)

ATOMIC 3.3                     ???      yes      |?|?|?|  -"-

ATOMIC 3.5                     ???      yes      |x|s|x|  -"-

Automation 2.31                ???      yes      | |s|x|(auto)

Automation 2.4                 ???      ???      |?|?|x|  -"-

Automation 2.51                ???      ???      |?|?|x|  -"-

Automation 5.01                ???      ???      |?|?|x|  -"-

BAPACK 1.0				 yes      | |?| |

ByteKiller 1.4                 1989     no       |?|?|?|

Bytekiller 2.0                          no       |?|7|?|

Bytekiller 3.0                          no       |?|g|?|(bk3)

DC Squish  1.4                          no!?     |?| |?|(DCSq)

Happy Packer                            yes      |x|?|?|

I-Tech No Quick Cruncher                no       |x|?|?|(see pack20)

JAM Packer 1.1 (.TTP version)           yes      | |s| |(jam)

JAM Packer 3.0 pre-release              yes(LZH) | |s|x| -"-

JAM Packer 3.0 release 1.1              yes      | |s|x| -"-

JAM Packer 4.0                          yes      | |s| | -"-

JEK Packer 1.2                 ???      yes      | |s| |(jek1)

JEK Packer 1.3                 ???      yes      | |s| |

JEK Packer 1.4               13.06.89   yes      | |s| |

L.S.D. Packer 1.2            18.01.89   yes      | |s| |

MCA-Crunch                              ???      |x|?|x|(mcacr)

Pack 2.0                                no       |x| | |(pack20)

Pack-Fire 1.01               03.08.90   yes      | | | |

Pack-Fire 2.01               19.02.91   yes      | | | |

Pack-Ice 1.13                20.05.90   yes      | | | |

Pack-Ice 2.12                22.06.90   yes      | | | |

Pack-Ice 2.20                21.11.90   yes      | | | |

Pack-Ice 2.31                19.02.91   yes      | | | |

Pack-Ice 2.40                14.08.91   yes      | | | |

PFX Packer 1.5                          yes      | | | |

PFX Packer 1.7                          yes      | | | |

PFX Packer 1.8				 yes 	  | | | |

Pompey Packer 1.9a                      yes      |x| |x|(pp19)

Thunder Packer 1.0                      yes      |x| |x|

------------------------------------------------------------------------

Legend (confusing, isn't it?):

R = adds relocation information

(x=yes, blank=no, ?=don't know)

M = unpacking routine accesses dangerous memory areas, such as just

below $78000 - usually to copy the unpacking routine there.

7 = uses area below $78000 (very dangerous! avoid!)

s = uses area below screen memory (a little risky)

g = uses gemdos buffer area (dangerous!)

blank = no

Most unpacking routines however DO access memory that doesn't belong

to them, which is after the data segment and is needed for the

decompression (as a buffer). This is not very dangerous though.

S = Packer is not programmed safely i.e. has problems with new TOS or

harddisks, ACC's etc.

(x=yes, blank=no, ?=don't know)

Packer-specific stuff:

----------------------

(atomic): Version 3.5 compresses better than any other packer I know of,

including Pack-Ice/Pack-Fire.

(auto): All these packers crash (when packing), so I can't test if

the files created can be uncompressed. Please try that for

me if you can...

Automation 2.31 almost the same as LSD packer and JEK Packers...

Automation 2.4 (actually Crown Of Creation Compacter v2.4)

packed programs crash on TOS 2.05 when unpacked.

(bk3): The Bytekiller v3.0 by M.Coates does not use a correct unpacking

routine, it can't unpack PC-relative programs. This is the

main reason it is not supported.

(DCSq): I WILL NOT SUPPORT THAT ONE!!! (Spaghetti-code!! looks like

a compiled-C unpacking routine!)

Strangely, some programs packed with DC Squish DO unpack!

(mcacr): This one crashes all the time (TOS 2.05), if you can use it,

please tell me if it works...

Actually, not even programs packed with MCA Crunch will unpack

and run on TOS 2.05!!!

(pack20): contains very stupid code, which would have to be checked for

separately, use the unpack.tos supplied with it instead!!

Also, this one doesn't like the fastload/TT bits... You must

remove those before packing with removsym.prg.

(pp19): Crashes all the time too. Should be able to unpack though...

Special message to the Pompey Pirates: GEM may have some bugs,

but it's not GEM's fault if you don't know how to use it...

Try to write a serious TOS-version-independant program!

(jek1): The JEK Packers 1.1 - 1.4 and the LSD Packer are very much

alike...

(jam): The JAM Packers v3.0 are now supported too. However, with TOS

2.05 the compressed files can not be loaded from the desktop,

only if uncompressed first with Naughty!!

Also, the JAM-packer 3.0 pre-release V2-compressed programs

cannot be unpacked with Naughty (at least the ones I tested)

as it seems because of a bug in the packer.

Please help me to complete this list, if you have any packers that

work (or don't) and are not listed here, please let me know!

6. My contact address(es):

--------------------------

The safest way to contact me, is to send a "common" letter to

following address:

Marinos Yannikos

Wulfeniaweg 3

A-9100 Vlkermarkt

AUSTRIA (Europe)

If my landlord doesn't throw me out soon, I can be reached faster

at this address (in Vienna, where I'm studying...)

Marinos Yannikos

Blumberggasse 14/1

A-1160 Wien

AUSTRIA

Alternatively, at least until the end of 1992, you can reach me by

e-mail with this address:

nino@vmars.tuwien.ac.at

And if you feel really daring, you can try to call me (after 9pm local

time, that's GMT+2 more or less...) at this number:

(+int) 0222/4550305

7. What now?

------------

OK, now it's time for the greetings (in no particular order):

Hi to:

All S.O.T.E. members (esp. Thomas), Eye of the Brainwash Company,

Michael Darlison, Richard Clarkson, the ST News team (esp. Richard),

John & Takis Pournaras in Greece, Kasper of Electra, Lazer,

New Mode, Slime, Karl, Daniel, Chaos Inc. (all of Delta Force),

Alexander Herzlinger, Emmanuel Moecklin, and the other IRC guys with

the nicknames Godsend, rosebud, Cep, Goemon, st-guru, Robo, Gero,

Jackintos, Pit,

all the guys I met at the Mega Leif Convention '92 (Nic, Powa, Kasper,

...?)

...and all the other guys who I'm sorry to have forgotten because

my brain is really burned out after some heavy C-coding at the

university and some 3 hours coding and writing this stuff! I'll

remember you next time!

Have fun!

Zork of S.O.T.E.