Xmodem, CRC Xmodem, Wxmodem

File Transfer Protocols

Please circulate this document anyway that you see

fit without alteration except on the page at the

end titled: "Notes and Comments".  It is requested

that anyone using these protocols within a commer-

cial product not charge for them as an option or

surcharge, but include XMODEM and its derivations

as part of the basic product.

Peter Boswell

June 20, 1986

People/Link email: TOPPER

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 2

----------------------------------------------------------------------

TABLE OF CONTENTS

1.   PREFACE  . . . . . . . . . . . . . . . . . . . . . . . . . . .   3

2.   INTRODUCTION . . . . . . . . . . . . . . . . . . . . . . . . .   5

3.   TERMINOLOGY  . . . . . . . . . . . . . . . . . . . . . . . . .   6

4.   XMODEM . . . . . . . . . . . . . . . . . . . . . . . . . . . .   7

4.1. Xmodem Hardware Level Protocol  . . . . . . . . . . . . .   7

4.2. Xmodem Initiation . . . . . . . . . . . . . . . . . . . .   7

4.3. Xmodem Data Transmission  . . . . . . . . . . . . . . . .   8

4.4. Xmodem Cancellation . . . . . . . . . . . . . . . . . . .   9

4.5. Xmodem Error Recovery and Timing  . . . . . . . . . . . .   9

5.   CRC XMODEM . . . . . . . . . . . . . . . . . . . . . . . . . .  13

5.1. CRC Calculation Rules . . . . . . . . . . . . . . . . . .  13

5.2. CRC Xmodem Initiation . . . . . . . . . . . . . . . . . .  14

6.   WINDOWED XMODEM (WXMODEM)  . . . . . . . . . . . . . . . . . .  15

6.2. Transparency and Flow Control Rules (Byte Level Rules)  .  16

6.3. Initial Handshake Rules . . . . . . . . . . . . . . . . .  18

6.4. Window Packet Transmission Rules  . . . . . . . . . . . .  18

6.5. Notes for X.25 Hosts  . . . . . . . . . . . . . . . . . .  22

7.   APPENDIX A - CRC CALCULATION RULES . . . . . . . . . . . . . .  23

7.1. IBM PC - 8088/8086 Data Structure . . . . . . . . . . . .  23

7.2. BASIC Implementation of Bit Shift Method  . . . . . . . .  23

7.3. BASIC Implementation of the Table Method  . . . . . . . .  26

8.   NOTES AND COMMENTS . . . . . . . . . . . . . . . . . . . . . .  28

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 3

----------------------------------------------------------------------

1.   PREFACE

In the years that have past since Xmodem was first developed as a file

transfer protocol, many thousands of people have been involved in

finding reasonable ways to move data via asynchronous telephone commun-

ications.  I appreciate the opportunity that I have had to meet and

learn from many of these people.  There is nothing in this document

that did not actually come from someone else.  Indeed, whether it is

WXMODEM, X.PC, Synchronous dial-up X.25, SNA, ZMODEM, Blast, Kermit or

any other protocol that becomes the dominant dial-up file transfer

protocol for personal and home computers is just not important.  What

is important is that the public domain have a high speed file transfer

protocol that is reasonably popular and  commonly available for many

types of personal computers, for bulletin boards and for services such

as People/Link, Delphi, CompuServe, GEnie and The Source.

Here are a few people that all of us should thank and I would espec-

ially like to recognize:

Ward Christensen  Ward, a true pioneer in the microcomputer

communications area, is the author of the original Checksum

Xmodem protocol.  Thanks for reminding me to "keep it simple

stupid".

Chuck Forsberg  Chuck has edited perhaps the best work on

Xmodem and has provided both YMODEM (1K Xmodem) and ZMODEM

(Windowed YMODEM) to the public domain.  Thanks for showing

me a protocol which would deal with the X-On/X-Off problem

and reminding me that there is such a thing as a DLE char-

acter.

Richard (Scott) McGinnis  Scott is the architect, the moving

force, for the People/Link software system.  His ideas,

comments and encouragement have been wonderful.  Wait until

you see his visual conference program for the IBM PC!

Thanks for showing me how to use a DLE.

Gene Plantz  Gene operates a major IBM PC bulletin board in

the Chicago area and has been active in the National SYSOP

Association.  Thanks for pushing me to do something about

performance.

In a historical perspective, there seems to be a common pattern in all

computer systems development that can shed some light on where we stand

and how we got here.  The pattern is function first, then integrity and

finally performance.

Any kind of software must first do something worthwhile.  There is no

point in being error free, or inexpensive to operate if we do not want

the function.  Back in 1977, Ward Christensen had a need to move data

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 4

----------------------------------------------------------------------

between microcomputers.  Within a year it became obvious that the

function Xmodem provided met a real need to many microcomputer users.

Once we have a new function and we accept it, there is a normal desire

for the function to be correct.  No one can't count the times that new

software users have pointed out ... "that new function is super, but

the results are wrong".  The effort changes from providing new function

to providing integrity.  The development of CRC Xmodem is a clear

response to the integrity phase of a service as it reduced undetected

transmission errors by many orders of magnitude.

After the integrity has been accepted, people begin to look toward cost

and performance.  XMODEM entered this phase in 1984-1985.  Chuck

Forsberg's YMODEM is a major step in this effort providing larger

block sizes, batch mode and more.  His ZMODEM is a major step toward

making XMODEM derivative protocols work effectively with Public Data

Networks and most importantly, provides for restart of a file transfer

at the point of failure.  WXMODEM, presented here, is an alternate

solution to ZMODEM which is, hopefully, an easier solution to the most

important performance problems.

No one really knows where XMODEM and the file transfer function will go

in the coming years.  Perhaps X.PC from Tymnet, MNP from Microcom or

Synchronous X.25 will slowly push XMODEM, et. al, into history.  I

think this will happen, but not for maybe 5 to 10 years.  Perhaps when

50% of the households outgrow the Commodore 64, or when modem manufac-

turers can provide a $50 synchronous modem we will see the beginning of

the end for XMODEM, but not today.

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 5

----------------------------------------------------------------------

2.   INTRODUCTION

XMODEM and its derivatives have become the primary method for file

transfer for personal computers.  Hopefully this document will help

people to understand these protocols and to implement them on their

own.  In particular, this document presents an additional XMODEM

derivation to the public domain: WXMODEM.

Why develop another file transfer protocol?

After working with bulletin boards, Public Data Networks such as Tymnet

and Telenet, and commercial host systems such as People/Link, Delphi,

CompuServe and others, a number of people came to believe that hobby-

ist, home and business users would benefit significantly from a new,

conceptually simple file transfer protocol which would provide improved

performance and fully support the public data networks such as Tymnet,

Telenet and Datapac.

But before WXMODEM can be presented, XMODEM and CRC XMODEM must be

described in detail.

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 6

----------------------------------------------------------------------

3.   TERMINOLOGY

I've elected to use two special terms: transmitter and receiver.  The

transmitter is the computer/software which is transmitting data packets

and receiving acknowledgement characters.  The receiver is the com-

puter/software receiving the data packets and transmitting acknowledge-

ment characters.

Here is a table of special ASCII characters that are used throughout

this paper:

Name      Decimal        Hexadecimal    Description

SOH          01           H001          Start Of Header

EOT          04           H004          End Of Transmission

ACK          06           H006          Acknowledge (positive)

DLE          16           H010          Data Link Escape

X-On (DC1)   17           H011          Transmit On

X-Off(DC3)   19           H013          Transmit Off

NAK          21           H015          Negative Acknowledge

SYN          22           H016          Synchronous idle

CAN          24           H018          Cancel

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 7

----------------------------------------------------------------------

4.   XMODEM

Xmodem is a popular error recovery type protocol for transferring files

between computers via serial, asynchronous communications.   Before

learning more about Xmodem, it is important to hear what its author has

to say:

"It was a quick hack I threw together, very unplanned (like

everything I do), to satisfy a personal need to communicate

with some other people.  ONLY the fact that it was done in

8/77, and that I put it in the public domain immediately,

made it become the standard that it is"....."People who

suggest I make SIGNIFICANT changes to the protocol, such as

'full duplex', 'multiple outstanding blocks', 'multiple

destinations', etc etc don't understand that the incredible

simplicity of the protocol is one of the reasons it survived

to this day in as many machines and programs as it may be

found in!"a

4.1. Xmodem Hardware Level Protocol

The protocol is Asynchronous, 8 data bits, no parity bit, one stop

bit.  Modems which are commonly used are AT&T 103 (300 baud), AT&T

212A (1200 baud) and CCITT V.22 (2400 baud).

Typically, the data in a file is transmitted without change (if a

7 bit machine, the left most, high order, bit is always zero)

except that CP/M and MS/DOS operating systems want a ^Z (decimal

26) to represent end-of-file.

4.2. Xmodem Initiation

Prior to entering the protocol, both the transmitting and receiv-

ing computer must know where to get the data (what file is to be

transmitted) and where to put the data (file to store the data or

buffer area).  In Xmodem one side of the file transmission is

always in charge (local computer), asking the other side (remote

computer) to either transmit a file or to accept a file.  Through

a dialog outside of Xmodem the local computer (your PC) first

sends commands to the remote computer to select a file name

to prepare to transmit or receive a file via XMODEM.  Once this is

completed the remote computer enters the XMODEM protocol.  Now the

local computer must be told what file to transmit or receive and

it enters the XMODEM protocol, and hopefully data starts moving.

a    Ward Christensen, quoted from a message posted on CompuServe

in 1985.  Edited by Chuck Forsberg, "X/Ymodem Protocol

Reference", unpublished, 10/20/1985.

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 8

----------------------------------------------------------------------

Upon entering the Xmodem protocol, the transmitting computer waits

between 10 seconds and a minute to receive an NAK character from

the receiving computer.  The receiving computer is said to drive

the protocol.  The transmitter may retry any number of times.  If

any character other than a NAK or CAN is read by the transmitter,

it is ignored.  The CAN character implies cancellation of the

Xmodem file transfer and that the transmitter should leave the

Xmodem protocol.  Once the receiver has sent a NAK, it will wait

10 seconds for data to begin to arrive.  If none arrives in 10

seconds, the receiver will send another NAK and continue to repeat

10 times at which point the receiver will leave the Xmodem

protocol (typically with a super cryptic error message such as

"aborted", "NAK retry maximum exceeded").

Transmitter                        Receiver

[wait for one minute]         <    [NAK]

[begin block transmission]    >

4.3. Xmodem Data Transmission

The transmitter takes the data, divides it into 128, 8 bit byte

pieces and places it in an Xmodem Packet.

The Xmodem Packet looks like this:

[SOH] [seq] [cmpl [seq] [128 data bytes] [csum]

SOH       Start of header character (decimal 1).

seq       one byte sequence number which starts at 1, and

increments by one until it reaches 255 and then

wraps around to zero.

cmpl seq  one byte 1's complement of seq.  This can be

calculated as cmpl = 255 - (255 and seq) or using

xor as cmpl = (255 and seq) xor 255.

data      128, 8 bit bytes of data.  Note than when sending

CP/M and MS/DOS files a ^Z (decimal 26) must be

added to then end of the file.  If the last block

of data is less than 128 bytes, the Xmodem packet

must be padded with characters, usually ^Z's.

csum      one byte sum of all of the data bytes where any

overflow or carry is discarded immediately.  For

example, if the first 3 bytes are 255, 5 and 6 the

checksum after the first 3 bytes will be 10.

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 9

----------------------------------------------------------------------

Once Xmodem Initiation has completed, the transmitter sends the

first Xmodem packet and then waits.  After the receiver has the

full packet, it will compare its own checksum calculation with the

checksum that was sent by the transmitter.  If the checksums

match, the receiver will send an ACK.  If the checksums are

different, the receiver will send a NAK.

After receiving an ACK the transmitter will send the next Xmodem

packet.  If a NAK is received, the transmitter will resend the

same XMODEM packet again.

Once the transmitter has sent the last Xmodem packet and has

received an ACK, the transmitter will send an EOT and then wait

for a final ACK from the receiver before leaving the Xmodem

protocol.  When the receiver sees an EOT instead of an SOH (the

first character the next packet), the receiver transmits an ACK

character, closes its files and leaves the Xmodem protocol.

Let's look at a three block file transfer:

Transmitter                                  Receiver

<<<<<          [NAK]

[SOH][001][255][...][csum]    >>>>>

<<<<<          [ACK]

[SOH][002][254][...][csum]    >>>>>

<<<<<          [ACK]

[SOH][003][253][...][csum]    >>>>>

<<<<<          [ACK]

[EOT]                         >>>>>

<<<<<          [ACK]

Seems easy, right?  And it is, until something goes wrong.

4.4. Xmodem Cancellation

It has become a defacto standard that the receiver may cancel the

file transfer by sending a CAN character and then leaving the

protocol.  If the transmitter receives a CAN character when

expecting either a NAK or ACK, the transmitter is to termin-

ate and leave the protocol.  Likewise, if the receiver sees a CAN

when expecting an SOH (start of packet) it should terminate the

file transfer.  Many implementations now require two CAN char-

acters before recognizing a cancel condition.

4.5. Xmodem Error Recovery and Timing

Error detection and recovery are the primary purposes of the

Xmodem protocol.  The transmitter and receiver should continue to

retry until 10 errors in a row have occurred.  Some of the common

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 10

----------------------------------------------------------------------

error conditions are listed below:

4.5.1.    Complement Error

If the sequence number does not match the complement

sequence number, the packet must be discarded and a NAK

sent to the transmitter.

4.5.2.    Duplicate packet condition

If the sequence number is the same as the sequence

number of the last packet received, the packet should be

discarded and an ACK sent to the transmitter.

4.5.3.    Out of sequence error

If the sequence number matches the complement sequence

number and it is neither the expected sequence number

nor the last sequence number, the receiver should send

two CAN characters and leave the Xmodem protocol

(e. g. abort the file transfer).

4.5.4.    Receive timeout errors

When expecting data, if 10 seconds ever pass without

receipt of a character, the receiver should send another

NAK.  This should be repeated 10 times.  Some implement-

ations will timeout after 10 seconds waiting for the

first character of a packet, SOH, and then reduce the

timeout for characters in a packet.  The timeout should

not go below 5 seconds if the protocol is to be used

with public data networks.

4.5.5.    Transmit timeout errors

In the original protocol, the transmitter would wait 10

seconds for an ACK, NAK or CAN and then retransmit the

last Xmodem packet as if a NAK had been received.  Most

implementations either have the transmitter wait for a

very long time (30 seconds to a minute) and then

terminate the file transfer if an ACK, NAK or CAN has

not been receive or wait about 30 seconds and retransmit

the last packet.

4.5.6.    Packet synchronization errors

Since extraneous characters are frequently generated

when using asynchronous communications, the receiver

should not count on receiving exactly 132 characters for

each Xmodem packet.  One algorithm for re-synchroniz-

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 11

----------------------------------------------------------------------

ation goes as follows:

Assume that the checksum algorithm will cause re-trans-

mission of Xmodem packets which contain extraneous

characters.

If the character received when expecting the start of a

packet is not a SOH then ignore the character and

continue to search for a SOH.

Once a SOH is found, assume that the next two characters

will be a valid sequence number and complement.  If they

are complements then assume that the packet has begun.

If they are not complements, continue to search for a

SOH.

Send a NAK if a timeout occurs while attempting to

re-synchronize (e.g. continue to process timeouts as

described above).

If no re-synchronization occurs within 135 characters

then send a NAK character and retry receiving the

packet.

4.5.7.    False EOT condition

When the receiver sees an EOT (which was not sent by the

transmitter, but generated out of a communications error)

instead of a SOH character, the receiver assumes incorrectly

that the complete file has been transmitted.  This is

typically an unrecoverable error and it does occur especially

when the transmitting and receiving UARTs are clocked

slightly differently.  An algorithm to detect false EOT might

return a NAK for the first EOT received and only assume true

end of transmission after receiving two EOT's.

Transmitter                   Receiver

[last block .. ]    >>>>>

<<<<<     [ACK]

[EOT]               >>>>>

<<<<<     [NAK]

[EOT]               >>>>>

<<<<<     [ACK]

Just in case the transmitter was not prepared to resend the

EOT, it might be wise to set the timeout to about 3 seconds

and retransmit the NAK up to 3 times and then issue a warning

message but assume end of transmission.

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 12

----------------------------------------------------------------------

4.5.8.    False CAN condition

Some Xmodem implementations will terminate on a single CAN

character.  Occasionally a CAN character will be generated by

a communications error and if this occurs and is seen by the

receiver between packets or is ever seen by the transmitter,

the file transfer will be incorrectly canceled.  Many

implementations now require two CAN characters in a row

before assuming that the file transfer is to be aborted.

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 13

----------------------------------------------------------------------

5.   CRC XMODEM

CRC Xmodem is very similar to Checksum Xmodem.  The protocol initiation

has changed and the 8 bit checksum has been replaced by a 16 bit CRC.

Only theses changes are presented.

One of the earliest and most persistent problems with Xmodem were

transmission errors which were not caught by the checksum algorithm.

Assuming that there is no bias in asynchronous communications errors,

we would expect that 1 out of every 256 erroneous complete or oversized

Xmodem packets to have a valid checksum.  With the same assumption, if

the checksum were 16 bits, we would expect 1 out of every 65,536

erroneous complete or oversized packets would have a valid checksum.

5.1. CRC Calculation Rules

Considerable theoretical research has shown that a 16 bit cyclical

redundancy check character (CRC/16) will detect a much higher

percent of errors such that it would only allow 1 undetected

bit in error for every 10^14 bits transmitted.  That's 1 un-

detected error per 30 years of constant transmission at 1 mega-

bit per second.  However, my personal experience indicates that

something around 10^9 to 10^10 is more realistic.  Why such a vast

improvement over the checksum algorithm?  It is caused by the

unique properties that prime numbers have when being divided into

integers.  Simply stated, if an integer is divided by a prime

number, the remainder is unique.  The CRC/16 algorithm treats all

1024 data bits in an Xmodem packet as an integer, multiples that

integer by 2^16 and then divides that 1040 bit number by a 17 bit

prime number.  The low order 16 bits of the remainder becomes the

16 bit CRC.

The 17 bit prime number in CRC Xmodem is 2^16 + 2^12 + 2^5 + 1 or

65536 + 4096 + 32 + 1 = 69665.  So calculating the CRC is simple,

just multiply the 128 byte data number by 65536, divide by 69665

and the low order 16 bits of the remainder are the CRC.  The only

problem is, I've never seen a computer which has instructions to

support 130 byte integer arithmetic!  Fortunately for us, Seephan

Satchell, Satchell Evaluations, published a specification a very

efficient algorithm to calculate the CRC without either 130 byte

arithmetic or bit manipulation.  Appendix A contains the source

code, in IBM/PC BASIC, for the calculation of a CRC.

Other methods of calculating CRC's for Xmodem involve bit level

logic. a.

a    Chuck Forsberg, "X/Ymodem Protocol Reference", available on

many bulletin boards.

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 14

----------------------------------------------------------------------

5.2. CRC Xmodem Initiation

The initiation of CRC Xmodem was designed to provide for automatic

fall back to Checksum Xmodem if the transmitter does not support

the CRC version.

The receiver requests CRC Xmodem by sending the letter C (decimal

67) instead of a NAK.  If the transmitter supports CRC Xmodem, it

will begin transmission of the first Xmodem packet upon receipt of

the C.  If the transmitter does not support CRC Xmodem, it will

ignore the C.  The receiver should timeout after 3 seconds and

repeat sending the C.  After 3 timeouts, the receiver should fall

back to the checksum Xmodem protocol and send a NAK.

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 15

----------------------------------------------------------------------

6.   WINDOWED XMODEM (WXMODEM)a

First, Xmodem provided the basic file transfer function, then CRC

Xmodem improved the data integrity, now we come to WXmodem which

provides better cost/performance.

6.1. WXmodem Design Criteria

A few people began discussing improvements to Xmodem with me in

late 1985, over time we developed the following criteria:

6.1.1.    The protocol must be as similar as possible to the

XMODEM originally developed by Ward Christensen.

The popularity of XMODEM, I believe, is based on

its conceptual simplicity.  More software writers

are familiar with this protocol than any other.

More files are transferred everyday by this

protocol than any other asynchronous protocol.

Simplicity here implies a limited number of rules

for timing, error recovery and initiation.

6.1.2.    The protocol must overcome the propagation delay

that is characteristic of the public data net-

works.  While the cost of long distance communi-

cation is 50 to 90% less via the public data

networks than via the public voice networks, the

propagation delays inherent in public data networks

both reduces the cost savings and increases the

aggravation that occurs while watching a computer

slowly perform a file transfer.

6.1.3.    The protocol must overcome the flow control

problems which are characteristic in many public

data network situations.  Basically, in most

situations, the X-On and X-Off characters must

always be used for flow control and only for flow

control when using public data networks.

6.1.4.    The protocol should improve error recovery by

simplifying the manner in which a programmer can

determine the beginning of an XMODEM block.  Since

the Start of Header character (SOH) can appear in

the data or CRC, the programmer must use a rela-

tively sophisticated method to determine if a SOH

actually represents the beginning of a XMODEM

block.

a    This section assumes that the reader is already familiar with

Xmodem and CRC Xmodem presented above.

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 16

----------------------------------------------------------------------

6.2. Transparency and Flow Control Rules (Byte Level Rules)

This protocol provides special public data network support for

non-X.25 hosts and PC-Pursuit access to bulletin boards.  In order

to accomplish this, the transmitter is not permitted to transmit

the X-On and X-Off characters in the Xmodem packets.  The reason

for this restriction is simple:

By the very nature of X.25 public data networks, without

flow control, buffer overruns and lost data are inevit-

able from time to time at any baud rate.

To avoid data loss public data networks must always

assume that any X-Off and X-On character is a flow

control character when supporting PC-Pursuit for

bulletin boards and when supporting non-X.25 host

systems.

Since many non-X.25 hosts, bulletin boards and communications

programs use X-On and X-Off as flow control characters, public

data networks must support those X-Off and X-On requests at the

point of connection where the X-Off is received by the public data

network.  Otherwise, as many as several hundred characters backed

up in the network would be transmitted by the public data network

before the X-Off used for flow control reached the transmitter.

The public data network has no way to know whether an X-On/X-Off

protocol or Xmodem is operational at any point in time.  Therefore

a Xmodem packet which contains an X-Off character and no succeed-

ing X-On character will cause the public data network to stop

forwarding the ACK or NAK.

In addition, error recovery requires sophisticated programming for

the receiver to determine the start of an XMODEM packet.  This

protocol simplifies this task by dedicating a special character as

an indicator that an XMODEM packet is about to begin.  The

SYN (synch, decimal 22) character is used for this purpose.

The presence of one or more SYN characters in a data stream always

indicates that the next non SYN character is the beginning of an

XMODEM packet (e.g. SOH).

The method used here to handle these situations is through the

insertion of the DLE character (H010, decimal 16, data link escape

character) as an indicator that the character following the DLE is

in fact a modified DLE, SYN, X-On, or X-Off character.

Rules:

6.2.1.    Whenever an X-On, X-Off, SYN or DLE character is

about to be transmitted as any part of an actual

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 17

----------------------------------------------------------------------

XMODEM packet including the CRC, the transmitter

will transmit instead a DLE character followed by

the original character which has been modified by

exclusive or'ing it with 64 to its value. 1

6.2.2.    The inserted DLE characters are not counted in the

128 byte data length of the data portion of the

XMODEM packet.  Indeed, it would be possible to

have a packet which is physically 264 bytes in

length because the Xmodem block sequence number

(or its complement), all of the 128 data characters

and two CRC characters are all either X-On, X-Off,

SYN or DLE characters.

6.2.3.    Neither the DLE nor the adjusted characters are

used in the CRC calculation, rather the original

character is always used in the CRC calculation.

6.2.4.    When the receiver sees a DLE character, it does not

count it in the XMODEM block length calculation,

nor compute it in the CRC calculation but discards

it and then remembers to exclusive or the next

character with 64 and to verify that the result

character is either a DLE, SYN, X-On or X-Off (the

receiver will reject the packet unconditionally, if

not one of those four characters) and then include

the character as part of the packet.

6.2.5.    Prior to transmission of a XMODEM packet, the

transmitter will send one or more SYN characters

(recommend two) as a positive indicator to the

receiver of the beginning of a Xmodem packet.2

6.2.6.    Except for the character received after a DLE, the

receiver will test each incoming character to see

if it is a SYN character.  If it is, it will

discard the character and assume that the next

character will be another SYN or SOH.  If a SYN

character is received in the middle of a packet,

the receiver will NAK that packet.  The purpose of

the SYN character is to simplify recognition of the

beginning of a XMODEM packet by the receiver.  Once

an out of synch condition occurs on incoming

data, the receiver can just ignore every incoming

character until it sees a SYN.  Existing XMODEM

code which already properly deals with this

situation could just always discard any SYN

character at time of receipt with no further

action.

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 18

----------------------------------------------------------------------

6.2.7.    The transmitter must support flow control char-

acters (X-On, and X-Off) during transmission of

packets.  Upon receipt of an X-Off it will wait 10

seconds for an X-On and will start transmission

again after 10 seconds or an X-On is received,

whichever occurs first.  Any extraneous X-On

characters received by the transmitter will be

ignored and discarded.  (Note that this does NOT

apply to X.25 host computers which use X.25 L2 and

L3 windows for flow control.)

6.3. Initial Handshake Rules

An initial handshake is provided to permit the receiver to

indicate to the transmitter whether it can support checksum

Xmodem, CRC Xmodem, or Windowed Xmodem:

6.3.1.    WXMODEM - The receiver will send a character W

(decimal 87) and wait 3 seconds for the beginning

of a Xmodem packet.  This will be repeated 3 times

and then the receiver will drop down to CRC Xmodem.

6.3.2.    CRC XMODEM - The receiver will send a character C

(decimal 67) and wait 3 seconds for the beginning

of a Xmodem packet.  This will be repeated 3 times

and then the receiver will drop down to Checksum

Xmodem.

6.3.3.    Checksum XMODEM -  The receivers will send a NAK

and wait up to 3 seconds for the beginning of a

Xmodem packet.  This will be repeated 4 times and

if no valid SOH is received, the receiver will

abort the file transfer request.

6.4. Window Packet Transmission Rules

In order to overcome the propagation delays inherent with public

data networks such as Tymnet, Telenet, Datapac, IPSS, Transpac and

dozens more, the protocol must permit the transmitter to send more

than one packet before receiving an acknowledgement from the

receiver.  The number of packets that the transmitter will send

before stopping transmission if an acknowledgement has not been

received is called the "window".  WXmodem uses a window of 4

packets for several reasons.  Most importantly, it uses a single

set of timing rules which would deal reasonably well with a wide

range of baud rates (that implied keeping the window fairly

small).  Secondly, the window sequence number is directly related

to the Xmodem packet sequence number which, hopefully, will

simplify implementation of windowing.

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 19

----------------------------------------------------------------------

Rules:

6.4.1.    The window is always 4 Xmodem packets.  That is,

the transmitter will send 4 unacknowledged pack-

ets.  Transmission will not cease and the time out

interval will not begin until 4 unacknowledged

packets have been transmitted.  Note that the

window may be less than 4 Xmodem packets for short

files or at end-of-file.

6.4.2.    The receiver will transmit acknowledgements in the

form:

ACK[sequence]

The [sequence] field is an 8 bit number where the

high order or most significant 6 bits are always

zero and the low order or least significant 2 bits

are always the same as the low order 2 bits of the

XMODEM block sequence number of the XMODEM packet

being acknowledged (value in decimal may range

from 0 to 3).

6.4.3.    The receiver does not have to acknowledge every

packet, but must acknowledge at minimum every

fourth packet.  The transmitter will accept one

ACK[sequence] for multiple XMODEM packets.  For

example, after an unknown number of packets:

Transmitter                             Receiver

....

....

....

[Block Sequence Number H0FE]

[Block Sequence Number H0FF]            ACK[H002]

[Block Sequence Number H000]            ACK[H003]

[Block Sequence Number H001]

[Block Sequence Number H002]            ACK[H001]

.....

Since some transmitters must close the window and

cease all communications before doing disk I/O to

read more data, it is suggested that acknowledge-

ments be sent for every packet (except when the

receiver can easily determine that another packet

is already being received at the point in time that

the ACK[sequence] is about to be sent).3

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 20

----------------------------------------------------------------------

6.4.4.    The receiver will reject a packet (request re-

transmission) by sending:

NAK[sequence]

Where [sequence] is then next window sequence

number (between H000 and H003) after the [sequence]

of the last good block.  The receiver will discard

up to 3 Xmodem packets received after the NAK is

transmitted until it receives the packet with the

sequence number that had previously been nak'ed by

the receiver.  The receiver will not send a second

NAK until another packet with the same sequence

number is received which is also invalid or a

timeout has occurred.

6.4.5.    When the transmitter receives a NAK[sequence], it

will complete transmission of any XMODEM block

currently being transmitted and then begin re-

transmission starting with the block which was

nak'ed.

6.4.6.    The receiver will discard duplicate packets but

count them in the window for purposes of deter-

mining the maximum receive window without an ACK in

response.  For example, if the receiver gets packet

sequence number 127 four times in a row, it must

send an ACK H003 even if the receiver has previous-

ly acked that block.

6.4.7.    The timeout intervals at various points in process-

ing are:

Waiting for a character on receive, start of packet

not yet recognized:      15 seconds

Waiting for a character on receive, start of packet

has been recognized:     15 seconds

Waiting for an Ack or Nak on transmit side after

the window has closed:   15 seconds4

Waiting for an X-On after receipt of an X-Off by

the transmitter:         10 seconds

6.4.8.    When the transmitter times out waiting for an ACK

or NAK when the window is closed (e.g. four blocks

have been transmitted), the transmitter will

retransmit the last block transmitted and wait

again.  Only after 10 consecutive timeouts have

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 21

----------------------------------------------------------------------

occurred will the transmitter cancel the trans-

mission.

6.4.9.    Where possible, it is recommended that the receiver

return an ACK[sequence] for every packet or at

least 50% of the Xmodem packets.  When the receiver

must wait for the window to close (e.g. receive 4

Xmodem packets without an acknowledgement),

some performance benefit will be lost.

If the receiver cannot overlap disk I/O and communications

I/O, the receiver can temporarily stop transmission by either:

"Closing the window" (e.g. receiving 4 blocks without sending

an ACK[sequence]) performing the disk I/O and then sending an

ACK[sequence].

Transmitting an X-Off followed by an X-On when the receiver

is ready to resume accepting data.  Note that the receiver

should be prepared to accept data for about a 1/4 of a second

after the X-Off is sent to cover situations where satellite

propagation delay may occur.  One possible implementation

would let the computer user set the "X-Off delay time" so

that in the normal case the X-Off delay could be set to 25

milleseconds.  A sophisticated implementation might set the

initial X-Off delay at 250 milleseconds and then reduce it

based on experience during the file transfer.

Each approach has its advantages, but the X-Off approach will

provide the best performance in most cases especially when

using a public data network.  Note, however, that some

computers, notably the Commodore 64 and the IBM PC Jr cannot

receive communications data while writing to disk.

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 22

----------------------------------------------------------------------

6.5. Notes for X.25 Hosts

Host computer systems which utilize the X.25 protocol

(examples: People/Link, Delphi, CompuServe, The Source) to

interface with the various public data networks may send special

control packets which change the manner in which the network will

communicate with the remote personal computer, bulletin board or

terminal.  For the purposes of this paper, it is assumed that the

X.25 host can already support CRC and/or Checksum Xmodem and

present only the changes for WXMODEM.

6.5.1.    When an X.25 Host is the transmitter, it must be

sure to set the distant X.3 PAD parameters to

assure that the receiver can use X-Off/X-On for

flow control.  This is accomplished by sending a

Q-Bit command packet to set X.3 parameter 12 to a 1

prior to the initial handshake.  Note that if the

receiver cannot support WXMODEM, the X.25 Host must

send the appropriate Q-Bit packet to reset para-

meter 12 to a 0 before transmitting the first CRC

or Checksum Xmodem packet.

6.5.2.    When an X.25 Host is the receiver and in WXMODEM

mode, it must be sure to set the distant X.3 PAD

parameters to assure that the network will use

X-Off/X-On for flow control between the network and

the transmitter to prevent its buffers from

overflowing.  This is accomplished by sending a

Q-Bit command packet to set X.3 parameter 5 to a 1

prior to the initial handshake.

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 23

----------------------------------------------------------------------

7.   APPENDIX A - CRC CALCULATION RULES

The purpose of this appendix is to give non-technical and non mathema-

tical software writers a cook book approach to calculating the CRC-16

used in Xmodem.  We have half accomplished that goal.  The BASIC code

in the examples below has been tested on an IBM PC and found to work

effectively even at 9600 with compiled Basic.  Some BASIC languages do

not offer an XOR function and others do not have MKI$ and CVI functions

which simplified the movement of data between data types.  Someday we

hope to provide a Commodore C-64/C-128 implementation which simulates

XOR, but not today!

My thanks go to Chuck Forsberg, Joe Noonan, John Byrns and Stephen

Satchell.  Without their help and public domain documents, this would

have never been possible.

7.1. IBM PC - 8088/8086 Data Structure

The Intel 8080 and upward has a feature, convenient only to some

electrical engineer somewhere, which places 2 byte (16) bit

integers in BYTE REVERSE order in memory.  That is, the least

significant byte is placed in memory before the most significant

byte for integer operations.  If A$ is one byte containing the

number 52 and it is assigned to I% using the ASC function, the

binary value (52) ends up in the first byte of I% and the second

byte is zero.

Result

I%=0                [x'0000']

I%=1                [x'0100']

A$="A"              [x'41']

I%=ASC(A$)          [x'4100']

B$=MKI$(I%)         [x'4100']  letter "A" then binary zero

I%=CVI(CHR$(0)+A$)  [x'0041']

A$=CHR$(65)         [x'41']

Once this is understood, many problems with these algorithms goes

away.

7.2. BASIC Implementation of Bit Shift Method

The bit shift method here was converted from the "C" logic

presented in Chuck Forsberg's "Xmodem/Ymodem" protocol reference

and from an old IBM two page reference guide that Joe Noonan

carries with him in his appointment calendar!

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 24

----------------------------------------------------------------------

Chucks' "C" code:

/*

* This function calculates the CRC used by the XMODEM/CRC Protocol

* The first argument is a pointer to the message block.

* The second argument is the number of bytes in the message block.

* The function returns an integer which contains the CRC.

* The low order 16 bits are the coefficients of the CRC.

*/

int calcrc(ptr, count)

char *ptr;

int count;

{

int crc, i;

crc = 0;

while (--count >= 0) {

crc = crc ^ (int)*ptr++ << 8;

for (i = 0; i < 8; ++i)

if (crc & 0x8000)

crc = crc << 1 ^ 0x1021;

else

crc = crc << 1;

}

return (crc & 0xFFFF);

}

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 25

----------------------------------------------------------------------

But in IBM PC BASIC, our implementation looks like:

100 DEFINT A-Z 'DEFAULT IS TWO BYTE INTEGERS

2000 REM * V$ CONTAINS 133 CHARACTER COMPLETE XMODEM PACKET

2010 REM * CRC$ IS TWO BYTE CRC WITH MOST SIGNIFICANT BYTE FIRST

2020 CRC$=CHR$(0)+CHR$(0)                      'START AT ZERO

2030 FOR I2=4 TO 131

2040   A$=MID$(V$,I2,1)

2050   GOSUB 4000

2060 NEXT I2

2070 REM * CRC$ CONTAINS CALCULATED CRC!

3000 IF CRC$=MID$(V$,132,2) THEN ....    'IT'S GOOD!!!

4000 REM * CRC BITWISE CALCULATION (WHAT A JOKE!)

4010 CRCH1=ASC(LEFT$(CRC$,1)) XOR ASC(A$)

4020 CRCL1=ASC(RIGHT$(CRC$,1))

4030 FOR I3 = 0 TO 7

4040   CARRY=0 : IF CRCH1 > 127 THEN CARRY=-1  'IS HIGH BIT ON IN CRC?

4050   CRCH1=(CRCH1*2) AND 255                 'CRCH << 1 AND 255

4060   IF CRCL1>127 THEN CRCH1=CRCH1+1 'IF CRCL CARRIES THEN INCR CRCH

4070   CRCL1=(CRCL1*2) AND 255                 'CRCL << 1 AND 255

4080   IF CARRY=0 THEN GOTO 4105               'IF HIGH BIT WAS ON,

4090   CRCH1=CRCH1 XOR 16                      'XOR WITH &H1021

4100   CRCL1=CRCL1 XOR 33

4110 NEXT I3

4130 CRC$=CHR$(CRCH1)+CHR$(CRCL1)

4140 RETURN 'WHEW

That routine will execute 128 * 7 + 128 * 9 * 8 BASIC statements

for each Xmodem packet or 10112 statements per Xmodem packet!  It

will work for low baud rates in compiled BASIC, but just is too

much for interpretive BASIC.

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 26

----------------------------------------------------------------------

7.3. BASIC Implementation of the Table Method

This method is based on routine M4 in Steven Satchell's paper,

"Test of CRC Routines for CRC-CCITT", but has some very signifi-

cant differences.  A table of 256 CRC's, originally calculated

with the bit shift method is used to avoid performing the bit

shift during communications.  The table contains the CRC's for

each byte value from 0 to 255 when the original CRC is zero.  The

result of this calculation is included in the DATA statements in

the code.

The comments are intended to show what is logically happening

rather than physically.  Because of the "byte reverse" nature of

integers in the 8088, a logical shift of 8 bits to the left is a

physical shift of eight bits to the right!

200 DEFINT A-Z  'ALL INTEGERS

210 DIM CRCTB(256)

300 GOSUB 9000 'INITIALIZE CRC TABLES

6200 REM * CRC CALCULATION USING TABLE METHOD, V$=XMODEM PACKET

6210 CRC$=CHR$(0)+CHR$(0)                 'INITIALIZE TO ZERO

6220 FOR Q=4 TO 131

6230   CRCH1=ASC(LEFT$(CRC$,1))           'CRC >> 8 AND 255

6240   CRCL2=CVI(CHR$(0)+RIGHT$(CRC$,1))  'CRC << 8 AND 255

6250   CRC1$=MKI$(CRCTB(CRCH1 XOR ASC(MID$(V$,Q,1))) XOR CRCL2)

6260   CRC$=RIGHT$(CRC1$,1)+LEFT$(CRC1$,1) 'SET IT BACK!

6270 NEXT Q

6280 IF CRC$ <> MID$(V$,N,2) THEN ....... 'GOTO ERROR ROUTINE

6290 REM * END OF CRC CALC

9000 FOR I%=0 TO 255 ' INITIALIZE CRC TABLE

9010   READ CRCTB(I%)

9020 NEXT I%

9025 RETURN

9030 DATA 0, 4129, 8258, 12387, 16516, 20645, 24774, 28903

9040 DATA -32504,-28375,-24246,-20117,-15988,-11859,-7730,-3601

9050 DATA 4657, 528, 12915, 8786, 21173, 17044, 29431, 25302

9060 DATA -27847,-31976,-19589,-23718,-11331,-15460,-3073,-7202

9070 DATA 9314, 13379, 1056, 5121, 25830, 29895, 17572, 21637

9080 DATA -23190,-19125,-31448,-27383,-6674,-2609,-14932,-10867

9090 DATA 13907, 9842, 5649, 1584, 30423, 26358, 22165, 18100

9100 DATA -18597,-22662,-26855,-30920,-2081,-6146,-10339,-14404

9110 DATA 18628, 22757, 26758, 30887, 2112, 6241, 10242, 14371

9120 DATA -13876,-9747,-5746,-1617,-30392,-26263,-22262,-18133

9130 DATA 23285, 19156, 31415, 27286, 6769, 2640, 14899, 10770

9140 DATA -9219,-13348,-1089,-5218,-25735,-29864,-17605,-21734

9150 DATA 27814, 31879, 19684, 23749, 11298, 15363, 3168, 7233

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 27

----------------------------------------------------------------------

9160 DATA -4690,-625,-12820,-8755,-21206,-17141,-29336,-25271

9170 DATA 32407, 28342, 24277, 20212, 15891, 11826, 7761, 3696

9180 DATA -97,-4162,-8227,-12292,-16613,-20678,-24743,-28808

9190 DATA -28280,-32343,-20022,-24085,-12020,-16083,-3762,-7825

9200 DATA 4224, 161, 12482, 8419, 20484, 16421, 28742, 24679

9210 DATA -31815,-27752,-23557,-19494,-15555,-11492,-7297,-3234

9300 DATA 689, 4752, 8947, 13010, 16949, 21012, 25207, 29270

9310 DATA -18966,-23093,-27224,-31351,-2706,-6833,-10964,-15091

9320 DATA 13538, 9411, 5280, 1153, 29798, 25671, 21540, 17413

9330 DATA -22565,-18438,-30823,-26696,-6305,-2178,-14563,-10436

9340 DATA 9939, 14066, 1681, 5808, 26199, 30326, 17941, 22068

9350 DATA -9908,-13971,-1778,-5841,-26168,-30231,-18038,-22101

9360 DATA 22596, 18533, 30726, 26663, 6336, 2273, 14466, 10403

9370 DATA -13443,-9380,-5313,-1250,-29703,-25640,-21573,-17510

9380 DATA 19061, 23124, 27191, 31254, 2801, 6864, 10931, 14994

9390 DATA -722,-4849,-8852,-12979,-16982,-21109,-25112,-29239

9400 DATA 31782, 27655, 23652, 19525, 15522, 11395, 7392, 3265

9410 DATA -4321,-194,-12451,-8324,-20581,-16454,-28711,-24584

9420 DATA 28183, 32310, 20053, 24180, 11923, 16050, 3793, 7920

This method uses 128 * 6 BASIC statements per Xmodem packet or a

miserly 768 BASIC statements per packet.  And, if you want, the

code can be tightened still more.  Unfortunately, any further

tightening that we could see would eliminate most of the already

limited readability of the code.

Xmodem, CRC Xmodem, WXmodem

June 20, 1986                                                 Page 28

----------------------------------------------------------------------

8.   NOTES AND COMMENTS

Please add your notes and comments here or send them to me and I'll get

them added to the current copy on People/Link.

1.   This was originally set up to ADD 32 to the character on transmit

and SUBTRACT 32 on receive.  By using exclusive or with 64, the

logic is the same on transmit and receive.

2.   The use of the SYN character was added at the request of several

people who have coded Xmodem routines and have struggled valiantly

to improve their error recovery routines.  Peter Boswell 6/10/86

3.   The suggestion that ACK[sequence] be sent for every block received

was added.          Peter Boswell       6/10/86

4.   The original value for the ACK/NAK timeout was 10 seconds.  This

was changed to 15 seconds the situation where the receiver is

operating at 300 baud and using X-Off to stop receipt of characters

during disk I/O.  Peter Boswell, 6/10/86