Quick Index 1.4 - the ST System Performance Evaluation Program

--------------------------------------------------------------

by Darek Mihocka and Ignac Kolenko

Ever wondered how fast your ST is? With the different versions of  TOS,

and all of the dozens of TOS "hooks" already available (e.g.  GDOS, Turbo

ST, UIS), floppy disks, RAM disks, and hard disks, almost every ST system

is unique.

With all these different ST systems, the time has arrived for a

benchmarking program to evaluate the efficiency of the ST.

Quick Index is such a benchmarking program that runs on any ST.

Quick Index measures the speed of 6 different parts of the ST:

- CPU performance in 4 categories

- VT52 text output

- GEM dialog boxes

- GEMDOS file creation

- raw disk DMA sector I/O

- disk speed (rotations per minute)

To run Quick Index, double click on the file QINDEX14.PRG. When the main

dialog box appears, click on the GO button. The tests will start. After

about one or two minutes, the results of the tests will be printed beside

the 9 total categories.

Each one of the 9 categories can be selected and deselected by clicking

on the category name with the mouse. Only selected categories will be

tested when GO is clicked.

By selecting which drive to test, you can measure the relative efficiency

of various disk drives. Even on the same hard disk, different partitions

will give very different GEMDOS results, depending on the size of the

partition and the number of files on it.

As a reference, we used a stock 520ST (with TOS 1.0) with a single sided

floppy  disk and color monitor (in low resolution). Such a system will show

speeds of 100% in each of the 9 categories. Other systems we used for

comparison were 520STs, 1040STs, and Mega 4s with TOS 1.2 and 1.4

installed, with and without Quick ST, Turbo ST, Deskcart, and GDOS, and

other accessories.

3 new CPU categories were added in this release of Quick Index, as we had

the opportunity to finally test a 16 MHz accelerator board. The results

were mixed. Only the CPU division test showed any significant increase in

speed, due to the fact that division on the 68000 is a very CPU intensive

operation. Having the CPU run twice as fast speeds this up. On the other

hand, the other CPU categories showed an increase of less than 10%, since

they tend to be more memory bound, and this particular board did not use

cached memory. When this board was slowed down to 8MHz, the CPU numbers

actually dropped to about 60%, rather than the normal 100%. So be careful

when selecting accelerator boards!

On average, the accelerator board we tested improved the speed of the

BIOS test by about 30%, while having almost no affect on the GEM test.

The disk speed category was also added in this release, and it simply

displays the speed of the selected disk drive in rotations per minute

(RPM). A floppy disk should be at 300 RPM, and a hard disk at 3600 RPM.

We also had the opportunity to test the Supra 10 Meg floppy disk drive.

It performed better than expected. The DMA test showed it to be about 5 to

10 times faster than a standard floppy disk, and only 2 or 3 times slower

than a hard disk. Considering each floppy holds 10 megabytes and is

"removable", it sure beats buying a second floppy drive.

One interesting observation was that a double sided disk formatted with

DC Formatter scored only 43% on the GEMDOS test, but 182% on the DMA

test. In other words, "twisted" disks may read faster but may cut down the

speed of other disk operations.

This release also fixes a small problem with the DMA and GEMDOS tests. In

earlier versions the numbers tended to vary slightly from test to test. The

tests are now performed a bit more accurately and should produce more

consistent numbers.

Another minor fix in this release is that the mouse is disable during the

tests, so that sudden movements of the mouse do not affect the results.

Remember that the BIOS and GEM tests are referenced against a low

resolution screen. The numbers you obtain in medium or high resolution will

be slightly higher. So if are trying to compare the efficiency of TOS 1.4,

Turbo ST, the blitter, or other screen accelerators, be sure to always

perform the test in the same resolution.

With about half a dozen companies now claiming to have 16 MHz accelerator

boards for the ST, we would like to see some more results. We challenge the

makers of these boards to run Quick Index on their hardware and show us

some pleasing numbers before we shell out $300 to $500 for such boards.

We'd like to see numbers in the 150%-200% range, and not less!

We hope that you will find Quick Index useful. Remember to support

shareware by sending us a contribution of $30. You then become a registered

user of all of the Quick programs (Quick View, Quick Index, Quick Find,

Quick Inf, Quick Print, etc...) and will be able to receive updates that

will not released to the public. Click on the HELP button in Quick Index

for more information.

This release of Quick Index may be copied, and this documentation file may

be reprinted, on the condition that the program and documentation are not

modified and are distributed at no cost. The ARCed file may be posted on

any free BBS or online service.