****************************************************************************

P S G  -  S O U N D C H I P  AY-3-8910  ( S T )

****************************************************************************

$FFFF8800 [R/-] Read Data

[-/W] Register Selection

$FFFF8801 [R/W] ******

$FFFF8802 [R/W] Write Data

$FFFF8803 [R/W] ******

$FFFF8800 [R/-] 76543210 .......................... PSG Register 14 (Port A)

||||||||

|||||||+-- Sideselect: 0:side 1/1:side 0

||||||+--- Drive A Select: 0:on/1:off

|||||+---- Drive B Select is not connected in FALCON 030

||||+----- Printer Select In

|||+------ DSP Reset: 0:no reset/1:reset

||+------- Strobe Parallelport

|+-------- internal Speaker: 0:on/1:off

+--------- IDE Reset: 0:no reset/1:reset(HD slows down)

NOTE: The PSG-Registers are now fixed at 2 addresses($8800.w/$8802.w).

Accessing the shadowregisters ($8804.w-$8900.w) cause a buserror.

****************************************************************************

A U D I O  -  S U B S Y S T E M

****************************************************************************

$FFFF8900 [R/W] ____3210 .............................. Sound-DMA-Control Hi

||||

||||   MFP I/O-Port-7 Request

||00-- no request

||01-- after playing a frame

||10-- after recording a frame

||11-- after playing or recording a frame

||

||     MFP Timer-A-Request

00---- no request

01---- after playing a frame

10---- after recording a frame

11---- after playing or recording a frame

$FFFF8901 [R/W] 7_54__10 ................................................ Lo

| ||  ||

| ||  |+-- 1: DMA-Play enable

| ||  +--- 1: DMA-Play frame repeat

| |+------ 1: DMA-Record enable

| +------- 1: DMA-Record frame repeat

+--------- 0: select playframe-adresses

1: select recordfame-adresses

$FFFF8902 [R/W] ******

$FFFF8903 [R/W] 76543210 ............................ Frame-Start-Address Hi

$FFFF8904 [R/W] ******

$FFFF8905 [R/W] 76543210 ................................................ Mi

$FFFF8906 [R/W] ******

$FFFF8907 [R/W] 7654321_ ................................................ Lo

$FFFF8908 [R/W] ******

$FFFF8909 [R/W] 76543210 ...........................Frame-Address-Counter Hi

$FFFF890A [R/W] ******

$FFFF890B [R/W] 76543210 ................................................ Mi

$FFFF890C [R/W] ******

$FFFF890D [R/W] 7654321_ ................................................ Lo

$FFFF890E [R/W] ******

$FFFF890F [R/W] 76543210 .............................. Frame-End-Address Hi

$FFFF8910 [R/W] ******

$FFFF8911 [R/W] 76543210 ................................................ Mi

$FFFF8912 [R/W] ******

$FFFF8913 [R/W] 7654321_ ................................................ Lo

HOW to access the play/record-frame:

You have to set bit 7 of $8901.w to select play- or record-shadowregister,

then access the frame-begin/end-registers! The play- and record-shadow-

register are two seperate registers; they appear only at the same

addresses!

$FFFF8920 [R/W] _654_210 ............................. Sound-Mode-Control Hi

||| |||

||| |00-- play 1 track

||| |01-- play 2 tracks

||| |10-- play 3 tracks

||| |11-- play 4 tracks

||| +---- ??unknown??

|00------ connect track 1 with speaker

|01------ connect track 2 with speaker

|10------ connect track 3 with speaker

|11------ connect track 4 with speaker

+-------- ??unknown??

$FFFF8921 [R/W] 76____10 ................................................ Lo

||    ||

||    00-- nute condition (on STE: 6258 Hz)

||    01-- 12517 HZ

||    10-- 25033 HZ

||    11-- 50066 HZ

|+-------- 0:  8 Bit

|          1: 16 Bit

+--------- 0: Stereo

1: Mono

$FFFF8922 [R/W] ________

$FFFF8923 [R/W] ________

$FFFF8924 [R/W] 76543210 ??unknown??

$FFFF8925 [R/W] 76543210 ??unknown??

$FFFF8930 [R/W] 76543210 ............................ Source-Device-Clock Hi

||||||||

||||||||   Source-Device: EXT-INP

|||||||+-- Handshaking: 0:on/1:off

|||||++--- Source-Clock

||||+----- ??unknown??

||||

||||       Source-Device: A/D-Converter

|||+------ ??unknown??

||+------- 0: internal 25.175 MHz-Clock

||         1: extermal Clock

|+-------- set to zero

+--------- ??unknown??

$FFFF8931 [R/W] 76543210 ................................................ Lo

||||||||

||||||||   Source-Device: DMA-PLAY

|||||||+-- Handshaking: 0:on/1:off

|||||++--- Source-Clock

||||+----- 0: if handshaking and destination=DSP-REC

||||       1: if destination<>DSP-REC

||||

||||       Source-Device: DSP-XMIT

|||+------ Handshaking: 0:on/1:off

|++------- Source-Clock

+--------- 0: Tristate, disconnect DSP from Multiplexer

(if you want to use the external SSI-Port)

1: connect DSP with Multiplexer

Source-Clock:   %00: internal 25.175 MHz-Clock

%01: external Clock

%10: intermal 32 MHz-Clock, do not use it for the CODEC

%11: not defined

$FFFF8932 [R/W] 76543210 ...................... Device-Destination-Matrix Hi

||||||||

||||||||   Destination-Device: EXT-OUT

|||||||+-- Handshaking: 0:on/1:off

|||||++--- Source-Device

||||+----- ??unknown??

||||

||||       Destination-Device: D/A-Converter

|||+------ ??unknown??

|++------- Source-Device

+--------- ??unknown??

$FFFF8933 [R/W] 76543210 ................................................ Lo

||||||||

||||||||   Destination-Device: DMA-REC

|||||||+-- Handshaking: 0:on/1:off

|||||++--- Source-Device

||||+----- 0: if handshaking on and source=DSP-XMIT

||||       1: if source<>DSP-XMIT

||||

||||       Destination-Device: DSP-REC

|||+------ Handshaking: 0:on/1:off

|++------- Source-Device

+--------- 0: Tristate, disconnect DSP from Multiplexer

(if you want to use the external SSI-Port)

1: connect DSP with Multiplexer

Source-Device : %00: DMA-PLAY

%01: DSP-XMIT (DSP send data)

%10: EXT-INP (External Input)

%11: A/D-Converter

$FFFF8934 [R/W] ____3210 ........................... Prescale external Clock

||||

++++--    0: ??switch to STE-compatible mode??

1-15: samplingrate=Clock/256/(prescalevalue+1))

$FFFF8935 [R/W] ____3210 ........ Prescale internal Clock (25.175 or 32 MHz)

||||

++++-- look above! According to the developer-

||||   documentation you can only use the following

||||   values for the CODEC(A/D- and D/A-Converter):

||||   0,1,2,3,4,5,7,9,11

0000-- switch to STE-compatible mode

0001-- CLK50K  49170 Hz

0010-- CLK33K  32780 Hz

0011-- CLK25K  24585 Hz

0100-- CLK20K  19668 Hz

0101-- CLK16K  16390 Hz

0110-- CLK14K  14049 Hz (invalid for CODEC)

0111-- CLK12K  12292 Hz

1000-- CLK11K  10927 Hz (invalid for CODEC)

1001-- CLK10K   9834 Hz

1010-- CLK09K   8940 Hz (invalid for CODEC)

1011-- CLK08K   8195 Hz

1100-- CLK07K   7565 Hz (invalid for CODEC)

1101-- CLK07K   7024 Hz (invalid for CODEC)

1110-- CLK06K   6556 Hz (invalid for CODEC)

1111-- CLK06K   6146 HZ (invalid for CODEC)

NOTE: This register is used for both sourceclocks, 25.175 MHz and 32 MHz.

$FFFF8936 [R/W] ____3210 .............................. Track-Record-Control

||||

||00-- record 1 track

||01-- record 2 tracks

||10-- record 3 tracks

||11-- record 4 tracks

++---- ??unknown??

$FFFF8937 [R/W] ____3210 ......................... CODEC-Hardwareadder-Input

||||                           (ADDRIN-register)

||||     Source-input of the 16-bit-hardwareadder

||||

|||+-- 1: input from A/D-Converter

||+--- 1: input from Multiplexer

|+---- ??unknown?? (XBIOS-CALL $8C use this bit)

+----- ??unknown??

NOTE: The CODEC-Hardwareadder-Input connects the D/A-Converter with the

multiplexer or the A/D-Converter. It is also possible to connect both.

In this case the 16-bit-Hardwareadder mix the two signals.

$FFFF8938 [R/W] 76543210 ............................... A/D-Converter-Input

||||||||                                 (ADCINPUT-register)

|||||||+-- 0: input from right MIC-channel

|||||||    1: input from PSG-channel

||||||+--- 0: input from left MIC-channel

||||||     1: input from PSG-channel

|||||+---- ??unknown?? (XBIOS-CALL $8C use this bit)

+++++----- ??unknown??

$FFFF8939 [R/W] 76543210 .......... Channel-Input-Amplifier in +1.5 dB steps

||||||||            (GAIN-register)

||||||||

||||++++-- 0-15: Gain right channel (RTGAIN)

++++------ 0-15: Gain of left channel (LTGAIN)

$FFFF893A [R/W] 76543210 ...... Channel-Output-Amplifier in -1.5 dB-steps Hi

||||||||        (ATTEN-register)

||||||||

||||++++-- 0-15: Attenuation of feft channal(LTATTEN)

++++------ ??unknown??

$FFFF893B [R/W] 76543210 ................................................ Lo

||||||||

||||++++-- ??unknown??

++++------ 0-15: Attenuation of right channel(RTATTEN)

$FFFF893C [R/W] ______10 ................................... CODEC-Status Hi

||

|+-- 1: right channel-overflow (read only)

+--- 1: left channel-overflow (read only)

$FFFF893D [R/W] 7654____ ................................................ Lo

||||

|||+------ ??unknown??

||+------- ??unknown??

|+-------- ??unknown?? (read only)

+--------- ??unknown?? (read only)

$FFFF893E [R/W] ******

$FFFF893F [R/W] ******

$FFFF8940 [R/W] _____210 .............................. GPx-Datadirection Hi

|||

+++-- ??unknown??

$FFFF8941 [R/W] ____3210 ................................................ Lo

||||

|+++-- bidirectional Dataportpath of the GP0-

|      GP2-Pins on the DSP-Connector

|      0: Pin set to Input (read data from GPx)

|      1: Pin set to Output (write data to GPx)

|         (normally %111)

+----- ??unknown??

$FFFF8942 [R/W] ____3210 ................................... GPx-Dataport Hi

||||

|+++-- Shadow-I/O-Data-Bits of $FFFF8943

+----- ??unknown??

$FFFF8943 [R/W] _____210 ................................................ Lo

|||

+++-- Input/Output-Data-Bits of the

GP0-GP2-Pins on the DSP-Connector. This

Pins can be used for userdef. operations.

############################################################################

################## research and documentation by AURA ######################

############################################################################