02200000000801

1

2

9[...............................................................]0

68000 MACHINE LANGUAGE COURSE PART III by Mark van den Boer

In  part  I & II the basics of programming the  68000  have  been

introduced.  In the following parts of this course I will discuss

the  instructions of the 68000.  To understand this part and  the

coming  ones  I suggest you keep a printed copy of part  II  (the

addressing modes) in close range, so you can look things up.

Before discussing the individual instructions of the 68000 I will

divide its instructions into eight classes.

These classes are:

- Data movement operations.

- Integer  arithmetic  operations  (on  two's  complement  binary

numbers.

- Logical operations.

- Shift and rotate operations.

- Bit manipulation operations.

- Binary Coded Decimal (BCD) operations.

- Program control instructions.

- System control operations.

In  this  part  and  the following  parts  all  classes  will  be

discussed.  This approach is somewhat different of the one chosen

by  other authors (the alphabetic order).  For an explanation  of

the source and destination addressing modes I refer to the  table

at the end of part II.

To  specify  the  operands  of  the  instructions  the  following

conventions are used:

Rn   stands for either a data- or addressregister

An   stands for an address register

Dn   stands for a data register

<ea> stands  for an effective address operand.  This  means  that

after the operand after evaluation must yield an address.

#    stands for immediate data

All other symbols which appear in either field of the

instruction's assembler syntax will be explained.

One  suggestion  before  the  explanation  starts:   Since  68000

instructions  may operate on bytes,  words and longwords,  it  is

wise  to always specify which size you want to  manipulate.  This

way will considerably reduce the amount of very hard to find bugs

in your program.  Since assemblers take defaults when no size  is

specified you can get very odd results.  E.g.  BSET  (instruction

will  be explained later) operates only on bytes  and  longwords.

The assembler makes the default for you. Suppose you want a word-

operation  to  be  performed  and you  don't  specify  the  size,

thinking  that the default is nearly always word.  Well  in  this

case the default isn't word, so the assembler thinks your code is

alright, but it doesn't do what you want.

Data movement operations

Instruction:   EXG

Syntax:        EXG Rn,Rn

Data sizes:    Long

Condition codes affected: None

Addressing modes:

source:   Dn

An

destination:

Dn

An

Function:      Exchange the contents of two registers

Example:

Instruction              Before         After

EXG a0,d0                d0=11111111    d0=22222222

a0=22222222    a0=11111111

Instruction:   LEA

Syntax:        LEA <ea>,An

Data sizes:    Long

Condition code affected: None

Addressing modes:

Source:

(An)

w(An)

b(An,Rn)

w

l

w(PC)

b(PC,Rn)

Destination:

An

Function:      Load an address register with a value.  LEA stands

for Load Effective Address.

Example:

Instruction              Before         After

LEA (a1),a0              a0=00000000    a0=12345678

a1=12345678    a1=12345678

LEA 9(a0),a0             a0=00111111    a0=00111119

LEA 5(a0,d0.w)           a0=10000100    a0=10001111

d0=0000100b    d0=0000100b

LEA $12345678,a0         a0=00000000    a0=12345678

Instruction:   LINK

Syntax:        LINK An,#

Data sizes:    None

Condition codes affected: None

Addressing modes:

source:   An

destination:

#w

Function:      Create  temporary  space  on  stack.   The  source

(address register) is pushed onto the  stack,  the

stack-pointer  (a7) is copied into the source  and

the destination is added to the stack-pointer. The

destination-operand  is usually  negative  because

the  68000-stack grows downward  in  memory.  This

instruction  is  used  very  much  by   high-level

languages such as C and PASCAL.  This  instruction

has  the  advantage  that  local  variables  in  a

subroutine  can  be accessed through  the  source-

address register.

E.g.:  Every C-compiler on the Atari St uses  this

instruction at the beginning of every subroutine.

Example:

Instruction              Before         After

LINK a6,#-4              a6=12345678    a6=44444444

a7=44444448    a7=44444440

Let's analyse: First  a6 is pushed onto the  stack:  a7=44444444.

Then the stack-pointer is copied into a6. Then the

destination-operand is added to a7.

Instruction:   UNLK

Syntax:        UNLK source

Data sizes:    None

Condition codes affected: None

Addressing modes:

source:   An

Function:      The opposite of LINK. Also used by all C-compilers

at the end of functions.

Example:

For values see LINK

Instruction              Before         After

UNLK a6                  a6=44444444    a6=12345678

a7=????????    a7=44444448

First a6 is copied into a7:  a7=44444444.  Then a6

is pulled from the stack,  which yields the  final

values of a6 and a7

Instruction:   MOVE

Syntax:        MOVE <ea>,<ea>

Data sizes:    Byte, Word, Long

Condition codes affected: N,Z (conditionally set/cleared),

V,C (always cleared)

Addressing modes:

source:   Dn

An

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

w(PC)

b(PC,Rn)

#x

destination:

Dn

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

Function:      Move   the   contents  of  the   source   to   the

destination.   This   instruction  is   the   most

frequently  used  68000-instruction  and  is  very

powerful.  Note  that the  PC-relative  addressing

modes are not allowed as destination-operand. This

seems illogical, but there's a good reason for it.

In the view of the designers at Motorola it is bad

practice to have self-modifying code in a program.

Example:

See part II of this course for examples on the MOVE-instruction.

Instruction:   MOVE to CCR

Syntax:        MOVE <ea>,CCR

Data sizes:    Byte

Condition  codes  affected:   All  as  a  direct  result  of  the

instruction.

Addressing modes:

source:   Dn

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

w(PC)

b(PC,Rn)

#x

Function: Set some condition codes. With this instruction you can

clear/set the N,Z,V,C and X flag.

Example: See part II

Instruction:   MOVE to SR

Syntax:        MOVE <ea>,SR

Data sizes:    Word

Condition  codes  affected:   All  as  a  direct  result  of  the

instruction. This instruction is priviliged .

Addressing modes:

source:   Dn

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

w(PC)

b(PC,Rn)

#x

Function: Set condition codes.

Example: See part II

Instruction:   MOVE from SR

Syntax:        MOVE SR,<ea>

Data sizes:    Word

Condition codes affected: None.

Addressing modes:

source:   SR

destination:

Dn

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

Function: Request   condition  codes.   This  function   is   not

priviliged on the 68000 and 68008, but is priviliged on

all other members of the 68000-family.

Example: See part II

Instruction:   MOVE to/from USP

Syntax:        MOVE USP,An   or   MOVE An,USP

Data sizes:    Long

Condition codes affected: None.

Addressing modes: see Syntax

Function: This instruction is priviliged.  The instruction exists

because the 68000 has in fact two stack-pointers.  They

are both contained in a7.  This is possible because the

68000 has two modes of operation: user- and supervisor-

mode.  When in usermode it is impossible to access  the

supervisor  stackpointer.  It  is rather  logical  that

there are in fact two stackpointers.  If there had been

only  one  it would have been possible to mess  up  the

stack  in  such  a way that  the  system  would  crash.

However, the supervisor must have the ability to access

the user-stackpointer (for parameter-passing etc.).

Example:

Instruction              Before         After

MOVE USP,a6              USP=12345678   USP=12345678

a6=33333333    a6=12345678

Instruction:   MOVEA

Syntax:        MOVEA <ea>,An

Data sizes:    Word, Long

Condition codes affected: None

Addressing modes:

source:   Dn

An

(An)

(An)+

-(An)

w(An)

b(An,Rn)

w

l

w(PC)

b(PC,Rn)

#x

destination:

An

Function: Load an address register with a value. If the operation

is word-sized then the address is  sign-extended.  Most

assemblers will accept MOVE <ea>,An as well. However, I

think it is better to write MOVEA instead of just MOVE.

When  you  intend  to  move a  value  into  an  address

register but you forget to specify an address  register

and  specify  something else,  the assembler  will  not

generate an error. E.g. you want to move $12345678 into

a0.  MOVE.L #$12345678,d0   will not generate an error,

while MOVEA.L #$12345678,d0 will generate an error.

Example: See part II

Instruction:   MOVEM

Syntax:        MOVEM register list,<ea>      or

MOVEM <ea>,register list

register list is a set of registers.  Suppose  you

want  to  specify  all  data-register.  You  could

write:  d0-d7  or d0/d1/d2/d3/d4/d5/d6/d7  or  d0-

d3/d4/d5-d7. A number of registers in a row can be

denotated with a minus ('-') between them.  Single

registers  can  be specified with  a  slash  ('/')

between them.

Data sizes:    Word or Long

Condition codes affected: None

Addressing modes:

destination:

(An)

-(An)

w(An)

b(An,Rn)

w

l

source:   (An)

(An)+

w(An)

b(An,Rn)

w

l

w(PC)

b(PC,Rn)

Example:

Instruction:             Before         After

MOVEM.W $1000,d0/d3      $1000=$1234    d0=$1234

$1002=$5678    d1=$5678

MOVEM.L d0-d7/a0-a6,-(a7) will push all registers onto the stack.

MOVEM.L (a7)+,d0-d7/a0-a6 will pull all registers from the stack.

Instruction:   MOVEP

Syntax:        MOVEP Dn,w(An)   or   MOVEP w(An),Dn

Data sizes:    Word, Long

Condition codes affected: None

Function: Transfer data from or to a peripheral. This instruction

has been specially designed for communication with  all

devices  which have been originally designed for  8-bit

micro-processors.  This  way the 68000 can  communicate

with these devices.  E.g.  in the Atari are two  ACIA's

present.  ACIA's  are designed for 8-bit  computers.  I

won't  explain the operation of this  instruction  here

for it is rather seldom used,  especially by beginners.

If  you  want  to know all  about  this  instruction  I

suggest you contact the writer of this article.

Instruction:   MOVEQ

Syntax:        MOVEQ #,Dn

Data sizes:    Long

Condition codes affected: See move.

Function: This  instruction  provides an efficient way  (both  in

space  and  time) to set a data register to  a  value).

Values range from -128 to +127 decimal. Most assemblers

will generate MOVEQ-instructions where possible.

Example:

Instruction              Before         After

MOVEQ #$34,d0            d0=ffffffff    d0=00000034

Instruction:   PEA

Syntax:        PEA <ea>

Data sizes:    Long

Condition codes affected: None

Addressing modes:

source:   (An)

w(An)

b(An,Rn)

w

l

w(PC)

b(PC,Rn)

Function: Push  an address onto the stack.  This  instruction  is

often used to write position independent code.  It  has

nearly the same function as MOVE.L <ea>,-(a7).

Example:

Instruction              Before         After

PEA (a6)                 a6=12345678    a6=12345678

a7=44444444    a7=44444448

Instruction:   SWAP

Syntax:        SWAP Dn

Data sizes:    None

Condition codes affected: V, C: always cleared

Z set if the result is 0

N set if bit 31 of the result is 1

Function: exchange upper and lower half of a data register.  This

instruction  is  often  used in  conjunction  with  the

divide operation (will be explained further)

Example:

Instruction              Before         After

SWAP d0                  d0=12345678    d0=56781234

Next  time I will discuss the integer arithmetic  operations.  By

the  way:  I'm beginning to believe that this course is the  best

there is,  since I don't get any comments!!! If you read this and

react you can earn yourself a surprise.

Originally published in ST NEWS Volume 2 Issue 2.