Programmer's Clinic

MATHEW Lodge chairs this month's programmer's problem solving forum covering

such diverse topics as queries about paddles, a fix enabling 0.5Mb and 1Mb

SIMMs to be mixed in STEs. Other items include IMG file routines, a STOS tip,

MIDI queries and a MINIX question.

Mixing SIMMs in the STE

------------------------

One clear fact arose from the recent confusion over STE memory upgrades: You

cannot mix 512K SIMMs with 1MB SIMMs. However, even this is not a fact any

more, since David Hoggan of Southsea, Devon has provided a software patch that

enables STE owners to mix the two types to give a 2.5Mb STE.

The reason that mixing the two types of SIMM normally doesn't work is that

there is a bug in TOS 1.6 such that the Memory Management Unit (MMU) is

incorrectly programmed during boot-up.

Memory in STEs (and STs too) is split into two banks. When the STE is

booted, it starts by assuming there is 2Mb in each bank, and by writing a

series of test patterns to various parts of memory it works out exactly how

much memory is fitted.

Unfortunately, TOS 1.6 in the STE adds up the memory and gets the wrong

answer - when 2.5Mb is fitted, it believes that 4Mb is present. This causes

problems as programs (particularly GEM) attempt to write to memory that doesn't

exist.

The following program should be assembled and placed in an AUTO folder.

Make sure that it is run before any other AUTO folder programs by copying it

into the folder before anything else is put there. Comments in the source

indicate the order in which the SIMM cards should be fitted.

------------------------------------------------------------------------------

A:\LISTINGS\SIMM.S

------------------------------------------------------------------------------

* STE SIMM mixer by David Hoggan

* Lets STE configure 2.5 Megs properly

* MUST BE FIRST IN AUTO FOLDER!

* This program asssumes 2Mb in bank 0 and 0.5Mb in bank 1.

* It has been found that the other way around doesn't work

* - the MMU configures to a 1Mb machine.

* Install your SIMMs as follows:

*

* (back of STE)

*

* |--  1meg SIMM  --|

* |-- .5meg SIMM  --|

* |--  1meg SIMM  --|

* |-- .5meg SIMM  --|

*

* (front of STE)

clr.l   -(sp)

move.w  #32,-(sp)       Switch to super mode

trap    #1

addq.l  #6,sp

cmp.l   #$280000,$42E   RAM <= 2.5 megs?

ble     already         If so, nothing to do

cmp.b   #9,$424         Is it installed properly?

bne     already         If so, nothing to do

move.l  #$280000,$42E   Set phystop

move.l  $4F2,a0         Find reset address

jmp     (a0)            Reset system

already: move.l d0,-(sp)        Returned stack

move.w  #32,-(sp)       Switch back to user mode

trap    #1

addq.l  #6,sp

clr.w   -(sp)           Back to TOS

trap    #1

------------------------------------------------------------------------------

David notes that: "While not the most elegant solution, I have had no

problems with compatibility while running any software or hardware combination.

Memory block zero must contain 1Mbit SIMM cards, otherwise the MMU will be

configured for a 1MB machine".

STOS tip

---------

Kevin Ware-Lane of High Wycombe, Buckinghamshire, has found an undocumented

feature in STOS BASIC:

"Create any file, and save it in the root directory of your working copy of

the STOS Basic disk. It must be saved under the file name AUTOEXEC.BAS. Now,

whenever you boot your working copy of STOS, that BASIC program will be loaded

and run."

GFA takes parameters

---------------------

Many programmers know how to read the command line passed to a .TTP program, or

the document name of an installed GEM application is when it double clicked on.

However, few people know how to do it in GFA BASIC - this month, Leighton

Cathcart from Bangor, Co. Down in Northern Ireland comes to the rescue.

"Accessing the command line is very easy as GFA BASIC contains a built-in

variable called BASEPAGE, which holds the address of the basepage of your

program. The command line string is stored 128 bytes further on from this

address.

The length of the command line string is stored at BASEPAGE+128 and the

string starts at BASEPAGE+129, terminated by a zero. The folowing program is an

example in which the command line is placed in the string ARG$. You should

compile it and give it a .TTP extension".

------------------------------------------------------------------------------

A:\LISTINGS\GFA_TTP.BAS

------------------------------------------------------------------------------

Rem Read command line in GFA

Rem By Leighton Cathcart

Length%=Peek(Basepage+128)

If Length%=0

Print"No command line"

End

Else

For Count%=129 To (129+Length%)

Char%=Peek(Basepage+Count%)

Arg$=Arg$+Chr$(Char%)

Next Count%

Endif

Print"Command line:";Arg$

Repeat

Until Inkey$=Chr$(13)

------------------------------------------------------------------------------

"This will work not only for .TTP programs, but also for GEM .PRG programs

as well. When you select Install Application from the desktop, the name of the

file you opened to run your application is also placed at this location in

memory."

What a GFA dump!

----------------

Another GFA user is Giulio Marchesi from Bergamo in Italy. Giulio has

provided a program in GFA BASIC V2 to produce better proportioned screen dumps

for Degas Elite .PI3 pictures. The program was written for an Epson FX-800

9-pin printer, but it should work on all compatibles.

------------------------------------------------------------------------------

A:\LISTINGS\GFA_DUMP.BAS

------------------------------------------------------------------------------

Rem Degas PI3 screen dump

Rem By Giulio Marchesi

' print Degas .PI3

INPUT "Filename: ";file$

BLOAD file$,XBIOS(2)-34

GOSUB print_degas

'

PROCEDURE print_degas

a$=SPACE$(800)        ! buffer for line data

g$=CHR$(27)+"*"+CHR$(1)+CHR$(800)+CHR$(800/256)

' FX 800 graphics mode, double density)

OPEN "",#99,"LST:"    ! open printer channel

FOR s%=XBIOS(3) TO s%+79 ! reads screen data

x%=VARPTR(a$)

FOR q%=s%+399*80 TO s% STEP -80

FOR i%=0 TO 1    ! double data for double density

POKE x%,PEEK(q%)

INC x%

NEXT i%

NEXT q%

PRINT #99,g$;a$;CHR$(13);       ! one line and CR

PRINT #99,CHR$(27);"J";CHR$(1); ! 1/216 line feed

PRINT #99,g$;a$;CHR$(13);       ! print line again

PRINT #99,CHR$(27);"J";CHR$(23);        ! 23/216 feed

NEXT s%

CLOSE #99

RETURN

------------------------------------------------------------------------------

Turn over a new leaf

---------------------

On now to computer literature, and Mark T. White of Toronto, Canada has a plea

for anyone recommending books for further reading: "May I make a plea to all

contributors: If you quote a book, please include its ISBN number, in addition

to the full name, author(s) and publisher.

"I found Katherine Peel's 'Concise Atari ST 68000 Programmer's Guide'

(Glentop), ISBN 1 85181 172 9, through an advert in ST World. I have been

unable to find Balmer and Fitler's 'Programmer's Guide To GEM' (Sybex) without

knowing the ISBN. Does anyone know it, or where I can get the book?".

STE is a joy to use

--------------------

Tony Goodhew writes from Birstall, Leicester on the subject of the new

controller ports found on the STE:

"The STE addendum sheet supplied with the new machine gives pinout

specifications, but no details about how to read the port. I am particularly

interested in pins 5 and 15 - the paddle inputs, as these must be connected to

a four channel analogue to digital converter chip. I would be most grateful for

some help in using these inputs from either HiSoft Power BASIC, or GFA BASIC

V2.

"A similar input on a BBC Micro has been put to good use in the past for

simple data logging applications, and I forsee many educational uses if this

port can be used in the same manner".

Atari's  STE Developer Information Addendum provides the necessary

technical data. Four new joystick ports were added for the STE, directly

interfaced to the 68000 bus, rather than using another IKBD controller. The

following table gives the locations of the hardware registers for the

joysticks, paddles and light gun/pen inputs:

STE CONTROLLER PORTS

Address Usage                Description

=================================================

FF9200  xxxx xxxx xxxx 3210  Fire buttons

FF9202  UDLR UDLR UDLR UDLR  Joystick directions

Joy3 Joy2 Joy1 Joy0

FF9210  xxxx xxxx NNNN NNNN  Paddle 0 X

FF9212  xxxx xxxx NNNN NNNN  Paddle 0 Y

FF9214  xxxx xxxx NNNN NNNN  Paddle 1 X

FF9216  xxxx xxxx NNNN NNNN  Paddle 1 Y

FF9220  xxxx xxNN NNNN NNNN  Light pen/gun X

FF9222  xxxx xxNN NNNN NNNN  Light pen/gun Y

Note that the joystick registers are bi-directional and can therefore be

written to as well as read. If the registers are  written, then the outputs are

driven until the registers are read again.

Two paddle controllers can be connected with eight bit resolution on each

axis, and the current position of each of the four paddles are reported in the

registers shown in the table. No information is given in the data sheets as to

the voltage range that the A/D converters function in, but I would guess that

it is 0-5V. The triggers on the paddles are bits zero and one of $FF9202

(joystick zero left and right.)

The light gun/pen position is reported in $FF9220 and $FF9222. The x-axis

accuracy is four pixels in low resolution, eight pixels in medium and 16 pixels

in monochrome. The y-axis position is accurate to within one pixel in all

modes.

The x position only ranges from 0 to 319, and therefore must be left

shifted for other screen modes. Again, no electrical information is provided on

how to implement a light gun/pen.

The key to the problem

-----------------------

Lloyd Patton provides help for Paul Margetson, who wants to write his own

keyboard interrupt handler in order to test for the depression of more than one

key at once. The problem is that if a replacement interrupt handler is written,

then the operating system does not receive the key press information, and

typing commands into a debugger becomes impossible.

"Simply placing the keystroke data into the keyboard buffer will be

sufficient. Each keystroke is stored in a circular-list buffer as a 32 bit

integer, in the format returned by the Bconin routine. The low word contains

the character, the high word has the scan code in the lowest 8 bits and the

kbshift variable in the highest 8 bits. Also remember to update the Iorec

structure and take care of rotation around the buffer".

Here is Lloyd's solution:

------------------------------------------------------------------------------

A:\LISTINGS\INSERT.C

------------------------------------------------------------------------------

/* Insert keypresses by Lloyd Patton */

#include <local.h>

#include <osbind.h>

main()

{

long UpperA = 0x001e0041; /* 'A' */

long Number0 = 0x000b0030; /* '0' */

long ch;

appl_init();

insert_char(UpperA);

ch = evnt_keybd();  /* test GEM call */

printf("\nkeystroke should be 0x1e41 and is 0x%lx\n",

ch);

insert_char(Number0);

ch = Bconin(2);   /* test BIOS call */

printf("\nkeystroke should be 0x000b0030 and is

0x%08lx\n", ch);

evnt_keybd();

appl_exit();

}

insert_char(keycode)

long keycode;

{

iorec *keybuf;

long stack;

keybuf = Iorec(1);  /* get Iorec address */

stack = Super(0L);  /* supervisor mode to access Iorec

*/

/* loop around buffer */

if ((keybuf->ibuftl += 4) >= keybuf->ibufsiz)

keybuf->ibuftl = 0;

/* if room in buffer store keystroke */

if (keybuf->ibuftl != keybuf->ibufhd)

*(long *)&keybuf->ibuf[keybuf->ibuftl] = keycode;

Super(stack);   /* back to user mode */

}

------------------------------------------------------------------------------

Speedy AUTO solution

---------------------

Lloyd's also supplies a solution to Pete Ellis' problem of setting the

double-click speed from the AUTO folder:

"The only call to set this is evnt_dclick() which can be used only after

GEM initialisation and thus after the AUTO folder programs have run. If the

Control Panel is a problem using up a valuable accessory slot then write a

program that is run from the desktop then use the following listing, but if the

problem is memory usage then use a smaller DA like ROCP:"

main()

{ /* the control panel's dialogue setting is unaffected.

However, the dclick value does come into effect.    */

appl_init();

evnt_dclick(4, 1);

appl_exit();

}

MIDI clock

-----------

Finally this month, a query from Rolf Nilsson of Genarp, Sweden, who has been

dabbling with MIDI in assembler and C. "Since I am a musician, my main interest

in programming is about MIDI. I have managed to program some simple MIDI

programs, like getting MIDI bytes in to the ST, then manipulating and sending

them back out again. I've also written a simple sequencer, but I need help.

"Can anyone suggest how to implement the clock that is the heart of any

sequencer? This is vital if MIDI replay is to be accurate. Is it done through

interrupts?

"There are some good books on MIDI programming for IBMs and compatibles,

but they don't deal with this problem because the MIDI interface (Roland

MPU-401 or equivalent) takes care of the clock."

Winding down

-------------

Clinic is more yours than it is mine, so if you know the answer to a problem

then write in. Keep the letters coming - especially the solutions and comments

on other people's problems and ideas, but don't forget that the clinic depends

on your problems too! Remember to include your full name and give your phone

number if possible.

If you have a listing longer than about 25 lines then please include it on

a disk. Putting the text of your letter on disk is helpful, too, 1st Word

format if possible, but straight ASCII is fine.

If you are sending a complete program then I also like to see it running

before putting it into the column, so please include a double-clickable version

of your program if at all possible. If you want the disk returning include a

stamped addressed envelope.

Mathew Lodge

"Programmer's Clinic"

"Maen Melin"

Holmes Chapel Road

Lach Dennis

Northwich

Cheshire

CW9 7SZ

JANET : SOCS18 @ uk.ac.york.vaxa