68060 FLOATING-POINT SOFTWARE PACKAGE (Kernel version)

-------------------------------------------------------

The file fpsp.sa contains the 68060 Floating-Point Software

Package. This package is essentially a set of exception handlers

that can be integrated into an operating system.

These exception handlers emulate Unimplemented FP instructions,

instructions using unimplemented data types, and instructions

using unimplemented addressing modes. In addition, this package

includes exception handlers to provide full IEEE-754 compliant

exception handling.

Release file format:

--------------------

The file fpsp.sa is essentially a hexadecimal image of the

release package. This is the ONLY format which will be supported.

The hex image was created by assembling the source code and

then converting the resulting binary output image into an

ASCII text file. The hexadecimal numbers are listed

using the Motorola Assembly Syntax assembler directive "dc.l"

(define constant longword). The file can be converted to other

assembly syntaxes by using any word processor with a global

search and replace function.

To assist in assembling and linking this module with other modules,

the installer should add a symbolic label to the top of the file.

This will allow calling routines to access the entry points

of this package.

The source code fpsp.s has also been included but only for

documentation purposes.

Release file structure:

-----------------------

(top of module)

-----------------

|		| - 128 byte-sized section

(1)  |   Call-Out	| - 4 bytes per entry (user fills these in)

|		| - example routines in fskeleton.s

-----------------

|		| - 8 bytes per entry

(2)  | Entry Point	| - user does "bra" or "jmp" to this address

|		|

-----------------

|		| - code section

(3)  ~		~

|		|

-----------------

(bottom of module)

The first section of this module is the "Call-out" section. This section

is NOT INCLUDED in fpsp.sa (an example "Call-out" section is provided at

the end of the file fskeleton.s). The purpose of this section is to allow

the FPSP routines to reference external functions that must be provided

by the host operating system. This section MUST be exactly 128 bytes in

size. There are 32 fields, each 4 bytes in size. Each field corresponds

to a function required by the FPSP (these functions and their location are

listed in "68060FPSP call-outs" below). Each field entry should contain

the address of the corresponding function RELATIVE to the starting address

of the "call-out" section. The "Call-out" section must sit adjacent to the

fpsp.sa image in memory.

The second section, the "Entry-point" section, is used by external routines

to access the functions within the FPSP. Since the fpsp.sa hex file contains

no symbol names, this section contains function entry points that are fixed

with respect to the top of the package. The currently defined entry-points

are listed in section "68060 FPSP entry points" below. A calling routine

would simply execute a "bra" or "jmp" that jumped to the selected function

entry-point.

For example, if the 68060 hardware took a "Line-F Emulator" exception

(vector #11), the operating system should execute something similar to:

bra	_060FPSP_TOP+128+48

(_060FPSP_TOP is the starting address of the "Call-out" section; the "Call-out"

section is 128 bytes long; and the F-Line FPSP handler entry point is located

48 bytes from the top of the "Entry-point" section.)

The third section is the code section. After entering through an "Entry-point",

the entry code jumps to the appropriate emulation code within the code section.

68060FPSP call-outs: (details in fskeleton.s)

--------------------

0x000:	_060_real_bsun

0x004:	_060_real_snan

0x008:	_060_real_operr

0x00c:	_060_real_ovfl

0x010:	_060_real_unfl

0x014:	_060_real_dz

0x018:	_060_real_inex

0x01c:	_060_real_fline

0x020:	_060_real_fpu_disabled

0x024:	_060_real_trap

0x028:	_060_real_trace

0x02c:	_060_real_access

0x030:	_060_fpsp_done

0x034:	(Motorola reserved)

0x038:	(Motorola reserved)

0x03c:	(Motorola reserved)

0x040:	_060_imem_read

0x044:	_060_dmem_read

0x048:	_060_dmem_write

0x04c:	_060_imem_read_word

0x050:	_060_imem_read_long

0x054:	_060_dmem_read_byte

0x058:	_060_dmem_read_word

0x05c:	_060_dmem_read_long

0x060:	_060_dmem_write_byte

0x064:	_060_dmem_write_word

0x068:	_060_dmem_write_long

0x06c:	(Motorola reserved)

0x070:	(Motorola reserved)

0x074:	(Motorola reserved)

0x078:	(Motorola reserved)

0x07c:	(Motorola reserved)

68060FPSP entry points:

-----------------------

0x000:	_060_fpsp_snan

0x008:	_060_fpsp_operr

0x010:	_060_fpsp_ovfl

0x018:	_060_fpsp_unfl

0x020:	_060_fpsp_dz

0x028:	_060_fpsp_inex

0x030:	_060_fpsp_fline

0x038:	_060_fpsp_unsupp

0x040:	_060_fpsp_effadd

Miscellaneous:

--------------

_060_fpsp_snan:

----------------

- documented in 3.5 of 060SP spec.

- Basic flow:

exception taken ---> enter _060_fpsp_snan --|

|

always exits through _060_real_snan <----

_060_fpsp_operr:

----------------

- documented in 3.5 of 060SP spec.

- Basic flow:

exception taken ---> enter _060_fpsp_operr --|

|

always exits through _060_real_operr <-----

_060_fpsp_dz:

----------------

- documented in 3.7 of 060SP spec.

- Basic flow:

exception taken ---> enter _060_fpsp_dz   --|

|

always exits through _060_real_dz   <----

_060_fpsp_inex:

----------------

- documented in 3.6 of 060SP spec.

- Basic flow:

exception taken ---> enter _060_fpsp_inex --|

|

always exits through _060_real_inex <----

_060_fpsp_ovfl:

----------------

- documented in 3.4 of 060SP spec.

- Basic flow:

exception taken ---> enter _060_fpsp_ovfl --|

|

may exit through _060_real_inex     <---|

or |

may exit through _060_real_ovfl     <---|

or |

may exit through _060_fpsp_done     <---|

_060_fpsp_unfl:

----------------

- documented in 3.4 of 060SP spec.

- Basic flow:

exception taken ---> enter _060_fpsp_unfl --|

|

may exit through _060_real_inex     <---|

or |

may exit through _060_real_unfl     <---|

or |

may exit through _060_fpsp_done     <---|

_060_fpsp_fline:

-----------------

- not fully documented in 060SP spec.

- Basic flow:

exception taken ---> enter _060_fpsp_fline --|

|

-------------------------------------------

|               |                     |

v               v                     v

(unimplemented   (fpu disabled)       (possible F-line illegal)

stack frame)           |                     v

|               v               special case "fmovecr"?

|         exit through                    |

|     _060_real_fpu_disabled        -------------

|                                   |           |

|               ^                   v           v

|               |                 (yes)        (no)

|               |                   v           v

|               |             fpu disabled?   exit through

|               |                   |         _060_real_fline

v               |             -------------

|               |             |           |

|               |             v           v

|               |-----------(yes)        (no)

|                                         |

|----<------------------------------------|

|

|

|----> may exit through _060_real_trace

|

|----> may exit through _060_real_trap

|

|----> may exit thorugh _060_real_bsun

|

|----> may exit through _060_fpsp_done

_060_fpsp_unsupp:

------------------

- documented in 3.1 of 060SP spec.

- Basic flow:

exception taken ---> enter _060_fpsp_unsupp --|

|

|

may exit through _060_real_snan      <----|

or  |

may exit through _060_real_operr     <----|

or  |

may exit through _060_real_ovfl      <----|

or  |

may exit through _060_real_unfl      <----|

or  |

may exit through _060_real_inex      <----|

or  |

may exit through _060_real_trace     <----|

or  |

may exit through _060_fpsp_done      <----|

_060_fpsp_effadd:

------------------

- documented in 3.3 of 060 spec.

- Basic flow:

exception taken ---> enter _060_fpsp_effadd --|

|

|

may exit through _060_real_trace     <----|

or  |

may exit through _060_real_fpu_disabled     <----|

or  |

may exit through _060_fpsp_done      <----|