The STOS STE Extension

By Asa Burrows

The New STOS STE Commands :

x = STE

This function returns 1 if the machine is an STE or 0 if it isn't. You

can test to see if it's an STE or not and adjust your game to suit,

giving STE owners the extra features but still making the program

compatible with all STs.

Joysticks

Your STE now has the ability to use up to six of the little wagglers.

x= LSTICK (j)

Returns the current status of the joystick's left

position. The value x holds either -1 if the joystick is in the left

position or zero if it isn't. You can access up to six joysticks, j

holds the number of the stick you want, one to six.

The following functions work exactly the same way for the other

directions and the <fire> button.

x= RSTICK (j) right

x= USTICK (j) up

x= DSTICK (j) down

x= FSTICK (j) fire

To access joysticks one & two you need to use these other

commands.

STICKS ON

This command activates the interrupt routine for twin joysticks

control on the ST and STE, it also disables the mouse.

STICKS OFF

This de-activates the interrupt routine and reactivates the

mouse.

The new extension is completely compatible with the twin sticks

extension, so if you have that you can use the same commands without

any bother. They have been included in the STE extension to maintain

compatibility.

x =STICK1 and x =STICK2

Returns the status of joysticks 1 and 2 respectively. The variable

holds one byte laid out as follows. This is a bit of a bind and it's

better to stick with the easier joystick commands.

Bit     description

0       UP      if bit set

1       DOWN    if bit set

2       LEFT    if bit set

3       RIGHT   if bit set

4       UNUSED

5       UNUSED

6       UNUSED

7       FIRE    if bit set

Light Gun/Pen

If you've a light gun or pen you can now write a shoot-'em-up for it.

x =LIGHT X and y =LIGHT Y

These two commands return the x and y co-ordinates of your gun or pen

when the <fire> button is pressed and holds it until it's pressed

again. You can check the button using the FSTICK (3) command because

the To check the <fire> button use the FSTICK (3) command, the button

is returned as joystick three.

Extended colour palette

The STE has a palette of 4096 colours rather than an ST's 512.

E PALETTE $RGB,$RGB,...(up to 16 colour values)

This command works exactly as the PALETTE command in STOS,

except that it uses a hex number for the colour values. Hex uses base

sixteen rather than the decimal base 10. Letters are used for values

over nine. Ten is represented by 'a', eleven by 'b' and so on. White

now becomes $fff.

E COLOUR colour,$RGB

The same as the COLOUR command but using hex values and access to the

4096 colours.

x= E COLOR (colour)

No it isn't a spelling mistake, it's just a limitation of

STOS. You can't use the same name for a command and function.

This command returns the RGB value of the colour number, 0 to 15.

STE DAC and Microwire Interface

The DAC is one of the best things added to the ST. This brilliant

Digital to Analogue Converter plays raw sample sound in stereo, and

with the Microwire Interface you can set left and right volume control

as well as treble and bass.

DAC CONVERT start address of sample, end address of sample

Because the STE DAC plays raw sampled sound you'll need to

convert your Maestro samples. DAC CONVERT does the job almost

instantly. Ideal for using the same sample data for the ST and STE.

DAC SPEED

Sets the speed of sample replay. 0=6Khz, 1=12.5Khz, 2=25Khz and 4 is a

pretty amazing 50Khz.

DAC RAW start address of sample,end address of sample

Plays your raw sample.

DAC MONO and DAC STEREO

Guess what, sets the sample to be mono or stereo.

DAC LOOP ON and DAC LOOP OFF

Want an endless wall of noise? Set the loop function on and off with

these commands.

DAC M VOLUME volume

DAC L VOLUME volume

DAC R VOLUME volume

DAC TREBLE volume

DAC BASS volume

These commands set the master, left and right volume as well as the

treble and bass. The volume can be anywhere from 0 to 12.

DAC MIX ON and DAC MIX OFF

These commands allow you to mix the PSG sound chip with the

STEs DAC.

DAC STOP

Tricky one this, it er, stops the DAC.

Hardware Scrolling

The STE has hardware scrolling which enables screen data to be shifted

in bytes rather than words using the hardware. All this adds up to

single pixel scrolling.

HARD SCREEN SIZE w,h,mode

Sets the screens logical size. w=width of screen and h=height of

screen ready for scrolling.

HARD SCREEN OFFSET x,y

This command tells the ST where to start displaying the screen.

x = HARD PHYSIC (screen address)

This command tells the ST where the screen is stored.

HARD INTER ON and HARD INTER OFF

Once you have set all the hardware scrolling commands you can turn on

the scrolling with these two. While the interrupt routine is running

you use the HARD SCREEN OFFSET to get scrolling effect. If an error

occurs while interrupt is on STOS turns hardware scrolling off.

Register Me

To get the most from this STE extension and the blitter extension then

register your copy with the author. You'll get upgrades,technical

support and a comprehensive manual with copious examples. All for 10.

Send cheques or postal orders to :

Asa Burrows

67 Malvern Road

Leytonstone

London E11 3DJ