68060 INTEGER SOFTWARE PACKAGE (Library version)

-------------------------------------------------

The file ilsp.s contains the "Library version" of the

68060 Integer Software Package. Routines included in this

module can be used to emulate 64-bit divide and multiply,

and the "cmp2" instruction. These instructions are not

implemented in hardware on the 68060 and normally take

exception vector #61 "Unimplemented Integer Instruction".

By re-compiling a program that uses these instructions, and

making subroutine calls in place of the unimplemented

instructions, a program can avoid the overhead associated with

taking the exception.

Release file format:

--------------------

The file ilsp.sa is essentially a hexadecimal image of the

release package. This is the ONLY format which will be supported.

The hex image was created by assembling the source code and

then converting the resulting binary output image into an

ASCII text file. The hexadecimal numbers are listed

using the Motorola Assembly Syntax assembler directive "dc.l"

(define constant longword). The file can be converted to other

assembly syntaxes by using any word processor with a global

search and replace function.

To assist in assembling and linking this module with other modules,

the installer should add a symbolic label to the top of the file.

This will allow calling routines to access the entry points

of this package.

The source code ilsp.s has also been included but only for

documentation purposes.

Release file structure:

-----------------------

The file ilsp.sa contains an "Entry-Point" section and a

code section. The ILSP has no "Call-Out" section. The first section

is the "Entry-Point" section. In order to access a function in the

package, a program must "bsr" or "jsr" to the location listed

below in "68060ILSP Entry Points" that corresponds to the desired

function. A branch instruction located at the selected entry point

within the package will then enter the correct emulation code routine.

The entry point addresses at the beginning of the package will remain

fixed so that a program calling the routines will not have to be

re-compiled with every new 68060ILSP release.

For example, to use a 64-bit multiply instruction,

do a "bsr" or "jsr" to the entry point defined by

the 060ILSP entry table. A compiler generated code sequence

for unsigned multiply could look like:

# mulu.l <ea>,Dh:Dl

# mulu.l _multiplier,%d1:%d0

subq.l	&0x8,%sp	# make room for result on stack

pea	(%sp)		# pass: result addr on stack

mov.l	%d0,-(%sp)	# pass: multiplicand on stack

mov.l	_multiplier,-(%sp) # pass: multiplier on stack

bsr.l	_060LISP_TOP+0x18 # branch to multiply routine

add.l	&0xc,%sp	# clear arguments from stack

mov.l	(%sp)+,%d1	# load result[63:32]

mov.l	(%sp)+,%d0	# load result[31:0]

For a divide:

# divu.l <ea>,Dr:Dq

# divu.l _divisor,%d1:%d0

subq.l	&0x8,%sp	# make room for result on stack

pea	(%sp)		# pass: result addr on stack

mov.l	%d0,-(%sp)	# pass: dividend hi on stack

mov.l	%d1,-(%sp)	# pass: dividend hi on stack

mov.l	_divisor,-(%sp) # pass: divisor on stack

bsr.l	_060LISP_TOP+0x08 # branch to divide routine

add.l	&0xc,%sp	# clear arguments from stack

mov.l	(%sp)+,%d1	# load remainder

mov.l	(%sp)+,%d0	# load quotient

The library routines also return the correct condition code

register value. If this is important, then the caller of the library

routine must make sure that the value isn't lost while popping

other items off of the stack.

An example of using the "cmp2" instruction is as follows:

# cmp2.l <ea>,Rn

# cmp2.l _bounds,%d0

pea	_bounds		# pass ptr to bounds

mov.l	%d0,-(%sp)	# pass Rn

bsr.l	_060LSP_TOP_+0x48 # branch to "cmp2" routine

mov.w	%cc,_tmp	# save off condition codes

addq.l	&0x8,%sp	# clear arguments from stack

Exception reporting:

--------------------

If the instruction being emulated is a divide and the source

operand is a zero, then the library routine, as it's last

instruction, executes an implemented divide using a zero

source operand so that an "Integer Divide-by-Zero" exception

will be taken. Although the exception stack frame will not

point to the correct instruction, the user will at least be able

to record that such an event occurred if desired.

68060ILSP entry points:

-----------------------

_060ILSP_TOP:

0x000:	_060LSP__idivs64_

0x008:	_060LSP__idivu64_

0x010:	_060LSP__imuls64_

0x018:	_060LSP__imulu64_

0x020:	_060LSP__cmp2_Ab_

0x028:	_060LSP__cmp2_Aw_

0x030:	_060LSP__cmp2_Al_

0x038:	_060LSP__cmp2_Db_

0x040:	_060LSP__cmp2_Dw_

0x048:	_060LSP__cmp2_Dl_