The XL operating system

=======================

I. General structure

The XL OS slightly differs in its structure from what you may know about other

computers. On a scheme it might look as follows:

+=====================+

| Application program | --------------------+

+=====================+                     |

|                                        |

v                                        |

+=======================+       +========================+    |

| Central I/O subsystem | ----> | Parallel I/O subsystem |    |

+=======================+       +========================+    |

|                                 |                     |

v                                 v                     |

+=====================+         +=====================+            |

| File device drivers | ------> | Block I/O subsystem | <----------+

+=====================+         +=====================+

|                        |             |

|                        v             v

|              +===============+  +===============+

|              | Serial I/O    |  | Parallel I/O  |

|              +===============+  +===============+

|                     |                   |

v                     v                   v

+==================================================================+

|                         H A R D W A R E                          |

+==================================================================+

II. The subsystems

The main XL OS subsystem, Central I/O (CIO), located in the ROM kernel, is

responsible for interfacing the application program with most of available

I/O devices. If the application program does not need to access a block I/O

device directly (as disk editors usually do), the CIO is the only part of the

XL OS used by applications for external communication. All CIO devices, such as

screen editor (text console), graphic screen, keyboard, printer, tape recorder,

disk filesystem, are handled as files. The CIO passes application's requests to

appropriate device drivers, those are responsible for performing the required

operation accessing the hardware directly or using the Block I/O subsystem.

The Block I/O subsystem is commonly referenced as SIO, the Serial I/O. This

traditional name is inappropriate, because parallel I/O operations, such as

transfers to/from a hard drive attached to the parallel bus, may be performed

here as well. From the operating system's point of view however, there's no

difference between a parallel or a serial drive, because the particular

operation depends only on the appropriate block device driver.

III. The XL OS ROMs

The Central I/O, the Parallel I/O, the Block I/O subsystems, device drivers for

the console, the graphic screen, the keyboard, the tape recorder, the printer,

the serial device driver are located altogether with interrupt services, two

bitmap fonts, autotest program and floating point calculation package in one

16k ROM memory.

The parallel I/O device driver, if a device like this is present, is located on

an external ROM memory. The XL OS supports a standardized way to initialize the

PIO driver at startup and select it to pass I/O requests when necessary. This

mechanism is _very_ similar to that used for modern plug&play devices on PC.

The external device ROM replaces a part of the XL OS (strictly speaking: the

math package is replaced) for the time when the operation is performed, so no

additional locations from (already short) 64k address space are used.

IV. The DOS

The DOS is loaded to the RAM at startup time. The main difference between the

XL OS and the rest of computers is that the DOS is _NOT_ the main part of the

operating system.

The DOS generally consists of two main parts. The first is the resident

filesystem driver registered in the CIO as responsible for disk operations the

same way as other CIO devices are installed. The filesystem driver communicates

with physical disk drives using the Block I/O subsystem located in the ROM

kernel.

The other part of the DOS is so called DUP, the Disk Utility Package. It's an

application program running as an operating system shell. Using the DUP, the

user can perform specific operations referring to the disks, such as program

execution, file deletion, renaming etc. The DUP is interfaced to the resident

part of the DOS via standardized CIO calls.

It is worth mentioning, that usage of the DOS is not necessary. If the 8-bit

system has no disk drive (using a tape recorder as a storage device), the DOS

is not needed: if the XL OS fails to boot from a floppy, it attempts to execute

the built-in Atari BASIC or external ROM cartridge. If this also fails, the

internal autotest program is called.