The 6502 processor

==================

After the emulator is aborted, an alertbox will appear providing some debug

information. Essentially, it's the current CPU (6502) state. The following

information is displayed:

A - accumulator value

X - index register X value

Y - index register Y value

S - stack pointer value. Default is $FF (empty stack)

Flag - processor status register

PC - program counter

Opcode - 6502 instruction in execution

CPU time - the time spent emulating 6502 in seconds.

The A, X and Y indicate the state of internal registers. See the end of this

text for mode detailed info. The Opcode displays a hex code for an instruction,

that caused the emulator to stop running (f.e. an illegal instruction was

encountered) or an instruction, that would be executed next, if the emulator

wasn't stopped by the user (F5 key). In both cases the PC points to this opcode.

The processor status register (P) contains flags indicating a result of last

executed instruction. From the left, bits are as follows:

N - negative, last operation set the highest (7) bit of a register.

V - overflow, last operation changed the 6th bit of a register

* - this bit is unused, should be always set.

B - BRK instruction was executed

D - decimal mode for integer arithmetics

I - interrupts (IRQ line) are disabled.

Z - zero, last operation zeroed a register

C - carry, last addition produced a result bigger than 255 in binary mode or

bigger than 99 in decimal mode. For subtraction, if the result was less

than a zero, this bit is cleared (it's borrowed).

The first difference between the 6502 and what you may know from Motorola

processors is, that the P register in the 6502 indicates the *processor*

status. That means, that this register will be altered by any calculations done

internally by the CPU as well as by any data loaded to a CPU register from the

memory, but it is NOT altered while writing data from a register to the memory.

For example, if you load a zero to the accumulator, the Z bit will be set. If

you load a $01 to the X register, the Z bit will be cleared. But if you store

the (previously zeroed) accumulator to the memory, the Z bit remains cleared.

Just because this operation doesn't alter the internal processor state.

The next difference is the C (carry) bit function. First of all, it's functions

for addition and for subtraction are different. Addition sets the C if the last

result was bigger than it could fit on 8 bits, while subtraction *clears* this

bit if the result was less than a zero (also comparisons clear the C that time).

The fourth big difference between 6502 and 68k series is that all operations

affect only flags referencing to results they can produce. That means, for

example, that logical operations like AND, OR, EOR affect NZ only, because such

an instruction simply can't cause an overflow, no result can be bigger than 255

or less than a zero.

The last big difference is that all 6500 processors are little endian, while

all 680x0 chips are big endian. This refers to the format of integer

representation. All little endians (6500, Z-80, 80x86) place the low

significant byte of an integer at the first position, while all big endian

processors place there the most significant byte.

The 6502 programming model provides six internal registers listed

above. All of them, except for 16-bit program counter, are 8-bit

wide. The accumulator is a general purpose and arithmetic register,

it accumulates results for all arithmetic and logical operations by

default. The X and Y registers are auxiliary, they're used as 8-bit

counters and indexes for indexed addressing modes. The processor has

no Motorola style address registers.

In addition to that, the CPU has an ability to access the first 256

bytes of RAM (so called "zero-page") using 8-bit addresses. The zero

page can be used by the processor as 256 bytes of fast access memory

to replace a larger number of internal registers. Zero page addresses

can be used as fast 8-bit variables or 16-bit pointers. About a half

of available addressing modes use the zero page to access the whole

64k of memory.

Thanks to this architecture, which allowed to simplify the internal

CPU structure a whole lot, the 6502 is a very efficient processor.

Average access to an internal register is 2 cycles, zero page access

3 cycles and main memory access 4 cycles long. Due to that a 2 MHz

6502 is much faster than a 4 MHz Zilog Z-80 or a 5 MHz 8086 (!).