Various Atari 8-bit system information

--------------------------------------

This file contains several information those aren't necessary to use

the EmuXL. However, it may help you to understand the 8-bit system.

Read it, if you're interested.

The XL operating system

=======================

I. General structure

The XL OS slightly differs in its structure from what you may know about other

computers. On a scheme it might look as follows:

+=====================+

| Application program | --------------------+

+=====================+                     |

|                                        |

v                                        |

+=======================+       +========================+    |

| Central I/O subsystem | ----> | Parallel I/O subsystem |    |

+=======================+       +========================+    |

|                                 |                     |

v                                 v                     |

+=====================+         +=====================+            |

| File device drivers | ------> | Block I/O subsystem | <----------+

+=====================+         +=====================+

|                        |             |

|                        v             v

|              +===============+  +===============+

|              | Serial I/O    |  | Parallel I/O  |

|              +===============+  +===============+

|                     |                   |

v                     v                   v

+==================================================================+

|                         H A R D W A R E                          |

+==================================================================+

The built-in Atari BASIC interpreter is considered an application

program. Neither the XL OS nor other applications do depend on its

presence or not.

II. The subsystems

The main XL OS subsystem, Central I/O (CIO), located in the ROM

kernel, is responsible for interfacing the application program with

most of available I/O devices. If the application program does not

need to access a block I/O device directly (as disk editors usually

do), the CIO is the only part of the XL OS used by applications for

external communication. All CIO devices, such as screen editor (text

console), graphic screen, keyboard, printer, tape recorder, disk

filesystem, are handled as files. The CIO passes application's

requests to appropriate device drivers, those are responsible for

performing the required operation accessing the hardware directly or

using the Block I/O subsystem.

The Block I/O subsystem is commonly referenced as SIO, the Serial

I/O. This traditional name is inappropriate, because parallel I/O

operations, such as transfers to/from a hard drive attached to the

parallel bus, may be performed here as well. From the operating

system's point of view however, there's no difference between a

parallel or a serial drive, because the particular operation depends

only on the appropriate block device driver.

III. The XL OS ROMs

The Central I/O, the Parallel I/O, the Block I/O subsystems, device

drivers for the console, the graphic screen, the keyboard, the tape

recorder, the printer, the serial device driver are located

altogether with interrupt services, two bitmap fonts, autotest

program and floating point calculation package in one 16k ROM memory.

The parallel I/O device driver, if a device like this is present, is

located on an external ROM memory. The XL OS supports a standardized

way to initialize the PIO drivers (number up to 8) at startup and

select it to pass I/O requests when necessary. This mechanism is

_very_ similar to that used for modern plug&play devices on PC. The

external device ROM replaces a part of the XL OS (strictly speaking:

the math package is replaced) for the time when the operation is

performed, so no additional locations from (already short) 64k

address space are used.

IV. The DOS

The DOS is loaded to the RAM at startup time. The main difference

between the XL OS and the rest of computers is that the DOS is _NOT_

a main part of the operating system.

The DOS generally consists of two main parts. The first is a resident

filesystem driver registered in the CIO as responsible for disk

operations the same way as other CIO devices are installed. The

filesystem driver communicates with physical disk drives using the

Block I/O subsystem located in the ROM kernel.

The other part of the DOS is so called DUP, the Disk Utility Package.

It's an application program running as an operating system shell.

Using the DUP, the user can perform specific operations referring to

the disks, such as program execution, file deletion, renaming etc.

The DUP is interfaced to the resident part of the DOS via

standardized CIO calls.

It is worth mentioning, that usage of the DOS is not necessary. If

the 8-bit system has no disk drive (using a tape recorder as a

storage device), the DOS is not needed: if the XL OS fails to boot

from a floppy, it attempts to execute the built-in Atari BASIC or

external ROM cartridge. If this also fails, the internal autotest

program is called.

The 6502 processor

==================

I. Debug info

After the emulator is aborted, an alertbox may appear providing some

debug information. Essentially, it's the current CPU (6502) state. The

following information is displayed:

A - accumulator value

X - index register X value

Y - index register Y value

S - stack pointer value. Default is $FF (empty stack)

Flag - processor status register

PC - program counter

Opcode - 6502 instruction in execution

CPU time - the time spent emulating 6502 in seconds.

The A, X and Y indicate the state of internal registers. See further

parts of this text for more detailed info. The Opcode displays a hex

code for an instruction, that caused the emulator to stop running (e.g.

an illegal instruction was encountered) or an instruction, that would

be executed next, if the emulator was stopped by the user (F5 key). In

both cases the PC points to this opcode.

The opcode field is followed by a three character mnemonic code for the

instruction. It is only an approximate information, because the

addressing mode is not displayed. The whole 65c02 instruction table is

covered, illegal opcodes are displayed as "ILL".

The processor status register (P) contains flags indicating a result of

last executed instruction. From the left, bits are as follows:

N - negative.

V - overflow.

* - unused.

B - break.

D - decimal.

I - interrupt.

Z - zero.

C - carry.

The N flag is set when the last operation produced a result from the

range $80-$FF. May be also set by BIT (bit test) instruction; it

means this time, that the tested operand is negative.

The V bit gets set, when an addition or a subtraction produced a

signed result that doesn't fit on 8-bits (i.e. exceeding the range -

127/+128). May be also set by the BIT instruction mentioned above,

what means that the operand is bigger than $3f (i.e. bit 6 is set).

The unused flag, referenced as `*', should be always set to 1, as in

the original 65c02 processor (the state of this flag is undefined in

the `normal', NMOS 6502). If the state of this bit is not a 1, it would

indicate a bug in the emulator.

The B bit indicates, that the processor encountered and executed the

software interrupt instruction BRK, what generated an IRQ (bit I gets

affected). The B bit has an informative purpose, it allows the OS to

differentiate the BRK interrupt from IRQs coming from the hardware.

The D bit, when set, switches the ALU (arithmetical-logical unit)

from binary to decimal mode.

The I bit, when set, means that the processor doesn't accept

(ignores) IRQ signals. It may happen either when the SEI instruction

was executed or (more often) while the processor is servicing an

interrupt.

The Z indicates that the last operation produced a zero result.

However, it is valid only for binary mode of the ALU.

The C indicates that last addition produced a result bigger than 255

in binary mode or bigger than 99 in decimal mode. For subtraction, if

the result was less than a zero, this bit is cleared (it's borrowed).

II. Differences and similarities between 6502 and 68000.

The first difference between the 6502 and what you may know from

Motorola processors is, that the P register in the 6502 indicates the

*processor* status. That means, that this register will be altered by

any calculations done internally by the CPU as well as by any data

loaded to a CPU register from the memory, but it is NOT altered while

writing data from a register to the memory. For example, if you load a

zero to the accumulator, the Z bit will be set. If you load a $01 to

the X register, the Z bit will be cleared. But if you store the

(previously zeroed) accumulator to the memory, the Z bit remains

cleared. Just because this operation doesn't alter the internal

processor state.

The next difference is the C (carry) bit function. First of all, it's

functions for addition and for subtraction are different. Addition sets

the C if the last result was bigger than it could fit on 8 bits, while

subtraction *clears* this bit if the result was less than a zero (also

comparisons clear the C that time).

The third big difference between 6502 and 68k series is that all

operations affect only flags referencing to results they can produce.

That means, for example, that logical operations like AND, OR, EOR

affect NZ only, because such an instruction simply can't cause an

overflow, no result can be bigger than 255 or less than a zero. So,

logical operations or data movements don't set or clear the C flag.

The fourth difference is, that the 6500 series uses unsigned

arithmetics in effective address calculations, while Motorola tends to

sign-extend addresses. The darkside of this 6500 behaviour is that you

can't use (for example) negative indexes counting data back from the

base address, but at the other hand the 6500 method looks more

consistent with normal arithmetics.

The last big difference is that all 6500 processors are little endian,

while all 680x0 chips are big endian. This refers to the format of

integer representation. All little endians (6500, Z-80, 80x86) place

the low significant byte of an integer at the first (i.e. low)

position, while all big endian processors place there the most

significant byte. Of course, it is worth to remember, that the 6502 is

a pure 8-bit CPU, so all single operations are 8-bit wide, unlike in

the 32-bit Motorola 68k.

Aside from that, both processors look quite similar and if one is an

experienced 6502 programmer (in assembly language, I mean), learning

m68k assembler is quite easy. Both processors have small instruction

set and quite big amount of addressing modes. Also a lot of mnemonics

are similar or identical.

II. Programming model

The 6502 programming model provides six internal registers listed

above. All of them, except for 16-bit program counter, are 8-bit wide.

The accumulator is a general purpose and arithmetic register, it

accumulates results for all arithmetic and logical operations by

default. The X and Y registers are auxiliary, they're used as 8-bit

counters and indexes for indexed addressing modes. The processor has no

Motorola style address registers.

In addition to that, the CPU has an ability to access the first 256

bytes of RAM (so called "zero-page") using 8-bit addresses. The zero

page can be used by the processor as 256 bytes of fast access memory to

replace a larger number of internal registers. Zero page addresses can

be used as fast 8-bit variables or 16-bit pointers. About a half of

available addressing modes use the zero page to access the whole 64k of

memory.

Thanks to this architecture, which allowed to simplify the internal CPU

structure a whole lot, the 6502 is a very efficient processor. Average

access to an internal register is 2 cycles, zero page access 3 cycles

and main memory access 4 cycles long. Due to that a 2 MHz 6502 is much

faster than a 4 MHz Zilog Z-80 or a 5 MHz 8086 (!).

III. Versions

There are three main versions of the 6502. The older, NMOS version,

referenced as 6502, is used in 8-bit Ataris. Slightly modified version,

referenced as 6510, is the heart of the main competitor to 8-bit Ataris

- the Commodore C-64. CMOS version of the 6502, known as 65c02, was

used in Apple computers.

The 65c02 is backward compatible with the NMOS 6502, so that about 99%

of the software written for the 6502 works on a 65c02 system. Only some

software that relies on NMOS 6502 specific features (design bugs), will

fail.

First of all, the NMOS version used to take undefined action

encountered an illegal opcode. Most of them don't make anything useful

or anything at all. Some of them may even lock the CPU so that it can't

return to normal operation without a reset (even NMI signals don't take

any effects). However, some of the illegal instructions, such as LAX

(LDA/TAX), ANX (AND/TAX), DCP (DEC/CMP) or ISB (INC/SBC) may look

useful and migh have been used by software designers. Of course, such a

software will fail after an upgrade to 65c02, where all undefined

opcodes are considered NOPs.

Another reason for a software to fail is a bug in the indirect JMP

instruction. If the jump address is located on a page boundary so that

the lower byte of the destination address is located at the last byte

of a page, the higher byte will be fetched from the first byte of this

page instead of the first byte of the next page. For example, for an

instruction like JMP ($05FF), the destination address will be fetched

from $05FF/$0500 instead of $05FF/$0600. The 65c02 corrects this bug.

The third big difference between NMOS and CMOS version is that the Z

(zero) bit in the processor status register is set properly by the CMOS

CPU after decimal calculations. This NMOS "bug" was caused by the way

the 6502 does all decimal operations. Namely, in the decimal mode the

CPU performs addition and subtraction in two steps: first it performs a

normal binary operation as usual, then does a decimal correction. On

the NMOS version the Z flag state reflects the first, binary stage of

the calculation, so it rarely makes any sense considering the decimal

result. It is quite unlikely however that this feature of the CMOS CPU

might cause the older software to fail.

Apart from that, the CMOS version features some more instructions and

addressing modes, those are missing in the NMOS version.

IV. EmuXL's processor

The Emu emulates a CPU that mostly has identical behaviour to the CMOS

6502 version (65c02). The main exception is the behaviour of the Z flag

after decimal operations - here it behaves like the NMOS version. At

the other hand, most of extended 65c02 instructions are implemented, so

CPU type tests may display various results depending on what features

they use to determine the processor type. NMOS illegal instructions

aren't (yet) implemented.

The EmuXL timings do not reflect the timings on the original computer.

The Emu wasn't ever planned as a full sync emulator, the main concern

was to gain as much speed as possible out of the host processor

(68030). So, there are less or more important differences between the

Emu and an 8-bit Atari.

1) the lower is the screen resolution, the CPU emulator runs slower.

An original computer runs faster, when the screen resolution is

lower.

2) instruction execution is not synchronized either, instructions are

just executed as fast as possible. So, for example, the TXS

(transfer X into stack pointer) is faster than TXA (transfer X

into accumulator), though on a true 6502 they both take the same

time.

3) instruction execution time may (= does) depend on the memory area

the instruction accesses. For a STA absolute timings will be

different for RAM, ROM and I/O access. Also different I/O

registers will cause different execution timings for the same

instructions.

4) a sequence of the same instructions will be executed faster, than

a sequence of different instructions, for example:

STA $0600

STA $0601

will be executed faster than:

STA $0600

STX $0601

though on an original 6502 both take fragments of code take

exactly the same time. This is an effect of the 68030 on-chip

caches.

However, regardless of the timing details, the main scheme of the

6502 timings is kept. Namely, when accessing data, the fastest are

registers, then the zero page at the second place, and the rest of

the memory at the third place.

The ANTIC

=========

ANTIC stands for "Alpha-Numeric Television Interface Controller". It is

the main (out of two) video chip in the 8-bit Atari computer series.

First of all, the ANTIC is, without any doubt, the most advanced and

powerful video chip ever built for popular 8-bit computers. Though its

display modes aren't too impressive today, its flexibility outperforms

even the Falcon video shifter (not to mention other chipsets, perhaps

except for Amiga).

The ANTIC is a microprocessor - it executes programs stored in the

memory. Such a program, called Display List, defines all basic screen

settings, such as display mode, screen memory location, active scrolls

and interrupts etc. independently for each horizontal line. Any part of

the Display List and any part of the screen memory (as small as 1

scanline) can be physically located anywhere in the memory in any

order. That means, that creating special effects, those are very hard

to get on other computers, like flipping the display upside down,

horizontal mirroring, independent scrolling, or generally customizing

the screen memory for programmer's requirements is very easy and

doesn't need any special techniques like using interrupts on the main

CPU or something. ANTIC can do many things without arranging the 6502

(an obvious example is displaying an animation while the 6502 is

totally halted).

Besides, the ANTIC has 14 registers mapped to the I/O area. They allow

to control further settings, such as screen width, special text effects

etc. Two registers (one 16-bit, the other 8-bit wide) control the

location of the Display List and the current character set. There are

more registers, but there's no place here for a detailed description.

Just to mention, as a curiosity, a register, called WSYNC, that allows

the programmer to request a halt on the main CPU until a horizontal

sync signal occurs.