Engineering Hardware Specification

of the

Atari ST Computer System

The Atari Corporation

7 January 1986

THE SCOPE OF THIS DOCUMENT is limited to a  description

of  the  ST  computer  system  hardware  characteristics and

idiosyncrasies.  This document does not provide  a  detailed

description  of ST component parts, peripheral devices, dev-

ice protocols, or software.   References  are  provided  for

detailed inquiry.

_1.  _S_y_s_t_e_m _A_r_c_h_i_t_e_c_t_u_r_e

----------                    ----------

|Graphics  |                  |Music     |

|Subsystem |                  |Subsystem |

----------                    ----------

|                            |

|         ----------         |

--------| Main     |--------

| System   |

----------

|

|         ----------

The ST                --------|Device    |

|Subsystems|

----------

The hardware architecture of the Atari  Corporation  ST

(Sixteen/Thirty-two) computer system consists of a main sys-

tem, a graphics subsystem, a music  subsystem,  and  several

device  subsystems (most of the device subsystems require ST

resident intelligence).  The ST is based on the  MC68000  16

bit  data/24  bit  address  microprocessor  unit  capable of

directly accessing up to 16 Mbytes of ROM  and  RAM  memory.

Hardware features of the ST computer system include:

Main System

o  16 bit data/24 bit address 8 MHz microprocessor unit

o  192 Kbyte ROM, cartridge expandable to 320 Kbyte

o  512 Kbyte RAM or 1 Mbyte RAM

o  direct memory access support

Graphics Subsystem

o  32 Kbyte BitMap video display memory (from above)

o  320 x 200 pixel, 16 color palette from 512 selections

o  640 x 200 pixel, 4 color palette from 512 selections

o  640 x 400 pixel, monochrome

Music Subsystem

o  programmable sound synthesizer

o  musical instrument network communication

Atari Corp Confidential

Device Subsystems

o  intelligent keyboard

o  two button mouse

o  RGB color and monochrome monitor interfaces

o  printer parallel interface

o  RS232 serial interface

o  MIDI musical instrument interface

o  on board floppy disk controller and DMA interface

o  hard disk drive DMA interface

The following is a  simplified  hardware  system  block

diagram of the Atari ST:

Atari Corp Confidential

----- Atari ST System Block Diagram ---------------

---------------

| MC68000 MPU   |<--

|               |   |

---------------    |

|                           ----------

|<------------------------>|192 Kbyte |<--->EXPAND

---------|------------------------->| ROM      |

|         |                           ----------

|         |                           ----------

|         |                          |512K or 1M|

|         |                       -->| byte RAM |<--

----------        |        ----------    |    ----------    |

| Control  |<----->|<----->| Memory   |<--                   |

| Logic    |-------|------>|Controller|<--                   |

----------        |        ----------    |    ----------    |

|||||            |        ----------     -->| Video    |<--  RF MOD

|||||            |<----->| Buffers  |<----->| Shifter  |---->RGB

|||||            |       |          |        ----------      MONO

|||||            |        ----------

|||||            |        ----------         ----------

|||||            |<----->| MC6850   |<----->| Keyboard |<--->IKBD

|||| ------------|------>| ACIA     |       | Port     |

||||             |        ----------         ----------

||||             |        ----------         ----------

||||             |<----->| MC6850   |<----->| MIDI     |---->OUT/THRU

||| -------------|------>| ACIA     |       | Ports    |<----IN

|||              |        ----------         ----------

|||              |        ----------         ----------

|||              |<----->| MK68901  |<----->| RS232    |<--->MODEM

|| --------------|------>| MFP      |<--    | Port     |

||               |        ----------    |    ----------

||               |                      |    ----------

||               |                       ---| Parallel |<--->PRINTER

||               |                       -->| Port     |

||               |        ----------    |    ----------

||               |<----->| YM-2149  |<--     ----------

| ---------------|------>| PSG      |------>| Sound    |---->AUDIO

|                |       |          |---    | Channels |

|                |        ----------    |    ----------

|                |                      |    ----------

|                |        ----------     -->| Floppy   |<--->FLOPPY

|                |<------| WD1772   |<----->|Disk Port |     DRIVE

|                |    -->| FDC      |        ----------

|                |   |    ----------

|                |   |    ----------         ----------

|                |    -->| DMA      |<----->|Hard Disk |<--->HARD

|                |<----->|Controller|       | Port     |     DRIVE

----------------------->|          |        ----------

----------

Atari Corp Confidential

_2.  _M_a_i_n _S_y_s_t_e_m

----------                    ----------

|          |                  |          |

|          |                  |          |

----------                    ----------

|                            |

|         ----------         |

--------| Main     |--------

| System   |

----------

|

|         ----------

--------|          |

|          |

----------

The main system includes the microprocessor unit,  main

memory  (both  ROM  and  RAM),  and general purpose DMA con-

troller.  The main system is  limited  to  memory  expansion

only and should not be considered an open system (except via

the high speed device interfaces).

_2._1.  _M_i_c_r_o_p_r_o_c_e_s_s_o_r _U_n_i_t

The ST computer system is based on an 8 MHz MC68000  16

bit  data/24 bit address microprocessor unit (with an inter-

nal 32 bit architecture).  Some features of the MC68000 are:

eight  32 bit data registers, nine 32 bit address registers,

a 16 Mbyte direct addressing  range,  14  addressing  modes,

memory  mapped  I/O,  five  data types, and a 56 instruction

set.  The MPU is directly  supported  by  an  MK68901  Multi

Function Peripheral providing general purpose interrupt con-

trol and timers, among other things.

_2._2.  _M_e_m_o_r_y _C_o_n_f_i_g_u_r_a_t_i_o_n

The configuration of main memory consists  of  five  64

Kbyte sets of ROM (standard set0 to set2, expansion set3 and

set4) and one configurable  bank  (standard  bank0)  of  128

Kbyte, 512 Kbyte, or 2 Mbyte RAM.  The configuration of main

memory ROM is ascertained through  software  identification.

The  configuration  of  main  memory RAM is achieved via the

programming   of   the   Memory    Configuration    Register

(read/write,  reset:  all zeros).  RAM configuration must be

asserted during the first steps of the power up sequence and

can  be  determined by using the following shadow test algo-

rithm:

Atari Corp Confidential

START   o  write 0x000a (2 Mbyte, 2 Mbyte) to the Memory

Configuration Register.

BANK0   o  write Pattern to 0x000000 - 0x0001ff.

o  read Pattern from 0x000200 - 0x0003ff.

o  if Match, then Bank0 contains 128 Kbyte; goto BANK1.

o  read Pattern from 0x000400 - 0x0005ff.

o  if Match, then Bank0 contains 512 Kbyte; goto BANK1.

o  read Pattern from 0x000000 - 0x0001ff.

o  if Match, then Bank0 contains 2 Mbyte; goto BANK1.

o  panic:  RAM error in Bank0.

BANK1   o  write Pattern to 0x200000 - 0x2001ff.

o  read Pattern from 0x200200 - 0x2003ff.

o  if Match, then Bank1 contains 128 Kbyte; goto FIN.

o  read Pattern from 0x200400 - 0x2005ff.

o  if Match, then Bank1 contains 512 Kbyte; goto FIN.

o  read Pattern from 0x200000 - 0x2001ff.

o  if Match, then Bank1 contains 2 Mbyte; goto FIN.

o  note:  Bank1 nonexistent.

FIN     o  write Configuration to the Memory Configuration

Register.

o  note Total Memory Size (Top of RAM) for future

reference.

RAM memory access cycles are  interleaved  between  the

microprocessor  unit and the video controller, thus allowing

video display memory to reside efficiently as part  of  main

memory (the MPU still maintains full memory bandwidth).

_2._3.  _D_i_r_e_c_t _M_e_m_o_r_y _A_c_c_e_s_s

A direct main memory RAM access channel  is  shared  to

provide  support  for  both low speed (250 to 500 Kbits/sec)

and high speed (up to 12 Mbits/sec) 8 bit  device  controll-

ers.   The  base address for the DMA read or write operation

is loaded into the DMA Base  Address  and  Counter  Register

(read/write,  reset:  all  zeros).   Since  only one counter

register and channel is provided, only one DMA operation can

be executed at a time.

The actual DMA operation is performed through a 32 byte

FIFO  programmed  via  the  DMA  Mode Control Register (word

access write only, reset: not affected) and DMA Sector Count

Register  (word  access  write only, reset: all zeros).  The

progress, success, or failure of a DMA operation is reported

through  the  DMA  Status  Register  (word access read only,

reset: one) which is cleared by toggling Write/_Read in  the

DMA Mode Control Register.

Atari Corp Confidential

Bus accesses are granted  to  the  DMA  controller  and

MC68000  MPU  on  an  egalitarian  first  come, first served

basis.  The access remains in effect until an  operation  is

complete or until control is otherwise relinquished.

Atari Corp Confidential

_3.  _G_r_a_p_h_i_c_s _S_u_b_s_y_s_t_e_m

----------                    ----------

|Graphics  |                  |          |

|Subsystem |                  |          |

----------                    ----------

|                            |

|         ----------         |

--------|          |--------

|          |

----------

|

|         ----------

--------|          |

|          |

----------

The basic components  of  the  graphics  subsystem  are

video  display memory and the video controller.  The defini-

tions of BitMap,  BitBlk,  BitBlt,  coordinate  system,  and

color  model are left to higher logical (software) levels of

system specification.

_3._1.  _V_i_d_e_o _D_i_s_p_l_a_y _M_e_m_o_r_y

Video display memory is configured as n logical  planes

interwoven  by  16  bit words into contiguous memory to form

one 32 Kbyte (actually 0x7d00) physical  plane  starting  at

any 256 byte half page boundary (in RAM only).  The starting

address of display  memory  is  placed  in  the  Video  Base

Address  Register  (read/write,  reset:  all zeros) which is

then loaded into the Video Address  Counter  Register  (read

only, reset: all zeros) and incremented.  The following is a

diagram of possible physical configurations of video display

memory:

--------

16 bit word   |        |

-------- -------- -------- -------- --------

4 plane       |plane 0 |plane 1 |plane 2 |plane 3 |plane 0 |

-------- -------- -------- -------- --------

2 plane       |plane 0 |plane 1 |plane 0 |plane 1 |plane 0 |

-------- -------- -------- -------- --------

1 plane       |plane 0 |plane 0 |plane 0 |plane 0 |plane 0 |

-------- -------- -------- -------- --------

Atari Corp Confidential

Display memory resides as part of main memory  and  has

an identical bit, byte, and word arrangement with the physi-

cal screen origin located at top left (bit 15):

high     low

-------- --------

word 0        | byte 0 | byte 1 |

-------- --------

fedcba98 76543210

_3._2.  _V_i_d_e_o _C_o_n_f_i_g_u_r_a_t_i_o_n

The ST possesses three modes  of  video  configuration:

320  x  200  resolution  with 4 planes, 640 x 200 resolution

with 2 planes, and 640 x 400 resolution with 1  plane.   The

modes  are  set through the Shift Mode Register (read/write,

reset: all zeros).  A sixteen word color lookup  palette  is

provided  with  nine  bits  of color per entry.  The sixteen

Color Palette Registers (read/write,  reset:  not  affected)

contain  three  bits  of red, green, and blue aligned on low

nibble boundaries.  Eight intensity  levels  of  red,  eight

intensity  levels  of  green,  and eight intensity levels of

blue produce a total of 512 possible colors.

In 320 x 200 4 plane mode all  sixteen  palette  colors

can  be  indexed,  while  in 640 x 200 2 plane mode only the

first four palette entries are applicable.   In  640  x  400

monochrome mode the color palette is bypassed altogether and

is instead provided with an inverter for inverse video  con-

trolled  by  bit 0 of palette color 0 (normal video is black

0, white 1).  Color palette memory is arranged the  same  as

main  memory.   Palette  color  0  is  also used to assign a

border color while in a  multi-plane  mode.   In  monochrome

mode the border color is always black.

The general flow of the video controller is as follows:

BitMap  planes are taken a word at a time from video display

memory and placed in the video shift register where one  bit

from  each  plane is shifted out and collectively used as an

index (plane 0 is the least  significant  bit)  to  a  color

lookup  palette  entry which is supplied to 3 bit digital to

analog converters to produce RGB output.  The following is a

block diagram of the video controller:

Atari Corp Confidential

----- Video Controller Block Diagram ---------------

--------          --------        --------   lsb  --------

| 3      |---- -->|0 1 2 3 |----->|      0 |----->| 16 x 9 |-

--------  |    |   |        |      |      1 |----->| Lookup |-|->R

| 2      |------    |        |      |      2 |----->|        |-

--------  | |    |   |        |      |      3 |----->|        |-

| 1      |--------    |        |      |        |--    |        |-|->G

--------  | |-     |    --------        --------   |   |        |-

| 0      |----------                                |   |        |-

|        | |-                                       |   |        |-|->B

|        | |                                        |   |        |-

|        |-                                         |    --------

|        |                                          |    --------

--------                                            -->|Inverter|->MONO

--------

Logical BitMap          Video Display   Video Shift     Color Palette

Planes                  Memory          Register        and 3 Bit DACs

Atari Corp Confidential

_4.  _M_u_s_i_c _S_u_b_s_y_s_t_e_m

----------                    ----------

|          |                  |Music     |

|          |                  |Subsystem |

----------                    ----------

|                            |

|         ----------         |

--------|          |--------

|          |

----------

|

|         ----------

--------|          |

|          |

----------

The ST music subsystem is composed  of  a  programmable

sound synthesizer and a musical instrument serial interface.

The interface provides high speed  serial  communication  of

musical data to and from more sophisticated synthesizer dev-

ice subsystems.

_4._1.  _S_o_u_n_d _S_y_n_t_h_e_s_i_z_e_r

The YM-2149 Programmable Sound Generator produces music

synthesis,  sound effects, and audio feedback (eg alarms and

key clicks).  With an applied clock input of 2 MHz, the  PSG

is  capable  of providing a frequency response range between

30 Hz (audible) and 125 KHz (post-audible).   The  generator

places  a  minimal  amount  of processing burden on the main

system (which acts as the sequencer) and has the ability  to

perform  using  three independent voice channels.  The three

sound channel outputs are mixed, along with  Audio  In,  and

sent  to  an external television or monitor speaker (the PSG

has built in digital to analog converters).

The sound generator's internal registers  are  accessed

via  the  PSG  Register  Select Register (write only, reset:

registers all zeros).  The tone generator registers  control

a  basic square wave while the noise generator register con-

trols a frequency modulated square  wave  of  pseudo  random

pulse  width.   Tones and noise can be mixed over individual

channels by using the mixer control register.  The amplitude

registers allow the specification of a fixed amplitude or of

a variable amplitude when used with the envelope  generator.

The  envelope  generator  registers  permit  the  entry of a

skewed attack-decay-sustain-release envelope in the form  of

a continue-attack-alternate-hold envelope.

Atari Corp Confidential

_4._2.  _M_u_s_i_c_a_l _I_n_s_t_r_u_m_e_n_t _C_o_m_m_u_n_i_c_a_t_i_o_

n

The Musical Instrument Digital Interface (MIDI)  allows

the  integration of the ST with music synthesizers, sequenc-

ers, drum boxes, and other devices  possessing  MIDI  inter-

faces.   High  speed  (31.25  Kbaud) serial communication of

keyboard and program information is provided by  two  ports,

MIDI  OUT  and  MIDI IN (MIDI OUT also supports the optional

MIDI THRU port).

The MIDI bus permits up to 16 channels in one of  three

network  addressing modes:  Omni (all units addressed simul-

taneously, power up  default),  Poly  (each  unit  addressed

separately),   and   Mono   (each   unit   voice   addressed

separately).  Information is communicated via five types  of

data  format  (data  bytes, most significant bit:  status 1,

data 0) which are prioritized from  highest  to  lowest  as:

System  Reset  (default  conditions,  should  not be sent on

power up to avoid deadlock), System Exclusive  (manufacturer

unique  data:   Sequential  Circuits,  Kawai,  Roland, Korg,

Yamaha), System Real Time (synchronization),  System  Common

(broadcast),  and  Channel  (note  selections, program data,

etc).

Atari Corp Confidential

_5.  _D_e_v_i_c_e _S_u_b_s_y_s_t_e_m_s

----------                    ----------

|          |                  |          |

|          |                  |          |

----------                    ----------

|                            |

|         ----------         |

--------|          |--------

|          |

----------

|

|         ----------

--------|Device    |

|Subsystems|

----------

The ST supports seven device subsystems:   an  intelli-

gent  keyboard,  video  interface, parallel interface, RS232

interface, MIDI interface, floppy disk interface,  and  hard

disk   interface.    Included  with  each  device  interface

description is a port pin assignment chart with the  ST  and

programmable  signals justified left (pins that are not con-

nected are not shown).  The connector  type  on  the  ST  is

shown  above  each  pin  list with an S designating a female

socket and a P designating a male plug.

_5._1.  _I_n_t_e_l_l_i_g_e_n_t _K_e_y_b_o_a_r_d

The Atari Intelligent Keyboard (ikbd) transmits encoded

make/break   key   scan   codes  (with  two  key  rollover),

mouse/trackball data, joystick data, and time of  day.   The

ikbd  receives commands as well, with bidirectional communi-

cation controlled on the ST side by an  MC6850  Asynchronous

Communications  Interface Adapter supplied with transmit and

receive clock inputs of 500 KHz.  The data transfer rate  is

a constant 7812.5 bits/sec which can be generated by setting

the ACIA Counter Divide Select to divide by  64.   All  ikbd

functions  such  as  key  scanning,  mouse tracking, command

parsing, etc. are performed by a 1 MHz HD6301V1 8 bit Micro-

computer Unit.

The ikbd is equipped with a combination  mouse/joystick

port  and  a joystick only port.  The Atari Two Button Mouse

is a mechanical, opto-mechanical, or optical mouse with  the

following  minimal performance characteristics: a resolution

of 100 counts/inch (4 counts/mm), a maximum velocity  of  10

inches/second  (250  mm/second),  and  a maximum pulse phase

error of 50 percent.  The Atari Joystick is  a  4  direction

Atari Corp Confidential

switch-type  joystick  with  one  fire button.  The ikbd can

report movement using one  of  three  mouse/joystick  modes:

mouse with joystick, disabled mouse with joystick, and joys-

tick with joystick.

----- Mouse/Joystick0 Port Pin Assignments ---------------

ST           DB 9P

----                                    ----

IKBD MATRIX       1 |<--- XB Pulse / Up Switch --------|

IKBD MATRIX       2 |<--- XA Pulse / Down Switch ------|

IKBD MATRIX       3 |<--- YA Pulse / Left Switch ------|

IKBD MATRIX       4 |<--- YB Pulse / Right Switch -----|

IKBD MCU          6 |<--- Left Button / Fire Button ---|

7 |---- Power ---------------------->|

8 |---- Ground ----------------------|

IKBD MCU          9 |<--- Right Button / Joy1 Fire ----|

----                                    ----

Signal Characteristics

mouse pins 1-4          TTL levels.

joystick0 pins 1-4      TTL levels.

pin 6                   TTL levels, closure to ground.

pin 7                   +5 VDC.

pin 9                   TTL levels, closure to ground.

Mouse Phase Directions

POSITIVE RIGHT (UP)             NEGATIVE LEFT (DOWN)

.    -----       -----                  -----       -----

XA (YA)     |     |     |     |                |     |     |     |

|     |     |     |                |     |     |     |

. --       -----       -----      -----       -----       --

.       -----       -----            -----       -----

XB (YB)        |     |     |     |          |     |     |     |

|     |     |     |          |     |     |     |

. -----       -----       --      --       -----       -----

Atari Corp Confidential

----- Joystick1 Port Pin Assignments ---------------

ST           DB 9P

----                                    ----

IKBD MATRIX       1 |<--- Up Switch -------------------|

IKBD MATRIX       2 |<--- Down Switch -----------------|

IKBD MATRIX       3 |<--- Left Switch -----------------|

IKBD MATRIX       4 |<--- Right Switch ----------------|

IKBD MCU          6 |<--- Fire Button -----------------|

7 |---- Power ---------------------->|

8 |---- Ground ----------------------|

----                                    ----

Signal Characteristics

pins 1-4,6              TTL levels.

pin 7                   +5 VDC.

_5._2.  _V_i_d_e_o _I_n_t_e_r_f_a_c_e

The ST video display interface supports low  resolution

(320  x 200 or 640 x 200) television receivers and composite

monitors, low resolution (320 x 200 or 640 x 200) RGB  moni-

tors, and medium resolution (640 x 400) monochrome monitors.

Both NTSC and PAL color encoding standards are available  in

two separate configurations of the video interface hardware.

For United States television  receivers,  the  modulated  RF

display  signal  is provided on two adjacent broadcast chan-

nels:  channel 2 (55.25 MHz) and channel 3 (61.25 MHz).   In

order  to  protect  against  monitor  damage while in medium

resolution monochrome mode, the  Monochrome  Monitor  Detect

signal  is  provided  as  an  interrupt on the MK68901 Multi

Function Peripheral and can be tested to determine the pres-

ence  of  a  monochrome monitor (active low, any change will

generate the interrupt if enabled).

External/Internal syncs and 50/60 Hz  field  rates  are

selected  via the Sync Mode Register (read/write, reset: all

zeros).  The YM-2149 Programmable Sound Generator I/O Port A

provides  a General Purpose Output for use in the control of

functions such as the remote selection of  monitor  external

or  internal  sync.  Two autovector interrupts are generated

to allow software synchronization with horizontal and verti-

cal  blanking  intervals.   The  Horizontal  Blanking (down)

Counter MFP Timer B has an active high input signal and pro-

duces  an  interrupt when the counter times out (Event Count

Mode).   The  horizontal  blanking  counter  actually   uses

display  enable, the first of which occurs at the end of the

Atari Corp Confidential

first display line.  The video display shift mode should  be

selected during the vertical blanking interval.

----- Video Port Pin Assignments ---------------

Television

ST           RCA Pin Jack

----                                    ----

core |---- RF Modulated Video --------->|

shield |---- Ground ----------------------|

----                                    ----

Monitor

ST           Circular DIN 13S

----                                    ----

1 |---- Audio Out ------------------>|

2 |---- Composite Video ------------>|

PSG I/O A         3 |---- General Purpose Output ----->|

MFP               4 |<--- Monochrome Detect -----------|

5 |<--- Audio In --------------------|

6 |---- Green ---------------------->|

7 |---- Red ------------------------>|

8 |---- Peritel Power -------------->|

9 |---- Horizontal Sync ------------>|

10 |---- Blue ----------------------->|

11 |---- Monochrome ----------------->|

12 |---- Vertical Sync -------------->|

13 |---- Ground ----------------------|

----                                    ----

Signal Characteristics

pin 1                   1 VDC peak to peak, 10 Kohm.

pin 2                   1 VDC peak to peak, 75 ohm.

pin 3                   TTL levels, assertion not assigned.

pin 4                   TTL levels, active low,

1 Kohm pullup resistor to +5 VDC.

pin 5                   1 VDC peak to peak, 10 Kohm.

pins 6-7                1 VDC peak to peak, 75 ohm.

pin 8                   +12 VDC pullup.

pin 9                   5 VDC active low, 3.3 Kohm.

pin 10                  1 VDC peak to peak, 75 ohm.

pin 11                  1 VDC peak to peak, 75 ohm.

pin 12                  5 VDC active low, 3.3 Kohm.

_5._3.  _P_a_r_a_l_l_e_l _I_n_t_e_r_f_a_c_e

The ST parallel interface  supports  Centronics  STROBE

from the YM-2149 PSG for data synchronization and Centronics

BUSY to the  MK68901  MFP  (ACKNLG  is  not  supported)  for

handshaking.   Eight  bits  of  read/write  data are handled

through I/O Port B on the PSG at  a  typical  data  transfer

rate of 4000 bytes/second.

----- Parallel Port Pin Assignments ---------------

ST           DB 25S

----                                    ----

PSG I/O A         1 |---- Centronics STROBE ---------->|

PSG I/O B         2 |<--- Data 0 --------------------->|

PSG I/O B         3 |<--- Data 1 --------------------->|

PSG I/O B         4 |<--- Data 2 --------------------->|

PSG I/O B         5 |<--- Data 3 --------------------->|

PSG I/O B         6 |<--- Data 4 --------------------->|

PSG I/O B         7 |<--- Data 5 --------------------->|

PSG I/O B         8 |<--- Data 6 --------------------->|

PSG I/O B         9 |<--- Data 7 --------------------->|

MFP              11 |<--- Centronics BUSY -------------|

18-25 |---- Ground ----------------------|

----                                    ----

Signal Characteristics

pin 1                   TTL levels, active low.

pins 2-9                TTL levels.

pin 11                  TTL levels, active high,

1 Kohm pullup resistor to +5 VDC.

_5._4.  _R_S_2_3_2 _I_n_t_e_r_f_a_c_e

The ST RS232 interface provides voltage level  synchro-

nous  or asynchronous serial communication.  Five EIA RS232C

handshake control signals are supported:   Request  To  Send

and  Data Terminal Ready are transmitted through the YM-2149

PSG I/O Port A while Clear To Send, Data Carrier Detect, and

Ring  Indicator  are  received through the MK68901 MFP.  The

MFP USART transmit and receive clock inputs  are  controlled

by  the  Baud  Rate  Generator MFP Timer D which is supplied

with 2.4576 MHz and can support asynchronous  data  transfer

rates  from 50 to 19200 baud.  One byte transmit and receive

data buffers are managed by the MFP  USART,  which  provides

monitoring of buffer conditions and communication errors.

----- RS232 Port Pin Assignments ---------------

ST           DB 25P

----                                    ----

1 |---- Protective Ground -----------|

MFP               2 |---- Transmitted Data ----------->|

MFP               3 |<--- Received Data ---------------|

PSG I/O A         4 |---- Request To Send ------------>|

MFP               5 |<--- Clear To Send ---------------|

7 |---- Signal Ground ---------------|

MFP               8 |<--- Data Carrier Detect ---------|

PSG I/O A        20 |---- Data Terminal Ready -------->|

MFP              22 |<--- Ring Indicator --------------|

----                                    ----

Signal Characteristics

pins 2-5                RS232C levels.

pins 8,20,22            RS232C levels.

_5._5.  _M_I_D_I _I_n_t_e_r_f_a_c_e

The ST MIDI interface provides current  loop  asynchro-

nous  serial communication controlled by an MC6850 ACIA sup-

plied with transmit and receive clock  inputs  of  500  KHz.

The  data  transfer rate is a constant 31.25 Kbaud which can

be generated by setting the ACIA Counter  Divide  Select  to

divide  by 16.  The MIDI specification calls for serial data

to consist of eight data bits preceded by a  start  bit  and

followed by one stop bit.

----- MIDI Port Pin Assignments ---------------

MIDI OUT/THRU

ST           Circular DIN 5S

----                                    ----

MIDI IN           1 |---- THRU Transmit Data --------->|

2 |---- Shield Ground ---------------|

3 |<--- THRU Loop Return ------------|

MIDI ACIA         4 |---- OUT Transmit Data ---------->|

5 |<--- OUT Loop Return -------------|

----                                    ----

MIDI IN

ST           Circular DIN 5S

----                                    ----

MIDI ACIA         4 |<--- IN Receive Data -------------|

5 |---- IN Loop Return ------------->|

----                                    ----

Signal Characteristics

current loop            5 ma, zero is current on.

_5._6.  _D_i_s_k _D_r_i_v_e _I_n_t_e_r_f_a_c_e

The ST floppy disk drive interface is provided  through

the  DMA  controller  to an on board WD1772 Floppy Disk Con-

troller.  A total of two daisy chained  floppy  disk  drives

(drive  0  or 1) can be supported.  Commands are sent to the

FDC by first writing to the DMA  Mode  Control  Register  to

select  the  FDC  internal command register and then writing

the desired one byte command to the Disk  Controller  Regis-

ter.   The  entire floppy disk DMA read or write sequence is

as follows:

o  select floppy drive 0 or 1 (PSG I/O Port A).

o  select floppy side 0 or 1 (PSG I/O Port A).

o  load DMA Base Address and Counter Register.

o  toggle Write/_Read to clear status (DMA Mode Control Register).

o  select DMA read or write (DMA Mode Control Register).

o  select DMA Sector Count Register (DMA Mode Control Register).

o  load DMA Sector Count Register (DMA operation trigger).

o  select FDC internal command register (DMA Mode Control Register).

o  issue FDC read or write command (Disk Controller Register).

o  DMA active until sector count is zero (DMA Status Register,

do not poll during DMA active).

o  issue FDC force interrupt command on multi-sector transfers

Atari Corp Confidential

except at track boundaries (Disk Controller Register).

o  check DMA error status (DMA Status Register, nondestructive).

The detection of floppy disk removal is  not  supported

in hardware.

The ST hard  disk  drive  interface  is  also  provided

through the DMA controller, however the Atari Hard Disk Con-

troller is off board and is  sent  commands  using  an  ANSI

X3T9.2  SCSI-like (Small Computer Systems Interface) command

descriptor block protocol.  The Atari  Hard  Disk  Interface

(AHDI)  supports  a minimal subset of SCSI commands (Class 0

OpCodes), which are dispatched using the following fixed six

byte  Atari  Computer System Interface (ACSI) command packet

format:

----- ACSI Command Descriptor Block ---------------

Byte 0  |xxxxxxxx|

||||||||

||| -------- Operation Code

----------- Controller Number

Byte 1  |xxxxxxxx|

||||||||

|||--------- Block Address High

----------- Device Number

Byte 2  |xxxxxxxx|

||||||||

----------- Block Address Mid

Byte 3  |xxxxxxxx|

||||||||

----------- Block Address Low

Byte 4  |xxxxxxxx|

||||||||

----------- Block Count

Byte 5  |xxxxxxxx|

||||||||

----------- Control Byte

The  following  is  a  summary  of  available   command

OpCodes:

Atari Corp Confidential

----- AHDI Command Summary Table ---------------

---------- --------------------

| OpCode   | Command            |

---------- --------------------

| 0x00     | Test Unit Ready    |

| 0x05     | Verify Track       |  *

| 0x06     | Format Track       |  *

| 0x08     | Read               |  *

| 0x0a     | Write              |  *

| 0x0b     | Seek               |

| 0x0d     | Correction Pattern |

| 0x15     | Mode Select        |

| 0x1a     | Mode Sense         |

---------- --------------------

*  multisector transfer with implied seek

NOTE:  subject to change.

Commands are issued to the Atari HDC in a manner  simi-

lar  to that of the FDC, with the major difference being the

handshaking of a multi-byte command descriptor  block.   The

entire hard disk DMA read or write sequence is as follows:

o  load DMA Base Address and Counter Register.

o  toggle Write/_Read to clear status (DMA Mode Control Register).

o  select DMA read or write (DMA Mode Control Register).

o  select DMA Sector Count Register (DMA Mode Control Register).

o  load DMA Sector Count Register (DMA operation trigger).

o  select HDC internal command register (DMA Mode Control Register).

o  issue controller select byte while clearing A0.

o  set A0 for remaining command bytes.

o  after last command byte select controller (DMA Mode Control

Register).

o  DMA active until sector count is zero (DMA Status Register,

do not poll during DMA active).

o  check DMA error status (DMA Status Register, nondestructive).

o  check HDC status byte and if necessary perform ECC correction

following a Verify Track or Read Sector command.

The format of both floppy and hard  disks  contain  512

byte data sectors.

Atari Corp Confidential

----- Floppy Disk Port Pin Assignments ---------------

ST           Circular DIN 14S

----                                    ----

1 |<--- Read Data -------------------|

PSG I/O A         2 |---- Side0 Select --------------->|

3 |---- Logic Ground ----------------|

4 |<--- Index Pulse -----------------|

PSG I/O A         5 |---- Drive0 Select -------------->|

PSG I/O A         6 |---- Drive1 Select -------------->|

7 |---- Logic Ground ----------------|

8 |---- Motor On ------------------->|

9 |---- Direction In --------------->|

10 |---- Step ----------------------->|

11 |---- Write Data ----------------->|

12 |---- Write Gate ----------------->|

13 |<--- Track 00 --------------------|

14 |<--- Write Protect ---------------|

----                                    ----

NOTE:  shield ground must not be connected on the ST side,

the cable must have Read Data and Write Data paired

with Logic Grounds.

Signal Characteristics

pin 1                   TTL levels, active low,

1 Kohm pullup resistor to +5 VDC.

pin 2                   TTL levels, active high,

high at system reset.

pin 4                   TTL levels, active low,

1 Kohm pullup resistor to +5 VDC.

pins 5-6                TTL levels, active low,

high at system reset.

pins 8-12               TTL levels, active low (inverted).

pins 13-14              TTL levels, active low,

1 Kohm pullup resistors to +5 VDC.

Atari Corp Confidential

----- Hard Disk Port Pin Assignments ---------------

ST           DB 19S

----                                    ----

1 |<--- Data 0 --------------------->|

2 |<--- Data 1 --------------------->|

3 |<--- Data 2 --------------------->|

4 |<--- Data 3 --------------------->|

5 |<--- Data 4 --------------------->|

6 |<--- Data 5 --------------------->|

7 |<--- Data 6 --------------------->|

8 |<--- Data 7 --------------------->|

9 |---- Chip Select ---------------->|

MFP              10 |<--- Interrupt Request -----------|

11 |---- Ground ----------------------|

12 |---- Reset ---------------------->|

13 |---- Ground ----------------------|

14 |---- Acknowledge ---------------->|

15 |---- Ground ----------------------|

16 |---- A1 ------------------------->|

17 |---- Ground ----------------------|

18 |---- Read/Write ----------------->|

19 |<--- Data Request ----------------|

----                                    ----

Signal Characteristics

pins 1-8                TTL levels.

pin 9                   TTL levels, active low.

pin 10                  TTL levels, active low,

1 Kohm pullup resistor to +5 VDC.

pin 12                  TTL levels, active low,

system reset.

pin 14                  TTL levels, active low.

pins 16,18              TTL levels.

pin 19                  TTL levels, active low,

1 Kohm pullup resistor to +5 VDC.

Atari Corp Confidential

_6.  _C_o_m_p_o_n_e_n_t_s

The standard configurations of the Atari ST  main  sys-

tem, graphics subsystem, music subsystem, and device subsys-

tems are made up of the following major hardware components:

Main            o  8 MHz MC68000 Microprocessor Unit

o  MK68901 Multi Function Peripheral

o  192 Kbyte ROM

o  512 Kbyte or 1 Mbyte RAM

o  Memory Controller Chip

o  Control Logic Chip

o  DMA Controller Chip

Graphics        o  32 Kbyte Display Memory (from main RAM)

o  Video Shift Register Chip

Music           o  YM-2149 Programmable Sound Generator

Device          o  Atari Intelligent Keyboard (ikbd)

1 MHz HD6301V1 Microcomputer Unit

o  Atari Two Button Mouse

o  2 MC6850 Asynchronous Communications

Interface Adapters

o  WD1772 Floppy Disk Controller

Atari Corp Confidential

_7.  _E_x_p_a_n_s_i_o_n

The Atari ST can be  expanded  by  cartridge  ROM  only

which can contain a maximum of 128 Kbyte.  The disclosure of

cartridge presence  or  memory  size  is  not  supported  in

hardware.   The  ROM  cartridge  slot  has the following pin

assignments (only the lower 15 address lines are available):

----- ROM Port Pin Assignments ---------------

ST           40S

----                                    ----

1 |---- Power +5 VDC --------------->|

2 |---- Power +5 VDC --------------->|

3 |<--- Data 14 ---------------------|

4 |<--- Data 15 ---------------------|

5 |<--- Data 12 ---------------------|

6 |<--- Data 13 ---------------------|

7 |<--- Data 10 ---------------------|

8 |<--- Data 11 ---------------------|

9 |<--- Data 8 ----------------------|

10 |<--- Data 9 ----------------------|

11 |<--- Data 6 ----------------------|

12 |<--- Data 7 ----------------------|

13 |<--- Data 4 ----------------------|

14 |<--- Data 5 ----------------------|

15 |<--- Data 2 ----------------------|

16 |<--- Data 3 ----------------------|

17 |<--- Data 0 ----------------------|

18 |<--- Data 1 ----------------------|

19 |---- Address 13 ----------------->|

20 |---- Address 15 ----------------->|

21 |---- Address 8 ------------------>|

22 |---- Address 14 ----------------->|

23 |---- Address 7 ------------------>|

24 |---- Address 9 ------------------>|

25 |---- Address 6 ------------------>|

26 |---- Address 10 ----------------->|

27 |---- Address 5 ------------------>|

28 |---- Address 12 ----------------->|

29 |---- Address 11 ----------------->|

30 |---- Address 4 ------------------>|

31 |---- ROM3 Select ---------------->|

32 |---- Address 3 ------------------>|

33 |---- ROM4 Select ---------------->|

34 |---- Address 2 ------------------>|

35 |---- _Upper Data Strobe --------->|

36 |---- Address 1 ------------------>|

37 |---- _Lower Data Strobe --------->|

38-40 |---- Ground ----------------------|

----                                    ----

Atari Corp Confidential

_8.  _M_e_m_o_r_y _M_a_p

The first 2 Kbyte of ST  memory  is  reserved  for  the

exception  vector  table  and  supervisor  stack.  This area

along with I/O space is protected for supervisor  references

only.   Accessing  supervisor  protected  areas while in the

user state will result in a bus error.  A 4 word portion  of

ROM  is  shadowed  at  the  start of RAM for the reset stack

pointer and program counter.  Writing to this  area  or  any

ROM location will also result in a bus error.  The following

is a map of ST memory:

Atari Corp Confidential

----- ST Memory Map ---------------

----------------

00 0000   | ROM            |   Reset:  Supervisor Stack Pointer

00 0004   | ROM            |   Reset:  Program Counter

00 0008   | RAM            |   0 Kbyte RAM

----------------

.

----------------

08 0000   | RAM            |   512 Kbyte RAM

----------------

.

----------------

10 0000   | RAM            |   1 Mbyte RAM

----------------

.

.

.

----------------

fa 0000   | ROM            |   320 Kbyte ROM

----------------

.

----------------

fc 0000   | ROM            |   Reset:  Supervisor Stack Pointer

fc 0004   | ROM            |   Reset:  Program Counter

fc 0008   | ROM            |   192 Kbyte ROM

fe ffff   | ROM            |   0 Kbyte ROM

----------------

.

.

.

----------------

ff 8000   | I/O            |   Configuration Registers

ff 8200   | I/O            |   Display Registers

ff 8400   | I/O            |   Reserved

ff 8600   | I/O            |   DMA/Disk Registers

ff 8800   | I/O            |   Sound Registers

----------------

.

----------------

ff fa00   | I/O            |   MC68xxx Registers

ff fc00   | I/O            |   MC68xx Registers

----------------

Atari Corp Confidential

_9.  _I/_O _M_a_p

The ST I/O space ranges from ff 0000 to ff  ffff,  with

MC68000 and MC6800 peripheral internal registers starting at

ff fa00 and ff fc00 respectively.   Accessing  reserved  I/O

addresses  will result in a bus error.  Bit values for vari-

ous read  and/or  write  registers  are  labeled  as  active

One/_Zero  (always mask out unused field bits).  The follow-

ing is a map of ST I/O space:

_____________

Configuration

ff 8001   R/W             |----xxxx|   Memory Configuration

||||

-------   Bank0      Bank1 (not used)

0000       128 Kbyte  128 Kbyte

0001       128 Kbyte  512 Kbyte

0010       128 Kbyte    2 Mbyte

0011       Reserved

0100       512 Kbyte  128 Kbyte

0101       512 Kbyte  512 Kbyte

0110       512 Kbyte    2 Mbyte

0111       Reserved

1000         2 Mbyte  128 Kbyte

1001         2 Mbyte  512 Kbyte

1010         2 Mbyte    2 Mbyte

1011       Reserved

11xx       Reserved

_______

Display

ff 8201   R/W             |xxxxxxxx|   Video Base High

ff 8203   R/W             |xxxxxxxx|   Video Base Low

ff 8205   R               |xxxxxxxx|   Video Address Counter High

ff 8207   R               |xxxxxxxx|   Video Address Counter Mid

ff 8209   R               |xxxxxxxx|   Video Address Counter Low

ff 820a   R/W             |------xx|   Sync Mode

||

| ----   External/_Internal Sync

-----   50 Hz/_60 Hz Field Rate

ff 8240   R/W     |-----xxx-xxx-xxx|   Palette Color 0/0 (Border)

||| ||| |||

||| ||| || ----   Inverted/_Normal Monochrome

||| ||| |||

||| |||  ------   Blue

|||  ----------   Green

--------------   Red

ff 8242   R/W     |-----xxx-xxx-xxx|   Palette Color 1/1

Atari Corp Confidential

ff 8244   R/W     |-----xxx-xxx-xxx|   Palette Color 2/2

ff 8246   R/W     |-----xxx-xxx-xxx|   Palette Color 3/3

ff 8248   R/W     |-----xxx-xxx-xxx|   Palette Color 4

ff 824a   R/W     |-----xxx-xxx-xxx|   Palette Color 5

ff 824c   R/W     |-----xxx-xxx-xxx|   Palette Color 6

ff 824e   R/W     |-----xxx-xxx-xxx|   Palette Color 7

ff 8250   R/W     |-----xxx-xxx-xxx|   Palette Color 8

ff 8252   R/W     |-----xxx-xxx-xxx|   Palette Color 9

ff 8254   R/W     |-----xxx-xxx-xxx|   Palette Color 10

ff 8256   R/W     |-----xxx-xxx-xxx|   Palette Color 11

ff 8258   R/W     |-----xxx-xxx-xxx|   Palette Color 12

ff 825a   R/W     |-----xxx-xxx-xxx|   Palette Color 13

ff 825c   R/W     |-----xxx-xxx-xxx|   Palette Color 14

ff 825e   R/W     |-----xxx-xxx-xxx|   Palette Color 15

ff 8260   R/W             |------xx|   Shift Mode

||

00       320 x 200, 4 Plane

01       640 x 200, 2 Plane

10       640 x 400, 1 Plane

11       Reserved

________

RESERVED

ff 8400           |----------------|   Reserved

________

DMA/Disk

ff 8600           |----------------|   Reserved

ff 8602           |----------------|   Reserved

ff 8604   R/W     |--------xxxxxxxx|   Disk Controller (Word Access)

ff 8606   R       |-------------xxx|   DMA Status (Word Access)

|||

|| ----   _Error Status

| -----   _Sector Count Zero Status

------   _Data Request Inactive Status

ff 8606   W       |-------xxxxxxxx-|   DMA Mode Control (Word Access)

||||||||

||||||| -----   A0

|||||| ------   A1

||||| -------   HDC/_FDC Register Select

|||| --------   Sector Count Register Select

|||0            Reserved

|| ----------   Disable/_Enable DMA

| -----------   FDC/_HDC

------------   Write/_Read

ff 8609   R/W             |xxxxxxxx|   DMA Base and Counter High

ff 860b   R/W             |xxxxxxxx|   DMA Base and Counter Mid

Atari Corp Confidential

ff 860d   R/W             |xxxxxxxx|   DMA Base and Counter Low

_____

Sound

ff 8800   R               |xxxxxxxx|   PSG Read Data

||||||||       I/O Port B

--------------   Parallel Interface Data

ff 8800   W               |xxxxxxxx|   PSG Register Select

||||

-------   Register Number

0000       Channel A Fine Tune

0001       Channel A Coarse Tune

0010       Channel B Fine Tune

0011       Channel B Coarse Tune

0100       Channel C Fine Tune

0101       Channel C Coarse Tune

0110       Noise Generator Control

0111       Mixer Control - I/O Enable

1000       Channel A Amplitude

1001       Channel B Amplitude

1010       Channel C Amplitude

1011       Envelope Period Fine Tune

1100       Envelope Period Coarse Tune

1101       I/O Port A (Output Only)

1111       I/O Port B

ff 8802   W               |xxxxxxxx|   PSG Write Data

||||||||       I/O Port A

||||||| -------   Floppy Side0/_Side1 Sele

ct

|||||| --------   Floppy _Drive0 Select

||||| ---------   Floppy _Drive1 Select

|||| ----------   RS232 Request To Send

||| -----------   RS232 Data Terminal Read

y

|| ------------   Centronics _STROBE

| -------------   General Purpose Output

--------------   Reserved

||||||||       I/O Port B

--------------   Parallel Interface Data

_______

MC68xxx

ff fa01                   |xxxxxxxx|   MFP General Purpose I/O

ff fa03                   |xxxxxxxx|   MFP Active Edge

ff fa05                   |xxxxxxxx|   MFP Data Direction

ff fa07                   |xxxxxxxx|   MFP Interrupt Enable A

ff fa09                   |xxxxxxxx|   MFP Interrupt Enable B

ff fa0b                   |xxxxxxxx|   MFP Interrupt Pending A

ff fa0d                   |xxxxxxxx|   MFP Interrupt Pending B

ff fa0f                   |xxxxxxxx|   MFP Interrupt In-Service A

ff fa11                   |xxxxxxxx|   MFP Interrupt In-Service B

ff fa13                   |xxxxxxxx|   MFP Interrupt Mask A

Atari Corp Confidential

ff fa15                   |xxxxxxxx|   MFP Interrupt Mask B

ff fa17                   |xxxxxxxx|   MFP Vector

ff fa19                   |xxxxxxxx|   MFP Timer A Control

ff fa1b                   |xxxxxxxx|   MFP Timer B Control

ff fa1d                   |xxxxxxxx|   MFP Timers C and D Control

ff fa1f                   |xxxxxxxx|   MFP Timer A Data

ff fa21                   |xxxxxxxx|   MFP Timer B Data

ff fa23                   |xxxxxxxx|   MFP Timer C Data

ff fa25                   |xxxxxxxx|   MFP Timer D Data

ff fa27                   |xxxxxxxx|   MFP Sync Character

ff fa29                   |xxxxxxxx|   MFP USART Control

ff fa2b                   |xxxxxxxx|   MFP Receiver Status

ff fa2d                   |xxxxxxxx|   MFP Transmitter Status

ff fa2f                   |xxxxxxxx|   MFP USART Data

______

MC68xx

ff fc00                   |xxxxxxxx|   Keyboard ACIA Control

ff fc02                   |xxxxxxxx|   Keyboard ACIA Data

ff fc04                   |xxxxxxxx|   MIDI ACIA Control

ff fc06                   |xxxxxxxx|   MIDI ACIA Data

Atari Corp Confidential

_1_0.  _I_n_t_e_r_r_u_p_t _T_a_b_l_e

The following tables list the ST interrupt  and  signal

priority assignments:

----- MC68000 Interrupt Autovector ---------------

--------------- --------------------------------

| Level         | Definition                     |

--------------- --------------------------------

| 7 (HIGHEST)   | NMI                            |

| 6             | MK68901 MFP                    |

| 5             |                                |

| 4             | Vertical Blanking (Sync)       |

| 3             |                                |

| 2             | Horizontal Blanking (Sync)     |

| 1 (LOWEST)    |                                |

--------------- --------------------------------

NOTE:  only interrupt priority level inputs 1 and 2 are used.

----- MK68901 Interrupt Control ---------------

--------------- --------------------------------

| Priority      | Definition                     |

--------------- --------------------------------

| 15 (HIGHEST)  | Monochrome Monitor Detect    I7|

| 14            | RS232 Ring Indicator         I6|

| 13            | System Clock / BUSY          TA|

| 12            | RS232 Receive Buffer Full      |

| 11            | RS232 Receive Error            |

| 10            | RS232 Transmit Buffer Empty    |

|  9            | RS232 Transmit Error           |

|  8            | Horizontal Blanking Counter  TB|

|  7            | Disk Drive Controller        I5|

|  6            | Keyboard and MIDI            I4|

|  5            | Timer C                      TC|

|  4            | RS232 Baud Rate Generator    TD|

|  3            | GPU Operation Done           I3|

|  2            | RS232 Clear To Send          I2|

|  1            | RS232 Data Carrier Detect    I1|

|  0 (LOWEST)   | Centronics BUSY              I0|

--------------- --------------------------------

NOTE:  the MC6850 ACIA Interrupt Request status bit must be tested

to differentiate between keyboard and MIDI interrupts.

Atari Corp Confidential

_1_1.  _C_a_s_e _D_e_s_i_g_n

The ST is primarily designed  as  a  keyboard  computer

with  external video display, disk drives, and power supply.

The top panel contains the keyboard, ventilation slots (bot-

tom  also),  and  power on LED.  The side panels contain the

expansion ROM slot and mouse/joystick ports.  The back panel

contains  the  on/off  switch,  reset button, channel select

switch, power connector, and all  remaining  device  connec-

tors:   video,  parallel, RS232, MIDI, and disk drive inter-

faces.

The device ports are labeled with  International  Elec-

trotechnical Commission-like symbols in combinations of pic-

tograms and phonetic transcriptions (captions).  In the case

of MIDI, the specification explicitly requires that the MIDI

connectors be labeled MIDI OUT, MIDI IN, and MIDI  THRU  (if

applicable).

Atari Corp Confidential

_1_2.  _P_o_w_e_r _S_u_p_p_l_y

An external DC power supply provides power to the  main

system  board,  keyboard,  expansion ROM, and expansion RAM.

All power levels are regulated for  over-voltage  and  over-

current  protection.  The following are minimal power supply

specifications:

AC Input                115 VAC 10% at 60 Hz (fused)

DC Output               +5 VDC at 3 A 5%

+12 VDC at .03 A 10%

-12 VDC at .03 A 10%

----- Power Port Pin Assignments ---------------

ST           DIN 7P

----                                    ----

1 |<--- +5 VDC ----------------------|

3 |---- Ground ----------------------|

4 |<--- +12 VDC ---------------------|

5 |<--- -12 VDC ---------------------|

6 |<--- +5 VDC ----------------------|

7 |---- Ground ----------------------|

----                                    ----

Atari Corp Confidential

_A_p_p_e_n_d_i_x _A -- _i_k_b_d _K_e_y_b_o_a_r_d _L_a_y_o_u_t

The surface of the Atari Intelligent Keyboard is formed

from  four  distinct  ergonomic  units:  a QWERTY typewriter

matrix, a function key array, a screen control cluster,  and

a  calculator numeric keypad.  As a whole, the United States

version of the keyboard layout is DEC  VT100-like  with  the

exception of the following items:

o  removed keys -- [break], [line feed], [no scroll], [set up].

o  repositioned keys -- [caps lock], cursor control keys,

programmable function keys.

o  added keys -- [alternate], [help], [undo], [insert], [clear/home],

10 programmable function keys.

o  improvements -- DEC VT200-style inverted T cursor control

keys, full calculator function numeric keypad.

Please note that the left [shift] key is  intentionally

large  to allow space for an extra ISO required key (see ISO

2530-1975).   Atari  ikbd  International  Layouts  will   be

designed for the following countries:

o  United Kingdom

o  United States

o  Finland

o  Norway / Denmark

o  Sweden

o  Japan

o  Germany

o  French Canada

o  France

o  Italy

o  Spain

NOTE:  the 'HELP' and 'UNDO' keys will be left untranslated.

Atari Corp Confidential

_A_p_p_e_n_d_i_x _B -- _R_e_f_e_r_e_n_c_e_s

General

A Hitchhiker's Guide to the BIOS

Digital Research GEM Software Documentation

Main System

Motorola MC68000 16-Bit Microprocessor  User's  Manual,

Fourth Edition

Mostek MK68901 Multi Function Peripheral Data Sheet

Graphics Subsystem

Adele Goldberg and David  Robson,  'Smalltalk-80:   The

Language and Its Implementation', Addison-Wesley, Read-

ing Massachusetts, 1983, Chapter 18.

Music Subsystem

General Instrument AY-3-8910 Programmable Sound Genera-

tor Data Sheet

MIDI Musical Instrument Digital Interface Specification

1.0

Device Subsystems

Atari Intelligent Keyboard (ikbd) Protocol and Specifi-

cation

Motorola MC6850 Asynchronous  Communications  Interface

Adapter Data Sheet

Centronics Parallel Interface Specification

Electronic Industries Association RS232C Standard

Western Digital WD1770/1772 Floppy Disk Controller Data

Sheet

Specification of the Atari  Computer  System  Interface

(ACSI)

Specification of the Atari Hard Disk Interface (AHDI)

Atari Corp Confidential

_A_p_p_e_n_d_i_x _C -- _N_o_t_e_s

General

851125A   An address error occurs when a word instruction is

used on a byte address.

Main System

841017B   The DMA Base Address and Counter Register must  be

loaded in low, mid, high order.

Graphics Subsystem

841017C   None.

Music Subsystem

841017D   The YM-2149 PSG I/O space and registers should  be

set up as critical regions in software.

Device Subsystems

841017F   Poll or service the Disk Drive  Controller  inter-

rupt on the MK68901 MFP General Purpose I/O Regis-

ter to detect the completion of a WD1772 FDC  com-

mand.   Do  not  poll  the  FDC Busy or DMA Sector

Count Zero status bits.

841017G   Select the Sector Count  Register  before  testing

the DMA Status Register Error bit.

841017H   Do not set the 30 ms Settling Delay bit on  WD1772

FDC type 2 and 3 command executions.

841017I   A force interrupt should be  issued  after  a  few

seconds  (ie time out) on all commands sent to the

WD1772 FDC.

841017J   Wait until the WD1772 FDC Motor On status  is  low

before deselecting a floppy drive.

841017K   A floppy disk drive configuration table should  be

maintained  in  software  to accommodate a diverse

selection of 3.5 inch  floppy  disk  drives.   Two

floppy disk drives currently under evaluation have

the following characteristics:

o  500 Kbyte unformatted, 80 cylinders, one  head,

3 ms stepping rate.

o  1 Mbyte unformatted, 80 cylinders, two heads, 3

ms stepping rate.

Atari Corp Confidential

Table of Contents

1.  System Architecture ...............................    1

2.  Main System .......................................    4

2.1.  Microprocessor Unit ...........................    4

2.2.  Memory Configuration ..........................    4

2.3.  Direct Memory Access ..........................    5

3.  Graphics Subsystem ................................    7

3.1.  Video Display Memory ..........................    7

3.2.  Video Configuration ...........................    8

4.  Music Subsystem ...................................   10

4.1.  Sound Synthesizer .............................   10

4.2.  Musical Instrument Communication ..............   10

5.  Device Subsystems .................................   12

5.1.  Intelligent Keyboard ..........................   12

5.2.  Video Interface ...............................   14

5.3.  Parallel Interface ............................   15

5.4.  RS232 Interface ...............................   16

5.5.  MIDI Interface ................................   17

5.6.  Disk Drive Interface ..........................   18

6.  Components ........................................   23

7.  Expansion .........................................   24

8.  Memory Map ........................................   25

9.  I/O Map ...........................................   27

10.  Interrupt Table ..................................   31

11.  Case Design ......................................   32

12.  Power Supply .....................................   33

Appendix A -- ikbd Keyboard Layout ....................   34

Appendix B -- References ..............................   35

Appendix C -- Notes ...................................   36

Atari Corp Confidential