Atari ST to Multisync Monitor Color/Mono Switch Box

by Scott Solberg                                        11/90

GEnie mail: S.SOLBERG

This  document  is intended to help those  who  wish  to

build their own converter cable and switch box, rather than buying

an Omniswitch (or equivalent).

I  decided  to build my own when the Omniswitch  3  that  I

ordered did not work with my Viewperfect 3 monitor  (#TX-14H8PTK).

The  Omniswitch sends the sync signals on the green video  signal.

While  this  is normally O.K.,  for some reason the  screen  would

flicker in color mode whenever I had a lot of green (and white) on

the screen (like the Desktop!!).   The only way to get rid of this

problem  was to actually reduce the green intensity on the  screen

by using the Control Panel Desk Accessory.  In order to completely

rule  out  monitor  malfunction,   I  decided  to  build  my   own

cable/switchbox.  While this task took longer than it should have,

it  was well worth the effort,  because I now can use my  monitor,

return the Omniswitch, and pocket a little change!

Initially,  I downloaded from GEnie files from  M.  Odegard

(#5617),  and W. Price (#12944).  I rigged a switch box that was a

hybrid of these two circuits,  and discovered in the process  that

they  are more complex than they need to be,  in addition  to  the

fact  that unnecessary resistors reduce the monochrome  signal  to

the monitor.

I made the following changes:

1) the connection from the Atari mono intensity signal  pin

to ground via the 100 Ohm resistor is deleted entirely.

2)  the  68 Ohm resistor between the Atari  mono  intensity

signal pin and the switch is removed.

The important features which I retained are:

1)  the  horizontal  and vertical  sync  signals  are  sent

directly  to the monitor.   This is why my homebrew  switch

box worked better than the Omniswitch.

2)  I  kept the 68 Ohm resistors on the  RGB  outputs  from

the Atari,  partly because they worked fine (I had just the

right amount of signal to my monitor), and partly because I

compared the RGB   signals  with these  resistors  to   the

signals from   the Omniswitch,    and   they    were almost

identical.

3)  I used a single 4-pole ON-ON switch,  which  is  easier

than using the two switches indicated by M. Odegard.

--- Making the Switchbox ---

This  is  my technique:

1)  buy  a  13-pin male-male Atari  monitor  cable  from  Best

Electronics @ (408) 243-6950 for $20 and cut off one end.

2)  attach a 9-pin female D-sub crimp-style connector  to  the

appropriate  wires  of  the 13-pin cable.   Make sure  you  use  a

shielded D-sub hood.   I found it is easiest to clamp the  cable's

metal-foil  shielding between the two halves of  the  hood,  which

avoids having to do any soldering.

3)  the  switch box itself has a female 9-pin  D-sub  jack  to

accept  the male multisync monitor plug,  and a male  9-pin  D-sub

jack to accept the female computer cable plug.   I did not want  a

long cable hardwired to my box because I thought this way would be

more versatile.

4)  I used a 4-pole ON-ON-ON switch from C&K  Components  Inc.

which  I picked up at a local electronics store (Radio Shack  does

not  carry any switches with more than 2 poles).   The  center  ON

position  is not needed,  and will only give you  a  funny-looking

screen!   When the switch is set to mono,  the mono detect wire is

grounded,  thus  causing  the ST to re-boot and come  up  in  high

resolution.

I have included DEGAS pictures of the schematics.

NOTE:

*  failure  to  properly  shield  your  cables  may   cause

interference when in Mono mode,  because the mono sync signals are

relatively  weak to begin with.  If you use a plastic or wood  box

for  your  enclosure,  you will have to solder  a  connector  wire

between casings of the 9-pin D-sub jacks.

*  By  the  way,  I am very happy  with  my  Viewperfect  3

monitor.   Half-height text in monochrome is very readable, and of

course  the  color is great.   The  horizontal/vertical  size  and

position controls are located on the top of the monitor,  which is

much  more  accessable than for other monitors such  as  the  ACER

which have the controls at the back of the monitor.   Actually,  I

find  that  I do not have to adjust the screen size  when  I  flip

between color and mono.  I bought my monitor for $460 + tax  at  a

local  computer show,  and it seemed to be the best in  its  price

range.