Pin Assignments and Cable Information All Interfaces

===================================================================

ST412 Interface (MFM and RLL) ST-506 was predecessor without buffered

seek

2 Cables: 1 each 34-pin Command Cable and 1 each 20-pin Data Cable

-------------------------------------------------------------------

J1 (34-pin Command) Host/Drive Pin Assignments

GROUND RTN     SIGNAL PIN     SIGNAL

PIN

_____________________________________________________

3

1              2          -HEAD SELECT 2

2

3              4          -HEAD SELECT 2

5              6          -WRITE GATE

7              8*         -SEEK COMPLETE

9             10*         -TRACK 0

11             12*         -WRITE FAULT

0

13             14          -HEAD SELECT 2

15             16          -RESERVED

1

17             18          -HEAD SELECT 2

19             20*         -INDEX

21             22*         -READY

23             24          -STEP

25 (29)**      26 (28)     -DRIVE SELECT 1 (2)

27 (27)        28 (26)     -DRIVE SELECT 2 (1)

29 (25)        30          -DRIVE SELECT 3

31             32          -DRIVE SELECT 4

33             34          -DIRECTION IN

* STATUS ENABLED WITH DRIVE SELECT

** TWISTED CABLE IN PARENTHESES

J2 (20-pin Data) HOST/DRIVE PIN ASSIGNMENTS

GROUND RTN     SIGNAL PIN     SIGNAL NAME

PIN

_____________________________________________________________

2              1          -DRIVE SELECTED

4              3          RESERVED

6              5          RESERVED

8              7          RESERVED

10              9          RESERVED

12             11          GROUND

13          +MFM (RLL) WRITE DATA

14          -MFM (RLL) WRITE DATA

16             15          GROUND

17          +MFM (RLL) READ DATA

18          -MFM (RLL) READ DATA

20             19          GROUND

==================================================================

ESDI (Enhanced Small Device Interface)

2 Cables: 1 each 34-pin Command Cable and 1 each 20-pin Data Cable

------------------------------------------------------------------

ESDI INTERFACE SPECIFICATIONS

ESDI CONTROL CABLE (J1/P1 PIN ASSIGNMENTS

(Disk Implementation - Serial Mode)

SIGNAL         GROUND

SIGNAL NAME                       PIN             PIN

________________________________________________________________________

-HEAD SELECT 2(3)                   2              1

-HEAD SELECT 2(2)                   4              3

-WRITE GATE                         6              5

-CONFIGURATION/STATUS DATA          8              7

-TRANSFER ACK                      10              9

-ATTENTION                         12             11

-HEAD SELECT 2(0)                  14             13

-SECTOR/BYTE CLOCK/

ADDRESS MARK FOUND                16             15

-HEAD SELECT 2(1)                  18             17

-INDEX                             20             19

-READY                             22             21

-TRANSFER REQ                      24             23

-DRIVE SELECT 2(0)                 26             25

-DRIVE SELECT 2(1)                 28             27

-DRIVE SELECT 2(2)                 30             29

-READ GATE                         32             31

-COMMAND DATA                      34             33

ESDI DATA CABLE (J2/P2) PIN ASSIGNMENTS

(Disk Implementation - Serial Mode)

SIGNAL         GROUND

SIGNAL NAME                        PIN            PIN

________________________________________________________________________

-DRIVE SELECTED                     1

-SECTOR/BYTE CLOCK/

ADDRESS MARK FOUND                 2

-COMMAND COMPLETE                   3

-ADDRESS MARK ENABLE                4

GROUND                                             5

+/-WRITE CLOCK                     7/8             6

+/-READ REF CLOCK                 10/11           9/12

+/-NRZ WRITE DATA                 13/14          15/16

+/-NRZ READ DATA                  17/18            19

-INDEX                              20

==================================================================

SCSI-1 (SMALL COMPUTER SYSTEM INTERFACE) CONNECTOR PIN ASSIGNMENTS

50-pin Cable

------------------------------------------------------------------

SCSI - Standard

SCSI (Small Computer System Interface) is an intelligent device

interface that allows up to 8 devices (usually 7 plus the host system)

to be connected together using up to a 6 meter cable. Unlike other

older interfaces, requests on a SCSI bus are just that, requests. For

example, rather than the low-level device commands like the familiar

STEP+ and STEP- of ST412, SCSI accesses data using the higher level

SEEK to a location ( LBA or logical block address on drives ).

Input voltages may vary from a low of 0VDC to a high of +5.25VDC.

These devices ARE NOT BUS COMPATIBLE with differential SCSI devices.

They are hovever command compatible. Attempting to share the SCSI bus

with a differential device will quite possibly damage the single-ended

SCSI device. Pin designations are as follows, and all signals are

active low (true is a voltage level between 0 and +0.8 VDC, and false

is defined as a voltage between +2 and +5.25 VDC).

�������������������������������������������������������������������Ŀ

�PIN NUMBER�SIGNAL �DESCRIPTION �PIN NUMBER�SIGNAL  � DESCRIPTION   �

�������������������������������������������������������������������Ĵ

�    1     � GND   �GROUND      �    26    �TERMPWR �TERMINATION PWR�

�������������������������������������������������������������������Ĵ

�    2     � DB0   �DATA BIT 0  �    27    �  GND   � GROUND        �

�������������������������������������������������������������������Ĵ

�    3     � GND   �GROUND      �    28    �  GND   � GROUND        �

�������������������������������������������������������������������Ĵ

�    4     � DB1   �DATA BIT 1  �    29    �  GND   � GROUND        �

�������������������������������������������������������������������Ĵ

�    5     � GND   �GROUND      �    30    �  GND   � GROUND        �

�������������������������������������������������������������������Ĵ

�    6     � DB2   �DATA BIT 2  �    31    �  GND   � GROUND        �

�������������������������������������������������������������������Ĵ

�    7     � GND   �GROUND      �    32    �  ATN   � ATTENTION     �

�������������������������������������������������������������������Ĵ

�    8     � DB3   �DATA BIT 3  �    33    �  GND   � GROUND        �

�������������������������������������������������������������������Ĵ

�    9     � GND   �GROUND      �    34    �  GND   � GROUND        �

�������������������������������������������������������������������Ĵ

�   10     � DB4   �DATA BIT 4  �    35    �  GND   � GROUND        �

�������������������������������������������������������������������Ĵ

�   11     � GND   �GROUND      �    36    �  BSY   � BUSY          �

�������������������������������������������������������������������Ĵ

�   12     � DB5   �DATA BIT 5  �    37    �  GND   � GROUND        �

�������������������������������������������������������������������Ĵ

�   13     � GND   �GROUND      �    38    �  ACK   � ACKNOWLEDGE   �

�������������������������������������������������������������������Ĵ

�   14     � DB6   �DATA BIT 6  �    39    �  GND   � GROUND        �

�������������������������������������������������������������������Ĵ

�   15     � GND   �GROUND      �    40    �  RST   � RESET         �

�������������������������������������������������������������������Ĵ

�   16     � DB7   �DATA BIT 7  �    41    �  GND   � GROUND        �

�������������������������������������������������������������������Ĵ

�   17     � GND   �GROUND      �    42    �  MSG   � MEGSSAGE      �

�������������������������������������������������������������������Ĵ

�   18     � DBP   �DATA PARITY �    43    �  GND   � GROUND        �

�������������������������������������������������������������������Ĵ

�   19     � GND   �GROUND      �    44    �  SEL   � SELECT        �

�������������������������������������������������������������������Ĵ

�   20     � GND   �GROUND      �    45    �  GND   � GROUND        �

�������������������������������������������������������������������Ĵ

�   21     � GND   �GROUND      �    46    �  C/D   � CONTROL/DATA  �

�������������������������������������������������������������������Ĵ

�   22     � GND   �GROUND      �    47    �  GND   � GROUND        �

�������������������������������������������������������������������Ĵ

�   23     � GND   �GROUND      �    48    �  REQ   � REQUEST       �

�������������������������������������������������������������������Ĵ

�   24     � GND   �GROUND      �    49    �  GND   � GROUND        �

�������������������������������������������������������������������Ĵ

�   25     � GND   �GROUND      �    50    �  I/O   � INPUT/OUTPUT  �

���������������������������������������������������������������������

ACK (Acknowledge): A signal driven by an Initiator to indicate an

acknowledgement for a REQ/ACK data transfer handshake.

ATN (Attention): A signal driven by an Initiator to indicate the

attention condition.

BSY (Busy): An "or-tied" signal which indicates that the bus is in

use.

C/D (Control/Data): A signal driven by a Target. It indicates whether

control or data information is on the data bus. True indicates

Control.

DB(7-0) (Data Bus): Contains the data that is sent from one device to

the other. Driven by either device, as determined by the state of the

I/O signal.

DB(P) (Data Bus Parity): Contains the parity bit for the data that is

sent on DB(7-0) from one device to the other during an information

transfer.

I/O (Input/Output): A signal driven by a Target which controls the

direction of data flow in the data bus, with respect to an Initiator.

True indicates input to the Initiator.

MSG (Message): A signal driven by a Target during the message phase.

REQ (Request): A signal driven by a Target to indicate a request for a

REQ/ACK data transfer handshake.

RST (Reset): An "or-tied" signal which indicates the reset condition.

SEL (Select): A signal used by an Initiator to select a Target or by a

Target to reselect an Intiator.

=====================================================================

SCSI - Differential

SCSI (Small Computer System Interface) is an intelligent device

interface that allows up to 8 devices (usually 7 plus the host system)

to be connected together. Unlike other older interfaces, requests on a

SCSI bus are just that, requests. For example, rather than the

low-level device commands like the familiar STEP+ and STEP- of ST412,

SCSI accesses data using the higher level SEEK to a location ( LBA or

logical block address on drives ).

Differential SCSI (Small Computer System Interface) varies from

standard SCSI at the bus level in that it is capable of operating up

to a 25 meter cable, and the electrical specifications are quite

different. Input voltages vary from a low of -7VDC to a high of

+12VDC. These high voltage changes are measured from the positive

( + or POS ) portion of a signal, in relation to the negative

( - or NEG ) signal of the same name.

These devices ARE NOT BUS COMPATIBLE with SCSI standard devices. they

are hovever command compatible. Attempting to share the SCSI bus with

a non-differential device will quite possibly damage the single-ended

SCSI device. The electrical specification follows the EIA RS-485-1983

standard. Pin designations are as follows :

�����������������������������������������������������������������������Ŀ

�PIN NUMBER�SIGNAL �DESCRIPTION    �PIN NUMBER�SIGNAL  � DESCRIPTION    �

�����������������������������������������������������������������������Ĵ

�    1     � GND   �SHIELD GROUND  �    26    �TERMPWR �TERMINATION PWR �

�����������������������������������������������������������������������Ĵ

�    2     � GND   �GROUND         �    27    �  GND   � GROUND         �

�����������������������������������������������������������������������Ĵ

�    3     �+DB0   �POS DATA BIT 0 �    28    �  GND   � GROUND         �

�����������������������������������������������������������������������Ĵ

�    4     �-DB0   �NEG DATA BIT 0 �    29    � +ATN   � POS ATTENTION  �

�����������������������������������������������������������������������Ĵ

�    5     �+DB1   �POS DATA BIT 1 �    30    � -ATN   � NEG ATTENTION  �

�����������������������������������������������������������������������Ĵ

�    6     �-DB1   �NEG DATA BIT 1 �    31    �  GND   � GROUND         �

�����������������������������������������������������������������������Ĵ

�    7     �+DB2   �POS DATA BIT 2 �    32    �  GND   � GROUND         �

�����������������������������������������������������������������������Ĵ

�    8     �-DB2   �NEG DATA BIT 2 �    33    � +BSY   � POS BUSY       �

�����������������������������������������������������������������������Ĵ

�    9     �+DB3   �POS DATA BIT 3 �    34    � -BSY   � NEG BUSY       �

�����������������������������������������������������������������������Ĵ

�   10     �-DB3   �NEG DATA BIT 3 �    35    � +ACK   �POS ACKNOWLEDGE �

�����������������������������������������������������������������������Ĵ

�   11     �+DB4   �POS DATA BIT 4 �    36    � -ACK   �NEG ACKNOWLEDGE �

�����������������������������������������������������������������������Ĵ

�   12     �-DB4   �NEG DATA BIT 4 �    37    � +RST   � POS RESET      �

�����������������������������������������������������������������������Ĵ

�   13     �+DB5   �POS DATA BIT 5 �    38    � -RST   � NEG RESET      �

�����������������������������������������������������������������������Ĵ

�   14     �-DB5   �NEG DATA BIT 5 �    39    � +MSG   � POS MESSAGE    �

�����������������������������������������������������������������������Ĵ

�   15     �+DB6   �POS DATA BIT 6 �    40    � -MSG   � NEG MESSAGE    �

�����������������������������������������������������������������������Ĵ

�   16     �-DB6   �NEG DATA BIT 6 �    41    � +SEL   � POS SELECT     �

�����������������������������������������������������������������������Ĵ

�   17     �+DB7   �POS DATA BIT 7 �    42    � -SEL   � NEG SELECT     �

�����������������������������������������������������������������������Ĵ

�   18     �-DB7   �NEG DATA BIT 7 �    43    � +C/D   �POS COMMAND/DATA�

�����������������������������������������������������������������������Ĵ

�   19     �+DBP   �POS PARITY BIT �    44    � -C/D   �NEG COMMAND/DATA�

�����������������������������������������������������������������������Ĵ

�   20     �-DBP   �NEG PARITY BIT �    45    � +REQ   � POS REQUEST    �

�����������������������������������������������������������������������Ĵ

�   21     �DIFFSEN�NOT USED       �    46    � -REQ   � NEG REQUEST    �

�����������������������������������������������������������������������Ĵ

�   22     � GND   �GROUND         �    47    � +I/O   �POS INPUT/OUTPUT�

�����������������������������������������������������������������������Ĵ

�   23     � GND   �GROUND         �    48    � -I/O   �NEG INPUT/OUTPUT�

�����������������������������������������������������������������������Ĵ

�   24     � GND   �GROUND         �    49    �  GND   � GROUND         �

�����������������������������������������������������������������������Ĵ

�   25     �TERMPWR�GROUND         �    50    �  GND   � GROUND         �

�������������������������������������������������������������������������

=================================================================

AT Interface

40-pin Cable

-----------------------------------------------------------------

AT INTERFACE SIGNAL PIN ASSIGNMENTS

PIN       NAME                     DESCRIPTION

_________________________________________________________________________

01       /Host        (From Host, Active Low) Reset signal from

Reset        the host.

02      Ground                     - - - -

03-18     - - -         Host data 0-15 to/from host. 16-bit tris-

tate, bidirectional data bus between host

and drive.  The lower 8-bits of host data

(0-7) are used for register and ECC access.

All 16-bits are used for data transfers.

19      Ground                    - - - -

20       Key          An unused pin, which is clipped off at the

drive to prevent incorrect cable attach-

ment.

21     Reserved                   - - - -

22      Ground                    - - - -

23      /HIOW         (From Host, Active Low) Host I/O write

strobe. Edge clocks data from the host data

bus to I/O register.

24       Ground                    - - - -

25       /HIOR         (From Host, Active Low) Host I/O read

strobe. Trailing edge clocks data from host

data bus to I/O register.

26       Ground                    - - - -

27      Reserved                   - - - -

28      Host ALE       (From Host,Active High) Host address latch

enable. Used to qualify host address lines.

Host addresses are latched on the trailing

this has been changed                        edge of Host ALE. The drive does not use

this signal.

29      Reserved                   - - - -

30       Ground                    - - - -

31       IRQ14         (To Host, Tristate, Active High) Interrupt

request from drive to host.  The host may

enable/disable the interrupt by clearing/

setting the /IEN bit of the Digital Output

register of the task file.  The signal is

in a high impedance state when the drive is

not selected or when the /IEN bit of the

Digital Output register is set.  The signal

is cleared when host performs a status read

from drive.

32       /HOST         (To Host, Tristate, Active Low) When active

I/016         it indicates to host that the 16-bit Data

register is addressed and the drive is

ready to send/receive a 16-bit word.

33        Host         (From Host, Active High) Host I/O address

ADDR1         line 1.

34      /PDIAG         (Active Low) Passed diagnostics.  Used by

Slave to signal to Master drive that Slave

has passed its internal diagnostics.  See

note 3.

35       Host          (From Host, Active Low) Host I/O address

ADDR0          line 0.

36       Host          (From Host, Active High) Host I/O address

ADDR2          line 2.

37      /HOST          (From Host, Active Low) Host I/O chip

CSO           select decoded from host address lines.

When active, one of the registers in the

range 01F0HEX through 01F7HEX is selected.

38      /HOST          (From Host, Active Low) Host I/O chip

CSI           select decoded from host address lines.

When active, one of the registers in the

range 03F0HEX through 0347HEX is selected.

39      /HOST          (To Host, Active Low) Dual purpose pin.

SLV/ACT         When drive is Slave (SLV), this pin is used

during a Diagnostic command to signal to

the Master that a Slave is present.  Drive

Activity to host: It is active when the

drive is executing a command.  May be used

by host drive an activity LED.

40      Ground                     - - - -

Notes:

1. Signal beginning with (/) is active low.

2. Reserved and Ground pins do not have directions.

3. /PDIAG and Host/SLV are used for communication between

Master and Slave.

=============================

XT Interface

(40-pin cable)

-----------------------------

1.0 XT HOST/DRIVE INTERFACE PIN ASSIGNMENTS

PIN        SIGNAL      DESCRIPTION

__________________________________________________________________

1           RES       (From Host,Active High) Bus Reset signal

3,5,7,9     DATA BUS     Host data to/from host.8-bit tristate,bi-

11,13                   directional data bus between host and drive.

15,17                   Used for transferring Status, Data and

Control information.

19           GND       Ground

20           Key       An unused pin, which is clipped off at the

drive to prevent incorrect cable attachment.

21           AEN       (From Host, Active High) Host address

enable, which is asserted during a DMA

cycle to disable the decoding of I/O port

addresses.

23          /IOW       (From Host, Active Low) Host I/O write

signal for writing data to an I/O port

address.

25          /IOR       (From Host, Active Low) Host I/O read

signal for reading data from an I/O port

address.

27          /DACK      (From Host, Active Low) DMA acknowledge

signal asserted in response to the DMA

Request signal. This signal enables DMA

data transfer when either /IOR or /IOW

signals are active.

29           DRQ       (To Host, Active High) DMA Request as-

serted by the drive to initiate a DMA

transfer.

31           IRQ       (To Host, Active High) Interupt Request

asserted by the drive to cause an interrupt

to the host.

33,35        SA1,SA0     (From Host, Active High) Host I/O address

lines 0 and 1 for selecting the drive's

I/O ports.

37           /CS       (From Host, Active Low) Card Select signal

asserted by the host to address the drive's

I/O ports.

39        /ACTIVE      (To Host, Active Low) Drive Activity signal

asserted by the drive when it is processing

a command.  Can be used to drive an external

LED indicator.

NOTE:  All other even-numbered pins are ground.