TUTORIAL.DOC

============

RUNNING FHDX

The FHDX disk contains the following files:

FHDX.PRG    factory hard disk utility program

FHDX.RSC    fhdx GEM resource file

TUTORIAL.DOC    this tutorial

README.DOC    a functional description of the program

WINCAP    configuration file for hard disk controller test,

daisy-chain test, format, partition.

Double-click on FHDX.PRG to run the hard disk  utility.

Move  the  mouse  cursor to the menu heading "Disk" and

select controller test, daisy-chain test, format  disk,

input  defect list, burn-in test, or ship.  Please note

that these operations should be performed in the  above

order.

April 13, 1989

- 2 -

CONTROLLER TEST

Click on the appropiate unit number and disk type. When

the  test  is finished, the screen will either turn red

if the disk failed  the  test,  or  stay  green  if  it

passed.   Hit  'Return'  or click on the message box in

the middle to get back to the main menu.

April 13, 1989

- 3 -

DAISY-CHAIN TEST

Click on the appropiate unit number and disk type. When

the  test  is finished, the screen will either turn red

if the disk failed the test (the message in the  middle

of  the screen will tell you whether the disk failed on

a write error,  a  read  error  or  a  data  comparison

error),  or  stay  green if it passed.  Hit 'Return' or

click on the message box in the middle to get  back  to

the main menu.

April 13, 1989

- 4 -

FORMAT DISK

A dialogue box with  available  drive  types  (such  as

MEGAFILE  60)  will  come up on the screen.  Select the

drive type by clicking on the  box  showing  the  model

number  of  the hard disk drive.  Click on OK to format

the disk, or click on CANCEL to abort.

Please note that the disk will be partitioned after  it

is  formatted.   The  default  partition schemes are as

follows:

MEGAFILE 60:

MCS5068:  16Mb - 16Mb - 16Mb - 16Mb

MSB5064:  15Mb - 15Mb - 15Mb - 15Mb

NO LABEL: 15Mb - 15Mb - 15Mb - 15Mb

MEGAFILE 44:  11Mb - 11Mb - 11Mb - 11Mb

MEGAFILE 30:  10Mb - 10Mb - 10Mb

SH205:  4Mb - 6Mb - 10Mb

SH204:  4Mb - 6Mb - 10Mb

April 13, 1989

- 5 -

INPUT DEFECT LIST

(1)  If a defect list already exists, a  dialogue  box  will

come  up  on the screen and ask if you want to keep the

defect list that already exists.  Click on Yes to  keep

the  current  list.   Click on No to ignore the current

list.

(2)  A dialogue box will come up on the  screen  to  inquire

the  type  of  tester  used  to  generate  the  defects

recorded by the hard disk vendor.  Select  MFM  or  RLL

and then click on Start to begin inputting, or click on

Cancel to abort.

(3)  A form will come up on the screen with blanks for  Head

number,  Cylinder  number and Byte offset.  Fill in the

blanks with the  appropiate  number  from  the  defects

recorded  by  the hard disk vendor.  To move the cursor

through the entries, use the 'Tab' key or  the  up/down

arrow  keys.   Click on Next (or hit 'Return') to enter

the next defect, or click on Done when you are finished

inputting.

(4)  A dialogue box will come up on the  screen  to  inquire

what  to  do  next.   You  should click on 'Examine the

defect list' if you have just  finished  inputting  the

defects.   Click  on 'Save the defect list' to save the

defects and leave.  Click on 'Quit without  saving'  to

leave without saving the defects just entered.

(5)  If you have clicked on 'Examine  the  defect  list'  in

step  4,  a  window will come up displaying the defects

that you have just entered.  Examine each  entry  care-

fully.   If not all of the defects are displayed, click

on the up/down arrows or the scroll  bar  to  move  the

content in the window up or down.

(6)  If you find an incorrect entry, click on the entry.   A

form  will  come  up with the entered numbers.  Correct

the wrong number(s).  Click on OK to save the corrected

numbers, click on Delete to remove that entry, or click

on Cancel to ignore the correction(s).  Deleted entries

will be disabled in the defect list window.

(7)  After examining all entries thoroughly,  click  on  the

close box on the upper left hand corner of the window.

(8)  A dialogue box similar to the one in step 4  will  show

up.  Click on the action you want to take (see step 4).

April 13, 1989

- 6 -

BURN-IN TEST

(1)  The burn-in test begins as soon as  you  click  on  the

menu entry.

(2)  A dialogue box will come up on the screen informing the

progress  of the test.  To suspend the test, hit <ctrl-

s>.  To  resume  the  test  after  suspending  it,  hit

<ctrl-r>.  To quit the test, hit <ctrl-q>.

(3)  After you quit the test, the screen  will  either  turn

red  if  the  disk failed the test, or stay green if it

passed.  Hit 'Return' or click on the  message  box  in

the middle of the screen to exit.

(4)  A window will then come up displaying the  defects  (in

form of head number, cylinder number and sector number)

that have been logged on the drive.  If there are  more

defects  than  the  window  could  display at one time,

click on the up/down arrows or the scroll bar  to  move

the  content  in  the  window up or down.  Click on the

close box on the upper left hand corner of  the  window

to get back to the main menu.

April 13, 1989

- 7 -

SHIP

An alert box will come up on the screen to  remind  you

to  turn  off  and  disconnect your hard disk after the

disk heads are parked.  You MUST switch off  the  power

of the disk once its heads have been parked.

April 13, 1989