This little program is also part of the new DEBJAG.IMG.

Currently there's only an uploader for TOS-based machines, but the protocol

is very simple so ....

Up-Loading :

Note: Allways sending longs (32bit).

1) Send magic $22071970.

2) Send start address (which is also used as destination address)

3) Send length in bytes (be sure its a multiply of 4 !!)+4 (for the checksum)

4) Send data (as longs !)

5) Send checksum long, which is; check = -SumOfData

6) The program will automatically start, if not there's a checksum error

Sending a long:

1) Wait for BUSY to be low => inactive

2) Send LSB

3) Set /STROBE to low => active

4) Wait for BUSY high => active

5) Set /STROBE to high

6) continue for the next three bytes

If you get checksum-problems, maybe the time between (3) and (4) is to short.