;----------------------------------------------------------------------------

;File name:	NEWSTUFF.DOC			Revision date:	1996.02.08

;Revised by:	URAn				Revision start:	1995.07.29

;----------------------------------------------------------------------------

;Original version:	STUFFER  ver: 0.9

;Revised version:	NEWSTUFF rev: 1.3 Beta

;

;NB:	This revision is released as freeware effort, which does not affect

;	the original copyright claim, included with the original document.

;	That (unchanged) document is contained further on in this file.

;----------------------------------------------------------------------------

;Revision purpose:

;

;  1:	Make the program compatible with modern TOS versions.

;	This is achieved.

;	Tested with TOS 1.04, TOS 2.06, MagiC 4.01

;

;  2:	Eliminate some bugs due to incorrect handling of vectors

;	and GEM functions (EG: zeroed 'work_in' for v_opnvwk).

;	This has largely been achieved, though some may remain.

;

;  3:	Implement XBRA protocol for all vector bending.

;	This is achieved.

;	Only BIOS and XGEMDOS vectors are bent,	with XBRA identifier 'Stuf'.

;

;  4:	Improve vector bending methods (aided by XBRA) so that no

;	vectors are bent into NEWSTUFF when resolution change occurs.

;	This is achieved, for the internal vectors.

;	Vector bending of other ACCs (from the \STUFFER\ folder) are a

;	different matter, so as yet such ACCs are not suited for NEWSTUFF.

;

;  5:	Improve exception handling, for compatibility to new 68xxx CPUs.

;	This should be achieved, but remains to be tested (I have none).

;

;  6:	Allow dynamic reloading of the \STUFFER\ folder.

;	This is achieved  (rev 1.3).

;	The (unimplemented) "Extras" button is now changed to "Reload",

;	and an alert is used to let user confirm or abort the reload.

;

;  7:	Allow NewStuff to start as non-ACC application, so as to allow

;	temporary use of ACCs without taking any of the normal ACC slots.

;	This is achieved  (rev 1.3).

;

;  8:	Allow access to the NewStuff popup from programs that have no menu,

;	or have disabled accessories in the menu.

;	This is achieved  (rev 1.3).

;

;----------------------------------------------------------------------------

;About revision 1.3:

;

;	The "Extras" button has been replaced by a "Reload" button, which

;	calls up an alert which allows you to confirm or abort the reload.

;	If you confirm, all 'stuffed' ACCs will be removed from RAM,

;	whereafter any accessories in the \STUFFER\ folder will be loaded.

;

;	NewStuff.Acc may now be renamed to NewStuff.Prg or NewStuff.App,

;	and then started as a normal (non-ACC) application.

;

;	NewStuff never creates a menu line, to avoid conflict with other

;	applications in Magic, and the popup can be activated by a 'hotkey'

;	combination (Altternate with Left_shift) if no menu line is active.

;

;	Another 'hotkey' combination (Alternate with Escape) is used to

;	exit from NewStuff when it was started as non-ACC application.

;	It is of course disabled when NewStuff is started as an accessory.

;

;	Both hotkeys will also work in singletasking TOS, which is useful

;	to reach NewStuff from GEM applications that do not allow ACCs.

;

;	The 'hotkey' codes reside at bytes 4-11 in the running code, which

;	corresponds to bytes 32-39 of the program file.

;

;	The codes consist of 4 words organized as follows:

;

;	Filepos  Purpose  Patch value

;	-------  -------  -----------

;	 32-33    Popup   Low 4 bits = the bios Kbshift value for 'hotkey'.

;	 34-35    Popup   Hi_byte = Key number or 0.  Lo_byte = ascii or 0.

;	 36-37    Exit    Low 4 bits = the bios Kbshift value for 'hotkey'.

;	 38-39    Exit    Hi_byte = Key number or 0.  Lo_byte = ascii or 0.

;

;	If a nonzero key number is used, the pressed ascii value is ignored.

;	If a zero key number is used, the pressed key number is ignored.

;	If key number and ascii are both zero, the Kbshift value is used

;	without requiring any other key to be pressed.

;

; NB:	If some other GEM program reads keypresses, a 'hotkey' with settings

;	using nonzero key number or nonzero ascii may be blocked.

;	This will never happen if those settings are both zero, since the

;	Kbshift value can be tested without calling aes function evnt_keybd.

;	This is why my default popup setting is Alternate+Left_Shift.

;

;----------------------------------------------------------------------------

;Future plans:

;

;  1:	Loading/dumping of individual ACCs, limited only by available RAM.

;

;  2:	AV-protocol interface for the loaded ACCs.

;

;  3:	Vector guardian for the loaded ACCs, to avoid rez-change problems.

;

;----------------------------------------------------------------------------

;Possible problem:

;

;	If any program (even a stuffed ACC) steals the XGEMDOS vector

;	without XBRA protocol, NewStuff is going to bomb !!!

;	This is not a problem with NewStuff, but a symptom of that other

;	program's serious BUG !!!

;

;IMHO:	There is no more serious bug in existence, than stealing vectors

;	without using XBRA protocol to tell the system what is going on.

;

;----------------------------------------------------------------------------

;The original documentation of STUFFER v0.9 follows immediately below

;----------------------------------------------------------------------------

DC STuffer is copyright 1988 Double Click Software.

DC STuffer is a SHAREWARE program.  ONLY *YOU* can insure

revisions and continued support from Double Click Software.

If you would like to receive version 1.0 of the DC STuffer

desk accessory, program, and complete documentation, please

send a contribution to:

DC STuffer

Double Click Software

P.O. Box 741206

Houston, Texas 77236-1206

DC STuffer programmed by Keith Gerdes.

DC STuffer allows you to load in more than six desk accessories (DAs) into

GEM.  DC STuffer acts just like the desktop, you can open up any desk

accessory, and if it does not take complete control of the desktop

(or the program you are running) you can go to DC STuffer again to open

another desk accessory.  This version has a limit of 32 DAs.  Version

allows you to load or unload DAs at will without the need to reserve memory

for the DAs.

INSTRUCTIONS for version 0.9

============================

We hope you enjoy using DC STuffer.  Here is how to use version 0.9:

Simply place STUFFER.ACC with your normal boot-up DAs.  Any DA you wish

to be loaded by DC STuffer must be placed in the folder STUFFER which

MUST be in the ROOT directory of your boot disk.

This version limits you to loading 32 DAs.  If you wish to abort the

loading, press the RIGHT SHIFT button at any time before or during the

loading sequence.

DC STuffer will ALWAYS leave at least 128K of memory for the operation of

programs and the desktop.  If the load stops prematurely, you have either

loaded in 32 DAs, or have 128K or less left of memory.

When you select DC STuffer from the DESK menu you will be presented with

either 0-20 DA names, or 18 DA names and arrows to scroll through the DA

names, in a drop-down menu similar to the desktop's menu.  The DAs will be

highlighted similar to the desktop menu.  When the DA you want to activate

is highlighted, press the left mouse button.

Pressing the right mouse button while at the DC STuffer menu will exit

DC STuffer, as will moving the mouse into the menubar area, or clicking

outside of the drop-down menu.

You can load up to 20 DAs and access them at a glance when DC STuffer is

selected.  If, however, you load in more than 20 DAs, you will see 18 DAs

and up and down scroll arrows.  Click on the arrows to go up or down in the

DA list.  If you can no longer go in the desired direction, the arrows will

be disabled and not selectable.

EXTRAS - this is not allowed in this version, but allows the ability to

load and unload accessories at will.

ABOUT... - Tells you a little about DC STuffer.

NOTE:

=====

Certain precautions are in order.

1) Changing resolution will consume more memory.  Version 1.0 does not have

this limitation.

2) Changing resolution is fatal if certain DAs are loaded.  These DAs will

usually crash if you change resolution while they are loaded from the

desktop (normal load).  This is usually because they are stealing

vectors from the OS, and since DAs are unloaded when a resolution change

occurs, the vector is left pointing to blank memory.

Version 1.0 has a means for alleviating this problem, and isolating and

reporting which DAs steal vectors.

3) We have trouble with a few DAs with v0.9, which are fixed in v1.0

JUGGLER II - if the program is not loaded, the DA hangs, and so

does STuffer.

TURBO ST - you cannot change rez with this (steals vectors)

ST DOCTOR - you cannot change rez with this (steals vectors)

We have fixed the Diablo Emulator problem in the 1.0 version.

4) Please report any problems with DAs to us at:

CompuServe      76356,2510

GEnie           M.VEDERMAN2

UseNet          uace0@uhnix2

BITNET          UACE0@UHUPVM1

HASTE           1-713-973-6555

5) Buy SHADOW, the TRUE background file transfer program.  Never will you

get a system slowdown while transferring files when you use SHADOW.  If

you do not own SHADOW, you owe it to yourself to buy a copy and SAVE

yourself VALUABLE TIME while transferring files.  SHADOW works with

GEM and TOS programs, and even survives resolution changes and RESETS

without stopping the file transfer or losing data.   SHADOW is available

from ANTIC for only $29.95!!  Ask for it by name!

6) Please support our SHAREWARE efforts as well.

;----------------------------------------------------------------------------

;End of file:	NEWSTUFF.DOC

;----------------------------------------------------------------------------