-----------------------------------------------------------------------------

File name:	INTERNAL.DOC			Revision date:	1998.12.05

Created by:	Ulf Ronald Andersson		Creation date:	1998.10.29

-----------------------------------------------------------------------------

Content:	Technical information on the NetD master server for STinG

-----------------------------------------------------------------------------

Each server must initialize most elements of its NetD_SRV struct before it

may call the ND_init_SRV function. Empty queues must be initialized with

a NULL pointer. Elements that should be initialized to non-NULL values are:

ND_SRV_connect,  ND_SRV_traffic,  ND_SRV_warning

They must all point to valid function subroutines in the server, except that

ND_SRV_warning may optionally be NULL, if the server code doesn't need

that function. Some other struct entries must be initialized to NULL, since

NetD STX makes a safety check of that.  Bad values => refused services...

The elements that must be set to NULL are:

ND_SRV_next,  ND_SRV_listen_q,  ND_SRV_turnon_q,  ND_SRV_active_q

Additionally, the header elements from ND_SRV_magic through ND_SRV_name

must also be initialized with appropriate constants.  (See code examples.)

For most servers all the initialization can be handled by assembly-time

constants, so this should not affect code complexity.

Each server must call ND_init_SRV before using any other NetD calls, so

NetD STX can access the server structures properly.  ND_exit_SRV serves

the opposite purpose, and should be used when the server no longer wants

any contact with NetD (eg: if server program is to be terminated). After

ND_exit_SRV the server may still call ND_init_SRV again, if it decides

that it does want services resumed, but it must then start from scratch,

just as it did in its earlier contacts with NetD.

As shown, each server structure contains three pointers to connection queues.

The NetD STX will allocate and update these queues automatically in response

to traffic, and in response to other calls made by the server.  Note that it

is not legal for the server itself to add or remove any entries, since that

could confuse the implementation.  The server should only affect the queues

indirectly, by the effect of calling NetD...SRV and NetD...CON functions.

This is normally done as described below:

First the server must reserve connection services for the needed ports, by

calling ND_init_CON once for each port to be serviced.  NetD STX then

calls STinG to open a listening connection, allocates a new NetD_CON struct,

and initializes that with data from the server plus the connection handle.

The pointer to the struct is then linked into the first queue of the server,

which is for passive connections awaiting incoming calls.  Internally the

STX repeats this process twice, so as to be ready for incoming call conflict.

The server may later call ND_exit_CON, to stop service of a given port.

NetD STX will then remove the NetD_CON struct from the queue, close the

associated STinG connection, and release the NetD_CON struct RAM. Internally

this is also done recursively, so as to release 'extra' connections used to

be ready for incoming call conflict.  Note however that this only applies

to connections in the 'listening' or 'activating' stage.  Connections that

have already been established must instead be closed individually by using

the ND_kill_CON function. So it is legal to 'close shop' for future service

without affecting connections already being serviced.

When STinG completes an incoming connection to one of the ports serviced,

NetD STX will notice this in the next call to its TIMER_call handler, and

it will then reopen a new passive connection, so that further calls to the

same port can be received. Then it moves the NetD_CON pointer of the old

(now activating) connection to the second queue of the server, which is for

activating connections. It then proceeds to allocate a new NetD_CON struct

for the listening queue, copying all data from the old struct except the

handle, which is naturally set to that of the new listening connection.

Then the new struct is linked into the listening queue, which completes

the structure administration for the incoming call on TIMER_call level.

At each GEM timer event NetD APP will call the ND_exec_APP function in

NetD STX.  One of the things this leads to is that NetD STX then calls

the ND_SRV_connect function of the server associated with each entry

in the aforementioned activation queue, whereafter each entry is moved

from that queue into the third queue, for fully active connections.

Further, in the GEM timer events described above, NetD STX will also call

the ND_SRV_traffic function of each server which has one or more active

connections (one call per server, not per connection).  That will proceed

until all servers have closed all their active connections, which they

must do by calling the ND_kill_CON function.  NetD STX will then call

UDP_close/TCP_close as needed, remove the pointer from the server queues

and release the NetD_CON struct RAM.

The ND_exit_SRV function may be used to abort all traffic of a server,

since it has code that purges queues of the server, closing connections

and releasing RAM as needed, and finally removes the server struct pointer

from the server queue of NetD STX.

Inside the traffic function of simple servers it is often convenient to

deal with connections without regard for whether these use TCP or UDP.

(That works fine for protocols using short messages.)  To simplify this

the STX provides the ND_send_CON function which can be called by a server

without having to test for TCP or UDP.

There is one more vectored function of servers not described in the above,

and that is the ND_SRV_warning function.  This will be called for all

servers installed whenever NetD APP calls the ND_exit_APP function in

NetD STX.  This should mean that NetD APP is about to terminate, so that

no further traffic will be possible.  To servers, the ND_SRV_warning

call tells them that traffic will soon become blocked.  Normally they

should then close any active connections by calling ND_kill_CON, but

it is also permitted to call ND_exit_SRV instead, as a much simpler way

of cleaning up all NetD structures related to a server.

Note that it is not mandatory for a server to terminate just because the

NetD APP does so.  It is possible that the NetD APP will be restarted

again later on, and then services will recommence automatically for any

servers that did not terminate (eg: TSRs can't really terminate).  This

kind of operation will be quite normal when switching resolutions, since

all APPs/ACCs are terminated for GEM resolution switches.

STinG itself is not affected by GEM limitations, and neither is NetD STX,

so if care is taken servers may even proceed with traffic after a switch

as described above, with no other effect than a brief halt in transfers.

-----------------------------------------------------------------------------

End of file:	INTERNAL.DOC

-----------------------------------------------------------------------------