-----------------------------------------------------------------------------

File name:	WPTOSFIX.DOC			Revision date:	1999.03.25

Created by:	Ronald Andersson		Creation date:	1999.03.22

Copyright:	(c)1999 Ronald Andersson	Released as:	FREEWARE

-----------------------------------------------------------------------------

Release notes:

This document file as well as the program binary of WPTOSFIX are released

as FREEWARE, without limitation on use or distribution.

The sources too are released as FREEWARE, again without limitation on either

distribution or use, including even blatant 'ripoff'...  :-)

Centek in particular are encouraged to distribute this program to users of

their CT2B board (for which this program was written), until such time as

they find another solution for the problem that it eliminates (see below).

-----------------------------------------------------------------------------

Program purpose:

When booting a Falcon equipped with CT2B in that accelerator's "TOS 4.x mode"

or in its "full compatibility mode", there is a problem with decoding of the

TOS ROM area.

On a standard Falcon a bus error exception will be generated when a program

attempts to write any data to the TOS ROM address range.  No such exception

is generated if the same thing is done with CT2B installed, and it has been

booted in either of the two modes mentioned above.

Some software depend on standard bus error generation to detect whether TOS

and/or GEM reside in RAM or in ROM (both can be relocated and/or remapped).

Such software does not work correctly with CT2B, due to missing hardware

generation of this bus error condition.  WPTOSFIX.PRG fixes that !

This fix is needed (for example) to use such programs as GEMRAM in either of

the affected CT2B boot modes, and GEMRAM (or eqiv.) is needed by WINX and

SHBUF.  (Some enhancements I always use when using singletasking TOS.)

-----------------------------------------------------------------------------

Suggested usage:

WPTOSFIX.PRG is intended for placement in the \AUTO\ folder, preferably so

that it starts immediately after your boot manager, if you use one. If you

do not, then it should be placed so that it starts before any of the other

\AUTO\ programs that may need it (eg: GEMRAM).

-----------------------------------------------------------------------------

What WPTOSFIX does:

First a check is made to see if the prgram is running on a Falcon equipped

with a 68030 CPU (cookies "_MCH" and "_CPU").  If this test fails on either

cookie, no remapping is done.

Then an extensive check of the PMMU registers and the first two levels of

the active PMMU map is made.  If any differences are detected from the

map or PMMU registers used by a CT2B boot in "full compatibility mode" and

in "TOS 4.x mode", then no remapping is done.

If all tests match however, then the page descriptor (stored at $000007F8)

used for access to TOS ROM (either as $00Exxxxx or as $FFExxxxx) is given

a write protect bit.  This is all that is altered...

The program will display a message telling its name and version number,

as well as whether or not any remapping was done. After that the program

will terminate.  Since the PMMU map used resides in reserved system RAM

and thus not in the program, this program does not remain resident like

many other PMMU manipulating programs have to. It terminates completely.

-----------------------------------------------------------------------------

Extra features:

At entry to WPTOSFIX it checks the Kbshift keys, and uses these to control

some extra features.  If the right shift key was pressed then the program

will jump to a primitive but useful hexdump program, with initial display

set to a buffer containing PMMU register values that were active before

any remapping was done.  The storage order is:

offset from start  bytes  Content

-----------------  -----  -------

0             4      TC

4             4      TT0

8             4      TT1

12             8      CRP

20             8      SRP

28            ...     end of register table

If all of the keys <Alternate> <Control> <l-Shift> <r-Shift> were pressed

at entry to WPTOSFIX, then the program jumps directly to the hexdump code

described above, without having changed any PMMU register or any map data.

This is useful to get information about mapping that it could be dangerous

to alter without sufficient knowledge.

Inside the hexdump program some simple key commands are available:

Key  Action

---  ------

A   select logical Address for a block of data to display

N   scroll to the Next block of data

P   scroll to the Previous block of data

E   Edit a single byte at any address (doesn't have to be on display)

Esc  Escape from the hexdump utility, whereafter WPTOSFIX exits

Each display block is exactly 128 bytes, but if you need to inspect single

bytes you can use the E command.  It will show old value and then ask for a

new value.  If you then press Escape the value will not be changed.

-----------------------------------------------------------------------------

What is missing:

The sources are not complete for reassembly, since they rely on some of my

macro libraries for bindings to system functions, as well as for some other

things.

Anyone interested in these macro libraries should get in touch via email to

request a copy.  They are not yet released on my homepage, except for some

very few and very old files included with some of my older softwares.

Those are not comparable to my _real_ libraries.

-----------------------------------------------------------------------------

Contact addresses:

mailto:		dlanor@ettnet.se

homepage_1:	http://www.ettnet.se/~dlanor/

homepage_2:	http://dlanor.atari.org/

Actually I only use one mailbox as well as a single homepage site, but the

alias may remain useful if I ever change to another ISP (not likely). If

that happens, the 'dlanor.atari.org' redirection will be changed so as to

lead to the new homepage, which will always give a useful email address.

-----------------------------------------------------------------------------

End of file:	WPTOSFIX.DOC

-----------------------------------------------------------------------------