------------------

The New GCR Tester

------------------

version 1.1

------------

INTRODUCTION

------------

Since many of people had problems with the "hi dan" tester built in

to the Spectre 2.3 software, we have put together a new "user-friendly"

GEM-based GCR tester.  The purpose of this tester is to test the ability

of the GCR to perform Mac/GCR floppy disk operations.  It won't check your

ROMs, nor will it test any functions of the Spectre software.

***** CHANGES IN VERSION 1.1 *****

1. You are now warned to insert a blank disk before a GCR format is about

to begin.

2. If an error occurs in the "Drive Test" on side 2, the error dialog is

now correct.

3. This documentation now includes a new section entitled 'WHAT TO DO IF

THE TEST FAILS' to help you determine the cause of any problems.

-------------

HOW TO USE IT

-------------

Make sure GCRTEST.PRG and GCRTEST.RSC are in the same folder as

SPECTRE.PRG and DRVR128.SYS.  Run GCRTEST.PRG by double-clicking on it (or

selecting it and choosing "Open" from the Desktop's "File" menu).

--------

THE MENU

--------

File

Quit            - quits the GCR tester

Tests

Complete Test   - performs all but the Drive Test

Drive Test      - check out the drive with standard ST functions

Format          - formats the disk with the GCR

Write           - writes to the disk with the GCR

Read & Verify   - reads the disk with the GCR and compares to data

written by the "Write" option

Read            - reads the disk with the GCR and doesn't check the

data

Options

Drive A         - selects drive A as the test drive

Drive B         - selects drive B as the test drive

Side 1          - test side 1 of the disk

Side 2          - test side 2 of the disk

Quick Test      - causes GCR tests to only check every 8th track

----------------

ADDITIONAL NOTES

----------------

The GCR tester does not check for the existance of a GCR.  If you don't

have a GCR installed, the machine may lock up.  This is done this way so

that you can check a GCR without ROMs installed (ROMs must be installed to

detect the presence of a GCR).

Using the "Side 1" and "Side 2" options allows you to check either side

individually of the other.  Be aware though that the GCR "Format" option

only checks the "Side 2" option to determine if the disk should be double-

or single-sided (i.e. if "Side 2" is selected, BOTH sides of the disk will

be formatted).

The "Quick Test" option causes the GCR "Write", "Read & Verify", and

"Read" to only check every 8th track.  This can speed up a basic checkout,

though a full checkout should be done to ensure everything is up to par.

"Read & Verify" is only useful after doing a "Write", since it checks

the data that it reads back against what it wrote.  Use "Read" to check if

a disk is readable when the contents are "unknown" (i.e. to check out a

real Mac disk).

The "Complete Test" option performs the following: "Format", "Write",

and "Read & Verify".  Again, the Quick Test option only affects the "Write"

and "Read & Verify" operations.  Note that the "Drive Test" is not performed

so you should do that before hand.

The "Drive Test" operation formats a series of tracks on the disk in

ST mode in a random order, then writes to them, then reads them back and

compares the data.  Once this is done, the drive speed will be checked.

This will take approximately 1 minute to complete.

The display for the GCR reads and writes consists of 5 columns of 16

boxes.  Each of these boxes represents one track of the disk.  Each box

contains 2 characters, one for each side of the disk.  As the tests progress,

the characters change.  The characters are as follows:

"-" - this track/side has not been checked

"V" - this track/side is being verified (after format)

"W" - this track/side is being written

"R" - this track/side is being read

"+" - this track/side passed the test

"1" - side 1 of the disk failed

"2" - side 2 of the disk failed

Each column of the display represent each of a Mac disk's "speed zones"

and are important for determining what is causing a problem.  Whenever the

test gets to the top of a column, the GCR will be changing the speed of its

oscillator.

----------------------------

WHAT TO DO IF THE TEST FAILS

----------------------------

If the "Drive Test" fails, try rerunning the test without the GCR

connected at all.  If the test still fails, take you computer and/or drive

into an Atari service center to be repaired.  If the test passes, you have

either a defective GCR or floppy cable.

If the test fails completely (it skips the format and/or fails on all

tracks), the first place to look is at the cable connections.  You may also

have a defective GCR or disk cable.

If the test fails only on the inner tracks (in the 9 or 10 secs/trk

column), there are a couple of possibilities:

1. If the drive is an internal drive (especially on Megas), you may

need to add additional shielding to the drive to block out

interference from the internal power supply.  You should contact

us to get documentation on how to accomplish this.

2. If the drive is a third-party (non-Atari) drive, you should check

to see if the drive mechanism is made by NEC.  If so, it will

probably have to be modified to work properly with Mac disks.

Please contact us for more information.

If the test fails at seemingly random locations, you should double-check

all cable connections to make sure they are snug.  You should also try moving

your monitor away from the drive to see if that makes a difference.  If that

doesn't help, you should try adjusting the potentiometer in the GCR.  The

potentiometer is the little blue square thing on the GCR board.  There's a

little "nub" on it showing where it's set.  It should initially be set to

the "12 o'clock" position (back towards the cable connectors).  Try running

the test with the potentiometer set to the 3 o'clock and 9 o'clock positions

and see if the results improve.  If you see an improvement, you should be

able to continue adjusting the potentiometer until you reach a positions

which works all the time.

If the computer locks up entirely when trying to format, read, or write

Mac disks, you probably have a defective GCR, which you can return to us

for testing and repair/replacement.

---------------

IMPORTANT NOTES

---------------

The GCR serves 2 functions:

1. It holds the Mac ROMs.

2. It performs Mac/GCR disk operations.

It's also important to note that these 2 functions are completely

independent.  If the Mac/GCR disk portion of the GCR is not working,

the GCR will still function the same as a Spectre 128 (and work with

Spectre floppies and hard disks).  If you don't have ROMs installed, or

the ROMs are failing (i.e. you get a "bad ROMs" message when trying to

switch into Mac mode), the GCRTEST should still run correctly.

The following are things the GCR cartridge does NOT do:

1. It does NOT read/write Spectre, ST, or MS-DOS disks.  Those operations

are handled by the floppy disk controller in the ST.

2. It does NOT run Mac software.  Those operations are handled by the

Spectre software (in conjunction with the ROMs on the GCR).

3. It does NOT handle hard disk access.  Those operations are handled by

the DMA chip in the ST.

---------

THAT'S IT

---------

Hopefully this will help some of you narrow down (or eliminate) some of

the problems that you may be encountering.  If you have any questions, feel

free to leave EMail on GEnie or CompuServe or call us at Gadgets.

Copyright 1989,1990,1991 Gadgets by Small, Inc.  All Rights Reserved.

Gadgets by Small, Inc.

40 W. Littleton Blvd, #210-211

Littleton, CO  80120

(303) 791-6098  Fax: (303) 791-0253