======================================================================

A memory test program for the Atari ST (tm) and Mega (tm) computers.

======================================================================

Version 1.0, 02-jan-1990

Text changed (shareware fee dropped, restrictions relaxed) 08-nov-1995

OVERVIEW

This text documents a program to test the random access memory of

Atari ST and Mega computers. The text consists of a discussion of

state-of-the-art random access memory testing in general followed by

some considerations about memory testing on the Atari ST and Mega

computers in particular and a description of the program. The program

and this text are NOT in the public domain. We encourage you to spread

copies of this text and the program to your friends and to electronic

bulletin board systems. However, we insist that this text and the

program are NOT CHANGED IN ANY WAY and that the program is always

DISTRIBUTED TOGETHER with this text.

Peter Knoppers, Harm Salomons.

WHY MEMORY TESTING ?

Some defects in computer memories result in immediate failure of the

computer system. Others are more subtle and can remain unnoticed for

a long time. Defects in computer memories can indeed be very hard to

find and the effects of a defect can be very expensive. In recent

years memory testing has transformed from an art into a science. Many

computers do not execute a memory test after power-on, or use one with

very questionable fault detection capabilities.

WHEN SHOULD YOU TEST MEMORY ?

Memory testing is performed to identify a failing memory chip or to

prove that there are no faults present. (As will be explained below,

absolute proof of absence of faults is unattainable.) In general, when

something about the memory configuration of a computer has been

changed it is advisable to test all memory of that computer. If a

computer system has sporadic malfunctions, the memory is usually a

good place to start looking for the cause. Subtle faults in the memory

of your computer can damage the integrity of your files which may be

very expensive. In the absence of parity check (or a similar hardware

mechanism), you should test memory as often as you should make backups

of your files.

If your computer has parity check or a similar mechanism that checks

for errors on every read operation, you have in effect a continuous

memory test running in your computer. All professional computer

systems use such a mechanism. In this sense the Atari ST and Mega

computers are not professional computers...

WHAT IS MEMORY TESTING ?

In general, testing is performed to verify that a system performs as

specified. A specification for a computer memory could be:

A system capable of storing all conceivable patterns consisting

of a fixed number (called the capacity) of ones and zeros for a

certain length of time and which allows for retrieval and update

of any part of the pattern of ones and zeros without affecting the

remainder of the stored pattern.

The definition suggests that to test a memory with a capacity of n

bits, one has to write all the 2**n different patterns in it, and for

each such pattern apply all possible transitions to other patterns.

Assuming that the memory is written with one bit at a time, there are

n possible transitions to be tested for each pattern, which results in

a total of n*2**n transitions that must be verified in order to prove

that the memory is free of defects. After each transition, the

contents of the entire memory must be verified (hoping that a read

operation does not influence the stored pattern). Clearly, this would

result in an astronomical number of read and write operations (taking

up excessive test time) even for very small values of n.

REDUCTION OF THE TEST TIME

When implementation aspects of computer memories are considered, it is

possible to identify large classes of faults that are extremely

unlikely to occur. For instance, if a memory is constructed from a

number of identical chips, it is unlikely that the contents of one of

these chips influences the behaviour of another. On a smaller scale,

one can argue that the contents of one cell on a memory chip can not

influence the behaviour of other cells on that memory chip, provided

that those other cells are separated from the first cell by a certain

minimum distance. All the cells that lie within this minimum distance

constitute the neighbourhood of the cell. Because the cells are

usually arranged in a rectangular grid, the neighbourhood of a cell

usually consists of the four or eight cells that lie adjacent to the

cell.

TESTING FOR NEIGHBOURHOOD PATTERN SENSITIVE FAULTS

There are known algorithms that can test for neighbourhood pattern

sensitive faults. Unfortunately, these tests are not generally

applicable, because the arrangement of the cells on most chips is not

very straightforward, it varies between different manufacturers and

sometimes even within the chips of one manufacturer. This is further

complicated by the fact that real memory chips are manufactured with a

few redundant rows or columns in the cell matrix. When a defect is

found during the manufacturing process, the wiring pattern on the chip

can be altered with a laser beam, so that one of the spare rows or

columns is used instead of the one containing the defective cell(s).

This means that even chips from the same batch can have different cell

array orderings and only the manufacturer can test with confidence for

neighbourhood pattern sensitive faults.

Normally the cell array is covered with a grid of row-select, column-

select and data lines. This means that a myriad of different shorts

are possible. Many of these cause pattern sensitive faults.

When the layout of the cells on the chips is unknown it is not

practical to test for pattern sensitive faults.

TESTING WHEN THE NEIGHBOURHOOD IS UNKNOWN

The next best thing (as testing for neighbourhood pattern sensitive

faults in general is unachievable) is to test for 2-coupling faults.

A 2-coupling fault between two cells A and B is a fault where a

transition in cell A causes a transition in cell B, irrespective

of the contents of all other cells in the memory.

(The last part of this definition is the clever bit.) Each transition

can be up, or down, resulting in 4 possible types of 2-coupling faults

(up -> up, up -> down, down -> up, down -> down). The first published

algorithm designed for testing 2-coupling faults is (to our knowledge)

the SACO (Stuck At and COupling) algorithm published in [Na79]. The

SACO algorithm uses 14 writes and 16 reads per memory location. In

[Su81] the march-B algorithm was published. This algorithm detects the

same faults as the SACO algorithm, but needs only 11 writes and 6

reads per memory location.

OTHER TYPES OF FAULTS

In addition to any number of 2-coupling faults, SACO and march-B will

detect stuck-at and transition faults and address decoder faults.

A stuck-at fault is a fault where a memory cell has a fixed value.

(This fixed value is either 0, or 1).

Stuck at faults faults can simply be detected by writing and reading

all addresses with the value 0 and with the value 1.

A transition fault is a fault where a memory cell fails to make

a certain transition.

(The failing transition can be either up or down.) This fault is very

similar to the stuck-at fault, however it is possible that after

powering up the system, the cell with the transition fault is in the

state that it can not reach via a transition.

Transition faults can be detected by writing and reading all addresses

with the values 0, 1 and 0 in succession.

An address decoder fault is a fault where different addresses

access the same memory cell, or one address accesses no cell at

all.

It can be shown that SACO and March-B detect all address decoder

faults (this is not very difficult to prove).

WHAT IS MARCH-B ?

March-B can best be described in a pseudo programming language. In the

program below every read operation is assumed to signal an error when

the value read differs from the expected value. Statement grouping is

indicated by indenting (like in OCCAM).

FOR address = lowest TO highest STEP 1

write(address, 0)               # initialize all locations to 0

FOR address = lowest TO highest STEP 1

read(address)                   # if not 0: error

write(address, 1)

read(address)                   # if not 1: error

write(address, 0)

read(address)                   # if not 0: error

write(address, 1)

FOR address is lowest TO highest STEP 1

read(address)                   # if not 1: error

write(address, 0)

write(address, 1)

FOR address is highest TO lowest STEP -1

read(address)                   # if not 1: error

write(address, 0)

write(address, 1)

write(address, 0)

FOR address is highest TO lowest STEP -1

read(address)                   # if not 0: error

write(address, 1)

write(address, 0)

Each FOR loop is called a march-element.

Under the assumption that read operations do not alter the contents of

any memory cell it can be proven that there is no combination of

stuck-at faults, transition faults, coupling faults and address

decoder faults) that is not detected by this sequence of accesses.

However, the march-B test algorithm does not determine the type of

each fault and some faults may be obscured by others. (For example a

stuck-at fault in a cell that is inaccessible due to an address

decoder fault is undetectable.) Finally, one fault may be detected

(and reported) several times.

March-B is currently (1989) the state-of-the-art memory test for all

cases where the physical layout of the chips is unknown. There is no

known algorithm that can test for the more complex pattern sensitive

faults without this knowledge in reasonable time. Also, there is no

known test algorithm that achieves the fault coverage of march-B in

less time.

OTHER TYPES OF MEMORY FAULTS

There are other classes of faults that can be present in memories that

have been neglected so far. These faults are sensitive for timing,

heat, humidity, vibrations, electrical discharges, noise on the power

lines, etc. Some of these faults can only be detected when memory is

accessed at very high (or very low) speed or under special (stressed)

environmental conditions. Many of these faults are intermittent, that

is, they can not be reproduced on demand.

The reason for testing memory repeatedly for a long time is usually

the hope that an intermittent fault will be caught given enough test

time. Sometimes the environmental conditions are stressed, so the

intermittent fault may become permanent (as long as the stressed

environmental conditions last). The following table suggest some ways

to stress the environmental conditions of your computer while running

a memory test. For some conditions you may have to partly disassemble

your computer. As always, be very careful with the high voltages which

are present in and around the power supply while the computer is

switched on. All tests which involve opening your computer should only

be attempted if you are experienced in the repair of computer systems.

Stressing environmental conditions is not entirely without risk to

your computer either. If you cause damage to your computer it is your

problem, you may have stressed it too hard, or you may have had a

latent fault waiting to become permanent. The damage may or may not

covered by your warranty. We can not assume any responsibility.

condition              low budget stress method

high temperature       Use a hair drier to heat up the memory chips.

Temperatures above 100 degrees Celcius should

be avoided. You will have to open your computer

for this test to get the heat on the memory

chips. Take care, as plastic parts may not be

able to withstand much heat.

low temperature        Use a freeze spray. You will have to open your

computer for this test to get the cold right

on the chips and not in the power supply, the

floppy disk drive or the keyboard.

noise on power lines   Use a variac to vary the mains voltage of the

computer by NO MORE THAN 20 %. Disconnect your

hard disk and other peripherals that are not

necessary for this test and remove any floppy

disks from their drives. The strangest things

can happen when the supply voltage drops too

low.

other electrical noise Use a piezo electric gas lighter close to the

computer. This causes a tremendous amount of

electrical noise. If the computer stops working

immediately, you have probably disturbed the

operation of the MC68000. If a short while

after your spark a fault is reported, you may

have discovered a memory problem. If the faults

are reproducible or most of them apply to a

single bit, you have probably a fault in a

memory chip. Do not open the computer for this

test. If you have other electrical equipment

that interferes with radio reception, or a

cordless phone you could also try to operate

this equipment near your computer.

vibrations             Drop your computer (excluding any hard disks)

from one inch height onto the table

high humidity          This condition is very hard to control. You

could run the memory test program on hot, moist

days to determine if this influences the

performance of your computer.

MEMORY TESTING WITH MARCH-B FOR THE ATARI ST COMPUTER

The march-B memory test for Atari ST computer is carried out by the

MC68000 processor. This has a number of consequences:

- The test program itself must be stored somewhere in the memory;

therefore it is not possible to test all the memory at once.

- The test program must have some way to report any detected errors to

the user. If the BIOS of the Atari ST computer is to be used, some

interrupt vectors and other data in low memory locations may not be

disturbed. The screen is memory mapped; if this memory must be

tested, the screen cannot be used for output.

- Some events such as address errors, bus errors, illegal opcodes,

privilege violations, cause immediate transfer of control to an

exception service routine whose address is stored in low memory

locations. While these locations are being tested, these events will

cause the system to hang. In unmodified Atari ST computers a bus

error is generated when the processor attempts to access memory

above 4 Mega bytes. Addresses below 8 are mapped to Read-Only-

Memory. On reset the MC68000 loads the initial values for the PC and

the SSP (supervisor stack pointer) from these addresses. A write to

these addresses causes a bus error (even in supervisor mode). While

in user mode any access to addresses from 0 to 2047 causes a bus

error.

- Some memory locations can be modified by interrupt routines (for

instance mouse movement interrupts modify the contents of the screen

and keyboard interrupts add characters to a keyboard input buffer).

- When the Atari ST computer or Mega is reset, the BIOS determines

the size of the memory chips present in bank 0 and bank 1. This

information is used to program the MMU chip. A defective memory chip

may cause the BIOS to program the MMU incorrectly. This means that

the amount of memory in your system is effectively reduced by some

amount.

HOW DOES THE PROGRAM WORK ?

The approach that is used in this memory test program is to use a

menu-driven part written in C to select memory limits, etc. and a part

written in MC68000 assembler that takes complete control over the

computer to test the memory. This part disables all interrupts and

directly accesses the parallel port or the RS232 serial port to output

progress reports and detected faults. If the memory from which the

memory test program is running is defective, there is an unavoidable

chance that address errors or other violations cause the system to

hang. The only way to cope with this possibility is to output progress

reports at regular intervals.

The memory test program knows which addresses it occupies and

automatically skips these addresses when testing memory. The program

has an option to run the test for a specified number of times,

changing position in memory after each run. It actually makes sense to

run this program from different addresses, because the memory where

the program itself is positioned is accessed at the highest frequency

that can be obtained under software control. Frequent accesses to one

(or a few) addresses can cause a small part of a memory chip to heat

up which may reveal a temperature dependent problem. The user of the

program will observe that the system hangs when such a fault occurs.

To cope with this problem the program can report its position before

each run.

The program accesses 16 memory chips at once, each chip belonging to a

different data line. The march-B program in pseudo code is written for

a 1 bit wide memory. March-B can be extended for 16 bit wide memories

by replacing the values 0 and 1 in the algorithm by any 16 bit word

and the one's complement value of that word. (The one's complement of

a word is obtained by replacing all ones by zeros and vice versa.)

There is not much reason for using patterns other than 00.....00 and

11.....11, except that some faults in the data line buffers and shorts

between data lines might be detected when more exotic patterns are

used. (Real shorts between data lines would probably prevent start up

of the system.) The memory test program has the option to use a few

other patterns than 00.....00 and 11.....11, it can even select a

pseudo-random value for each successive run.

HOW TO USE THE MEMORY TEST PROGRAM

The program is started from the desktop or from your favourite command

line interpreter. A menu is presented which allows selection of the

memory range to test and many other parameters. The use of the menu is

intended to be self-explanatory.

The entries "start address" and "end address" determine the address

range that will be tested. The highest address accessed is JUST BELOW

the "end address". Both addresses must be entered as hexadecimal

numbers. The table below may help you to translate a decimal number of

bytes, Kilo bytes or Mega bytes to a hexadecimal number of bytes.

decimal  hexadecimal  Mega bytes  Kilo bytes   which Atari computer

0           0       0             0

524288       80000       0.5         512            520 ST

1048576      100000       1          1024       1024 ST or Mega 1

2097152      200000       2          2048            Mega 2

2621440      280000       2.5        2560     user-upgraded 1024 ST

4194304      400000       4          4096            Mega 4

Above the menu, the program displays the amount of memory that your

system appears to have as determined by the BIOS at startup. The MMU

chip limits access to memory above the 4 Mega byte limit. Some memory

extensions circumvent this limit. The program displays the address

where the MMU chip generates a bus error. If you select a higher value

for "end address" the memory test will be aborted by a bus error and

your computer may hang. No bombs will be displayed if the trap vectors

in low memory are overwritten.

The program displays how the MMU was programmed at boot time when the

BIOS determined the memory configuration. The BIOS tries to read and

and write specific patterns to certain addresses to determine the

amount of memory chips in bank 0 and bank 1. Some faults in the memory

chips may cause the BIOS to decide that less memory is installed than

is actually true, or it may even decide that no memory is installed in

bank 1. If bank 1 contains no chips, the MMU is (probably) programmed

for 2 Mega byte. Writes to this nonexistent memory have no effect,

reads yield amazingly reproducible values which seem to depend on the

instruction used to read them.

The "maximum run count" entry in the menu determines the number of

runs that the memory test program performs, unless the "maximum error

count" (see below) is reached. When the memory test program exits

(either because the "maximum run count" or the "maximum error count"

is reached), the system is rebooted. If the "maximum run count" value

is 0, the program will run forever, or until the "maximum error count"

is exceeded.

The entry "first execution address" determines the address to which

the memory test program is moved before it starts. The routine that

moves the test program to its destination cannot handle overlapping

moves. (The target and the source areas must be disjunct.) Therefore,

some values for the "first execution address" are not allowed.

It is informative to let the program run in the screen memory, where

you see a few pixels change as the program goes from one march element

to the next. The changes are small because the most used counters and

variables are kept in the registers of the microprocessor. If you have

no RAM disks or other resident programs that change the screen address

when they are installed, the screen memory ranges from the top of

memory in the computer minus 8000 (hexadecimal) to the top of the

memory.

The entry "relocation distance" determines how far the program is

moved in memory between successive runs. (If the "maximum run count"

is 1, the value of the "relocation distance" is irrelevant.) Certain

small values for the "relocation distance" are not allowed, because

the relocation routine cannot handle overlapping moves. Therefore the

"relocation distance" must be bigger than the amount of memory that is

occupied by the test program. If the program is about to be moved over

the "end address", it is moved to the "start address" instead.

The entry "patterns to use" selects the combinations of ones and zeros

to use as test patterns. There are four standard patterns, but the

program can also select a pseudo random pattern for each run.

The entry "output device" allows selection of the parallel port or the

RS232 serial port of the Atari ST computer for progress and error

reports. If the serial port is to be used, the baud rate and other

parameters must have been set before the memory test program was

started. The control panel accessory supplied by Atari can do this. If

not set explicitly, the baud rate will be 9600 baud, 8 databits, no

parity, one stopbit. The X-on/X-off option of the serial port will not

work with the memory test program.

If the parallel port is used, the memory test program will wait

forever for the printer to be put on-line. After about 30 seconds, it

will however use the sound generator of the Atari to summon attention.

The entry "report frequency" controls the amount of output generated

by the program (apart from reports about detected errors). Progress

reports can be generated once per run, once per march element (5 march

elements per run), or only when the program starts and exits.

All error reports contain the address where the error occurred, the

value that was actually read from that address and the value that the

program expected to read.

The "maximum error count" entry limits the maximum number of errors

reported by the program. Setting this limit is highly desirable when

you want to limit the amount of paper that might otherwise be filled

with error reports. When the "maximum error count" value is set to 0,

there is no limit.

With the last entry of the menu the memory test can be started, or the

parameters of the memory test can be saved on disk (the filename is

always MEMTEST.DAT), or recalled from disk. Once started, the memory

test program can only be stopped by pressing the reset button.

The memory test itself takes about 18.3 seconds per Mega byte per run.

This is quite good. The 8 MHz MC68000 can access memory no more often

than every 500 nano seconds. (Times were measured on an 8 MHz machine

with an ordinary stopwatch.)

DISCLAIMER

This program is provided `as is' without warranty of any kind. We will

not be liable to anyone for any damages or losses arising from the

use, or inability to use this program or this document.

This memory test has been written and tested with great care and we do

believe that it implements the march-B algorithm in an efficient

manner. As explained before, theory suggests that there are some kinds

of faults for which it is not possible to create a program or an

algorithm that detects these faults within a reasonable amount of

time.

Summarizing:

We can not guarantee that the memory in your Atari ST computer is

fault free, even when this memory test program reports no errors.

On the other hand, there may be faults in your Atari ST computer

that exhibit themselves as memory faults, but which are actually

faults in other components.

Our names and addresses are:

Peter Knoppers                     Harm Salomons

Prins Mauritsstraat 17             Antonia Veerstraat 38

2628 SR  Delft                     2614 HV  Delft

The Netherlands                    The Netherlands

Email: knop@duteca.et.tudelft.nl

REFERENCES

Na78:   Nair, R., Thattle, S.M., and Abraham, J.A., "Efficient

Algorithms for Testing Semiconductor Random-Access Memories",

IEEE Transactions on Computers vol. C-27, no. 6, pp. 572-576,

June 1978.

Su81:   Suk, D.S. and Reddy, S.M., "A March Test for Functional Faults

in Semiconductor Random Access Memories", IEEE Transactions on

Computers, vol. C-30, no. 12, pp. 982-985, December 1981.