OmniCom: VT100 Emulator/Kermit/XMODEM for OMNIVIEW

This release of OmniCom features a limited but quite functional  VT100

emulation, XMODEM, Kermit, capture mode, and screen dump.  We plan  to

continually improve this package as time goes on.  You can always  get

the latest version by sending us $10.00.

Every attempt has been made to make OmniCom compatible  with  850-like

and 835/1030 devices.  For there to  be  any  hope  that  it  will  be

compatible with other incompatible devices  (MPP,  in  particular)  we

will need technical data on the device.  Feedback on whether it  works

with these and other devices is requested:

CDY Consulting

421 Hanbee

Richardson, TX 75080

(214)235-2146

The OMNIVIEW has been updated to support the  entire  ASCII  character

set.  In particular, the characters for left and right  braces,  grave

accent,   and  tilde  have  been  substituted  for  the  corresponding

(worthless) characters in the ATARI character set.  If you bought your

OMNIVIEW before these changes you can update to the latest version  by

sending us the old chip with $10.00.

Getting OmniCom to run

ATARISRS.232 is an RS232 handler for the 850,  835,  and  1030  and  I

believe it originated from Russ Wetmore.  It provides an  'R:'  device

that will work with any of these ATARI modems.  If you have an 835  or

1030 you should probably use this as your RS232 handler.  If  you  are

using a P:R:Connection you will want to use the handler supplied  with

that device.

To get this program into an executable form you will  need  to  append

OC.OBJ    to   your  RS232  handler.    For  example,  if  you  choose

ATARISRS.232, then you should make a copy of it to a file called 'OC'.

From DOS this is done as follows:

'C (RETURN) ATARISRS.232,OC (RETURN)'.

Then append 'OC.OBJ' with the following command:

'C (RETURN) OC.OBJ,OC/A (RETURN)'.

Now you can load and execute 'OC' with the 'L' option of DOS:

'L (RETURN) OC (RETURN)'.

Operation of OmniCom

First of all I must give credit to David Bailey and others who  worked

on the public domain version of  VT10SQ,  a  simple  80  column  VT100

emulator.  Their work resulted in a minimal VT100 emulation  that  was

satisfactory for many applications.  It was used as an early model for

page 1

OmniCom: VT100 Emulator/Kermit/XMODEM for OMNIVIEW

OmniCom, which has much improved screen and keyboard processing, along

with  file  transfer  protocols  and  the  ability  to  customize  the

communication parameters.

The operation of OmniCom was designed to be simple and  yet  flexible.

If you are running a 300 baud modem then you can  just  start  typing.

If you are running a 1200 baud modem then you will want to change  the

baud rate.  This and other parameters are changed via the  OPTION  key

under the '(P)arameters' selection.  Using the  OPTION  key  you  will

also find other goodies: a BREAK generator, capture mode, an  835/1030

dialer, file transfer, dumb ATASCII or smart VT100 terminal selection,

and the ability to quit to DOS.  Feel free to  experiment.    You  can

answer any question with a RETURN without having any effect.

Although OmniCom should be able to keep up with 1200 baud most of  the

time, it may have trouble keeping up if several screens of  continuous

data are received, especially if the lines  are  short,  resulting  in

lots of scrolling.  In this case it will  use  XON/XOFF  to  keep  the

input buffer from overflowing.    You  will  know  this  is  happening

because the screen border will blink everytime an XOFF is  sent.    If

you want to avoid missing any characters on the screen, be  sure  that

the computer at the other end has XON/XOFF activated.  It is even more

critical that XON/XOFF be activated  during  capture  mode  since  the

capture buffer is quite small (256  bytes).    If  the  host  you  are

communicating with cannot be made to recognize XON/XOFF  then  capture

mode will work very poorly.

The keys to generate the characters for left and right  braces,  grave

accent, and tilde (which ATARI left out of  their  character  set  for

some dumb reason) are as follows:

Left brace  : control-9

Right brace : control-0

Grave accent: control-7

Tilde       : ATARI key (lower right corner)

The other thing you should be aware of is the simulation  of  the  DEC

VT100 keypad.  It is accessed by holding down  the  SELECT  key  while

typing the following keys:

Mapping of VT100 Keypad to ATARI Keyboard

+-------+-------+-------+-------+

| PF1/1 | PF2/2 | PF3/3 | PF4/4 |

+-------+-------+-------+-------+

|  7/Q  |  8/W  |  9/E  |  -/R  |

+-------+-------+-------+-------+

|  4/A  |  5/S  |  6/D  |  ,/F  |

+-------+-------+-------+-------+

|  1/Z  |  2/X  |  3/C  |       |

+-------+-------+-------+ENTER/V|

|      0/B      |  ./N  |       |

+-------+-------+-------+-------+

page 2

OmniCom: VT100 Emulator/Kermit/XMODEM for OMNIVIEW

If you do not like this mapping you are free to redo it to  suit  your

tastes.  Read about that in  the  next  section  on  customizing  your

keyboard.

ATARI     VT100 keypad         Xmitted

Sequence

(application)(numeric)

<SELECT ->     UP ARROW   <ESC O A>  <ESC [ A>

<SELECT =>   DOWN ARROW   <ESC O B>  <ESC [ B>

<SELECT *>  RIGHT ARROW   <ESC O C>  <ESC [ C>

<SELECT +>   LEFT ARROW   <ESC O D>  <ESC [ D>

<SELECT 1>        PF1     <ESC O P>   (same)

<SELECT 2>        PF2     <ESC O Q>   (same)

<SELECT 3>        PF3     <ESC O R>   (same)

<SELECT 4>        PF4     <ESC O S>   (same)

<SELECT Q>        7       <ESC O w>   (same)

<SELECT W>        8       <ESC O x>   (same)

<SELECT E>        9       <ESC O y>   (same)

<SELECT R>        -       <ESC O m>   (same)

<SELECT A>        4       <ESC O t>   (same)

<SELECT S>        5       <ESC O u>   (same)

<SELECT D>        6       <ESC O v>   (same)

<SELECT F>        ,       <ESC O l>   (same)

<SELECT Z>        1       <ESC O q>   (same)

<SELECT X>        2       <ESC O r>   (same)

<SELECT C>        3       <ESC O s>   (same)

<SELECT V>        ENTER   <ESC O M>   (same)

<SELECT B>        0       <ESC O p>   (same)

<SELECT N>        .       <ESC O n>   (same)

ATASCII/VT100 Selection

If you wish to communicate with an ATARI BBS system, you may  want  to

do so in ATASCII mode to take advantage  of  the  ATARI  line  drawing

graphics and editing functions.  Some ATARI BBS's  use  this  mode  to

draw pretty boxes and do animation.  The other characteristic of  this

mode is that all CR characters ($0D) are translated to $9B.   However,

a Hayes compatible modem will only recognize CR to terminate commands.

Thus, when you are talking directly to a Hayes  compatible  modem,  as

when you are dialing (ATDT...), you must be in VT100 mode.  Switch  to

dumb ATASCII mode after you get  the  "CONNECT"  indication  from  the

modem but before typing anything else (the  BBS  will  use  the  first

RETURN character to determine whether you  are  in  ATASCII  or  ASCII

mode).  You do so by pressing OPTION and typing "T2".

Sending BREAK

Some systems require the remote terminal to send a BREAK in  order  to

set communication parameters.  This can be done by pressing OPTION and

then typing 'B'.

page 3

OmniCom: VT100 Emulator/Kermit/XMODEM for OMNIVIEW

Capture Mode

IMPORTANT: The computer you  are  communicating  with  must  recognize

XON/XOFF in order for this feature to work!  XON  and  XOFF  are  what

OmniCom uses to control the speed at which the other computer sends it

data in order to prevent it from overflowing the capture buffer.

Capture mode allows you to save everything that is being sent  to  you

from the other computer to any output device.  Usually you  will  want

to capture the data to a disk file or to the printer.  This is done by

pressing OPTION and typing 'C'.  You can supply a  file  specification

or just hit RETURN if you want the output to go to  a  printer.    You

terminate capture mode by pressing OPTION and typing 'C' again.   This

will cause the output file to be closed.  Using 'Q'  to  quit  OmniCom

will also close the file.

It is important to note that capture mode is designed to be used  only

with ASCII data.  The reason for this is that it is not reliable  like

the file  transfer  protocols  XMODEM  and  Kermit.    Also,  it  will

translate all incoming CR characters ($0D) to  $9B  and  discard  LF's

($0A).  It is most useful for grabbing mail or documentation where  an

occasional missing or garbled character is not critical.

If you are capturing to a printer attached  to  an  ATR8000  you  will

notice that the printing will be sporadic and appear not  to  keep  up

with the screen.  This is because the ATR8000  is  also  used  as  the

modem interface and cannot print while  serial  I/O  is  active.    To

deactivate serial I/O press OPTION.  This will  take  OmniCom  offline

and allow the data stored in the ATR's printer buffer to  flow  freely

to the printer.  This should only be done when you expect no new  data

from the remote computer since it will be lost until  you  hit  RETURN

again to take OmniCom back online.

Screen Dump

You can make an exact copy of the screen to the printer at any time by

holding down SELECT and then  pressing  OPTION.    Any  data  arriving

during the interval that the screen is being  copied  to  the  printer

will be lost.  If you own an ATR8000 then you will also want  to  read

the last paragraph under Capture Mode.

Customizing your Keyboard

If you type OPTION and 'P', you will be asked if you want to edit  the

key mapping.  Answering 'N' to this question  (or  just  RETURN)  will

take you into the  section  to  adjust  the  communication  parameters

(explained in the next section).  If you do  want  to  customize  your

keyboard then answer 'Y'.

Now you are free to assign one or more characters (up to 17)  to  each

key or key in combination with CONTROL, SHIFT,  or  both  CONTROL  and

SHIFT.  Do that by typing the desired key when prompted.  Now you will

be asked for the code(s) that you wish that key to produce.   You  can

page 4

OmniCom: VT100 Emulator/Kermit/XMODEM for OMNIVIEW

do that in hex if it is to produce only one byte.  Otherwise  you  can

delete the hex byte and hit RETURN to specify a sequence of characters

up to 17 bytes long.  When specifying the character sequence  you  are

free to use the normal ATARI editing controls.  If you want to  insert

a  CONTROL  character  (like  CR)  hold  down  CONTROL  and  type  the

corresponding letter of the alpahbet.  For example,  if  you  want  to

insert a CR ($0D), type CONTROL-M.  One key many people will  need  to

change is DELETE/BACKSPACE.  As it comes, OmniCom produces a DEL ($7F)

when this key is  pressed  and  a  BS  ($08)  when  control-DEL/BS  is

pressed. You may want to reverse these definitions.

An entirely different set of key codes are accessed when you hold down

SELECT while typing a key.  SELECT activates the keypad mode.  In this

mode a VT100 generates a particular sequence of characters everytime a

key is pressed.  This sequence  is  ESC-'O'-code  where  the  code  is

different for every key of the keypad.  You can edit these codes also,

but you are only allowed to enter one byte in hex.

As you customize your keyboard you will want to save these changes  to

disk so that your keyboard will always act the same when  you  execute

OmniCom.     To  save  your  customized  keyboard  (along  with  other

parameters you may have changed, like screen color)  just  hit  RETURN

when prompted for the next key to edit.  Now you  will  be  given  the

opportunity to either save the changes you just made  or  restore  the

previous parameters, negating any recent changes.    These  parameters

are stored in a file called OC.OPT.  This file must be on drive 1 when

OC is loaded for your options to be in effect.

Using these techniques you can even alter the keyboard  for  use  with

PROFS on an IBM mainframe.  This has been done for you and is  on  the

disk under PROFS.OPT.  Rename this file to OC.OPT  and  your  keyboard

will act according to the following mapping (thanks  to  David  Bailey

for coming up with this convention):

page 5

OmniCom: VT100 Emulator/Kermit/XMODEM for OMNIVIEW

Yale Protocol        ATARI     VT100 keypad   Xmitted

Converter                                  Sequence

PF1          <SELECT 1>        PF1     <ESC O P>

PF2          <SELECT 2>        PF2     <ESC O Q>

PF3          <SELECT 3>        PF3     <ESC O R>

PF4          <SELECT 4>        7       <ESC O w>

PF5          <SELECT 5>        8       <ESC O x>

PF6          <SELECT 6>        9       <ESC O y>

PF7          <SELECT 7>        4       <ESC O t>

PF8          <SELECT 8>        5       <ESC O u>

PF9          <SELECT 9>        6       <ESC O v>

PF10         <SELECT 0>        1       <ESC O q>

PF11         <SELECT CLEAR>    2       <ESC O r>

PF12         <SELECT INSERT>   3       <ESC O s>

PA1          <SELECT Q>        PF4     <ESC O S>

PA2          <SELECT W>        -       <ESC O m>

PA3          <SELECT E>        ,       <ESC O l>

NA           <SELECT R>        0       <ESC O p>

NEWLINE        <RETURN>        RETURN  <^M>

ENTER          <ATARI> (Logo)  LF      <^J>

CLEAR          <CONTROL-CLEAR> ENTER   <ESC O M>

FIELD TAB      <TAB>                   <^I>

DELETE         <DELETE/BACK S>         <DEL> ($7F)

ERASE EOF      <CONTROL-DELETE/BACK S> <ESC DEL>

TOGGLE INSERT  <CONTROL-INSERT>.       <ESC O n>

HOME           <CONTROL-H>             <^H>

RE-DISPLAY     <CONTROL-V>             <^V>

<CTL-7>   Grave Accent <$60>

<CTL-8>        Tilde   <$7E>

<CTL-9>     Left Brace <$7B>

<CTL-0>    Right Brace <$7D>

Changing Communication Parameters

If you press OPTION and then answer 'P' and 'N', you will be given  an

opportunity to adjust some communication parameters,  set  the  screen

colors, cause  local  echo,  and  then  save  your  changes  to  disk.

Remember, if you want to leave a parameter the way  it  is,  just  hit

RETURN.

In selecting the baud rate,  you  can  either  choose  300,  1200,  or

customize.  If you choose to customize then you can specify the values

for AUX1 and AUX2 during the XIO 36 and XIO 34 when the modem  channel

is opened.  These two XIO commands control the  baud  rate,  the  word

size, the number of stop  bits,  carrier  detect,  and  other  control

lines.  Refer to your interface manual to  find  out  which  of  these

parameters are recognized by your interface:

XIO 36: AUX1: bits 0-3 - baud rate ($0A=1200, $08=300, $0E=9600)

bits 4-5 - word size (8 to 5 bits  -> 0 = 8 bits)

bit  7   - 1 or 2 stop bits (0 = 1 stop bit)

page 6

OmniCom: VT100 Emulator/Kermit/XMODEM for OMNIVIEW

AUX2: bit  0   - monitor CRX  * Interface will return an    *

bit  1   - monitor CTS  * error if one of these lines *

bit  2   - monitor DSR  * is selected but not on.     *

XIO 34: AUX1: bits 0-1 - 0 -> do not change XMIT

2 -> set XMIT to SPACE (0)

3 -> set XMIT to MARK (1)

bits 4-5 - 0 -> do not change RTS

2 -> turn RTS off

3 -> turn RTS on

bits 6-7 - 0 -> do not change RTS

2 -> turn DTR off

3 -> turn DTR on

In selecting the parity you can either  choose  none,  even,  odd,  or

custom.  If you choose to customize then you will be able  to  specify

the AUX1 and AUX2 to be used during the XIO 38 when the modem  channel

is opened.  This XIO command controls parity, translation and linefeed

options:

XIO 38: AUX1: bits 0-1 - 0 -> do not change parity bit

1 -> set output parity odd

2 -> set output parity even

3 -> set output parity to 1

bits 2-3 - 0 -> ignore and do not change parity bit

1 -> check for odd parity, clear parity bit

2 -> check for even parity, clear parity bit

3 -> do not check parity, clear parity bit

bits 4-5 - 0 -> light ATASCII/ASCII translation

1 -> heavy ATASCII/ASCII translation

2 -> no translation

bit  6   - 0 -> do not append LF

1 -> append LF after CR

After setting the modem parameters you  can  set  the  screen  colors.

These values are stored in the color  registers  at  $2C5,  $2C6,  and

$2C8.  You may have to experiment to find the  best  colors  for  your

tastes.  You can also activate local echo if the computer at the other

end is not echoing your typing.

Use of XMODEM File Transfer

XMODEM is a simple, fairly  reliable  way  of  transferring  files  to

(upload) and  from  (download)  other  computers  and  bulletin  board

systems.  It uses a very simple handshaking mechanism for transferring

the file in 128 byte blocks whereby each  block  that  is  transmitted

must be acknowledged by the receiving computer with a single byte ACK.

If noise on the line causes the block to arrive  corrupted  then  this

will usually be detected by a checksum that is appended to the  block,

in which case a NAK is returned instead so  that  the  block  will  be

retransmitted.  A sequence number is also part of the block to  insure

that everything arrives in order and that no block is lost.

page 7

OmniCom: VT100 Emulator/Kermit/XMODEM for OMNIVIEW

To start the transfer you must get the remote computer started  first.

For example, if you wish to download a file (from the remote  computer

to yours) you must tell it  that  you  wish  to  download.    It  will

probably ask you for the filename and then after a short interval tell

you that it is ready.  At that point you can go to the OPTION menu and

choose (F)ile-xfer and then  (D)ownload.    You  must  then  supply  a

filename, which can be different from the  filename  supplied  to  the

remote computer.  After that your computer will  send  a  NAK  to  get

things started and you should see  the  transmit  and  receive  lights

blinking, assuming that your modem has status lamps.   You  will  also

see a number representing the  retransmission  count  printed  to  the

screen for every block transferred.  When the transfer is complete the

file transfer menu will appear again and you can (Q)uit or hit  RETURN

to go back to the terminal mode.

Before an upload or download is  attempted  you  should  first  decide

which mode to do the transfer in,  ASCII  or  ATASCII,  and  make  the

appropriate choice under the (O)ptions selection  in  the  (F)ile-xfer

menu.  ATASCII will transfer the file as is, without any  translation.

It is mainly used for transferring programs and other non-text  files.

In ASCII mode during upload, all $9B bytes are converted to two bytes:

$0D $0A (CR LF).  In ASCII mode during download, all  $0D  (CR)  bytes

are converted to $9B and all $0A (LF) bytes are thrown  away.    ASCII

mode is used for transferring text, especially if the remote  computer

is not an ATARI. If you are unsure  which  mode  to  use,  ATASCII  is

probably the safest, especially if you are communicating with an ATARI

computer.

Another option concerns the method in which the XMODEM  at  the  other

end handles the end-of-file.  OmniCom defaults to using the byte count

of the last block to pad the last block (which seems to  be  the  most

popular technique in the ATARI world).  However, many systems pad  the

last block with ctrl-Z ($1A).  If you know this to be the case  or  if

you discover extraneous ctrl-Z characters at the end  of  a  file  you

downloaded, be sure to enable this option.

Under the same (O)ptions selection in the  (F)ile-xfer  menu  you  can

also make a couple of other choices.  If you would like  you  can  put

OmniCom in debug mode so that all XMODEM transactions are  printed  to

the screen. You may find this interesting but it is not good to  leave

it in this mode all the time as it will slow down your file transfers.

Another option is the default disk drive when you  do  not  supply  it

with the filename at the beginning of a transfer.  Once you have these

options set the way you like them you will probably want to save  them

to disk under the (P)arameters selection in the OPTION menu.

Kermit Operation

For file transfer over a packet switched network like  the  ARPANET  a

more robust protocol like Kermit is usually preferred, even though  it

is slower than XMODEM.  The Kermit in OmniCom is  particularly  robust

because it will not only retry individual packets but, if a particular

page 8

OmniCom: VT100 Emulator/Kermit/XMODEM for OMNIVIEW

file fails for any reason, it will keep retrying that file until it is

finally successful.  If the remote computer is in server mode then you

can be fairly certain that your transfer will be successful,  even  if

left unattended.  If you want to abort a transfer you  can  do  so  by

holdng down START/SELECT/OPTION for one packet time.    You  can  also

cause a premature timeout or retransmission by typing any key.

A complete description of Kermit operation is beyond the scope of this

manual.  However, for most cases you can put the  remote  computer  in

server mode by starting the remote Kermit and typing "server".    Then

you can control things  from  your  end  by  using  either  (S)end  or

(R)eceive.  When prompted for a filename wildcards are  allowed  (this

is the one big advantage over XMODEM). Use  the  (F)inish  command  to

take the remote computer out of server mode.  You can  also  send  and

receive files one at a time by starting the operation at each end  for

every file.  Notice that the filename is part of the  transfer  so  it

does not do any good to specify the filename on the receive end.

The Kermit protocol is more elaborate than that of  XMODEM,  involving

the exchange of communication parameters and the filename in  addition

to the user data.  It also uses a quote character ('#')  to  transform

all control characters (<$20) into printable ASCII characters  because

some communications  media  and  networks  are  sensitive  to  control

characters.  There are also provisions for quoting bytes with the  8th

bit on and for repeat counts but these options are not yet implemented

in OmniCom.  If you are curious, you can activate the debug mode under

the 'Options' selection to see the protocol in action.   However,  you

should not always run in  this  mode  because  it  slows  things  down

considerably and may cause unneeded retransmissions.

A couple of other things may be of help.  To transfer binary files  or

other files where the 8th bit of the characters  is  necessary,    you

will need to use the '(O)ptions' selection to tell Kermit to pass  all

8 bits.  Also, you must not have parity enabled (under 'Parameters' in

the VT100 OPTION menu) if you want your modem to pass all  eight  bits

intact.  Likewise, you may have to do something special to  your  host

to make it store 8 bit files.  For example,  if you are going  through

a TAC onto a host on the ARPANET, you will need to make the  TAC  pass

all   8  bits  with  the  following  commands:  '@b  o  s(RETURN)@b  i

s(RETURN)'.  On a TOPS-20 operating system the  command  to  make  the

Kermit store 8 bit files is: 'SET FILE 8-BIT'.

Also under the (O)ptions selection  you  will  find  that  the  Kermit

timeout between retransmissions can be set.  If you are  communicating

over a network with large  delays  (possibly  because  of  congestion)

resulting in many timeouts you may want to lengthen this parameter.

Responses To Characters Received By the Terminal Emulator

Unknown  escape  sequences  of the form "ESC char" are absorbed by the

emulator without further effect.

Escape Seq     Mnemonic       Description of Action

page 9

OmniCom: VT100 Emulator/Kermit/XMODEM for OMNIVIEW

ESC D          IND            Index, cursor down 1 line, can scroll

ESC E          NEL            Cursor start of line below, can scroll

ESC H          HTS            Set one tab at current position

ESC M          RI             Rev Index, cursor up 1 line, can scroll

ESC Z          DECID          Indentify term (response is ESC[?1;0c)

ESC 7          DECSC          Save cursor position and attributes

ESC 8          DECRC          Restore cursor from saved position

ESC [ Pn A     CUU            Cursor up Pn lines, does not scroll

ESC [ Pn B     CUD            Cursor down Pn lines, does not scroll

ESC [ Pn C     CUF            Cursor forward, stays on same line

ESC [ Pn D     CUB            Cursor backward, stays on same line

ESC [ Pn; Pn H CUP            Set cursor to row, column (same as HVP)

ESC [ Ps J     ED             Erase in display:

0= cursor to end of screen, inclusive

1= start of screen to cursor, inclusive

2= entire screen, cursor does not move.

ESC [ Ps K     EL             Erase in line:

0= cursor to end of line, inclusive

1= start of line to cursor, inclusive

2= entire line

ESC [ Pn; Pn R CPR            Cursor report (row, col), sent by term

Example: home position yields ESC[1;1R

ESC [ Pn c     DA             Device attributes (reports ESC [?1;0c)

ESC [ Pn; Pn f HVP            Set cursor to row, col (same as CUP)

ESC [ Ps g     TBC            Tabs clear, 0 = here, 3 = all

ESC [ 20 h     LNM            Set newline mode (lf = cr/lf)

ESC [ 20 l     LNM            Reset newline mode (lf = lf)

ESC [ ? Ps;...;Ps h  SM       Set mode, see table below

ESC [ ? Ps;...;Ps l  RM       Reset mode, see table below

Ps     mnemonic       mode           set            reset

1      DECCKM         cursor keys    application    numeric

3      DECCOLM        Columns         clear  screen      clear

screen

5      DECSCNM        Screen         reverse video  normal

7      DECAWM         Autowrap       on             off

ESC [ Ps;...;Ps m  SGR        Select graphic rendition

0 = reverse video off

7 = reverse video

ESC [ Ps n     DSR            Device Status Report.

Response from VT100: 0 = ready.

To VT100: 5 = report status with DSR,

6 = report cursor position using CPR

ESC [ Pn; Pn r DECSTBM        Set top and bottom scrolling margins

ESC ( A  or  ESC ( B          G0 points to ASCII symbols

ESC ( 0 or ESC ( 1 or ESC ( 2 G0 points to line drawing graphics

ESC ) A  or  ESC ) B          G1 points to ASCII symbols

ESC ) 0 or ESC ) 1 or ESC ) 2 G1 points to line drawing graphics

^G             BELL           Beep and blink border color

^H             BS             Backspace, cursor left one character

^I             HT             Horizontal tab, cursor to next tabstop

page 10

OmniCom: VT100 Emulator/Kermit/XMODEM for OMNIVIEW

^J             LF             Linefeed, move cursor down one line

^L             FF             Formfeed, treated as a line feed

^M             CR             Carriage return, move cursor to col 1

^N             SO             Invoke G1 character set

^O             SI             Invoke G0 character set

David Young 04/01/87

page 11