An Input/OUTPUT Port for the Cartridge Slot of the 520ST

By Douglas P. B. Renaux - Feb 27, 1986.

This  document  describes the hardware and software of a  8  lines

input/16  lines  output port.  The circuitry is connected  to  the

Cartridge Slot of the ATARI 520 ST.

IMPORTANT!!! - INFORMATION IN THIS DOCUMENT CAN BE FREELY USED AND

COPIED.  HOWEVER,  NO CHANGES CAN BE MADE TO IT WITHOUT CONSULTING

THE  AUTHOR  FIRST,  NOR THE AUTHOR'S NAME CAN BE OMISSED  IN  THE

COPIES.

Foreword

Although   the  Cartridge  Slot  is   intended  for  data   input,

specifically  from a ROM,  it can be used as an  output  port.  To

substitute the READ/WRITEline,  that is not available on the slot,

I used a WRITE ENABLE LATCH. This latch is set by READING a memory

location. The consequence of setting this latch is that the output

latches will read the data bus during the next instruction,  which

will be a memory WRITE that outputs the desired data.

1 - The signals available on the Cartridge Slot

D0..D15 - DATA BUS

A1..A15 - The 15 least significant addresses from the ADDRESS BUS

ROM 3 SELECT - Active when a READ instruction accesses $FB0000  to

$FBFFFF UDS',LDS' - Upper and Lower Data Strobe

2 - The I/O PORT The block diagram of the I/O PORT is in the  file

BLOCKDIA.PI3.  Circuit diagrams are in ADDRESS.PI3 and IN'OUT.PI3.

If you don't have these drawings then in appendix A you can read a

description  of  the  circuit.  The circuit of  the  I/O  PORT  is

standard.   A  address decoder enables the out put latches or  the

input buffers when the correct address is accessed.   However, the

output   latches  are  only  enabled  if  previously   the   WRITE

ENABLELATCH was set.

Address  Decoder:  The lines A3..A15 are connected to  NAND  gates

that  detect when allthese lines are at high level.  The lines  A2

and A1,  select one of the four outputs of a decoder.   The output

of  the  NOR gate "a" is active when we  read  from  $FBFFFE.   To

decode  a read to this address we use the outputs of the two  NAND

gates,the  output three of the decoder (active when A1 and A2  are

high) and the line ROM 3 SELECT.   The output of NOR gate "a" sets

the  WRITE ENABLE LATCH.

The output of NOR gate "b" decodes the address $XXFFFC.  To decode

anaccess to these addresses (read or write), we use the outputs of

the NANDgates, output two of the decoder and the line LDS'.  ROM 3

SELECT  cannot  be used here because it is only  active  on  read.

When  a  write  instruction accesses  the  address  $10FFFC  (non-

existent  in 1MByte ST's) then the output of NOR gate "b"  becomes

active.   An AND gate enables the output latches to read the  data

bus if theWRITE ENABLE LATCH is set AND the output of NOR gate "b"

is active.

The output of NOR gate "b" is delayed (by four inverters) and then

resets  the WRITE ENABLE LATCH,  just after the latches have  read

the databus.  NOR gate "c" decodes a read to address  $FBFFFA,  as

gate "a" does,  but using output one of the decoder. The output of

gate "c" enables the buffersto copy their inputs to the data  bus.

3 - Components

2- 4068 NANDgates for address decoding

2- 4002 NOR gates  for  address decoding

1- 4556 1 OF 4 DECODER  for  address decoding

1- 4043 SR FLIP-FLOP write enable latch

1- 4049 INVERTER

1- 4081 AND2- 40373 LATCHES output latches

1- 74LS244 BUFFERS input buffers

4 - Expansion

This I/O PORT is easily expandable to as many I/O lines you  want.

Onlyone  WRITE ENABLE LATCH is necessary if the expansion is  made

in  a  way  thatany write to port instruction  resets  the  latch.

Another solution is to havea reset latch address (as we have a set

latch address).  In this case the write to port instructions don't

need  to  reset the latch,  and you can outputa block of  data  to

several  addresses  without needing to set the latch  in  between.

Only at the end of the block transfer this would be necessary. You

can  have another 8 inputs by just connecting another  74LS244  to

D8..D15 of the data bus.

5 - Software To write to the output lines the contents of D0:

MOVE.W $FBFFFE,D1 ;D1 is destroyed, latch is set

MOVE.W D0,$10FFFC ;D0 is latched To read the input lines to D0:

MOVE.W  $FBFFFA,D0  6 - Connection to the ST.

I  could not find a experimentor PC Board with an  edge  connector

that fits to the ST.   So I made a convertor from the 40 pin  edge

connector  (withspacing  of 2mm) to a 64 pins  euroconnector  (DIN

41612).  This convertor is simply a double sided PCB with 20 lines

(on  each side) of 1mm spaced by 2mm(center of one line to  center

of  next  line).  These  lines  widen to 2.54  mmto  fit  the  DIN

connector. Line one of the ST goes to line 4 of the DIN connector.

The tabs of the DIN connector are on the lower side.

______

||| |. .|

1 |. .| 2 |. .| 3

+ |. .| 4 + Data 15 |. .| 5 Data 14

|. .|

:| | : :| | :

| |Ground |. .| 23 Ground

| | |. .| 31

|. .| 32 |||

DIN connector seen from front (connection) side

------  I  intend  to connect some of the  free  lines  of  this

connector to inter-nal signals of the ST.  Would be nice to have a

standard  connector  between ST users.   Whoever has  ideas  about

which  signals to connect and to which pin should let  the  others

know about it.  7 - Conclusion It is possible to open the ST !!And

this  opens  a  lot  of  possibilities  in  hardware  design.  The

68000'spower  can  be used outside the ST's  box.  But  a  complex

machine  like  the  ST cannot be easily  expanded  without  having

information about it.  The information that ATARI delivers only to

software houses.  Please ATARI,  let everybody know about the  ST.

Let the POWER be available to all users.

Douglas P. B. Renaux

Outshoornstraat 55

5622GX Eindhoven - NL Tel 31-40-436930

(Address valid until May 25, 1986)

People  dealing  with  hardware design are  cordially  invited  to

contact me.

APPENDIX A Circuit Description. Block Diagram:

write enable latch ___________ ______|| Read $FBFFFE

| |_____ |

|----------------------|S Q|------------| & |

A15..A1 | ADDRESS | $XXFFFC _________ | |

||---| LDS ----| DECODER |-----T---| Dela

y |----|R| |----||| ROM3 SEL||

|--------- ------ | -----|

|| |__________________________

___|| ||

|

||-----------|

| ----------- Read |

|

$ FBFFFA |

|---------

---------

------------

| || |

| INPUT

| D15 | OUTPUT|

D7..D0-----|BUFFERS|---INPUTS --| LATCHES |--OUT

| | D0

| | | |

| | ---------

------------

Address Decoder Description:

Address lines A15 to A3 are connected to the inputs of two 4068(8-

input  NAND).  Inputs are pins 2,3,4,5,9,10,11,12.  One  of  these

NANDgates has three inputs tied high. Power supply is connected to

pin  14  (+)and pin 7 (ground).  The address lines A2 and  A1  are

connected to the select lines of the4556 (1 of 4 decoder),  pins 3

(A2) and 2 (A1).  The enable line (pin 1) is tied low.  Pins 16(+)

and 8 (ground). The outputs of the decoder are: output 0 at pin 4,

output 1 (pin 5),  output 2 (pin 6),  output 3 (pin 7). The second

level  of the address decoding is done by three NOR  gatesof  four

inputs  (4002).   Two  chips  are  necessary.   These  gates   are

labelled"a",   "b"   and  "c".

Pinning  of  the   4002:

inputs  2,3,4,5;output  pin  1  inputs  9,10,11,12;output  pin  13

power:  pin  14 (+);  pin 7 (ground).  Two inputs of gate "a"  are

connected to the outputs of the 4068 (pin13).  The third input  is

connected to ROM3 SELECT and the fourth input goes to output 3  of

the decoder.  The output of gate "a" is connected to the  SETinput

of the WRITE ENABLE LATCH.  Gate "b".   Inputs: the two outputs of

the  4068s,  output 2 of the decoder and LDS.  The output of  this

gate is the line $XXFFFC of the blockdiagram,  it goes to a  chain

of four inverters (4049) to produce a delay,and it goes to an  AND

gate. Gate "c".

Inputs:  the two outputs of the 4068s, output 1 of the decoder and

ROM3  SELECT.   The output of gate "c" is inverted by a  4049  and

then connected to the enable line of the input buffers  (74LS244).

Delay  chain:  The hex inverter 4049 (inputs  pins  3,5,7,9,11,14;

outputspins 2,4,6,10,12,15 respectively) is used.  Four  inverters

are chained togive the desired delay and one inverter is used  for

the  enable of the input buffer.  Power is on pin 1 (+) and pin  8

(ground).  Note that + is NOT onpin 16! Write enable latch: A 4043

(SR-latch) is used.  SET is on pin 4,  RESETon 3 and Q on 2. Pin 5

(output  enable)  is  tied  low.  Power is on  pins  16  (+)and  8

(ground).  SET is connected to the output of gate "a".  RESET goes

tothe output of the delay chain.  AND gate (4081).

Inputs:

Q  from the latch (connected to pin 1) andthe output of  gate  "b"

(connected to pin 2).  The output of this gate (pin 3)enables  the

output  latches.  Power is on pins 14 (+) and 7  (ground).  Output

latches.   Two  40373  (octal  latches)  are  used.  Inputs  (pins

3,4,7,8,13,14,17,18) are connected to the DATA BUS.  Outputs (pins

2,5,6,9,12,15,16,19 respectively) are the outputs of the  circuit.

Pin 1 (output enable)is tied low.  Pin 11 (enable latches) goes to

the output of the AND gate.  Power to pins 20 (+) and 10 (ground).

Input  buffers.  The 74LS244 (inputs  2,4,6,8,11,13,15,17  outputs

18,16,14,12,9,7,5,3 respectively). The inputs of these buffers are

the  inputs ofthe circuit.  The outputs are connected to the  DATA

BUS  (D0..D7).  Pins 1 and19 (output enable) are connected to  the

inverted output of gate "c".  Powerto pins 20 (+) and 10 (ground).

I used the +5V from the cartridge slot to drive the circuit and 16

ledsthat monitor the outputs. (The leds are not connected directly

to the 40373,but via buffers 4049).