Using STOS Floating Point Routines in Machine Code Programs

===========================================================

By Peter Hibbs

It is sometimes useful to  have  floating  point arithmetic routines in

machine code, unfortunately these are very difficult to write and there

is very little information available  on  how  to  write them. The STOS

Basic language has an extension library called FLOAT.BIN which contains

all the main floating point arithmetic  routines as a binary file which

can be linked into STOS Basic when required. This document shows how to

use these routines in your  own  machine  code programs. (Note that the

FLOAT102.BIN file which is on the ST  User cover disk of Sept 1993 does

not work as it is a different version).

The routines available are as follows :-

1   Add two floating point numbers.

2   Subtract one floating point number from another.

3   Multiply two floating point numbers together.

4   Divide one floating point number into another.

5   Return the sine of a floating point number.

6   Return the cosine of a floating point number.

7   Return the tangent of a floating point number.

8   Return the exponential of a floating point number.

9   Return the naperien log of a floating point number.

10  Return the base 10 log of a floating point number.

11  Return the square root of a floating point number.

12  Convert an ASCII string into a floating point number.

13  Convert a floating point number into an ASCII string.

14  Convert a floating point number into an integer.

15  Convert an integer value into a floating point number.

16  Tests if two floating point numbers are equal.

17  Tests if two floating point numbers are not equal.

18  Tests if a floating point number is greater than another one.

19  Tests if a fp number is equal to or greater than another one.

20  Tests if a floating point number is less than another one.

21  Tests if a fp number is less than or equal to another one.

22  Return the arc sin of a floating point number.

23  Return the arc cos of a floating point number.

24  Return the arc tan of a floating point number.

25  Return the hyperbolic sin of a floating point number.

26  Return the hyperbolic cos of a floating point number.

27  Return the hyperbolic tan of a floating point number.

28  Return the integer part of a floating point number.

29  Return the value of a number raised to the power of another one.

The floating point operations use  numbers  in  the  IEEE 64 bit format

with a numerical range of 10E-307  to  10E+308. For more information on

the mathematical use of these  routines  please  consult your STOS User

Guide.

To use the  binary  file  in  your  own  program  first  copy  the file

FP_STOS.S into your source code  and  the  file FLOAT.BIN from the STOS

language disk into your library folder  on your hard disk/working disk.

The STOS library file is 'included' in the source file as a binary file

using the 'incbin' pseudo-op and is  labelled  as fp_data. You may also

need to set up the pathname  of  the 'incbin' pseudo-op as appropriate.

Note that  the  ninth  instruction  in  the  'float_point'  sub-routine

locates the function look-up table by adding $398C to the start address

of the data file. I don't  know  if  there  is more than one version of

this file in existence but  if  there  is,  this  value  may need to be

changed. The FLOAT.BIN file I am using has a size of 15976 bytes.

The FP_STOS.S file contains  two sub-routines, 'init_float' initialises

the binary file by changing the  longword addresses in conjunction with

the re-location table at the end  of  the  data. This routine should be

called near the start of the program once to set up these addresses.

The 'float_point' sub-routine  is  called  each  time  a floating point

operation is required. In  most  cases  registers  d0  and  d1 hold one

number, registers d2 and d3  hold  the second number (where applicable)

and register d4 holds the  function  number (0-28). The values returned

are always held in registers d0 and d1, no other registers are changed.

Note that the original STOS routines  pass  the numbers to the routines

in registers d1/d2 (or d3/d4)  but  return  the results in registers d0

and d1  which  makes  it  very  messy  to  program.  The  start  of the

'float_point' routines swaps  the  registers  around  so  that they are

always in d0/d1 or d2/d3.

The standard  IEEE  format  for  the  double  precision  floating point

numbers is as follows: registers d0.l and d1.l together form a complete

64 bit floating point number (registers  d2  and d3 are formed the same

way). Register d0.l holds  the  bottom  half  (bits 0-31) and registers

d1.l holds the top half (bits 32-63). Bits 0-51 form the mantissa part,

bits 52-62 form the exponent part and  bit 63 holds the sign. The exact

format of the numbers is not really important for the programmer (and I

don't know what it is anyway) since  there are routines to convert from

ASCII or integer  formats  available.  Just  remember  when moving data

about that the top and bottom halves  of  the numbers are copied to the

correct registers as longwords.

A typical program  to  multiply  two  floating  point  numbers in ASCII

format and display the results in ASCII format might look like this :-

main initialisation

.

bsr     init_float              initialise fp code

.

first convert ASCII numbers to fp format (if not already done)

.

lea     first_no,a0             convert 1st No to fp

move    #11,d4                  ASCII to fp function

bsr     float_point             d0/d1=fp No

move.l  d0,d2                   and save in d2/d3

move.l  d1,d3

lea     second_no,a0            convert 2nd No to fp

bsr     float_point             d4=11 still, d0/d1=fp No

.

now multiply fp numbers together, result in d0/d1

.

move    #2,d4                   multiply function

bsr     float_point             result in d0/d1

.

now convert fp numbers back to ASCII for display

.

lea     fp_buffer,a0            convert fp to ASCII

move    #'3',d2                 3 digits after decimal pt

move    #12,d4                  fp to ASCII function

bsr     float_point             result in fp_buffer

.

now display ASCII numbers, if required

.

move.l  a0,-(sp)                display results with

move    #9,-(sp)                c_conws BIOS call at

trap    #1                      current cursor posn

addq    #6,sp

.

.

first_no    dc.b    '123.456',0             1st No=123.456

second_no   dc.b    '987.654',0             2nd No=987.654

.

fp_buffer   ds.b    50                      output buffer

.

The format for each function  with  the  input and output parameters is

shown below :-

1   Add two floating point numbers.

ENTRY d0.l and d1.l hold first fp number

d2.l and d3.l hold second fp number

d4=0

EXIT  d0.l and d1.l hold result of d0/d1 plus d2/d3

2   Subtract one floating point number from another.

ENTRY d0.l and d1.l hold first fp number

d2.l and d3.l hold second fp number

d4=1

EXIT  d0.l and d1.l hold result of d0/d1 minus d2/d3

3   Multiply two floating point numbers together.

ENTRY d0.l and d1.l hold first fp number

d2.l and d3.l hold second fp number

d4=2

EXIT  d0.l and d1.l hold result of d0/d1 times d2/d3

4   Divide one floating point number into another.

ENTRY d0.l and d1.l hold first fp number

d2.l and d3.l hold second fp number

d4=3

EXIT  d0.l and d1.l hold result of d0/d1 divided by d2/d3

5   Return the sine of a floating point number.

ENTRY d0.l and d1.l hold floating point number

d4=4

EXIT  d0.l and d1.l hold sine value

6   Return the cosine of a floating point number.

ENTRY d0.l and d1.l hold floating point number

d4=5

EXIT  d0.l and d1.l hold cosine value

7   Return the tangent of a floating point number.

ENTRY d0.l and d1.l hold floating point number

d4=6

EXIT  d0.l and d1.l hold tangent value

8   Returns the exponential of a floating point number.

ENTRY d0.l and d1.l hold floating point number

d4=7

EXIT  d0.l and d1.l hold exponential value

9   Returns the naperien log of a floating point number.

ENTRY d0.l and d1.l hold floating point number

d4=8

EXIT  d0.l and d1.l hold naperien log value

10  Returns the base10 log of a floating point number.

ENTRY d0.l and d1.l hold floating point number

d4=9

EXIT  d0.l and d1.l hold base10 log value

11  Returns the square root of a floating point number.

ENTRY d0.l and d1.l hold floating point number

d4=10

EXIT  d0.l and d1.l hold square root value

12  Converts an ASCII string into a floating point number.

ENTRY a0=start address of buffer which holds value in ASCII

d4=11

EXIT  d0.l and d1.l hold floating point number

13  Converts a floating point number into an ASCII string.

ENTRY d0.l and d1.l hold floating point number

d2 holds number of digits required after decimal point

as an ASCII character

d4=12

a0=start address of buffer (ensure buffer is big enough)

EXIT  d0=number of characters (excluding the NUL chr) in string

a0=start address of output buffer

14  Converts a floating point number into an integer.

ENTRY d0.l and d1.l hold floating point number

d4=13

EXIT  d0.l holds integer value

15  Converts an integer value into a floating point number.

ENTRY d0.l holds integer number

d4=14

EXIT  d0.l and d1.l hold floating point value

16  Tests if two floating point numbers are equal.

ENTRY d0.l and d1.l hold first floating point number

d2.l and d3.l hold second floating point number

d4=15

EXIT  d0=1 if numbers are equal or 0 if not

17  Tests if two floating point numbers are not equal.

ENTRY d0.l and d1.l hold first floating point number

d2.l and d3.l hold second floating point number

d4=16

EXIT  d0=1 if numbers are not equal or 0 if they are

18  Tests if a floating point number is greater than another one.

ENTRY d0.l and d1.l hold first floating point number

d2.l and d3.l hold second floating point number

d4=17

EXIT  d0=1 if 1st number is greater than 2nd number or 0 if not

19  Tests if a fp number is equal to or greater than another one.

ENTRY d0.l and d1.l hold first floating point number

d2.l and d3.l hold second floating point number

d4=18

EXIT  d0=1 if 1st number is equal to or greater than 2nd number or

0 if not

20  Tests if a floating point number is less than another one.

ENTRY d0.l and d1.l hold first floating point number

d2.l and d3.l hold second floating point number

d4=19

EXIT  d0=1 if 1st number is less than 2nd number or 0 if not

21  Tests if a fp number is less than or equal to another one.

ENTRY d0.l and d1.l hold first floating point number

d2.l and d3.l hold second floating point number

d4=20

EXIT  d0=1 if 1st number is less than or equal to 2nd number  or

0 if not

22  Returns the arc sin of a floating point number.

ENTRY d0.l and d1.l hold floating point number

d4=21

EXIT  d0.l and d1.l hold arc sin value

23  Returns the arc cos of a floating point number.

ENTRY d0.l and d1.l hold floating point number

d4=22

EXIT  d0.l and d1.l hold arc cos value

24  Returns the arc tan of a floating point number.

ENTRY d0.l and d1.l hold floating point number

d4=23

EXIT  d0.l and d1.l hold arc tan value

25  Returns the hyperbolic sin of a floating point number.

ENTRY d0.l and d1.l hold floating point number

d4=24

EXIT  d0.l and d1.l hold hyperbolic sin value

26  Returns the hyperbolic cos of a floating point number.

ENTRY d0.l and d1.l hold floating point number

d4=25

EXIT  d0.l and d1.l hold hyperbolic cos value

27  Returns the hyperbolic tan of a floating point number.

ENTRY d0.l and d1.l hold floating point number

d4=26

EXIT  d0.l and d1.l hold hyperbolic tan value

28  Returns the integer part of a floating point number.

ENTRY d0.l and d1.l hold floating point number

d4=27

EXIT  d0.l and d1.l hold integer part value

29  Returns the value of a number raised to the power of another one.

ENTRY d0.l and d1.l hold first floating point number

d2.l and d3.l hold second floating point number

d4=28

EXIT  d0.l and d1.l hold value of 1st number raised to the power of

the 2nd number

See also FP_MACRO.DOC file  for  macro  definitions. Programmers should

note that although it would be  OK  to  use the FLOAT.BIN file in their

own programs it could  be  a  breach  of  copyright  to  use  it in any

published programs. ICTARI accepts  no  responsibility  for programs or

programmers who may be  in  breach  of  copyright  in  respect of these

routines.