Together with this file you should find the following:

ENHANCER.PRG  The program to put into the  AUTO  folder - after any Speedo-

GDOS. Not needed with NVDI 2.5+ !

It should  work  with  any  standard  Atari  computer  screen

(ST/STE/TT/Falcon) though probably  not  with graphics cards.

For these latter ones, however,  there may exist special NVDI

versions (or other Enhancer compatible software) that support

the Enhancer functions.

ENHFIXED.PRG  A bug-fixed version of the above.

� Don't distribute without the original plus this text.

� Don't blame any  problems  with  this  program  on  2B, but

report them to me. Though, needless  to say, I won't accept

any legal responsibility for it.

Fixes:

- v_opnbm able to transform pre-allocated bitmaps

- vq_scrninfo(2) always writes documented 272 words, not 273.

And for STF colour returns 3 bits/colour component, not 4.

- v_get_pixel should now hopefully work on Falcon high-colour

bitmaps too (though I haven't been able to test this one).

+ some smaller irregularities fixed too, and I have increased

the speed of clearing  and  copying  (during  v_opnbm) a bit.

Plus I took the liberty  of  translating  a message in German

into English.

ENHANCER.TXT  Notes from the authors in German.

ENHN_ENG.TXT  The above translated by me into English.

NEW_FUNC.TXT  Documentation (in English)  on  the  new  VDI  functions made

available by the VDI Enhancer (or  NVDI). (As it isn't signed

with any names, I don't  know  from  whom  or from where this

came, but it was on the CD where I found the rest.)

Also see the function descriptions by me below.

The programmer only has  to  make  sure  that  a  cookie 'EdDI' is present,

before starting to use the new VDI functions.

My guess is that EdDI stands for "E(nhance)d( V)DI".

For anyone who doesn't know how the cookie jar works:

1)  Load the longword at address $5A0 -  in  SUPERVISOR mode! If it is zero

there is no jar (nor cookies), otherwise it is a pointer to the jar.

2)  Starting with the first longword  of  the  jar  and ending on finding a

zero longword, compare EVERY OTHER longword  with the cookie ID you are

interested in ('EdDI').

3)  If you encounter a zero longword before finding the looked-for one, the

cookie isn't there. Otherwise it is  present and the longword following

the ID is its "value" (the value  of  EdDI is a pointer to a subroutine

that returns the version number of the Enhancer if called with register

D0 cleared; this Enhancer version returns hexadecimal $100 for v1.00).

~~~~~~~~

M�rten

for

Ictari

***********************

THE NEW VDI FUNCTIONS

***********************

by:  M�rten Lindstr�m

VDI Enhancer v1.00 (or NVDI 2.5+) makes available the functions

v_opnbm       Open bitmap (as a VDI workstation)

v_clsbm       Close bitmap

vq_scrninfo   Inquire screen info ("extended vq_extnd")

and in addition claims to fix a  faulty behaviour of the old V_GET_PIXEL on

Falcon high-colour screens (?).

You will of course have  to  use  the  low-level  method of calling the new

functions (filling in the VDI  arrays  "by  hand")  if you cannot write new

bindings (or Devpac macros) for them.

The new functions use  the  same  main  OP  code  as v_opnvwk, v_clsvwk and

vq_extnd respectively (which internally  are  called  by the Enhancer). And

the parameters are also the same  -  including the GRAF_HANDLE to contrl[6]

for v_opnbm/v_opnvwk. But each of them  in addition require the SUB-OP code

(contrl[5]) to be set =1.

V_OPNBM

=======

v_opnbm in addition differs  from  v_opnvwk  in  taking  a  pointer, in the

longword in contrl[7..8], to  a  standard  20-byte  "MFDB"  (see VDI raster

operations) for the bitmap, and in taking  9 extra intin (or work_in if you

like) elements for a total of 20 intins; make sure to set contrl[3]=20:

intin[11]:      Width-1  (will be rounded up to 15, 31, 47, 63 etc.)

intin[12]:      Height-1

intin[13]:      Width  of a pixel in �m  OR  =0 for screen resolution

intin[14]:      Height of a pixel in �m  OR  =0 for screen resolution

intin[15..19]:  =0 (reserved)

If you clear  the  contents  of  the  MFDB  before  passing  it  to v_opnbm

(actually only the first longword + the  word  at offset 12 need be zeroed)

then v_opnbm will allocate  memory  for  the  bitmap,  clear  it (= make it

WHITE, which on a high-colour bitmap  means  setting  all bits) and fill in

the MFDB. The bitmap will have the same number of colours as the screen.

Alternatively, you can always set the (bits/pixel) word at offset 12 to =1,

to open a MONOCHROME bitmap.

And, finally, you have the option of setting the first longword in the MFDB

to point to a pre-allocated bitmap,  in  which  case v_opnbm will NOT clear

it. Instead the word  at  offset  10  (the  format  flag)  will  be used to

determine whether the bitmap  needs  to  be  transformed  from VDI standard

format to device specific format.

USING THE BITMAP

================

A bitmap opened with  v_opnbm  works  just  like  a normal VDI workstation,

though some function calls will be ignored:

� All VDI escape functions

� Mouse showing and hiding with v_show_c and v_hide_c

� Setting/getting of VDI pen colours  with  vs_color and vq_color, except

for bitmaps in high or true colour.

It should also be noted that the pixel size set in intin[13..14] is ignored

whenever setting the text  size  (the  bitmap  is  treated  like having the

screen resolution), so  I  don't  really  understand  the  point  of at all

allowing the pixel size to be set.

Raster copy operations work the same  way  when using bitmap handle as when

screen (virtual) workstation handle is used,  the difference being that any

zeroed pointers in MFDBs are  taken  to  refer  to  the  BITMAP and not the

screen. This is a little unfortunate  (ideally a zero pointer should always

refer to the screen) but it would have  been hard to implement it any other

way.

Also, any clipping will of course only be applied as set for the used work-

station, when copying to it.

Like with any use of multiple workstations, you must remember to change the

VDI workstation handle according to what settings you want applied (or even

where you want the output to go).

IMPORTANT: All off-screen bitmaps  are  in  device specific format. Meaning

that 100% clean programs in  principle  shouldn't directly manipulate them,

but rely solely on VDI calls.  VR_TRNFM can convert between device specific

and device independent formats. Regarding  this  a  number of things can be

said though:

1)  If opening a pre-allocated bitmap (see above)  you can specify it to be

in device independent format (word at  offset  10 in MFDB =1), in which

case v_opnbm will automatically convert  it into device specific format

(and clear flag).

You are thus free to draw,  low-level and still 100% cleanly, something

on the bitmap BEFORE opening it as a workstation.

(A word of  warning  here:  this  feature  was  bugged  in the original

Enhancer, and what's worse: it  doesn't  seem  to work with NVDI 3(.02)

either. On the other hand,  I  have  fixed  it in the "Ictari version",

ENHFIXED.PRG, and hopefully 2B will fix it in NVDI too.)

2)  When using a MONO BITMAP, the  risk  must  be insignificant - even with

graphics cards - for  there  to  be  any  difference between the device

specific and device independent formats  (smaller  even  than on a mono

SCREEN).

3)  The call VQ_SCRNINFO, also provided by  the VDI Enhancer, can give info

on the device format.

VQ_SCRNINFO

===========

vq_scrninfo is the same as vq_extnd, except for the required sub-op code =1

mentioned above, and for allowing the new  mode 2 (in intin[0]), which will

result in hardware details of  the  screen  and colour representation being

written to the intout array.

Word of warning: vq_extnd (with mode 2) produces its output as an unusually

large number of intout elements: 272  (=16+256)  of them, which is way more

than the 128 words traditionally  reserved.  Depending on how your compiler

has arranged the VDI arrays, the only result  of this may be for some other

general output or input  array  to  be  overwritten  (ptsin  and ptsout for

instance) which isn't much to worry  about,  but  in general it is probably

best to redirect the  intout  array  (temporarily)  to a specially reserved

272-word area (for users of GFA Basic 3 I have included a routine that does

that).

intout[0]:  The device specific screen and raster format:

0 = word-interleaved bitplanes (standard ATARI palette-colour)

1 = whole bitplanes          (= the device independent format)

2 = pixel-packed               (example: Falcon High colour)

-1 (or anything else) = unknown format

intout[1]:  CLUT (Colour LookUp Table, i.e. palette)

0 = No CLUT

1 = hardware CLUT (Palette colour. Returned even for ST mono.)

2 = software CLUT (used for VDI pens with high/true colour)

intout[2]:  Number of bits/pixel

intout[3..4]: (longword): Number of colours (or 0 if more than $FFFFFFFF)

intout[5..7]: reserved (0)

intout[8]:  Number of bits red    (For ST/STE mono, 0 is returned)

intout[9]:  Number of bits green

intout[10]: Number of bits blue

intout[11]: Number of bits alpha channel

intout[12]: Number of bits genlock

intout[13]: Number of bits unused

intout[14..15]: reserved (0)

intout[16..271] With CLUT:  Colour "hardware" indexes for VDI pens 0-255

(e.g. intout[16] gives index for VDI pen 0)

Hi/true colour: intout[16..31] gives the numbers within a pixel

for each of the  red  bits  0-15  (bit 0 is the

least significant bit).  E.g.  intout[16] gives

the pixel bit number  for  red  bit  0 (=11 for

Falcon  high  colour).   For   unavailable  bit

numbers (e.g.  5  and  above  in  Falcon  high-

colour) -1 is returned.

intout[32..47]: same for green bits 0-15

intout[48..63]:  - " -   blue  bits 0-15

intout[64..79]:  - " -  alpha channel bits 0-15

intout[80..95]:  - " -  genlock bits 0-15

intout[96..127]: - " -  unused bits 0-31

intout[128..271]: reserved (0)

ENHANCER VERSION

================

The VERSION of the VDI Enhancer  can  be deduced by first clearing register

D0 and then making a subroutine jump  to  the address found as the value of

the 'EdDI' cookie. On return, D0 will contain for instance $100 for version

1.00. It is important that D0 is cleared before the call, since it is taken

to be an Enhancer function number  (function  0  = GET VERSION  is the only

such function yet implemented).

Since everything described above is  implemented already from version 1.00,

there may not be much point in checking the version number, but if there is

any later Enhancer version that we can  get  hold of - WITH documentation -

things will change of  course.  (NVDI  3.02  returns  v1.10  as the version

number of its "built-in VDI Enhancer".)