BLACK BOX ADDENDUM

The power supply for your Black Box uses a seven pin power connector. It is VERY important this connector is inserted properly; else damage to the BB will result

There are red markings on the top of the power connector. Be sure than* are ON TOP. Line up the pins on the BB's 7 pin connector with the holes '18 the connector, and press firmly. It is a good idea to plug in this connector before plugging in the supply to your AC wall outlet, .

With version 1.3 of the Black Box firmware, a new feature has been added*,; added.

Hardware RTS/CTS flow control may be enabled for the RS232 port. This 14

primarily used for 9600 baud modems, where the modem can buffer incoming

data. With flow control enabled, no loss of incoming data should ever

occur, even if you're not in term mode! Remember, this only works with

high speed modems supporting hardware flow control, and when using

correctly-wired cables.

In the Port Statuses configuration page, the RS232 port will cycle.

between OFF, ON, and FLOW. If you experience lockups when in terminal

mode, disable the flow control (by setting this parameter to ON instead

FLOW).

Due to the addition of the new feature, bit 6 of the byte at. $D6ED (page 26 of the BB manual) has been moved to bit 6 at SD6F2. The flow control F flag is bit 7 of this same byte.

One other undocumented feature of the BB is the coldstart key. By holding the HELP key as you press and release RESET, the computer will act as if it were turned off and back on, without losing anything that may be in a RAMdisk.

Black Box Add-ons

Currently, we have the Black Box Enhancer available as an add-on to the stock RD. It enhances the graphic screen dump by using dither patterns to reproduce 16 shades of gray on your printer. It also uses a slightly different pattern for each different color, so that the best possible representation of what's on the screen may be had.

The BB enhancer includes a graphics driver that allows all ATASCII

characters (inverse and graphic) to be printed on a graphics-compatible

printer when output is sent to the P9: device (similar to ANALOG'S G:

device).

The last and best addition the BB Enhancer provides is the built-in Task

Master sector editor. This powerful editor features a built-in sector copier (that uses all available memory), sector editor, disassembles, and more. Each function will access the full 16 mew range of hard disk partitions!

The BB Enhancer is available for $49.95, and as always, installation is free .if you send in your BB.