AtariServer V1.0

What is AtariServer:

AtariServer is a Windows 95/NT application which emulates one or more Atari floppy drives for one or more Atari computers.    It executes as a native multithreaded Win32 application.    You can put it in your 95/NT startup folder and it can run in the background, leaving you in Windows (or at the DOS prompt) to do other things.    When minimized, the main window disappears to prevent clutter on the desktop.    At the lower right, on the start menu bar, AtariServer inserts a small icon.    At any time, the user may doubleclick it to re-display the AtariServer window and manipulate the connections and floppy drives.

The AtariServer window allows the user to connect one or more Atari computers to the PC and "mount" up to 4 disks for each to use.    The number of Atari computers concurrently connected is limited by the number of free serial ports on your PC (and at some point your computer's performance).

AtariServer stores images of floppy disks in "disk files".    Each disk file represents exactly one 90k single density or exactly one 180k double density Atari floppy.    AtariServer provides a facility to create a blank disk file.    The disk image is not formatted because the Atari filesystem information is not written to it.    To put data onto it, the user can employ an Atari and floppy drive, then either sector copy to the floppy image or format the AtariServer disk file using Atari DOS and then copy files.    AtariServer disk files never have bad sectors and cannot emulate them at this time.    Once a disk image is formatted, it will work just as a single density 810 (except lots faster), but does not support any SIO commands other than than read/write sector, format and status.    This makes AtariServer generally unsuitable for archiving copy-protected programs (cracked programs should work OK, however).

Each disk image has its own description field and write protect setting.    If set as write protected, Atari format and write sector commands will fail- but this setting does NOT protect the disk image file itself from being deleted by the PC user!

What AtariServer doesn't do:

At this time, AtariServer does not emulate any devices other than a single density 810 type drive.    It does have support for the 815 double density drive, although it is buggy and I do not recommend its use for other than testing.    There is no provision for printer or serial devices at this time.    Technically, there is no reason why AtariServer cannot support these, in addition to other types of floppy emulation.    I am interested in hearing what people would like to implement (that is, if anyone actually cares... ;)      ).

If you want any advanced capabilities like happy, copy protection support, 3rd party drive support, alternative disk formats, for example- AtariServer will not work for you at this time, instead I suggest you consider APE, which is a very worthy program.    If you already use APE (or another), it is almost certain AtariServer won't give you anything you don't already have (other than a Windows interface).

Why have AtariServer?

Because I felt like writing it, mostly.    I've always wanted to do a disk emulator, so I wrote one.    The previous version is a Dos program, it worked pretty well but I was never happy with its serial port performance.    I made the second version a Windows program, mostly to take advantage of the better user interface tools and to make it a bit more usable.    Version 1 wouldn't run under Windows, therefore limiting the use of the PC when running AtariServer.

AtariServer is essentially beta software at this time.    Its nominally reliable, but continues to need updates and fixes.    If you want a platform to experiment with the Atari SIO bus, AtariServer offers a substantial advantage (the source code is freely available).

AtariServer is unique for a couple reasons;

- It is released via the GNU license, so you can use it for free and update it as you see fit.

- You can connect as many Atari computers as you have SIO2PC cables and serial ports on your PC.

- You can share a disk between multiple Ataris connected to the same PC, allowing a primitive form of networking/file sharing.    You can save arrangements of serial ports and disk mounts, allowing you to boot Windows and automatically have a convient arrangement of disks set up for you, or quickly change around which floppies are mounted for your connected Atari computers.

- AtariServer has a pretty useful dialog box which allows you to sort a library of disks by disk name, description or directory contents.    That lets you quickly find which of your images has a particular file.    One thing I'm particularly interested in adding is a decent indexing system for classes of programs.

- Since AtariServer is a Win32 program, it is quite suited to provide TCP/IP service to Atari systems.    That will require writing the Atari handler, in addition to the AtariServer enhancement.    This is something I'm particularly interested in and will continue to pursue it.

What do I need to use AtariServer:

A computer running at least Windows 95.    I suggest a Pentium 150 or equivalent as a reasonable minimum.    A fast 486 should work, but Windows imposes a lot of CPU overhead and the Atari SIO bus has rather critical timing constraints which may cause errors on slower systems.    I suggest at least 16 megs of memory and a few megabytes of free disk space.    The Atari floppy files can be stored on a network- the image data is held in RAM while the disk is mounted, so there are minimum performance limitations.

Although I've never tested this directly, AtariServer should work OK with standard SIO2PC cables.    I built my own, but made the same pin assignment choices, so there should be no problems.    If there are conflicts, I will immediately make the appropriate changes to AtariServer.    For what its worth, AtariServer does not honor the COMMAND pin of the SIO cable (because the stupid Windows serial port driver won't let me get hold of its value at the right time.)

If you want to work with the AtariServer source code, you will need Microsoft Visual C 5 or greater.    Version 4.5 may work OK because the MFC version is the same, but the I don't know if the project and ClassWizard files are at all portable.    AtariServer uses MFC, and requires the programmer to have nominal familarity with threads, the Win32 API and Windows software architecture principles (such as they are...).    Sorry about that, I'm teaching myself Linux and soon hope to be done with Windows.

What problems will I have:

Windows 95/NT do not multitask well, therefore anything being performed on your PC while AtariServer is interacting with an Atari will disturb the SIO protocol timing, potentially causing errors.    If Windows is sitting idle, things should work OK.    This is a shortcoming of Windows which I've been trying to work around, but it may be impossible to prevent in the current generation of Windows products.    Although Windows offers a realtime, critical thread priority, its execution scheduling becomes erratic if something else is being done on the PC- even something as innocuous as scrolling a web browser.

Can AtariServer work with APE (or others):

Yes, but likely not simultaneously on the same PC, (for instance APE is a DOS program which should not be run under Windows).    However, APE on one PC could emulate several floppies/printers/serial ports while AtariServer on another PC emulates another couple floppies, for example.    The Atari computer could copy disks from one emulator to another in this case.

AtariServer uses its own format for disk images (lots simpler, probably not better) than other disk emulator software.      Conversion utilities can be written and incorporated as time and circumstances permit.    The AtariServer floppy image has a fixed size header followed by contiguous disk sectors of the appropriate size.    The header contains the disk image version, description, write protect status, floppy type and a bunch of space for expansion.    This does impose some architecture problems if you want to support more esoteric disk formats, so direct .PRO or .ATR support will likely be useful at some point.

AtariServer Files:

The latest AtariServer can be found at the following addresses:

http://www.erols.com/menkesjg/AtariServerSrc.zip		- Source Only

http://www.erols.com/menkesjg/AtariServerApp.zip		- Executable only

http://www.erols.com/menkesjg/AtariServer.doc		- This file in Word 6 format.

(I'll put up a homepage when I convince my wife to do it for me.... ;)      )

Installation of the executable version:

Use Winzip to decompress the .zip file onto your C: drive.    The output is a directory; C:\AtariServer, within which you'll see AtariServer.exe.    On your desktop or in a folder/menu, create a shortcut to it and you're all set.    Run AtariServer, select the View option and select Settings.    The dialog which appears indicates the location of the disk files used by AtariServer.    Click the button with three dots in it to raise a dialog allowing you to select drives and subdirectories.    Select the DiskFiles subdirectory under the directory where AtariServer.exe is located and hit OK.

Installation of the source code:

Use Winzip to decompress the .zip file.    One complication here is caused by the Visual C project files.    Path references are stored in the project, I do VC development in V:\Projects\AtariServer, so if you put the files on a different drive you will likely have to adjust the project files to find the source code.    In the end, it should work OK, but converting can be tedious.    If possible, create a file share instead; share the directory in which Projects is located as V: and the paths will work out OK.

To use AtariServer

1.	First, select the File/Open New Comm Port option.    Choose the COM port to which the SIO2PC cable is connected and give it a short description.

2.	Note the screen displays a little "control-panel" for the port you just opened.    Each of the 4 buttons labeled "Mount" will raise a dialog box where a given disk image can be selected.    The first button represents D1 and the last D4.    If no disk is mounted for any of the 4 entries, AtariServer will ignore disk requests for those drives.    Press the Mount button which represents the Atari drive you want AtariServer to emulate (if you're booting off D1, choose the second button representing D2, for example).

3.	Since you don't have any disk images yet, choose "Create New".

4.	Select the disk type, enter a filename (just the filename, no path), and enter a description for the disk.

5.	This option creates and mounts a blank, unformated disk.    Boot your Atari and use a sector copier to copy an Atari diskette onto the new, mounted disk image.

6.	When you choose File/Save, the configuration of serial ports and disk image mounts is saved.    Later you can File/Open to reload the full set of ports and images.

7.	Once you have bootable disk imaged created, you can rely on AtariServer to boot your Atari by emulating D1 and Mounting the appropriate disk image.

Please forgive the terseness of the documentation and source code comments.    This is very much a work in progress.

I may be reached at: 'menkesjg@erols.com'.

Thanks,

Greg Menke