DESKTOP.PRG & DESKTOP.DOC

Copyright 1990-1991  ICD, Inc.

+--------------------------------------------------------------+

NOTE: This file is a SUPPLEMENT to the AdSCSI ST manual which

contains more extensive documentation on DESKTOP.

+--------------------------------------------------------------+

DESKTOP.PRG was previously a desk accessory (DESKTOP.ACC).  It

is now a program so it only uses memory when you actually run

it.

Note that you must have version 5.0.0 or greater of ICDBOOT

installed in order to install and run this version of

DESKTOP.PRG.

When running DESKTOP.PRG from NEODESK, you need to be aware of

the fact that it will not put the partition names in the icons,

or change paths in open windows.  Maybe we can do this later.

Otherwise, it works fine with NEODESK.

A check mark under the Boot column in the Available partitions

window indicates that this partition's boot sector contains

bootable code and that ICDBOOT.SYS can be loaded from this

partition when it is the first GEM partition enabled in the

system.  This does NOT necessarily mean that the AUTO folder and

the ACCessories will be loaded from this drive.

An asterisk ('*') next to a drive partition's name in the

Logical drives window indicates that the drive is the current

Autoboot drive.  This is the drive you selected in HDUTIL when

you set up your hard disk to Autoboot.  This is the drive where

the AUTO folder and ACCessories are loaded from.

To revert to "pristine" logical drive order, click on Unswap.

This makes your first partition C, next D, etc.

To save your swap order permanently, click on Save.  This will

write the swap order into ICDBOOT.SYS.  Next time you boot, you

will be using the new swap order.  If you swap your Autoboot

drive (the drive where the system looks for the AUTO folder and

ACCessories), you will be asked if you want to save a copy of

your DESKTOP.INF file on the new Autoboot drive.  This is usually

a good idea.  If you have swapped any drives corresponding to

open windows in your DESKTOP.INF file, you will be asked to

update the DESKTOP.INF.  This is also a good idea; otherwise, you

will come up without your windows open.

When DESKTOP.PRG terminates, the new swap order will be

reflected in the drive icons; also any open windows affected by

the new swap order will be changed.

A check mark under the On column in the Available partitions

window means that that partition is currently enabled (or

'visible' to ICDBOOT).

To enable or disable active partitions, click on the appropriate

line under the On column.  Up to 14 GEM partitions may be enabled

at any given time.  You can have a total of 64 partitions for a

physical hard drive (LUN) with a maximum of 128 partitions in the

system.  Actually more than 14 partitions can be enabled, but

ICDBOOT and TOS will only see the first 14 GEM/BGM partitions.

To name or rename a partition, click on any GEM or BGM partition

name in the Available Partitions window.  To get out of rename

mode, click anywhere else.  You can only edit (name/rename) GEM

and BGM volume names.  DESKTOP.PRG has no way of knowing how

non-GEM partitions keep their volume names, hence you can't edit

these.

Bear in mind, your volume names may be upper or lower case, with

spaces.  If you do a Show Info on the partition, the volume name

is truncated at the first space.  In the DESKTOP.PRG dialog,

volume names are shown with periods at the ninth character.  This

is strictly in keeping with the GEM convention of showing a

volume name as a file name.  The period will NOT show up in the

icon.  Be advised that it is NOT a good idea to use the pound

sign ('#') character as part of a volume name.  This confuses

TOS.

To turn on the write verify TEMPORARILY click on the On or Off

button as desired.  This is the same function provided by our old

VERIFY.ACC, and does not affect the Write Verify configuration in

ICDBOOT.  It DOES affect the verify on your floppy disk drives,

so you should probably leave it On most of the time.  The verify

does slow down file transfers, however it is usually worth the

extra time to insure good data.

To toggle the write cache function, click on the appropriate On

or Off as desired.  When turning write cache off, all buffers

will be flushed if needed, and write caching is then disabled

until rebooting or until you explicitly turn it back on.  This is

done for those using the Supercharger PC emulator.  The

Supercharger MUST have write caching disabled, otherwise it will

NOT write to the hard disk.  Once again, this is TEMPORARY.  It

does not make any permanent changes to your ICDBOOT file.  It

only changes the booter in memory.

While in DESKTOP.PRG, you can force a rescan of all hard drives

by clicking on the Rescan button.  This is equivalent to running

DESKTOP.PRG from the beginning.

You MUST finalize any actions undertaken here by clicking on OK

or Reboot.  If you click on Cancel, we will undo what we can.

ICD, Inc.

Note:  We have tested this program thoroughly and believe it

to be free from problems.  However under no circumstances will

ICD, Inc. be held responsible for any damage to information on

your hard drives.