AHDI README.DOC (July 21 1992 - ml.)

===============

AHDI is the hard disk driver for the Atari 680x0-line of computers.

It now supports the ACSI, the SCSI and the IDE interfaces.

The hardware dependent code of AHDI is also used by programs like

HDX, FHDX and TSTDRIVE.  Those programs will need all the .H files,

and all the .S files except DRIVER.S and INSTALL.S

To build the driver, just run "make" with the MAKEFILE in this

directory.

Refer to AHDI 3.00 Release Notes for a detail explanation of the

patchable variables, the extended GEMDOS partition and the big

GEMDOS partition.

The main difference between version 5.00 and version 6.00 is the

support of Sparrow SCSI.  The new files are SPSCSI.H and SPSCSI.S.

The driver code consists of :

ACSI.H	    BLITTER.H	DEFS.H	    ERROR.H	IDE.H

MACROS.H    MFP.H	SCSI.H	    SPSCSI.H	SYSVAR.H

DEFS.H	    - Most of the conditional assembly and global equates

are here.

When compiling for programs like HDX and FHDX, set

the "DRIVER" conditional assembly flag to 0.  This

will include routines (e.g. format) which are not

needed in the resident driver.

68030.S	    - Macros to access the cache registers when running on

the 68030.

ACSI.S	    - Set up the ACSI DMA channel to send commands, to send

and to receive data.  The clean up code is also used by

the Sparrow SCSI code since Sparrow SCSI uses the same

channel.

When the standard _rcvacsi() and _wrtacsi() routines

(e.g. the format routine) are not used to do the I/O,

remember to set and clear "flock" appropiately.  If

not, the floppy activities may interfere with the hard

disk activities.

CMDBLK.S    - Stuff the command block for all SCSI and ACSI commands.

COOKIE.S    - Allan Pratt's getcookie() code.

DOIT.S	    - Decide which interface (SCSI, ACSI, or Sparrow SCSI) to

use for a command, and send it on its way.

DRIVER.S    - Most changes made to the driver are documented at the

beginning of this file.

This file contains the hardware independent file-system

level code, all the global variables, and the entry

points for the BIOS.

This file MUST be the FIRST file when linking, because

it contains the patchable variables which locations are

published.

IDE.S	    - Set up the IDE channel to do either programmed or

BLiTTER I/O.

DON'T do BLiTTER I/O on the STBOOK!  To do BLiTTER I/O

on the Sparrow, the "badcombo" flag initialized in the

file DRIVER.S needs to be set to 0.

INSTALL.S   - This file contains the code to look for devices and

initialize the globals.

This file MUST be the LAST file when linking, because

the Mshrink() or Pterm() done at the end of the driver

uses a label in this file to calculate how much of the

driver to keep.

SCSI.S	    - Set up the TT SCSI channel for either programmed or

DMA I/O.

The Falcon 040 SCSI should be able to use most of the

TT SCSI code.  There are a couple of fixes that I know

of now.  First, there is no need to use the residue

register anymore, the hardware will flush the FIFO.

I think the register will stay for compatibility.

Second, the TT SCSI now bus errors if you try to DMA

to the end of memory.  That should be fixed in the

Falcon 040 SCSI.  For the most up-to-date information

about these issues, contact John Hoenig.

SPSCSI.S    - Set up the Sparrow SCSI channel for either programmed

or DMA I/O.

Sparrow SCSI is accessed by talking through the ACSI

DMA chip.  Refer to the macros in SPSCSI.H to access

the SCSI registers.

When the standard _rcvspscsi() and _wrtspscsi() routines

(e.g. the format routine) are not used to do the I/O,

remember to set and clear "flock" appropiately.  If

not, the floppy activities may interfere with the hard

disk activities.

Refer to the comments in the source files for a detail explanation of

the different functions.