DMACXROM.DOC - DOCUMENTATION VERSION 2.7

(C) 1988 HAPPY COMPUTERS, Inc.

This document describes the usage of the DMMXFER program, and also

describes the usage of the ROM/EPROM sockets option, the LO/HIGH

program select switch option, the cartridge jack option, and provides

some technical information regarding the power consumption and address

space used by the DISCOVERY CARTRIDGE & options.

DMMXFER PROGRAM:

This program will operate with all DISCOVERY CARTRIDGES.  No optional

hardware is needed to run DMMXFER.  First, we provide some definitions:

A MACINTOSH format disk is a 3.5 inch 400K MFS, or 800K HFS disk that

is usually generated on a MACINTOSH computer made by APPLE Computer

Corporation.  Without special additional hardware, a standard Atari

ST/MEGA computer cannot read or write disks in the MACINTOSH format.

A MAGIC format disk is a 3.5 inch disk specifically formatted on an

Atari ST/MEGA computer, for use with the MAGIC SAC product made by

DATA PACIFIC.  A standard Atari ST/MEGA computer can read and write

disks in the MAGIC format directly, without any additional disk

drive hardware.

The MAGIC SAC is a product made and sold by DATA PACIFIC.  DATA

PACIFIC claims that the MAGIC SAC product allows your Atari ST/MEGA

to run programs originally written for the MACINTOSH computer.

HAPPY COMPUTERS cannot offer any assistance concerning the usage

of DATA PACIFIC's products, or the execution of MACINTOSH programs

on your computer.  HAPPY COMPUTERS is not affiliated with DATA PACIFIC.

DATA PACIFIC offers a TRANSLATOR product that DATA PACIFIC claims will

permit your ST to directly access MACINTOSH format disks.  HAPPY COMPUTERS

has found that the most efficient use of the MAGIC SAC product is with

MAGIC FORMAT disks, without the need for a TRANSLATOR.

Reading and writing MACINTOSH formatted disks with DATA PACIFIC's

TRANSLATOR is much slower than reading and writing MAGIC format disks.

The DMMXFER program allows your Atari to directly read a disk formatted

for the MACINTOSH computer, and converts this into the MAGIC format.

The MAGIC format disk contains the same information that was on

the MACINTOSH format disk, but the disk can be used efficiently by

the MAGIC SAC product, without a TRANSLATOR or DISCOVERY CARTRIDGE.

The transfer in the opposite direction, from MAGIC format to MACINTOSH

format, is also provided by the DMMXFER program.

At present, the DISCOVERY CARTRIDGE's software does not provide access to,

or transfer of individual files on MACINTOSH or MAGIC format disks.

Entire disks must be transferred.  All files on the disk are transferred.

We understand that there are "shareware" programs available which

allow the ST/MEGA computer to directly access the files on a MAGIC

format disk, without the need for the MAGIC SAC.  HAPPY COMPUTERS

cannot provide any information concerning these programs.

The operation of the DMMXFER program is straight forward.  The

first menu allows selection of transfer type.

The options menu allows the user to select the source and destination

drives, which must both be 80 track floppy drives.  Up to 4 different

source and destination drives are possible if the DRIVE 3/4 option

is installed in your DISCOVERY CARTRIDGE.  See the DDRIVE.DOC file for

further information.  If the conditions for accessing a 3rd and 4th

drive are not met, then only drive A and B can be selected as the

source and destination drives.

The options menu allows the user to select single or double sided

transfers.  The DMMXFER program does not determine if a single or

double sided transfer is needed.  The user must determine this,

based on the user's knowledge of the content of the source disk.

The options menu allows the user to specify "BUFFER FILE" mode.

Normally, the DMMXFER program uses the RAM in the computer for

temporary storage of the disk data during the transfer operation.

This is fine for most uses where there is sufficient RAM, and/or

you are using different drives for the source and destination

drive.  If you only have one drive for both the source and

destination drive, you always have to swap the source and

destination disk during the transfer process.  If your computer

doesn't have very much RAM available for temporary storage, you

may have to make several swaps between the source and destination

disk.  The "BUFFER FILE" mode allows the use of a temporary file

on your hard disk drive, to require only one insertion of the

source and destination disk.  To use this feature, the program must

have been loaded from a hard drive.

MACINTOSH disks that contain bad sectors or copy protection can be

transferred to MAGIC format by DMMXFER.  When a bad sector is found,

the DMMXFER program informs the user and gives the option to proceed

or abort the transfer operation.  A bad sector is never produced on

the destination disk, so copy protection is not copied by this program.

The MAGIC FORMAT used by the MAGIC SAC does not provide for the

emulation of copy protection with bad sectors.  If you permit the

program to continue the transfer when a bad sector is encountered,

the sector written to the destination disk will have a good status, but

there will probably be a loss of data in that sector.  Bad sectors

that are found when reading the MAGIC format source disk, are handled by

DMMXFER in the same manner.

NOTE: There are different quantities of sectors per track on MAGIC

disks when compared to MACINTOSH disks.  Therefore, the program may

write a different quantity of tracks than it reads in.  Also note that

the 2 sided transfer may appear to read and write in different track

and side sequences.  This is all normal.

HARD DISK 20

When using MAGIC format disks which are created from an HFS single

or double sided disk, your startup disk must contain the file HARD

DISK 20, or a 64K MACINTOSH emulator, such as the MAGIC SAC or a SPECTRE

with 64K roms, will not recognize the MAGIC format disk.  This does not

apply to a SPECTRE with 128K ROMS, since the newer 128K operating system

automatically recognizes the HFS disk format.

PROBLEMS READING AND WRITING MACINTOSH DISKS

You may find that a particular drive will not read or write MACINTOSH

format disks.  The specification for ATARI drives does not require

that they are able to do this.  It is a nice coincidence that the

DISCOVERY CARTRIDGE will work with about 90% of all Atari drives

for the purpose of reading and writing disks in the MACINTOSH format.

The DISCOVERY CARTRIDGE hardware and software has already been optimized

to permit reading and writing of disks in the MACINTOSH format.  It

has many improvements that make it superior to the TRANSLATOR, sold

by DATA PACIFIC, for this purpose.  If you have a drive that doesn't

properly read or write disks in the MACINTOSH format, you can't blame

the drive, since it probably wasn't designed to do this.  You can't

blame the DISCOVERY CARTRIDGE, since it will work if the drive doesn't

filter the data.  Sorry, it's really a coincidence that this whole

thing works at all!  Don't beat your head against the wall, just find and

use a drive that will work.

ROM/EPROM SOCKETS OPTION:

The DISCOVERY CARTRIDGE can have ROM/EPROM sockets installed.  This

feature is included in FACTORY installed options 1 and 2.  HAPPY

COMPUTERS does not currently supply any ROMS or EPROMS for installation

in these sockets.  These must be obtained elsewhere.

There are two sockets, and both sockets are usually used.  A variety of

chip sizes can be used.  More information on chip sizes and addressing

is presented later.  It is important to note that the chip plugged

into the U4 socket on the DISCOVERY CARTRIDGE will be in the hi byte

(D15-D8) of the word accessed by the computer.  The chip plugged into

the U5 socket on the DISCOVERY CARTRIDGE will be in the lo byte (D7-D0).

If your are plugging in 64K MACINTOSH ROMS, the -0220 chip plugs into U4,

and the -0221 chip plugs into U5.

The user must plug in their own EPROMS or ROMS and provide for the

setting of the pin 1 state, as described below.  ROM / EPROM 28 pin

sockets may also be installed on your DISCOVERY CARTRIDGE outside

our factory.  Only the sockets are needed to provide this option.

It may be necessary to unsolder the holes in the board where the sockets

are to be mounted.  Please be aware that any modification made to your

DISCOVERY CARTRIDGE, including but not limited to the installation

of options outside HAPPY COMPUTERS' factory, will void your warranty.

Plugging in ROMS/EPROMS into sockets that were factory installed

by HAPPY COMPUTERS does not void the WARRANTY on your entire DISCOVERY

CARTRIDGE.  However, replacement of factory installed sockets, that are

damaged by negligent handling, is not covered under warranty.

If EPROMS are used, they must be programmed before they are used

in the DISCOVERY CARTRIDGE.  The DISCOVERY CARTRIDGE does not have

the hardware needed to write a program into an EPROM.

The most likely candidate for these sockets are MACINTOSH 64K OS

ROMS, which would be intended for use with the MAGIC SAC product.

HAPPY COMPUTERS cannot guarantee the performance of another company's

product when used with the DISCOVERY CARTRIDGE.  HAPPY COMPUTERS

has found that the current version of the software portion of the

MAGIC SAC product from DATA PACIFIC will treat a DISCOVERY CARTRIDGE

which has a battery backed up clock option and MACINTOSH ROMS, just

like the MAGIC SAC's hardware, and allow the MAGIC SAC program to

operate, but we cannot guarantee this.

The user could achieve a superior system by substituting the DISCOVERY

CARTRIDGE for the MAGIC SAC hardware cartridge.  The battery backed

up clock in the DISCOVERY CARTRIDGE is much more accurate than the clock

in the MAGIC SAC cartridge, and the DISCOVERY CARTRIDGE allows the

user to directly convert disks to and from the MACINTOSH format.

The MAGIC SAC cannot do this.  DATA PACIFC wants to sell you a slow and

expensive separate box to do this, called the TRANSLATOR.

HAPPY COMPUTERS cannot offer a legal opinion concerning the usage of

APPLE COMPUTER's programs in other computers.  We can state that the

DISCOVERY CARTRIDGE has not copied any copyrighted portion of the

MAGIC SAC product.  The primary reason that the MAGIC SAC is able to

run MACINTOSH programs is by virtue of the MACINTOSH ROMS.  There is

no computer program written by DATA PACIFIC inside the MAGIC SAC

CARTRIDGE.

The DISCOVERY cartridge does not rip off the MAGIC SAC.  We do

not provide any product which allows running MACINTOSH PROGRAMS.

The user must purchase this separately; contact your dealer.

The EPROM/ROM sockets could be used by a limitless variety of

other computer programs.  In the future, HAPPY COMPUTERS may offer

ROM based programs for use in the DISCOVERY CARTRIDGE.  These may

also be available as public domain, or "shareware".

We are not predicting the future, but it is possible that DATA

PACIFIC may attempt to prevent future versions of their MAGIC SAC

product from operating with a MACINTOSH ROM equipped DISCOVERY CARTRIDGE.

This may consist of some hardware lockout which looks for the HART chip,

and stops the MAGIC SAC software from running. The DISCOVERY CARTRIDGE

has a feature which hides the HART chip to account for such cases.

Our current software release does not utilize this feature.

128K CONTIGUOUS BYTES and SPECTRE 128

The DISCOVERY CARTRIDGE can be used to contain a ROM or EPROM program

which requires a total of 128K bytes in a contiguous address space.

The SPECTRE 128 product, which is sold by Gadgets by Small, requires

this 128K contiguous address space.  The implementation of a 128K

contiguous address space, for ROM or EPROM based programs plugged into

the DISCOVERY CARTRIDGE, is discussed in the documentation file

D128CROM.DOC.

NOTE: A JUMPER WIRE OR LO/HIGH SELECT SWITCH IS REQUIRED!

The logic state of pin 1 on the ROM/EPROM sockets must be defined by

the user.  Either a jumper wire or a switch can be used to do this.

The factory installed options package 1 requires that the user must

install a jumper wire, which is not provided by HAPPY COMPUTERS.

The board connects pin one of both sockets together, so a jumper to

either socket will also define the state of pin 1 on the other ROM/EPROM.

If you are using MACINTOSH 64K ROMS, pin one must be jumpered

to 0 volts.  Jumper MAC ROM pin 1 to MAC ROM pin 14.  Most EPROMS

require that pin 1 be jumpered to +5 volts.  Jumper EPROM pin 1 to

EPROM pin 28.  The EPROM data sheet for the particular EPROM manufacturer

and type used should be consulted to identify the correct state required

for pin 1, to enable reading from that particular EPROM.

This jumper may consist of a solid wire of 30 gauge or smaller which

is pushed into the socket along with the chip.  If properly installed,

this type of jumper can make good contact.  It is also possible

that a jumper which is improperly installed will make poor contact and

even damage the socket.

This jumper may also be soldered directly between the pins on the chip,

or may be soldered to the circuit board in the position where SW2

could be located.

The Program LO / HI bank select switch, which is provided as part of

FACTORY installed option 2, eliminates the need for a jumper.  This

switch allows setting the logic state of the ROM/EPROM pin 1 by the

selected position of the switch.  Select the LO position for 64K

MACINTOSH ROMS.  Most 256K EPROMS need pin 1 set HI; consult the data

sheet for your particular EPROMS.  If 512K EPROMS are used, this

switch allows the user to select one of two 256K banks to be

accessed by the computer at that time.  For example, 512K EPROMS

could be programmed with the MACINTOSH 64K program in one bank, and

some other program in the other bank.  Copying MACINTOSH ROMS into EPROMS

may be a violation of APPLE's copyright.  HAPPY COMPUTERS cannot

advise you on the legality of doing this.  We are cautioning that

it may be illegal.

There is a position on the DISCOVERY CARTRIDGE board near the ROM/EPROM

sockets for a capacitor C9.  This capacitor is not installed by HAPPY

COMPUTERS.  Some EPROMS may not operate properly without additional

decoupling capacitance.  HAPPY COMPUTERS cannot determine which

manufacturers or types of EPROMS/ROMS may require additional

decoupling capacitance, nor can we provide the capacitance value.

NOTES ON EPROM TYPES AND SIZES:

We will discuss the most common usage.  Always use chips that have an

access time of 200 nanoseconds or faster; consult the manufacturer's

data sheet for the speed rating.  Almost any size of EPROM up to

512K bits per chip can be easily used in the DISCOVERY CARTRIDGE.

Compare the manufacturer's data sheet pin out for address, data,

power, and select connections against the pinout provided to the EPROM

by the DISCOVERY CARTRIDGE as detailed on our schematic.

27C256-2 chips can be used.  In this case, each 256K bit chip provides

32K by 8, for a total of 64K bytes.  All of the 64K bytes can be

directly accessed by the computer at one time using addresses

$FA0000 to $FAFFFF with the factory configured jumpers.

27C512-2 chips can be used.  In this case, each 512K bit chip provides

64K by 8, for a total of 128K bytes.  Only 64K bytes can be accessed

directly by the computer at one time.  SW2 or additional bank select

circuitry must be provided to determine which 64K byte bank is being

accessed.  SW2 or the bank select circuitry defines the logic state

of the A15 address pin, which is the most significant address line on

the EPROM chips.  Currently, HAPPY COMPUTERS cannot provide any

information concerning the bank select circuitry.

USE LOW POWER ROMS OR EPROMS

To reduce the power consumption, HAPPY COMPUTERS recommends using

CMOS ROMS or EPROMS.  It may be a violation of copyright to copy

a computer program from one chip to another, even if this increases

the energy efficiency and increases the reliability.  HAPPY COMPUTERS

can't offer you a legal opinion on this matter.

SECOND CARTRIDGE JACK:

CAUTION: Do not plug any CARTRIDGE into this jack without first

determining that there is no electrical conflict with the DISCOVERY

CARTRIDGE.  It is possible that the conflicting cartridge and/or the

DISCOVERY CARTRIDGE will be damaged.

The cartridge jack is provided as part of the FACTORY installed options

3 and 4.  HAPPY COMPUTERS does not provide the cartridge to be plugged

into this jack.  HAPPY COMPUTERS does not guarantee that any cartridge

will work properly in this jack.  The user must determine this.  The

jack may also be installed on your DISCOVERY CARTRIDGE outside our

factory.  Only the jack is needed to provide this option.  It may be

necessary to unsolder the holes in the board where the jack is to be

mounted.  Please be aware that any modification made to your DISCOVERY

CARTRIDGE, including but not limited to the installation of options

outside HAPPY COMPUTERS' factory, will void your warranty.  Plugging

a cartridge into a jack that is factory installed can void your warranty

since it is possible for a cartridge to electrically conflict with

the signals of the DISCOVERY CARTRIDGE.

HAPPY COMPUTERS has determined that the MAGIC SAC and the DESKCART

cartridges do not electrically conflict with the DISCOVERY CARTRIDGE,

and will work properly when used as a second cartridge with the

DISCOVERY CARTRIDGE; however, we cannot guarantee this.  A particular

MAGIC SAC or DESKCART may be different from the unit HAPPY COMPUTERS has

evaluated.  HAPPY COMPUTERS has no control over the products manufactured

by other companies.

HAPPY COMPUTERS cannot determine if your particular cartridge is

suitable for use as a second cartridge.  Cartridge manufacturers may be

reluctant to supply information to allow the user to determine this.

HAPPY COMPUTERS therefore provides the complete information below,

and a schematic, which identifies the address space requirements

of the DISCOVERY CARTRIDGE.  Since it may not be easy for other

manufacturers to modify the selection address of their cartridge,

HAPPY COMPUTERS' software and hardware design does allow for this.

NOTE:  It is simply a matter of unplugging the DISCOVERY CARTRIDGE

and plugging in the CARTRIDGE of your choice, where electrical

conflicts are uncertain or cannot be resolved.

USING OTHER SECOND CARTRIDGES INCLUDING SPECTRE 128

It is possible to use other cartridges as a second cartridge plugged

into the DISCOVERY CARTRIDGES's second cartridge jack.  The information

needed to implement and use this ability is provided in the documentation

file D128CROM.DOC.

DISCOVERY CARTRIDGE POWER CONSUMPTION AND ADDRESSING:

First, you must consider the electrical power requirements.  Any

additional circuit will consume more electrical power, including

circuitry you may add on the DISCOVERY CARTRIDGE board, circuitry

you have added inside your computer, and circuits you wish to

plug in that are located inside a second cartridge.  HAPPY COMPUTERS

cannot help you determine the power available, or the power consumption

of any of these circuits.  It is up to the user to determine if

sufficient power is available to power the desired circuits.

HAPPY COMPUTERS cannot directly help you customize your DISCOVERY

CARTRIDGE to resolve address conflicts.  We can only provide the

following information which could help an experienced logic designer

customize the DISCOVERY CARTRIDGE.  The reasons why one may need to do

this are beyond the scope of this document.

WARNING:

Spare yourself by not reading the documentation below unless you

really need to!

Inside the DISCOVERY CARTRIDGE, the HART chip, the ROM/EPROM sockets,

and the 2nd cartridge port all connect to the various signals of

the computer's cartridge port.  HAPPY COMPUTERS has selected a

particular set of addresses which correspond to selecting these

various elements which share the tri-state data bus.

First of all, the ROM3* ($FB0000), and ROM4* ($FA0000) are the only

selection lines available at the cartridge port besides the address

lines.  There are a set of jumpers on the DISCOVERY CARTRIDGE board

which could be cut and changed to effect different addressing.  Also,

additional circuitry could be added to increase the selectivity

of addressing.  The factory installed "jumpers" are really traces

on the DISCOVERY CARTRIDGE board.  The trace would need to be cut

to effect a change in the jumper.  The silkscreen legend on the

board shows the location of where the trace would need to be cut.

Sometimes, the trace is on the bottom while the silkscreen is always

on the component side of the board.  Verify the connection with an

ohmmeter before and after cutting it.

The DMFMBKUP, DMMXFER, DCLOCK, DDRIVE, and DTEST programs can all be

modified to cause a different base address to be used for accessing

the HART chip and battery backed up clock.  A program which "patches"

files would be needed.  HAPPY COMPUTERS claims that all of the

privileges of copyright are maintained.  No portion of our programs

may be modified in any way, by any user, for any purpose, with the

exception of the information in the rest of this paragraph.  At offsets

$20-$23 within all of these files is normally a $00FB0000, which is the

base address these programs use to access the battery backed up clock in

the DISCOVERY CARTRIDGE.  At offsets $24-$27 within all of these files

is normally a $00FB0200 which is the base address these programs use to

access the HART chip in the DISCOVERY CARTRIDGE.  None of the

DISCOVERY CARTRIDGE programs access the EPROM/ROM sockets, or the

second cartridge port intentionally.  If you change the selection

address of these, no adjustment of our software is needed.

REMEMBER THE WARNING ABOVE? It gets worse here!

You'll really need to study the schematic while reading the section below.

Jumper S1 selects that the HART chip can be selected during ROM3

cycles.  This is the factory configuration.  Jumper S2 would select

ROM4 cycles to access the HART chip.  Jumper S3 selects that the HART

chip does its disk data I/O when A9 is high, and does its clock

I/O when A9 is low.  The output CKXSLN from the HART chip is low

when the ROMX* pin and the HPYSEL HART chip input pins are both low.

The data readback from the clock only appears on D15.  If the

battery backed up clock is not installed on the DISCOVERY CARTRIDGE

board, which is the case detected by having R4, and the 8573 chip

missing, then the HART chip will not drive the D15 data bus signal

when A9 is low.  This allows the CKXSLN to be used as the ROM3 select

for the 2nd cartridge port.  With the factory installed jumpers, and

no battery backed up clock on the DISCOVERY CARTRIDGE board, the

2nd cartridge port sees ROM3 and ROM4 just as though the cartridge

were plugged directly into the computer, except the 2nd cartridge's

ROM3 cycles are inhibited when A9 = 1, that is when the HPYSEL input pin

to the HART chip is high.

NOTE: Whenever the ROMX* HART chip input is low, and the HPYSEL

HART chip input is high, the HART chip is selected, and no other device

may be selected to drive the D0-D15 tri-state data bus.  The HART chip's

D15-D0 and A8-A1 lines must connect to the corresponding signals on

the cartridge port.  The software does not allow reconfiguring these

lines.

NOTE: The CKXSLN signal to the second cartridge is activated even

if there is a clock circuit on the DISCOVERY CARTRIDGE.  Having no

clock circuit just inhibits the HART chip from outputting data on

D15 while CKXSLN is low.  Therefore, if the 2nd cartridge did not

utilize the D15 line, it could coexist with the battery backed up

clock on the DISCOVERY CARTRIDGE.

NOTE: The user is not inhibited to only use the jumper selections

possible from our circuit board design; other selection schemes

could be used, including other decoding logic, and bank select

of EPROM banks for 512K bit and larger EPROMS.  There may be a

small area on the DISCOVERY CARTRIDGE board where the user can

mount and wire additional circuitry.

If any additional decoding circuitry is added, the access time of all

circuitry must be enhanced to make up for the loss of time through

the decoding circuits.  The HART chip should be capable of tolerating

an additional delay of 30ns on the ROMX* input pin, and the valid state

of the HPYSEL pin must proceed the active low going state of the ROMX*

pin by at least 15ns; but we cannot guarantee these as specifications.

Jumper S4 selects that the EPROM/ROM sockets on the DISCOVERY CARTRIDGE

are selected during ROM4 cycles.  This is the factory configuration.

Jumper S3 would select that these sockets are selected during ROM3

cycles.

Jumper S8 selects that the 2nd cartridge jack's ROM3 signal is active

as controlled by the CKXSLN signal from the HART chip as explained above.

This is the factory configuration.  Jumper S9 would select that all

ROM3 cycles activate ROM3 cycles on the 2nd cartridge jack.

Jumper S7 selects that the 2nd cartridge jack's ROM4 signal is active

during all ROM4 cycles.  This is the factory configuration. Jumper S6

would select that the 2nd cartridge jack's ROM4 signal was controlled

by the CKXSLN signal from the HART chip as explained above.

In the factory configuration, a second cartridge with any circuitry

that became selected by ROM4, would directly conflict with the optional

ROMS/EPROMs in the DISCOVERY CARTRIDGE.

#