D128CROM.DOC - DOCUMENTATION REVISION 2.7

(C) 1988 HAPPY COMPUTERS, Inc.

This document describes circuit modifications to the revision D and E

DISCOVERY CARTRIDGE boards that will allow the use of more cartridges

in the second cartridge port, such as the SPECTRE 128.  Other sections

detail how to use 27C512 EPROMS in a DISCOVERY CARTRIDGE to have

a 128K byte contiguous program address space.  This general purpose 128K

byte address space could be further customized to allow using the

MACINTOSH 128K ROMS and the SPECTRE 128 product.  A section details

how this may be accomplished.  This section is provided for educational

purposes.  Also, the function and usage of the program R128FILE.TOS

is discussed.

******* WARNING:

Any modification to your DISCOVERY CARTRIDGE will void the warranty from

HAPPY COMPUTERS.  Always check all connections before cutting traces

or applying power!  Use #26 or smaller solid (not stranded) wire with

high temperature insulation.  Use a low wattage, fine tip soldering

pencil which has a grounded tip, at a static free work station.

Please re-read the warranty before making modifications!

****** USING MORE CARTRIDGES

With the 2nd cartridge port installed, the MAGIC SAC and DESK CART

cartridges can be plugged in and used.  See the document DMACXROM.DOC for

an introduction to this information.  Due to address conflicts, many

cartridges cannot be used as a second cartridge, unless a switch is

added which disables the HART chip, and completely enables the second

cartridge.  This section describes the custom wiring needed for this

switch.  The 40 pin J1 cartridge jack must be mounted on the board.  This

jack is included in factory installed options 3 and 4.

****** Circuit used when DISCOVERY CARTRIDGE ROM/EPROM sockets not used

The circuit modification described below is included in factory

installed options number 5 and 6.

Cut trace S1.  The white paint for S1 is on top of the board.  Trace S1

is on the bottom under the board, directly beneath.

Cut trace S8.  The white paint for S8 is on top of the board.  Trace S8

is on the bottom under the board, directly beneath.

A SPDT switch is needed.  This can have a center off position, but the

center off position is not utilized.  The switch can be mounted at

position SW2 on the board, or alternately mounted at some other custom

location.  If the switch is mounted at SW2, then the switch must not

make contact with the standard SW2 connections.  Either the traces

going to SW2 must be cut, or the leads from the switch should not be

inserted into the board.  We will designate this switch as SW3.

One side of a newly added 10K ohm 1/4 watt or 1/8 watt 5% tolerance

resistor, which we will designate as RG, is connected to pins 17, 18, and

19 of the HART chip.  This connection should be made to the hole in the

board on the S1 trace which was cut, on the side of the trace closest to

the HART chip.  A wire from this point is connected to one side of the

switch SW3 described above.

One side of a newly added 10K ohm 1/4 watt or 1/8 watt 5% tolerance

resistor, which we will designate as RH, is connected to pin 31 of the

J1 second cartridge jack.  This connection can be made to the hole in

the board on the S8 trace which was cut, on the side of the trace which

is NOT closest to the HART chip.  A wire from this point is connected

to other side of the switch SW3.

The other end of RG and RH connect to the VCC +5 supply.  A convenient

place to make this connection is the left hand side of either R2 or R6 or

R7.

The center connection of SW3 connects to pin 31 of P1, which is the

ROM3* signal from the computer.  This connection should be made to the

hole in the board on the S1 trace which was cut, on the side of the trace

which is not closest to the HART chip.

***** USING THE ABOVE MODIFICATION

One position of the switch is used to access the DISCOVERY CARTRIDGE

hardware.  The other position of the switch is used to access the

hardware device plugged into the 2nd cartridge port.  HAPPY COMPUTERS

has tested and found that the SPECTRE 128 cartridge can be used as a

second cartridge when this custom wiring is installed.  However, we

cannot guarantee the performance of another company's product.

NOTE: The switch should not be moved while the computer's power is

turned on.

NOTE: When the DISCOVERY CARTRIDGE hardware is deselected, the third and

fourth floppy drives, if present, cannot be accessed through software.

The drive option switch will still function, if present.  No DISCOVERY

CARTRIDGE software will operate when the switch has the 2nd cartridge

enabled.  Even though the clock on the DISCOVERY CARTRIDGE cannot be

read, the clock still keeps proper time.

****** Circuit used when DISCOVERY CARTRIDGE ROM/EPROM sockets ARE used

This circuit is not currently available as "factory installed".

It is possible to use a double pole switch for SW3 that also switches

the ROM4* signal either to the DISCOVERY CARTRIDGE's ROM/EPROM sockets

or the second cartridge port.  The first pole of the switch would be

connected as shown above.  A similar connection scheme would be used

with the second pole of the switch for the ROM4* signal.  Pullup resistors

are also needed for the signals switched on this second pole.  Traces

S4 and S7 would be cut and wired through the switch's 2nd pole.

One position of the switch would disable both selection lines to the

2nd cartridge port, and fully enable selection of the DISCOVERY CARTRIDGE

hardware.  The other position of the switch completely disables all

circuitry on the DISCOVERY CARTRIDGE, and provides both the ROM3 and ROM4

selection to the second cartridge port.

****** USING 128K byte ROM/EPROM programs inside the DISCOVERY CARTRIDGE

As described in the DMACXROM.DOC file, 27C512 EPROMS can be used with

the DISCOVERY CARTRIDGE.  In the standard configuration, the SW2 switch

is used to select one of 2 possible 64K byte banks of program, for

access by the computer.

In some cases, it is desirable to have the entire 128K byte program

available to the computer in one contiguous address space, without the

need to throw a switch.  Unfortunately, there are only 128K addresses

available at the cartridge port.  A switch must be added to select

whether the HART chip is selected, or the 128K ROM/EPROM program is

selected.

It is also necessary to add a logic gate to enable both ROMS / EPROMS

for all accesses when either ROM3* or ROM4* are enabled.  This gate is

needed since the Atari cartridge port provides separate enable

signals for each bank of 64K bytes, and the ROMS / EPROMS need to

be enabled for all 128K bytes.

****** HARDWARE modification for contiguous 128K byte ROM / EPROM

Although it is possible to also have a second cartridge port along with

128K on board ROMS / EPROMS, this document assumes that the second

cartridge port is not present on the particular DISCOVERY CARTRIDGE

being modified.

This circuit modification is included in factory installed option #7

Any options except the 2nd cartridge jack can be present.  These

instructions are specific for using a 128K program in two 27C512 EPROMS.

Please note that some particular 512K bit mask programmed ROMS may have

different pin connection and selection requirements.  HAPPY COMPUTERS

cannot provide information about any masked programmed ROMS that we have

not examined.

CAUTION:

Read all of the instructions, and collect all components first.  Determine

the position for all new components before mounting any new component!

The two 28 pin ROM sockets are installed in positions U4 and U5 as usual.

These are included in factory installed options #1 and #2.

Refer also to the original documentation DMACXROM.DOC, regarding the

optional C9 decoupling capacitor.

Cut trace S1.  The white paint for S1 is on top of the board.  Trace S1

is on the bottom under the board, directly beneath.

Cut trace S4.  The white paint for S4 is on top of the board.  Trace S4

is on the top of the board.  The trace may be concealed by the white

paint dashed line.

A SPDT switch is needed.  This can have a center off position, but the

center off position is not utilized.  The switch can be mounted at

position SW2 on the board, or alternately mounted at some other custom

location.  If the switch is mounted at SW2, then the switch must not

make contact with the standard SW2 connections.  Either the traces

going to SW2 must be cut, or the leads from the switch should not be

inserted into the board.  We will designate this switch as SW4.

For units with a factory installed switch, the leads to the switch

should be carefully cut away from the circuit board and bent up so they

are accessible.

One side of a newly added 10K ohm 1/4 watt or 1/8 watt 5% tolerance

resistor, which we will designate as RJ, is connected to pins 17, 18, and

19 of the HART chip.  This connection should be made to the hole in the

board on the S1 trace which was cut, on the side of the trace closest to

the HART chip.  A wire from this point is connected to one side of the

switch SW4 described above.

One side of a newly added 10K ohm 1/4 watt or 1/8 watt 5% tolerance

resistor, which we will designate as RK, is connected to pin 20 of both

sockets U4 and U5.  This connection can be made to the hole in the board

on the S4 trace which was cut, on the side of the trace which is NOT

closest to the HART chip.  A wire from this point is connected to the

other side of the switch SW4.

The other end of RJ and RK connect to the VCC +5 supply.  A convenient

place to make this connection is the left hand side of either R2 or R6 or

R7.

A 74LS08 or 74ALS08 IC is mounted in the user customization space

provided on the DISCOVERY CARTRIDGE board.  The orientation is up to

the user.  It may be necessary to temporarily remove the battery while

doing the wiring.  It may be necessary to relocate both C4 adjustable

and C4 fixed, if present.

Pin 14 of the IC is connected to VCC +5; the same connection points

mentioned above for RJ and RK can be used.

Pin 7 of the IC is connected to power ground.  The power ground trace

runs along the right side of the board near C4.

Pin 1 of the IC is connected to pin 31 of P1, which is the ROM3* signal

from the computer.  This connection should be made to the hole in the

board on the S1 trace which was cut, on the side of the trace which is

not closest to the HART chip.

Pin 2 of the IC is connected to pin 33 of P1, which is the ROM4* signal

from the computer.  This connection should be made to the hole in the

board on the S4 trace which was cut, on the side of the trace which IS

closest to the HART chip.  This ROM4* signal must also connect to

pin 1 of both U4 and U5.  This connection point may be made to the

center terminal position of SW2, which is not otherwise utilized.

Pin 3 of the IC is connected to the center terminal of switch SW4.

***** USING 128K contiguous ROMS/EPROMS modification from above

One position of the switch is used to access the DISCOVERY CARTRIDGE

hardware.  The other position of the switch is used to access the

128K bytes in the two ROM/EPROM chips.  The 128K MACINTOSH ROMS cannot

be directly plugged in and utilized with the SPECTRUM SOFTWARE.  Further

consideration is needed; see below.  When 27C512 EPROMS are used, they

must be 200ns or faster (dash numbers less than or equal to -20).

HAPPY COMPUTERS cannot offer assistance regarding selection of ROMS or

EPROMS.  HAPPY COMPUTERS does not currently provide any ROM or EPROM

based programs for use with the DISCOVERY CARTRIDGE.  The user must

provide these.

The 128K byte program present in the ROMS/EPROMS will be accessible at

contiguous locations $FA0000 through $FBFFFF.  Note that the ROM4* line

and therefore the ROM/EPROM A15 pins are high when addresses $FB0000

through $FBFFFF are selected.

NOTE: The switch should not be moved while the computer's power is

turned on.

NOTE: When the DISCOVERY CARTRIDGE hardware is deselected, the third and

fourth floppy drives, if present, cannot be accessed through software.

The drive option switch will still function, if present.  No DISCOVERY

CARTRIDGE software will operate when the switch has the ROMS/EPROMS

enabled.  Even though the clock on the DISCOVERY CARTRIDGE cannot be

read, the clock still keeps proper time.

****** USING 128K MACINTOSH ROMS and SPECTRE 128 SOFTWARE

The SPECTRE 128 product is made by GADGETS BY SMALL.  They are

not affiliated with HAPPY COMPUTERS Inc.

Using the SPECTRE 128 software with a hardware device other than the

SPECTRE 128 hardware may be a violation of a licensing agreement between

you and Gadgets by Small, if any.  Gadgets by Small may insist that you

only use their hardware device to hold the MACINTOSH ROMS.  However,

there is no copyrighted computer program written by Gadgets by Small

contained within the SPECTRE cartridge.  HAPPY COMPUTERS cannot advise

you on the legal purpose of using the SPECTRE SOFTWARE with a hardware

device other than the SPECTRE cartridge.  The laws governing this will

vary for each locality.  HAPPY COMPUTERS provides the following

information for educational purposes only.  You should consult with

legal counsel familiar with the laws in your locality before implementing

or using any of the following information.

Ultimately, if legal in your locality, it would be more convenient to

not have to change cartridges to select running MACINTOSH programs with

the SPECTRE, or converting MACINTOSH disks with the DISCOVERY CARTRIDGE.

USING THE MACINTOSH 128K ROMS

IN YOUR DISCOVERY CARTRIDGE WITH SPECTRE SOFTWARE

1) Install the general purpose 128K contiguous modifications shown above.

2) Make changes to emulate the SPECTRE HARDWARE differences.  The SPECTRE

128 does not use a one for one connection on the data D0 to D15 lines.

A) To directly use the MACINTOSH 128K ROMS, you must determine and

implement the different wiring needed to the U4 and U5 sockets

on the data lines.  HAPPY COMPUTERS cannot provide this

information.  This would require examining a SPECTRE cartridge

to determine the actual wiring for D0 - D15 that the SPECTRE

needs, and cutting traces and rewiring the DISCOVERY CARTRIDGE

to reflect these changes.  CAUTION: the data lines must not be

changed going to the HART chip.  Make the changes after the

data lines pass the HART chip.

**  OR **

B) Read the scrambled data from original MACINTOSH ROMS as they

appear to the computer while they are plugged into a SPECTRE

cartridge.  Place this data in files, and write the resultant

data files to 27C512 EPROMS using an EPROM programmer.  With

the 27C512 EPROMS placed into the DISCOVERY CARTRIDGE, it will

appear to the computer the same as the SPECTRE appears, since

the same scrambled data will be present.  CAUTION:  It may be

a violation of APPLE's copyright to read out their ROMS and

copy them into EPROMS.  HAPPY COMPUTERS provides this information

solely for educational purposes.

****** PROGRAM R128FILE.TOS function and usage

The R128FILE.TOS program provides a convenient way to read out 128K

byte EPROMs that you have programmed while these EPROMS are plugged

into the DISCOVERY CARTRIDGE.  The U4F128K file corresponds to the data

read out from the U4 27C512 EPROM.  The U5F128K file corresponds to the

data read out from the U5 27C512 EPROM.  It is expected that the above

modification for accessing contiguous 128K byte EPROMS has been installed,

and that the switch is in the position which selects the EPROMS.

The program R128FILE.TOS reads 128K bytes (131072) from the Atari ST's

cartridge port.  All possible cartridge port addresses are read out.

The entire range $FA0000 through $FBFFFF is read out.  All data bytes

read from even addresses are stored in a file U4F128K.  These are the

high bytes from each word.  All data bytes read from odd addresses are

stored in a file U5F128K.  These are the low bytes from each word.

Each of these files will be 65536 bytes long as a result.  The files

will be written to the same drive and directory that the R128FILE.TOS

was executed from.  There must be sufficient room for the resultant files,

and the disk must not be write protected.  The data read out is not

examined or checked for validity in any way.  The program does not

report any error conditions.  If the files already exist, they are

replaced with the new contents read out.  The program has run properly

to completion if the two files of correct size are created.

#