DBACKUP.DOC - DOCUMENTATION REVISION 2.6

(C) 1988 HAPPY COMPUTERS, Inc.

This document describes the usage of the program DMFMBKUP.TOS.  This is

the program for backing up disks using the DISCOVERY CARTRIDGE.

DISK BACKUP, LEGALITY AND ETHICS

HAPPY COMPUTERS cannot offer a legal opinion.  Please consult your own

legal counsel for accurate information.  We have heard that in the United

States, there is a "FAIR USE" provision of the the copyright laws.

It may be considered fair use to make a backup of a copyrighted

computer program you have purchased, and use the backup in place of

the original.  We have heard that it may be illegal to make copies and

distribute them in any manner.

Software authors work very hard to produce the marvelous programs that

make your computer so much fun and so useful.  Generally speaking,

they are not doing this for the fun of it.  They do it to make money.

They have to eat, make rent or mortgage payments, and purchase cars,

VCRS, and clothes (no order of importance is implied).  They need to be

compensated for their work, like any other person.  Copy protection is

placed on disks as an attempt to ensure that authors receive fair

compensation for their work.  Our disk backup system was created to

allow users to make backups.  It is not our intention to deprive

authors of income.  With a powerful backup system such as ours, the

copy protection on the disk will at least serve as a reminder that

authors should be compensated.

There really is a need for backups.  Disks can and do fail, for a whole

variety of reasons.  Almost anyone who has used computers and floppy

disks for some period of time can attest to this.  The safest thing to

do is make backups, and put the original away for safe keeping.  It's

best if you also make a backup to stash away just like the original.

Check the laws of your own locality before copying any computer program.

The DMFMBKUP program, in conjunction with the DISCOVERY CARTRIDGE,

creates a disk backup system more useful and powerful than most other

backup systems available for almost any personal computer system.  This

system can directly copy disks using copy protection that certainly

defies all "SOFTWARE ONLY" copy programs.  It also copies disks that most

other hardware products can't touch, even those on other computers.

The program can also be used to copy disks that are not copy protected.

When writing disks that are not copy protected, the HART chip has a

speed advantage over the disk controller chip in the your computer.

The HART chip can format and write in one revolution of the disk.  The

disk controller in your computer needs 1 disk revolution to format, and

another disk revolution to write the data.

When writing to the destination disk, this program verifies all of the

sector data written.  There is no option to continue if a verify error

occurs.

The program has a feature for efficiently making many copies of the

same disk, providing you are using 2 drives, and your computer's

memory is big enough to read the entire program in at one time.  This

feature is called "ALTERNATE DESTINATION DRIVES".  It is described in

the section below on BACKUP CONTROL FILE CONTENTS.

The program has a feature called "BACKUP CONTROL".  Instead of selecting

parameters that control the backup process from menus, the program allows

the user to select a backup control (BC) file.  The backup control "tells"

the program how to read the source disk.  Some backup control files

are provided by HAPPY COMPUTERS.  The users may also write their own

backup control files.  These backup control files do NOT bear any

resemblance to the PDB files of the 8 bit HAPPY.  All programs copied

by the DISCOVERY CARTRIDGE do not require a DISCOVERY CARTRIDGE to run

the copy.

Many copy protected disks can be copied without the need to specify

a BC file at all.  Other copy protected disks may require the use of a

particular BC file to generate a working backup.  Some backup control

files are tailored to providing fast copying.  Other BC files can used to

copy a whole variety of copy protected disks that otherwise won't copy.

The DMFMBKUP program is written to provide good copying speed while

being able to copy many disks without specifying any backup control.

We could have written the program to be more automatic, eliminating

the need for backup control files in more cases.  If this were the

case, the program would be much slower in operation, almost intolerable.

The use of BC files is a trade off between copying speed, and the

complexity of the backup process.

COPYING DISKS - STEP BY STEP PROCEDURE

1.  Execute the DMFMBKUP.TOS program.

2.  Select the OPTIONS menu.

3.  Select the source and destination drives.

4.  Select the copy mode, the simpliest case is FLOPPY TO FLOPPY.

5.  Select a BACKUP CONTROL FILE, the simplest case is "(none)".

6.  Exit from the OPTIONS menu.

7.  Begin the copying process.

8.  Insert the source and destination disks as instructed by the

most recent message displayed at the bottom of the screen.

9.  During the copying process, you may abort the copying operation by

holding down the ALTERNATE key.  You may have to hold down the

ALTERNATE key for several seconds before the program stops.

10. The program will report any error conditons.  If an error occurs,

it is likely that the disk has not copied correctly.

OPTION DETAILS

The first menu allows the user to begin copying, or select the options

menu, or exit the program.  The options menu allows selection of copying

mode and backup control files, discussed below.  The options menu also

allows the user to select the source and destination drives.   Up to

4 different source and destination drives are possible if the DRIVE

3/4 option is installed in your DISCOVERY CARTRIDGE.  See the DDRIVE.DOC

file for further information.  If the conditions for accessing a 3rd and

4th drive are not met, then only drive A and B can be selected as the

source and destination drives.

FLOPPY TO FLOPPY MODE

The copy mode selection lets you select copying from FLOPPY TO FLOPPY.

In this case, the disk read from the drive specified as the source drive

will be copied to the disk in the drive specified as the destination

drive.  There is no image file generated or utilized.

FLOPPY TO FILE MODE

Another mode selection is to copy from FLOPPY TO FILE.  In this case,

the image of the disk read from the drive specified as the source drive

is copied into a file.  The file written is named "MFMBTEMP".  The file

is written to the same drive that the DMFMBKUP program was loaded from.

In the current version, there is no way to instruct DMFMBKUP to use a

different drive or file name for this purpose.  No destination disk

is created.  There must be sufficient space for the file on the disk

where the file is written.  It is not possible to tell in advance

exactly how much space is needed.  Usually, over 400K will be needed

for the image of an 80 track single sided disk.  It is possible that

up to 9 megabytes can be required for some disks.  In most cases, the

image file for an 80 track single sided disk will not exceed the 720K

disk space available for file storage on one double sided floppy.

If the file MFMBTEMP already exists, it is erased and replaced with

the new image file.  The disk used for the file must be formatted in

advance.  You must not insert or remove the disk that contains the file

while the drive select light is on.

There are many different reasons for copying to an image file.  The first

would be to allow the program to be transmitted by modem.  The DMFMBKUP

program makes no attempt to compact the size of the file it writes.  We

suggest that you "ARC THE FILE" to compact it before transmission.  HAPPY

COMPUTERS does not currently supply instructions on what this means or

how to accomplish this.  Please note that it may be illegal to transmit

a copy protected computer program by modem.  The compacted file may

allow several file images to be placed on one floppy.  It would also

reduce the space required to store the image of the file on your

hard drive.

Another reason to copy to an image file would be to allow editing of

the contents.  Image files contain a definition of the disk format

along with the data.  Currently, HAPPY COMPUTERS does not supply an

editor.  We do supply the information which documents the contents

of the image file, see below.

Another reason to copy to an image file is to save time.  The process

of reading and analyzing a copy protected disk takes longer than

the time it takes to read back in an image file you have already

created.  If you were going to make copies of a program at different

times, you may want to save the program as an image file rather than

as an executable backup.  Each time you wanted to make an executable

copy, you could do the job faster by reading from the image file rather

than reading the original disk.

FILE TO FLOPPY MODE

Although this program can create a file which represents the image of

the copy protected disk, the copied program cannot be executed while

it is in the image file form.  A executable backup copy can be created

from the image file by using the FILE TO FLOPPY copying mode.

In this mode, the image file named "MFMBTEMP" is read from the drive

that the DMFMBKUP program was loaded from.  This file specifies the

format and data of each track that will be written on the destination

disk, which is inserted in the drive specified as the destination drive.

With the current version of DMFMBKUP it is not possible to specify a

different name for this file, or a different drive for reading this

file from.

The image file may have been created by the FLOPPY TO FILE mode of this

program.  It may have been edited or completely created by the user.

It may have been received over a modem, and "EXTRACTED FROM AN ARC FILE".

In any case, the data in this file must conform to the format which

is described below.  If there is some problem in the format, an "ERROR

IN WRITE CODE PROCESSING" can occur.  Unmodified files that are produced

by the FLOPPY TO FILE mode will not cause this error.

While in the FILE TO FLOPPY mode, the backup control file specified has

no control over the format and data written to the destination disk.

Complete control over the format and data written is determined by the

contents of the image file.  For this reason, in the current software

version, the ALTERNATE DESTINATION DRIVES mode, which is selected by

the backup control file, is not available with the FILE TO FLOPPY mode.

The backup control file that was used when the file was created, if any,

does not have to be selected to recreate a backup from a file.  The

file itself contains all of the information needed to reconstruct

the backup.

SELECTING BACKUP CONTROL FILES

The file "DBKUPCF.S" is one large file which contains all of the

BACKUP CONTROL (BC) FILES that can be accessed from the DMFMBKUP

program.  We suggest that you print out this whole file, and read

through the english language descriptions.  One BC file may include

a whole list of program names that do not appear in the title

of the file.  If you find a particular file that backs up a particular

program that is not listed, make a note of this for your own future

convenience.

Software companies may change their disk protection method from

time to time.  The disk that HAPPY COMPUTERS has examined may not

have the same copy protection as the disk you have, even though the

title and even the version number may be the same.  For this reason,

a program may not copy with the backup control file even though the

instructions say to use it.

ERROR MESSAGES

Some error messages may just need a common sense solution.  Unusual

error messages may imply that something is wrong with your computer,

disk drive, destination disk, or DISCOVERY CARTRIDGE.  Error messages

may also imply a bug in the program.  Before contacting HAPPY COMPUTERS

for help in resolving errors, you must have ready the specific set of

circumstances that produced the error.  Computer type, other resident

programs, disk drive type, diskette manufacturer, serial number of

DISCOVERY CARTRIDGE, name of program being copied, statistics concerning

how often the error occurs, are all necessary before we can begin to

resolve the problem.  It really does help to try a different set of

hardware before placing fault on any one particular item.  You may just

be using the product incorrectly, please read our documentation first!

Getting write / verify errors will usually mean the destination disk

or drive has some defect.  Also, check to make sure that the cables are

all plugged in correctly before calling us!

PROBLEMS COPYING A PROGRAM?

Please see the section in the file DISCOVER.DOC titled WHAT TO DO IF

YOU CAN'T BACK IT UP for help in this area.  Do this before you call

our office!

TECHNICAL INFORMATION - BACKUP PHILOSOPHY

HAPPY COMPUTERS Inc. has the philosophy that a backup program should

always do the most to ensure the integrity of the resultant copy.

The copy produced should operate just like the original.  Our software

does not remove the protection, or modify the program being copied

in any way.  Checksums are usually written for each segment of data on

floppy disks.  HAPPY COMPUTERS believes that the backup program must

do the upmost to check that the copy produced is correct, with correct

checksums.  For these reasons, HAPPY COMPUTERS believes that dumb

"bit" copiers should be avoided.  Our DMFMBKUP program follows in the

steps its successful predecessor on the 8 bit Atari, the HAPPY BACKUP

PROGRAM.  The DMFMBKUP program does not just blindly copy bits or

flux spacings.  The DMFMBKUP completely analyzes the disk being copied

track by track, to create a complete description of each track.  The

DMFMBKUP program uses this decoded description to recreate each track

on the destination disk.

COPYING OTHER FORMATS

Analyzing the data read really does make a difference.  Other

hardware copying devices we have seen that do not completely analyze

the data are so erratic, you wind up never knowing if the copy you made

will work.  Our system produces consistent results.  This also

imples that software must be written to handle each format properly.

In its present form, this program can copy SINGLE DENSITY, HIGH DENSITY,

MACINTOSH FORMAT, AMIGA FORMAT.  However, the program is not currently

designed to analyze the data read for these formats.  There would be a

risk that copies produced may not consistently be correct.  As time

goes on, HAPPY COMPUTERS' software library will expand to properly

analyze and copy more disk formats.  The DISCOVERY CARTRIDGE hardware

will never be the limiting factor.  Cast your vote for a new feature by

writing your request in a letter mailed in.  Phone calls are not

considered votes, just passing desires.

TECHNICAL INFORMATION - BACKUP CONTROL FILE CONTENTS

The following is a description of the contents of the file "DBKUPCF.S"

which contains all of the backup control files that the user can select

while operating the DMFMBKUP program.  HAPPY COMPUTERS provides this

information for several reasons.  First, the user can examine the

BC files provided by HAPPY COMPUTERS as examples of how these are

written and integrated together.  Second, the user may be able to utilize

this information to generate their own BC files.  Aside from the

information presented here, HAPPY COMPUTERS cannot provide additional

help regarding the content of BC files.

The DBKUPCF.S file can be printed to the printer, and edited with an

ASCII text file editor, which HAPPY COMPUTERS does not supply.  Use

the editor you are comfortable with.  There are no special control

characters, other than standard ASCII characters.  We suggest that

you do not modify the BC files provided by HAPPY COMPUTERS.  We suggest

that you copy any section you wish to modify to the end of the file, and

make your modifications there.  This will effectively add new selections.

The selection numbers that we have provided will not be modified.

Each BC file set within DBKUPCF.S starts with a title line.  All of

the characters through the first CR or LF will be displayed as the title

for the user to make a selection from.  The selection number is computed

by the DMFMBKUP according to the position within the file.  The selection

numbers are not contained in the DBKUPCF.S file.  For example, if you

add one new BC file set at the start of the DBKUPCF.S file, all of the

previous selections will have their selection number increased by one.

For compatibility with low resolution, HAPPY COMPUTERS has limited all

text in our file to 40 characters per line.  The title line of the

currently selected BC file or "(none)" is displayed by the DMFMBKUP

options menu.

Following the title line is an optional information display area.  This

information is displayed by the "I" command from the option selection

menu of the DMFMBKUP program, and is also displayed just after a

particular BC file is selected.  All of the text starting with the title

line through, but not including the first line that has a single

exclamation mark as the first character on the line, is considered the

information display area.  Characters within the information display

area do not control the read process.

The first line after the line which had the exclamation mark, begins

the area where the backup control commands are located.  You must

use the exact upper case character shown.

ALTERNATE DESTINATION DRIVES mode

This mode is selected by having the letter A as the first character on

the first line following the ! terminator.  This mode can be selected for

any BC file by adding the line with the letter A, provided that the

other conditions for using it are met.  With the DMFMBKUP program in

this mode, once the source disk is read in, the source and destination

drives alternate as destination drives.  This allows the user very

efficient utilization, to write many disks at one sitting.  The system

writes to the disk in the other drive, they alternate as soon as the

disk just written is ejected, with no keyboard entry required.  Be sure

that the disk in the other drive is ready to be written before

ejecting the disk that just completed.  The source disk is not read

in again until the user selects to terminate this mode.

After writing to a disk is completed, the system sounds one bell, and

flashes the select light continuously on the drive that just completed

writing.  The user ejects the disk from the drive with the flashing

light, puts that disk in the "done" box, and inserts another disk.  In

the meantime, just as soon as the current disk has ejected, the system

begins writing on the other drive.  The message on the screen also

announces that the writing has completed correctly.

If a write / verify error occurs, the system reports this on the screen,

and besides flashing the drive select light on the drive with the bad

disk, the system also keeps ringing a bell tone through the monitor

speaker.  This audio indication should be sufficient to cause the

operator to place the disk just ejected into the "BAD DISKS" box.  The

operator should only need to observe the disk drive select lights, and

pay attention to the bell sound, to copy disks in this mode.

Conditions required to use this mode are as follows:

1) the 'A' must be the first control line in the backup control set

2) the source and destination must be different drives

3) the whole source disk must fit and read into memory at one time

WARNING

There are some defective drives that do not always recognize the change

in the WRITE PROTECT status that this system uses to determine when the

destination disk is ejected.  If you have a drive that sometimes does

not allow the system to recoginize the disk eject, usually the eject

will be recognized when the disk is re-inserted and re-ejected.

However, it is strongly recommended that this drive not be used with

GEM DOS files, since GEM can really screw up the contents on the disk

if it doesn't recognize that the disk has been changed.

SIDE SELECTION

Unless otherwise specified, all reading commands are directed to the

first side of the disk.  Most copy protected disks do not utilize

the back side of the disk, for compatibility with single sided drives.

To direct the program to read from the second side, the letter S is

placed as the first character on one line by itself.  After the

occurrence of this line with the letter S, all backup control command

lines that follow, will read from the disk's second side.  There is no way

to switch back to the first side.  With no backup control specified, only

the first side of the disk is read.

COMMENT

A line that begins with an asterisk "*" is considered to be a comment

line.  The text on that line will be ignored by the backup control system.

TRACK BY TRACK CONTROL

If a line begins with a number "tt", which is allowed to be between 0

and 99, then this is considered to be a track number, which will be

followed by a colon ":", and further backup control specifications.

The "tt" numbers do not have to be in ascending order, but this may

be more logical.  The order in which reading is specified is the

order in which the data will be written to the destination disk.

With no backup control specified, the DMFMBKUP program will read and

analyze tracks 0 through 79.

tt : R xx

The tt is the track number, the ": R" is the read mode specifier.

This is for fast reading of unprotected tracks.  The xx value is the

quantity of sectors per track, values from 8 to 11 are allowed.  The

sectors should be normal MFM data with 512 bytes per sector.  These

tracks are efficiently read by the disk controller in the ST, without

the need of any special twister format.

tt : D

The tt is the track number, the ": D" is the read mode specifier.

This specifies that the current track is read and analyzed by the HART

chip.  This is used to read tracks when you are not sure if they are

protected or not.  This type of reading takes about 5 times as long

as the "R" type reading.  This is the same type of reading that would

be done for each track in the case where no backup control had been

specified.  However, this can be used to read from tracks higher than

track 79, and it can be used to read from the second side.

tt : U t bbb

The tt is the track number, the ": U" is the read mode specifier.

This specifies that the program should do HART chip reading and analysis

for the current track, and also look for data which may be "under the

index hole".  The "t" field is the type of reading.  Currently only

type "t" = 1 is supported.  For "t" = 1, the program will attempt to

synchronize by finding the logical end of the last sector on the track

whose addr/data field falls under the index hole.  The end of writing

is the end of the data field of this sector, plus bbb bytes.  This permits

copying some extra bytes that may follow the sector, if needed.  With

type "t" =1, it is not mandatory to find data under the index.

Therefore, this type of copying can be blindly specified for any track,

but it  will increase the time required for reading the disk by

about 25%.  When used blindly, set "bbb" = 0.

tt : W t gg bbbb sss

The tt is the track number, the ": W" is the read mode specifier.

This specifies that a portion of the disk will be read directly as

flux transition spacings, and stored as direct flux spacing control

bytes for the $000A writing command.  There is no analysis,

decode, checksums, or filtering performed which corresponds to any

particular type of disk recording method.  A large quantity of storage

space is used by the resultant data.  It is possible that if too much

of a track is read using this method, and the data rate is faster than

500K bits per second, that the data will not fit in the buffers,

and the program will not function.  This will not usually happen with

3.5" MFM double density.  Even high density disks will not usually cause

this.

One purpose for using this feature is to read and write sectors that have

"WANDERING WEAK BITS".  These are non repeatable CRC error sectors that

have some bytes that change, and some bytes that don't change that are

mixed together.  It is a special disk protection method that is only

handled through the "W" reading and $000A writing.

Some copy protected tracks may stretch the bits out, causing certain

timing values.  This feature could be used to copy those tracks.

Another use would be to copy tracks that are not FM or double density

MFM, such as MACINTOSH or high density.

This method should be avoided for reading normal sectors, as the loss

of filtering, checksums and write precompensation makes the resultant

sector data written less reliable.

The "t" specifies the type of syncronization used to switch to this

method of reading.  The "gg" specifies the quantity of good crc sectors

that are expected on this track without retrying.  Since you must specify

the expected quantity of good sectors expected, and since this quantity

must match the actual quantity found on the source disk, the "W" type

reading cannot be blindly applied unless "gg" is zero, and you read the

whole track as flux spacings.  The "bbbb" specifies the equivalent qty of

MFM bytes that are to be read.  If the total flux spacing of all

transitions read is added up and divided by 32E-6 seconds, this will

approximately equal "bbbb".  This is not the quantity of bytes that

will result in the buffer.  The maximum value allowed is "bbbb"=6450,

corresponding to reading from a drive that is turning about 3% slow.

If the end of track is encountered before the desired byte count is

reached, there is no error.  This allows using a "bbbb" = 6450 to

read in the whole track.  The "sss" value must be 0 < sss < 2048.

For "t" = 0, the flux spacing reading method begins at the start of track,

immediately upon the occurrence of the index hole from the drive.  In

this case, since no MFM bytes need to be read for synchronization, almost

any type of disk data can be read.  Please note that the practical limit

for this program would be to read flux spacings that are at least 2usec

apart.  This would include high density MFM data.  In any case, the

"bbbb" byte count is based on a 32usec byte time, so specifying 6250

bytes, would actually read the equivalent of 12,500 bytes from a high

density 500K bit / sec disk.  The "sss" value is not utilized by

type "t" =0, but still must be in the correct range shown above.

With tt : W 0 0 6450 1, the whole track can be blindly read in as

flux transitions.  This could experimentally be used to copy certain

tracks that couldn't copy in any other way, or could be used to copy

MACINTOSH or HIGH DENSITY tracks that were aligned to the index hole.

For "t" = 1, the program switches to the flux spacing reading method,

immediately following the "sss" occurance of 3 of the special $A1

missing clock sync marks in a row.  More than one of this type of "W"

field may be on one line.  In this case the "gg" qty for both must

be the same, and the "sss" for each is relative to the previous.

The "sss" for the first "W" on a line is the qty of mark triplets

from the start of the track determined by the index hole.

REPEAT THROUGH TRACK NUMBER

The form of the command line is:

R : tt

This causes the previous command line to be repeated through track "tt".

This substantially condenses the number of lines required.

END OF A PARTICULAR BACKUP FILE SPECIFICATION

Following all of the commands which control the reading process, a line

which begins with the right parenthesis ")" terminates the current

backup control file.  The next line in the file DBKUPCF.S must either

be the title line for the next BC file, or the END MARKER below.

END OF DBKUPCF.S MARKER

Following the line which has the ")" which terminates the last BC

file specification within the DBKUPCF.S file, the very last line which

indicates the end of the DBKUPCF.S file is one that has the right bracket

"]" as the first character on the line.  Any lines after this in the

file DBKUPCF.S will not be accessed by the program.

THIS CONCLUDES OUR DESCRIPTION OF THE BACKUP CONTROL FILES!

TECHNICAL INFORMATION - CONTENTS OF THE MFMBTEMP FILE

This section describes the contents of the file MFMBTEMP that is

produced by the FLOPPY TO FILE mode, and is read as input for the

FILE TO FLOPPY MODE.

The data in the file is binary, not ASCII.  It cannot be edited by

an ASCII text editor.  HAPPY COMPUTERS does not currently supply

an editor for this purpose.

The data in the file consists of blocks of data; each block begins

on an even byte boundry.  Each block begins with a code from

$0000 to $000F.  There should be an even quantity of bytes in the

file.  HAPPY COMPUTERS cannot currently provide further information

on this data, beyond what is presented here.

*** $0008 S0TT NNNN NNNN

Each track's specification begins with this code.  The quantity of bytes

in this track's definition is NNNN NNNN.  If NNNN NNNN was added to

the location of the $0008 code word, this sum would point to the

location of another $0008 code word for the next track, or the sum

would point to the location of a $0007 code.  The S0TT words specifies

the side and track to be written.  The side is in bit 15.  If bit 15 of

the S0TT word is zero, the first side of the disk is written.  If

bit 15 of the S0TT word is one, the second side will be written.

The TT value is the track to be written

*** $0007

This $0007 marks the end of the specification for the destination disk.

*** $0009 SSGG 00gg

This $0009 code specifies that the track consists of normal MFM address

and data fields.  The following 512 * SS bytes are for a track with SS

normal sectors.  The gap between the data field and the next address

field is GG bytes.  This is the gap that follows the CRC1 CRC2 $FF

after the data field, and preceeds the 12 $00 before the addr field sync.

This code $0009 is the definition for an entire track, it must follow a

$0008 code.  The gg gap is the gap between the CRC1 CRC2 bytes of the

addr field and before the 12 $00 bytes before the data field sync marks.

Recommended values: (generated by the tt:R xx backup control command).

If the destination drive RPM exceeds that shown, the track can't write.

SECTOR PER TRACK   GG gap  gg gap   bytes/track    MAX RPM --- % over

8 (min)      176      22        6008          312.1      4%

9             92      22        6003          312.3      4%

10             38      22        6130          305.8      2%

11 (max)        1       8        6182          303.3      1%

NOTE:

The 11 sector track can only be written with the HART chip, individual

sectors can not later be re-written, since they will collide over the

next sector's address field.  These are "READ ONLY" tracks.

*** $000E AABB CCDD EEFF NNNN

Must follow the $0008 code.  This will be the code generated for

tracks read by the HART chip.  Only the byte needs to be accurate

for track writing.  The other values are statistics.

STATISTICS: note: a minus byte value is an overflow value...

AA is the qty of addr fields found with good CRC.

BB is the qty of addr fields found with bad CRC.

CC is the qty of data fields found with good CRC.

DD is the qty of data fields found with bad CRC.

EE is the qty of $A1 sync marks found that were < 3 in a row.

FF is the qty of at least 3 $A1 sync marks in a row that did not

preceed an addr or data mark.

NNNN is the qty of MFM bytes plus byte spaces found during read decoding.

At 300 RPM and 4usec per byte, this is usually 6250.  This is not the

quantity of bytes in the buffer, it tends to indicate the bit width and

RPM of the drive/system that wrote this track.  If the user modifies

the length of data on the track, the NNNN value should be updated

to represent the actual bytes contained, as the program will use this

to match the writing length to the destination drive speed.

**** NOTE:

The remainder of codes shown below will follow the $000E code.

*** $0000 NNNN ...(NNNN bytes of DD data)

Following the header, there are NNNN bytes of normal double density

data.  NNNN may be odd, but the next block must start at an even address.

These bytes do not have missing clocks.  If FM single density data is

read, it may appear as code $0000 blocks.

*** $0001 NNNN ...(NNNN bytes of CCDD clock data)

Following the header, there are NNNN/2 pairs of bytes which are CC clock

and DD data.  These bytes may have a missing clock pattern according to

the coding rules of MFM data.  The last CCDD word in the block collected

may not have a missing clock.

*** $0002 SSSN CCDD

There is a short byte with only N bits with CC clock and DD data.

The CC DD bytes will each be right justified with 8-NNNN bits of

leading zeroes in each byte.  If the least significant bit of the

DD byte is set, then the last transition of this short byte was at the

data point in a bit cell.  This 0002 code is generated to complete

a short byte when there is => 2 bit times (about 9usec) between flux

transitions.

The SSS 12 bit portion will be 100x xxxx xxxx if there is some

flux spacing value between transitions x xxxx xxxx, where this 9

bit value is just like a $0003 code had followed the $0002 code.

The SSS 12 bit portion will be 0xxx xxxx xxxx if there is no

transition for at least xxx xxxx xxxx count, just like having

a code $0004 following the $0002, with a limit of $7FF for

the code $0004.  The code $0002 is generated when there are

some normal MFM bit spacings followed by excessive flux spacing.

*** $0003 NNNN

There is an interval of NNNN*62.5E-9ns between flux transitions.

This is generated by reading analysis when a flux spacing exceeds that

which is usually allowed for MFM, = 2 bits times, or > 8usec.  The

maximum value generated by the reading analysis will be $13E, or a

count of 318, which is 19.875usec.  For user generated code, the $0003

will accept NNNN values between 50 ($32) and 390 ($186), or 3.125 usec to

24.375usec.  Use of the $000A will provide greater efficiency and

flexibility for user generated flux spacing values.  A $0004 code will be

generated by the reading analysis when the spacing exceeds $13E.  A post

reading analysis process could compress groups of $0003 and $0004 type

codes into a smaller number of bytes in the $000A code, but this is not

done by this program.

*** $0004 NNNN

There is an internval of NNNN*62.5E-9ns with no flux transition.

NNNN will be greater than $FF and no more than $FFFF (1 word).

Reading analysis will generate a value between $100 and $FFFF.

During writing, values of $64 to $FFFF will be accepted.

It may be more efficient and accurate to use the "W" type reading,

and $000A type writing for non MFM code spacings.

*** $0005 NNNN

This marks the special $A1 MFM sync byte with a missing clock bit as

expected.  NNNN is the qty of bits of the previous byte that were

really part of the sync mark (0-7).

*** $0006

This will only preceed a $0005 code.  It indicates that the sync mark

needs to be advanced in writing by 1/2 bit cell time.  It is caused by

having the sync mark detected in the clock pattern rather than the

data pattern.

*** $000A NNNN NNNN WWWW WWWW ...(NNNN NNNN data bytes of XX)

Switch to direct flux spacing control mode.  The WWWW WWWW field has

the total HART chip count spanned by the flux spacings generated, which

is useful during read analysis to determine this section's contribution

to the track length, and is unused during writing.  NNNN NNNN is the

qty of bytes in this block = block length.

The XX bytes specify the spacing between flux transitions.  Each count

is 62.5 nanoseconds.  The nominal 3.5" DD bit width is 64 counts = 4usec.

Flux spacings normally generated from MFM data block codes 0000, 0001

and 0002 include filtering and write precompensation, and adjustment

to match original the track length.  The flux spacings written by the

$000A are taken verbatim, with none of these adjustments.  Valid counts,

when a single byte specifies a flux spacing, are 25 to 255, corresponding

to 1.5625usec to 15.9375usec spacing.

A $00 byte specifies that the 2 bytes following the $00 are (hi lo)

a count in which there is no flux transition.  The "W" reading can

generate values from $00 $00E8 to $00 $FFFF.  The $000A writing will

accept values from $00 $0064 to $00 $FFFF.

A $01 byte specifies that the 2 bytes following the $01 are (hi lo)

a count at the end of which there is a flux transition.  The "W" reading

can generate values from $01 $0100 to $01 $013E.  The $000A writing will

accept values from $01 $0032 to $01 $0186.  It is wasteful to use the

$01 three byte sequence for counts < $100, since a single byte could do

this.

Byte values $02 to $18 (2 to 24) are not allowed, unless they are in

the hi lo pair following a $00 or $01.

*** $000B XXXX

This code is utilized during read analysis.  It is not utilized during

the writing process.  It follows the occurrence of 3 type $0005 sync

mark code blocks.  The XXXX will be 0000 if the field following is

neither an addr nor a data field.  The XXXX field is $FFFF if the field

following is a data field with a bad CRC.  The XXXX field is $7FFF if the

field following is an address field with a bad CRC.

*** $000C NNNN

During reading analysis, some $000B code blocks are replaced with

$000C code blocks.  Specifies that the first NNNN bytes of the

following $0000 code block were checked and did have a valid CRC,

which is contained in the buffer at the usual location of CRC bytes.

The length includes the data or addr mark byte and the CRC bytes,

so a 512 byte data sector needs a length of 512 + 1 + 2 = 515.  An

address field needs 7.

If the $0000 data block following the $000C is modified, the $000C

should be replaced with a $000D to allow the program to recompute the

correct CRC, or a verify error will occur.  In the case of data fields,

the read analysis will place a $000C over the $000B, only if a

verify readback with the FDC will succeed.

The requirements include the following:

1) valid data field mark & CRC computed based on length in addr field

2) data field follows an addr field with valid CRC

3) data mark follows an addr field's CRC  within 43 bytes

*** $000D nnnn

This code is not generated by the reading process.  It can be inserted

by the user to cause the program to generate valid CRC's when the

user has created or edited the data.  The length includes the

data/addr mark byte and the CRC bytes, so a 512 byte data sector

needs a length of 512 + 1 + 2 = 515.  An addr field needs 7.

A $0000 code block of sufficient length must follow this block.

The first byte of the $0000 block following, must be either an addr

mark $FE, or a data mark $F8 thru $FB.  An addr field will specify

a track and sector number that may be verified.  If the data field

has a $000D code preceeding it, a verify/readback will be done, so

the addr and data field must form a proper pair, or a verify error

will occur.

*** $000F WWWW

This code instructs the writing procedure to delay the start of writing

by WWWW bytes after the index hole.  This delays the start of writing,

allowing the data stream written to continue under the index hole.

The WWWW qty does not enter into the track length matching calculation

scheme.  The $000F code block must be the last block code for the track,

preceeding $0008 or $0007.  WWWW is 2047 maximum.  A byte is considered

to be 32usec here, or 1/6250 of a revolution.

*** THIS IS THE END OF THE DOCUMENTATION FOR THE MFMBTEMP FILE CONTENTS

OPTIONAL DISPLAY?

There are some optional display modes built into the DMFMBKUP program.

Currently, the information displayed is only for use by HAPPY COMPUTERS.

Future versions may document the information displayed, and how to control

the display process.

WRITING SPEED IMPROVEMENT

The actual speed improvement obtained using the HART chip to format,

write, and verify is detailed below.

WITHOUT HART CHIP, 4 revolutions required

1) 1 revolution = move head to next track, plus head settle delay

2) 1 revolution = write the address fields of the track (format)

3) 1 revolution = write the data fields of the track

4) 1 revolution = verify the data written

With the HART chip, this is reduced to 3 revolutions, since the

operation of writing the address and data fields is done in one

revolution.  A "software only" based program cannot be as fast

unless it does not properly verify.

#