DDRIVE.DOC - DOCUMENTATION REVISION 2.6

(C) 1988 HAPPY COMPUTERS, Inc.

This is the documentation for using a third and fourth drive with

your ST/MEGA computer that has a DISCOVERY CARTRIDGE.  This

documentation is divided into two sections.  The first section

describes the normal usage of the DDRIVE.PRG program, the

DINSTALL.TOS program and the DRVTBL file, and some related matters.

The second section covers some of the more technical aspects of

this feature.

In order to use this feature, your DISCOVERY CARTRIDGE must have the

DRIVE 3/4 option installed.  This is part of factory installed

option numbers 1, 2 and 4.  The drive plugged into the "DRIVE 3/4"

connector on the DISCOVERY CARTRIDGE will be the "third" drive,

regardless of how many drives you have plugged into the other

connectors.  A "fourth" drive may be plugged into the third drive.

The standard Atari designation is that drives A and B are physically

the first 2 floppy drives in this system. This remains unchanged.

HAPPY COMPUTERS does not require any particular letters for the 3rd

and 4th drives.  The user may designate these.

HAPPY COMPUTERS supplies the program DINSTALL.TOS on your DISCOVERY

CARTRIDGE Software Utilities disk.  This program allows the user

to select the letters desired for the 3rd and 4th drive.  The program

permits the user to select a letter between C: and P: for the 3rd

and 4th drive.  You must select a letter for a drive which is not

already in use by any hard drive or RAM disk.  The program does not

check for any conflict with a hard drive or RAM disk.  The program

also allows the user to select a "STEP RATE".  If you are using

standard Atari SF314 or SF354 drive(s), or any other drive(s) that

are Atari compatible for your 3rd and 4th drive, enter the number

"03" for the step rate.  More information concerning step rate is

provided in the technical section below.

The program DINSTALL.TOS creates the file DRVTBL on the root directory

on the disk DINSTALL was booted from, so make sure that writing is

enabled on this disk before running DINSTALL.  This DRVTBL file

contains the information that is needed by other HAPPY COMPUTERS

software programs to determine which drive letters you have selected,

and what the step rate is for these drives.

The program DDRIVE.PRG must be placed in the AUTO folder of the

disk you boot your computer with, to enable the usage of the 3rd

and 4th drive with programs that access files and other drive

commands.  The DRVTBL file must be in the root directory of that

disk.  If DDRIVE.PRG finds a proper DRVTBL file, it will enable

the usage of the 3rd and 4th drive on bootup, and momentarily

displays a message on the screen to that effect.  If DDRIVE cannot

find a valid DRVTBL file, it will report an error message and

you must press return before booting will continue.

The programs DMFMBKUP.TOS and DMMXFER.TOS will allow the usage of

any of the 4 floppy drives, A or B, plus the letters designated for

the 3rd and 4th drive, as source and destination drives.  These

programs will first test to see if DDRIVE became active when the

system was booted.  If so, these programs will use the drive letters

and "step rate" specified by the DRVTBL in the root directory of the

disk that booted the system.  If DDRIVE is not already active, these

programs will attempt to read the file DRVTBL from the root directory of

the disk that the program is currently being loaded from.  If the

DRVTBL is found there, it will specify the drive letters and

"step rate" for the 3rd and 4th drives.  If these programs cannot

find the information for the 3rd and 4th drive in either of these

places, then the 3rd and 4th drive will not be available.

Therefore, if DDRIVE is already activated, the DRVTBL originally

found by DDRIVE takes priority over a DRVTBL file on the disk from

which the DMFMBKUP.TOS or DMMXFER.TOS program is loaded.

It is not necessary that DDRIVE.PRG be loaded for DMMXFER.TOS

and DMFMBKUP.TOS to use the 3rd and 4th drive.  If DDRIVE hasn't

been loaded, these programs just need to find the DRVTBL file in

the root directory of the disk they are loaded from.

With DDRIVE activated, programs that access drives by using the GEM,

BIOS, or XBIOS calls should work okay with the 3rd and 4th drives.

Programs that "talk" directly to the hardware registers in the computer

to access drives will not work properly with the 3rd and 4th drives.

Some programs that are specifically written for floppy drives may

not allow you to specify drives other than drives A & B.

It is not possible for HAPPY COMPUTERS to modify programs from other

parties to make them work with the 3rd and 4th drives.  Usually,

only programs for analysis or copying of protected disks need to

talk to the hardware registers.  Since you have a DISCOVERY CARTRIDGE,

and our package is superior to any other disk copying software

for your ST, this should not be a problem.

To access the 3rd and 4th drives from the GEM DESKTOP, you must

create an ICON for that drive.  To do this from the DESKTOP,

first click once on a floppy drive that's already there, then

select "Options", and then "Install drive".  Select a drive

letter that corresponds to the letter you selected when you

ran the DINSTALL program.  Select a name for the device that

pleases you.  Move the icon(s) you have created to the desired

location on your DESKTOP, and be sure to save the DESKTOP to

your booting disk, by using "SAVE DESKTOP".

The only thing the standard DESKTOP won't let you do with the 3rd

and 4th drives is to FORMAT a disk in them.  It only allows you

to format drives A: and B:.  If you have a drive 3 swap switch

option installed in your DISCOVERY CARTRIDGE, you can format on

drive 3, while the system thinks (?) it is formatting on drive

A: or B:.  More information on this is described in the section

which documents that option.  You may also have access to an

improved DESKTOP accessory program that allows you to format

other drives.  HAPPY COMPUTERS does not presently supply this.

NOTE: There is a DRVTBL already created, along with DESKTOP ICONS

for a 3rd and 4th drive, on the DISCOVERY CARTRIDGE Software Utilities

disk.  We have selected drive letters J: and K:, and a step rate

of "06", and our ICONS are called "Happy Disk".  You can use the

DRVTBL we have supplied, or you can create your own.

IMPORTANT NOTE: Some ST computer operating systems have a bug which

incorrectly ignores a CRC error when reading the disk.  Correct

operation would be to retry the read, or at least report the error.

It is beyond the scope of this document to discuss the ramifications

of this bug, or how to determine if your ST computer has this bug.

We only document the impact this bug has on the usage of our product.

With the DDRIVE program installed, the CRC error is not ignored, it

is handled correctly, just like computers that do not have the bug.

You need to be aware that there can be a DIFFERENCE.  Without DDRIVE,

your computer may ignore a CRC error while reading, whereas with

DDRIVE, an error message can be reported, and the operation terminated.

This difference can prevent you from accessing disks and files that

seemed perfectly okay before you installed DDRIVE.  This difference

applies to all floppy drives including A and B, and your 3rd and

4th drive.  The problem does not affect hard disks.  The solution is

to deactivate DDRIVE temporarily (rename it to DDRIVE.NOP), and

attempt to copy the files from the questionable disk to a newly

formatted disk.  Then reactivate DDRIVE.  If you can access a disk

with DDRIVE deactivated, but cannot access the disk with DDRIVE

activated, then there is a good chance that your computer has ignored

a CRC error sector.  A CRC error means that the computer cannot

tell if the data has been read correctly.  There may or may not be

a loss of the data.  HAPPY COMPUTERS cannot provide any additional

assistance on this matter.

DRIVE 3 SWAP SWITCH USAGE:

The drive 3 swap switch is an option.  It requires no additional

software to support it.  This option is supplied by HAPPY COMPUTERS,

along with the drive 3/4 option.  The drive 3/4 option can be

installed after the unit leaves our factory, without adding the

drive 3 swap switch.  Without the switch, the drives will respond

the same as if the switch were in the Normal position.

The switch has two functional positions, Normal (left), and

Option (right).  If your drive 3 swap switch has 3 positions,

the center position is the same as Normal.

In the Normal position, the drive plugged into the "Drive 3/4"

connector on the DISCOVERY CARTRIDGE can only be accessed by

using HAPPY COMPUTERS software.  By using the software, this

drive will respond as the third physical floppy drive.  Other drives

also respond as usual.  This is just as if there were no switch.

If your computer has an internal drive that responds as drive A, as

it usually would, the only floppy drive that could plug directly in

your computer would be drive B.  If you have a drive B, it should now

be plugged into the "DRIVE A/B" connector on the DISCOVERY CARTRIDGE.

With the switch in the Option position, any access to drive B will

now be directed to the drive plugged into the "Drive 3/4" jack on the

DISCOVERY CARTRIDGE.  Using this switch position, software that only

accepts drive designations A & B, can now operate with your 3rd drive.

For example, you can now format disks on your 3rd drive from the

GEM DESKTOP.  With the switch in the Option position, the drive "B"

plugged into the "DRIVE A/B" connector cannot be accessed.  Your

internal drive A and a 4th drive can still be accessed.

It may be possible to re-configure your ST to have your internal

drive be drive B, and the first external drive be drive A.  This

would allow booting off the 3rd drive, as it does for ST computers

that do not have an internal drive.  This is beyond the scope of

what HAPPY COMPUTERS can help you with.

If your computer has no internal drive, the drive plugged into

the "Drive A/B" connector on the DISCOVERY CARTRIDGE would be

drive A.  With the switch in the Option position, any access to

drive A will now be directed to the drive plugged into the

"Drive 3/4" jack on the DISCOVERY CARTRIDGE.  Any operation

which you normally could only perform on drive A can also be done

with your 3rd drive.  This includes formatting, booting, and other

software that normally only lets you designate drive A or B as

floppy drives.  With the switch in the option position, your drive

A will not respond.  Drive B and your 4th drive will still respond.

While the drive that could be swapped (A / B) is being accessed by

the computer, the DISCOVERY CARTRIDGE ignores changes in the

position of the drive 3 swap switch. The following explains this

further. If the computer starts accessing the swapable drive, which

may either be drive A or B depending on which is your first external

drive, while the switch is in the normal position, then even if you

accidentally move the switch to the option position, the computer

will still continue to access the same drive, not drive 3.

By the same logic, if the computer starts by accessing drive 3, caused

by having the switch is in the Option position, then even if you move

the switch to the Normal position, the computer will continue to access

drive 3.  This "lock out" holds until the current drive operation is

finished, and the drive is deselected.  This is a safety lockout

feature.  You would not deliberately want to swap drives in the middle

of an operation.

The third drive will still respond normally to commands directed

to the third drive, even if the switch is in the Option position.

IMPORTANT NOTE:

DO NOT USE THE DRIVE 3 SWAP SWITCH feature if your third drive

cannot step at the 3ms step rate required for all drives that

are 100% Atari compatible.  The step rate you select with the

DINSTALL.TOS program does not apply to a "swapped" third drive.

The step rate applied to drives A & B is used instead, since the system

believes that you are accessing drive A or B.

TECHNICAL INFORMATION

DDRIVE does not necessarily have to be in the AUTO folder.  It can be

activated just by executing the program, if it has not already been

executed.

USING OTHER TYPES & SIZES OF DRIVES WITH YOUR COMPUTER

HAPPY COMPUTERS recommends that unless you are a technician familiar

with disk drive interfacing, that you purchase the type and size

drive you need that is already set up to be 100% compatible with

your computer.  At present, the only technical assistance HAPPY

COMPUTERS can offer on this matter is the information below.

DRIVE CONNECTION AND POWER:

*** STOP, read this! ***

HAPPY COMPUTERS does not represent any of the following

to be accurate.  It is the reader's responsibility to make

this judgement.

The drive power supply must conform to the grounding and isolation

requirements used by the other components in your computer system,

conforming to local electrical code requirements.

The DISCOVERY CARTRIDGE buffers all of the signals to and from the

"Drive 3/4" connector with the exception of the WRITE PROTECT coming

back from the drive.  The series and pull-up resistors used by the

DISCOVERY CARTRIDGE match that used in the ST computer, so use of

100% Atari compatible drives is assured.  Non Atari drives may have

an internal pull-up resistor on the WRITE DATA and WRITE GATE lines

that will not work with the 47 ohm series resistors RE and RF in the

DISCOVERY CARTRIDGE.  Either the series resistor needs to be changed,

or the pull-up resistor in the drive needs to be changed.

An acceptable value for the pull-up resistor on these signal lines

is 1K ohms, when the 47 ohm series resistors are present.

The drive must always respond to the MOTOR* control line, even if the

drive is not selected.  Some drives have a jumper which determines

this.  With the ST/MEGA all floppy's motors turn on and off at the

same time.

While the drive is selected, its WRITE PROTECT status should change

during the insertion of a disk at the moment a disk passes the

sensor.  Some drives do not activate their WRITE PROTECT status until

the disk is inserted almost all the way in.  This drive should not be

used with the ST/MEGA as the operating system needs to see the

change in write protect in order to update its "MEDIA CHANGED"

status.  Use of this drive can produce disasterous results if

the disk is changed.  The system may completely ruin the disk's

directory if it does not sense a change in the disk.  If the drive

was used solely as a source or destination drive with HAPPY COMPUTERS'

DMMXFER or DMFMBKUP programs, and never used with files, this

drive would be acceptable.

Use as short a cable between the drive and the DISCOVERY CARTRIDGE as

possible.  Use twisted pair with ground, or ribbon cable with ground on

every other wire for the STEP, WRITE DATA, WRITE GATE, READ DATA, and

INDEX signals, or better yet, for all signals.  At the DISCOVERY

CARTRIDGE end of your cable where there is a 14 pin male connector,

tie the shield connection to the logic grounds, and use this as the

ground for all of the above signals.  At the drive, connect these to

all of the signal ground connections available.

The "step rate" option provided by the DISCOVERY CARTRIDGE software

allows the user to select the head stepping rate for the 3rd and

4th drive.  This rate does not apply when the 3rd drive is selected

from the drive 3 swap switch.  In this case, the system step rate

applies, see below.  HAPPY COMPUTERS cannot determine the correct

step rate for a particular drive.  The specification sheet for the

drive, as published by the manufacturer or supplier of the drive

has this information.  If in doubt, select a larger (slower) rate.

The DMFMBKUP and DMMXFER programs will always use a step rate of at

least 12ms for all drives, regardless of what the system step rate or

the DRVTBL file specifies.  Some Atari supplied 3.5" drives actually do

not step at the 3ms Atari standard.  They have a special microprocessor

that slows down the step rate.  Since there is no efficient way to

determine if a particular drive has a "SLOW DOWN" processor, our programs

always allow at least 12ms for the head to settle before beginning

a write or read operation.  If the specification in the DRVTBL file

calls for a step rate slower than 12ms, these programs will use that

slower rate.  This does not seriously slow down these programs since

they only do a few random seeks.  Most of the time they just step

from one track to the next.  The time added to copying a disk by our

always using at least a 12ms step rate is less than 2 seconds.

When entering the STEP RATE with the DINSTALL.TOS program, you must

enter a 2 digit decimal value between 02 and 40, which is the decimal

quantity of milliseconds for the program to wait after each step of

the head, before beginning another step, or a read / write operation.

The step rate for the A & B drives can also be selected.  Location

$440 in the computer's memory controls the step rate for the A & B

drive.  This location is really useless for most systems that do

not use DDRIVE, since the location is always reset to $03 on

system reset or reboot, and is only looked at by the system just

after the system sticks this $03 value in there.  There is another

location that holds the currently active step rate for drives A & B.

Unfortunately, this location changes for the different versions

of Atari's operating system, so it should not be accessed by

application software.  Remember the rules for writing programs that

are not "OPERATING SYSTEM VERSION" dependent!

Here comes DDRIVE to the rescue.  When DDRIVE is first executed, it

reads the value out of location $440 and uses this as the step rate

control for drives A & B.  Therefore, you have the opportunity to

set location $440 first, and then execute & install DDRIVE to

establish a different step rate for drives A & B.  DDRIVE would

have to execute after this was done.  To have DDRIVE in the AUTO folder,

you would need to have a special program in the AUTO folder which

changed location $440 before DDRIVE executed.  Otherwise DDRIVE

should not be loaded from the AUTO folder.  HAPPY COMPUTERS cannot

provide this special program.  Valid values for location $440 are

$00, $01, $02, or $03, corresponding to 6, 12, 2, or 3 milliseconds, which

is applied to both drives A & B.

With the proper application of drive interfacing and step rate,

connection of a variety of drives becomes possible.  The most popular

drive to connect would be a 5 1/4" 40 track double sided drive that

is compatible with most IBM PC type computers or clones.

Even though a drive is connected and the step rate is accounted for,

the software must still be configured to account for the quantity of

tracks that a drive has, if this is any different from the 80 tracks

that standard ST drives have.  At present, HAPPY COMPUTERS cannot

offer any assistance regarding the configuring of software for a

particular drive.

The DISCOVERY CARTRIDGE can accomodate data rates up to 800K bits per

second.  This is sufficient to handle the "High density" 5 1/4" and

3.5" drives now in use.  However, our current software does not

easily support these higher densities.  The disk formatter / controller

IC built into the ST cannot accomodate these higher densities.

Future versions of our software will offer support of the higher

densities.

PROGRAMING ACCESS TO DRIVES 3 and 4

The DISCOVERY CARTRIDGE is likely to be in wide spread use by Atari

computer enthusiasts.  Software authors would be wise to not prohibit

the use of the 3rd and 4th drive features that this system offers.

After DDRIVE is installed, the extra drives respond through the TRAP

#13 and #14 calls.  The drive number passed to these is the logical

drive number computed as the drive letter minus "A".

If you wish to directly access the 3rd and 4th drives through the

hardware, first you should lock FLOCK by setting location $43E to

$FFFF, then you must deselect drives A & B through PORT A, in the

usual manner.  Execute the code shown as follows to select or de-select

the third and 4th drives.  NOTE: Whenever the 1st drive select signal

coming into the "COMPUTER" connector on the DISCOVERY CARTRIDGE is

selected, drives 3 and 4 are automatically deselected.  This is done

to prevent a conflict with software that is not aware of the 3rd and 4th

drives.  With FLOCK unlocked, the vertical blank routine will ensure

that the 3rd and 4th drive deselect, since it will at some point,

select drive A and B.

MOVE.L #$FB0200,A0 ;GET THE BASE ADDRESS OF THE HART CHIP

MOVE.W $188(A0),D0 ;SELECT THE 3RD DRIVE, IGNORE THE DATA READ INTO D0

MOVE.W $190(A0),D0 ;SELECT THE 4TH DRIVE, IGNORE THE DATA READ INTO D0

MOVE.W $180(A0).D0 ;DE-SELECT BOTH 3RD & 4TH, IGNORE DATA READ INTO D0

If you have DDRIVE loaded, DDRIVE's vertical blank handler will

restore the correct drive select after doing its "WRITE PROTECT" and

"MEDIA CHANGED" tests.  If DDRIVE has not been loaded, the standard

operating system's vertical blank handler will cause a selected 3rd or

4th drive to become de-selected.  Having a $FFFF in FLOCK $43E will

prevent the vertical blank routine from changing the drive selects.

It is best to leave FLOCK set to $FFFF until you are finished accessing

the 3rd and 4th drives, and the drive motors have turned off.

USER INSTALLED THIRD AND FOURTH DRIVE OPTION PARTS REQUIRED:

NOTE: **** HAPPY COMPUTERS CANNOT SELL PARTS SEPARATELY ****

In an ongoing quest to improve performance, HAPPY COMPUTERS reserves

the right to change components, connections and/or values without notice.

The part values shown are for use with standard Atari compatible drives.

J4 - ATARI 14 pin female drive connector

RB - 220 ohm 1/4 watt 5% resistor

RC - 1 Kohm 1/4 watt 5%resistor

RD - 1 Kohm 1/4 watt 5% resistor

RE - 47 ohm 1/4 watt 5% resistor

RF - 47 ohm 1/4 watt 5% resistor

U6 - 74LS14 TTL IC

U7 - 7406 TTL IC

DRIVE 3 SWAP OPTION, only a switch is needed:

SW1 - SPST PC mount switch that fits the mechanical requirements.

If a pc mount switch is not available, you might consider

securing the switch to the board with some thick wire.

It may be necessary to unsolder the holes where the components

are to be installed.

Please be aware that any modification made to your DISCOVERY CARTRIDGE,

including but not limited to the installation of options outside

HAPPY COMPUTERS' factory, will void your warranty.

#