DOPTIONS.DOC - DOCUMENTATION REVISION 2.7

(C) 1988 HAPPY COMPUTERS, Inc.

The following is a list of the factory installed option configurations.

The option number is marked in the space provided on the bottom of the

unit.  Refer to the rest of the documentation for a description of what

the options do, if more information is needed.

----------

OPTION 0

No extra factory installed options, nothing below is included.

This unit backs up software and converts disks to and from MACINTOSH

format.

----------

OPTION 1

battery backed up time and date clock

3rd and 4th drive circuitry and connector

3rd drive option switch

ROM/EPROM sockets

The ROM/EPROM sockets can be used with a 64K byte program in ROMS or

EPROMS.  The MACINTOSH 64K ROMS could be used, allowing the MAGIC SAC

software to operate.  A jumper must be added by the user to select the

state of pin 1 on the ROMS or EPROMS.  No ROMS or EPROMS are provided.

The MAGIC SAC software is not included.

A 3rd and 4th floppy drive can be connected, allowing a total of 4

floppy drives to be accessed from the desk top and most other software.

----------

OPTION 2

battery backed up time and date clock

3rd and 4th drive circuitry and connector

3rd drive option switch

ROM/EPROM sockets

ROM/EPROM bank select switch

The ROM/EPROM sockets can be used with a 64K byte program in ROMS or

EPROMS.  The MACINTOSH 64K ROMS could be used, allowing the MAGIC SAC

software to operate.  No jumper is required.  128K byte ROMS could be

used.  The switch selects a bank of 64K which will be enabled at one time.

No ROMS or EPROMS are provided.  The MAGIC SAC software is not included.

A 3rd and 4th floppy drive can be connected, allowing a total of 4

floppy drives to be accessed from the desk top and most other software.

----------

OPTION 3

jack for plugging in a second cartridge

HAPPY COMPUTERS has tested and found that the MAGIC SAC and DESKCART

will operate as a second cartridge with OPTION 3.  Other cartridges

may also work.  The SPECTRE 128 does not work.  Also, see options 5 or 6.

We do not guarantee that any cartridge will work as a second cartridge.

Some L shaped cartridges may not even fit as a second cartridge.

----------

OPTION 4

jack for plugging in a second cartridge

3rd drive option switch

3rd and 4th drive circuitry and connector

HAPPY COMPUTERS has tested and found that the MAGIC SAC and DESKCART

will operate as a second cartridge with OPTION 4.  Other cartridges

may also work.  The SPECTRE 128 does not work.  Also, see options 5 or 6.

We do not guarantee that any cartridge will work as a second cartridge.

Some L shaped cartridges may not even fit as a second cartridge.

A 3rd and 4th floppy drive can be connected, allowing a total of 4

floppy drives to be accessed from the desk top and most other software.

----------

OPTION 5

jack for plugging in a second cartridge

2nd cartridge enable switch

3rd drive option switch

3rd and 4th drive circuitry and connector

HAPPY COMPUTERS has tested and found that the MAGIC SAC and DESKCART

and SPECTRE 128 will operate as a second cartridge with OPTION 5.

Many other cartridges may work with option 5 that do not work with

options 3 or 4.  We do not guarantee that any cartridge will work as

a second cartridge.  Some L shaped cartridges may not even fit as a

second cartridge.  The 2nd cartridge switch allows the user to select

whether the 2nd cartridge or the DISCOVERY CARTRIDGE is enabled.

A 3rd and 4th floppy drive can be connected, allowing a total of 4

floppy drives to be accessed from the desk top and most other software.

----------

OPTION 6

jack for plugging in a second cartridge

2nd cartridge enable switch

HAPPY COMPUTERS has tested and found that the MAGIC SAC and DESKCART

and SPECTRE 128 will operate as a second cartridge with OPTION 6.

Many other cartridges may work with option 6 that do not work with

options 3 or 4.  We do not guarantee that any cartridge will work as

a second cartridge.  Some L shaped cartridges may not even fit as a

second cartridge.  The 2nd cartridge switch allows the user to select

whether the 2nd cartridge or the DISCOVERY CARTRIDGE is enabled.

----------

OPTION 7

battery backed up time and date clock

3rd and 4th drive circuitry and connector

3rd drive option switch

ROM/EPROM sockets

128K ROM/EPROM enable switch

The ROM/EPROM sockets can be used with a 128K byte contiguous program

contained in ROMS or EPROMS.  The MACINTOSH 128K ROMS could be used,

but the SPECTRE 128 software still will not operate without some

additional changes, which are listed in the documentation.  HAPPY

COMPUTERS cannot currently provide a DISCOVERY CARTRIDGE which

allows running the SPECTRE 128 software by simply plugging in the

MACINTOSH 128K ROMS.  No ROMS or EPROMS are included.  No SPECTRE 128

SOFTWARE is included.

A 3rd and 4th floppy drive can be connected, allowing a total of 4

floppy drives to be accessed from the desk top and most other software.

#