DCLOCK.DOC - DOCUMENTATION REVISION 2.6

(C) 1988 HAPPY COMPUTERS, inc.

This is the documentation for the battery backed up clock for the

DISCOVERY CARTRIDGE.  This feature is optional.  The first part

of this documentation describes the use of the software that HAPPY

COMPUTERS supplies on the DISCOVERY CARTRIDGE Software Utilities

disk for use with this battery backed up clock.  The second part

of this document describes the hardware portion of this option

including high accuracy calibration.

Your DISCOVERY CARTRIDGE must have the optional clock hardware

circuitry installed, in order to use the features described in this

document.  FACTORY installed options 1 and 2 include this.

The program DCLOCK.PRG should be placed in the AUTO folder on your

booting disk.  Once your computer is booted with DCLOCK.PRG in the

AUTO folder, a message will confirm that DCLOCK has been activated

upon bootup.  If the program can read a valid time from the clock,

it will report this, and this message stays on the screen only

momentarily.  In this case, DCLOCK reads the time and date from the

DISCOVERY CARTRIDGE's clock, and stores them in your computer.

The message that the clock has been set correctly can be held on

the screen as long as you want by holding down either CONTROL,

or ALTERNATE, or the LEFT SHIFT key.

If DCLOCK cannot read a valid time from the DISCOVERY CARTRIDGE's

clock, an error message will be reported, and that message will

stay on the screen until you press return.  DCLOCK may report

that the time values need to be set.  In this case, simply set the

clock, and if the clock hardware and its battery are okay, the time

will be correct the next time you boot your computer.

NOTE: A new DISCOVERY CARTRIDGE with a factory installed clock option

will probably need to have its clock set.  Our manufacturing test

program forces this condition to ensure that you set your clock for

your time zone.  Your dealer may have set your clock for you.

SETTING THE CLOCK:

With the Atari supplied file CONTROL.ACC in the root directory on

the disk you booted, you can call the Control Panel Desk Accessory.

First set the date, then enter the time value for the next minute

which is about to come up.  HOLD ONE FINGER ON THE RIGHT SHIFT KEY,

and when the time is exactly that which you typed in, press RETURN.

If you do not hold down the right shift key while pressing the

return key, the GEM software clock will be set, but the DISCOVERY

CARTRIDGE's clock will not be set.  The DCLOCK.PRG must have been

installed and activated on boot up prior to setting the clock for

the DISCOVERY CARTRIDGE's clock to be set.

Once the clock has been set, its battery holds the time even when

your ST computer is turned off.  It should not need to be set again

unless one of the following occurs:

1) BATTERY FAILURE - this is usually detected by DCLOCK on boot up.

2) HARDWARE problem - this is usually detected by DCLOCK on boot up.

3) CLOCK GLITCHED - if the display mode of DCLOCK.TOS (see below)

is interrupted by SYSTEM RESET or POWER DOWN, instead of a

correct exit, or if some program incorrectly accesses the

clock, the proper time values in the clock can be lost.

DCLOCK may or may not be able to detect this on boot up.

4) LOSS OF TIME ACCURACY - only the most exotic and expensive

clocks are sufficiently accurate to never need resetting.

HAPPY COMPUTERS has a clock that sets itself automatically

based on the WWV clock signal from Colorado and Hawaii.

I guess I had enough arguments with my wife about what the

time really was, so I bought the HEATHKIT WWV clock kit!

A typical battery backed up clock, which has not had high

accuracy calibration, will usually be accurate to within

a few seconds a week.  So after a few months, the time

DISPLAYED may be a couple of minutes off.  If you want,

you can always just reset the time.  DCLOCK cannot detect

loss of time accuracy on boot up.

DCLOCK may detect that it cannot read a correct time at all from

the DISCOVERY CARTRIDGE.  In this case, check the battery first.

Some repair may be needed.

Although the software for the MAGIC SAC made by DATA PACIFIC will

recognize the DISCOVERY CARTRIDGE clock circuit as though it were

a MAGIC SAC clock circuit, the MAGIC SAC program for setting the

clock should not be used with a DISCOVERY CARTRIDGE, if you intend

to used the DCLOCK program.  If you set the clock with the MAGIC

SAC program, you may get an error message from DCLOCK when DCLOCK

is executed from the auto folder.  However, you may set the clock with

DCLOCK, and use the clock with the MAGIC SAC program.

The MAGIC SAC uses the same clock chip as the DISCOVERY CARTRIDGE.

However, the clock circuit design in the DISCOVERY CARTRIDGE is far

more accurate than the one in the MAGIC SAC.  Also, the DCLOCK

software is more accurate than the software with the Magic Sac,

since the DCLOCK software will exactly adjust to the seconds

change of the clock chip.  The DCLOCK program will not recognize

the MAGIC SAC, even though it uses the same clock chip, since the

MAGIC SAC does not have the powerful HART chip.

TECHNICAL INFORMATION ON DCLOCK SOFTWARE:

Including the DISCOVERY CARTRIDGE's clock, the ST has at its

disposal a total of 3 clocks.  There is a hardware clock inside the

keyboard.  There is a software clock based on interrupts kept

by GEM DOS.  There is a battery backed up clock in the DISCOVERY

CARTRIDGE.  The newer MEGA computers also have a battery backed

up clock.  Both the hardware clock in the keyboard and the

software clock in GEM DOS are set any time you set the clock from

the control panel.  The DISCOVERY CARTRIDGE's clock is set when

you also hold down the right shift key while setting the other

clocks.

If you don't have a DISCOVERY CARTRIDGE clock, GEM will

try to read a valid time from the keyboard's clock on bootup.

Bootup means either the first time since the power to the

computer is turned on, or after pressing the reset button.

On bootup the first time the computer is turned on, GEM will

not read a valid time from the keyboard, so GEM's time is not set.

If you press reset on your computer without turning the power

off, GEM will set its software clock from the hardware clock

in the keyboard, if this value had been set.

Let's say DCLOCK was run from the auto folder, and GEM's and the

keyboard's clock were therefore set, and later you reboot using the

reset button, and you boot a disk which does not have DCLOCK in

the auto folder.  In this case, GEM's clock will still be set from

the clock values still contained in the keyboard, even though they

were originally set from the DCLOCK program previously run at some

time since the computer was first booted.

SPECIAL DCLOCK DISPLAY MODE:

A special display mode of DCLOCK can be entered.  First we recommend

that you make a copy of DCLOCK.PRG and rename it to DCLOCK.TOS.

This will avoid problems with the mouse overwriting the display.

Your screen should be in medium or high resolution to use this mode.

Execute the DCLOCK.TOS program, and before it completes loading,

hold down the CONTROL and ALTERNATE and RIGHT SHIFT keys all at the

same time.  DCLOCK will then continuously display the time and date

from all three clocks that the ST has at its disposal.  If there

is a problem reading the clock value from the DISCOVERY CARTRIDGE,

this display mode will show a display with no times printed.

You will notice that the time display for the GEM clock and keyboard

clock both increase 2 seconds at a time.  There may be a time

difference of several seconds at first, which may increase if it

has been some time since the computer was booted.  This is because

of accuracy differences between the software interrupt clock, and the

oscillator in the keyboard clock.

Programs that disable the interrupts will usually cause GEM's clock

to become slower. The DMMXFER and DMFMBKUP programs disable interrupts

for long periods of time.  However, when you exit these programs, they

will automatically re-read the DISCOVERY CARTRIDGE's clock and reset

GEM's time, providing that the DCLOCK program was already executed

and resident.

You will also notice that the display of the DISCOVERY CARTRIDGE

clock is incremented every 30 seconds, and a bell tone is

sounded each time this display changes.  The 8573 clock chip used

does not have a readout for the seconds.  DCLOCK can still

determine exactly when it is 0 or 30 seconds past a minute change

and displays this.

Since the DISCOVERY CARTRIDGE clock does not read out seconds

directly, the DCLOCK program first reads out the clock and sets

the computer's GEM and keyboard clocks to within 30 seconds, and

then waits until the exact change to 0 or 30 seconds past the

minute occurs, and then updates GEM's and the keyboard's clock

to these exact values.

Therefore, when the system is first booted, the clock may be

at most 30 seconds slow, and within 30 seconds, the clock will

be corrected to coincide exactly to the time measured by the

DISCOVERY CARTRIDGE's clock.  This entire process is usually

invisible to the user, but may be observed by booting the system

and entering the display mode of DCLOCK at just the right moment.

Once this 30 seconds correction has taken place, the clock value

in the DISCOVERY CARTRIDGE is not utilized by GEM DOS again.

GEM maintains its clock using the computer's interrupt system.

As already stated, DMMXFER and DMFMBKUP will re-read the DISCOVERY

CARTRIDGE's clock and reset GEM's time if DCLOCK was already loaded.

You may also re-run DCLOCK to cause the DISCOVERY CARTRIDGE's clock

to be read, to update GEM's clock.  This permits you to manually update

GEM's clock to the accurate time kept in the DISCOVERY CARTRIDGE.

IMPORTANT:

If you use a system reset or power down to exit the special display

mode of the DCLOCK program, the time values stored in the clock chip

can be lost.  Exit the special display mode only by holding down

CONTROL and ALTERNATE and LEFT SHIFT together.

You will also notice a year/flag and status value displayed.  These

values are used by the DCLOCK program to determine what the year

is and when to update it.  Also, the DCLOCK program can sense a

loss of battery power by looking at these.  They do not provide

information for the user.

The DCLOCK program is about 3900 bytes long.  Once DCLOCK is run,

it installs a resident program that requires about 1700 bytes.

This resident program provides the 30 second correction mentioned

above, and sets the clock chip in the DISCOVERY CARTRIDGE when you

follow the procedure shown above.

USER INSTALLED CLOCK PARTS REQUIRED:

NOTE: **** HAPPY COMPUTERS CANNOT SELL PARTS SEPARATELY ****

In an ongoing quest to improve performance, HAPPY COMPUTERS reserves

the right to change components and or values without notice.

C4 - 5-50pf variable cap, see text

C5 - 12pf NPO cap, see text

D1 - 1N914 or equivalent

D2 - 1N914 or equivalent

R4 - 2.2K resistor 1/4 watt 5%

R5 - 4.3K resistor 1/4 watt 5%

R6 - 15K resistor 1/4 watt 5%

R7 - 4.3K resistor 1/4 watt 5%

U3 - 8573 clock chip

X2 - 32768 Hz crystal resonator, "watch tuning fork"

Battery & holder = 3 volt lithium button type.  A wide variety

of batteries and holders can be used.  These may differ in size and

life.  A CR2032 type should last about a year.  It is even possible

to use a high capacity cylindrical lithium battery, with extension

wires added to meet the holes in the board.  Make sure the battery

and holder you have selected will fit inside the DISCOVERY CARTRIDGE

case before you solder it in.  A button type battery and holder are

usually mounted on the bottom of the board.  Pay careful attention to

the battery polarity shown on the PC board silkscreen.  Two holes are

provided for the positive terminal, to allow mounting two different

popular size button battery holders.

Please be aware that any modification made to your DISCOVERY CARTRIDGE,

including but not limited to installation of options outside of

HAPPY COMPUTERS' factory, will void your warranty.

It may be necessary to unsolder the holes where the components

are to be installed.

On rev D boards, a trace is missing which must be effected by adding

a jumper wire.  This trace should have connected pin 16 of U3 to C4

and C5.  Factory installed clock option rev D boards have a jumper

installed which makes this connection.  DISCOVERY CARTRIDGES which

did not have a factory installed clock did not have the jumper

installed by HAPPY COMPUTERS.  Installation of the clock option

on REV D boards requires adding this jumper.  Boards that are REV

E or higher, do not require this jumper.

On rev E boards, positions are provided to have C5 consist of 2

capacitors in parallel.  Also, positions are provided to have

C4 be an adjustable cap or a fixed cap, or both of these in parallel.

This has been done to facilitate high accuracy calibration.

HIGH ACCURACY CALIBRATION

The key items to calibration are accurate frequency of the

oscillator, and stability versus time and temperature.

Using standard commercial tolerance components at the values shown

for C4, C5 and X2, and without adjusting C4 at all, this circuit will

typically have an error of several seconds per week, although this

may be worse or better.

The first item required for calibration is a digital frequency

meter which measures frequency as "one over the period".  Pin

11 of U3 outputs a 128Hz signal (7812.5000 microseconds period)

which is the current oscillator value.  Do not measure the 32768

HZ at the XTAL, as any measuring probe connected will throw the

oscillator off its center value.  Measure only at pin 11.  This

period measuring device should have a calibrated "oven stabilized"

reference standard.  The typical period accuracy of such a device

will be 0.1 ppm.  A device with worse accuracy can be substituted,

and the resulting calibration will of course be less accurate.

A meter which cannot measure frequency as 1/period will not

work at all for this purpose.

If you solder any component, you must wait for the component to

reach room temperature before measuring the result.

First C4 is set to the minimum capacitance setting (minimum period)

setting.  Then the X2 and C5 are chosen to bring the value as

close as possible to 7812.5000usec, without exceeding it.  If

measuring frequency, that would be greater than or equal to 128Hz.

Use at most 2 capacitors for C5 in parallel to accomplish this.

Both capacitors must be of the NPO type which have high stability

and low temperature drift.  The value for C5 depends on the X2

crystal.  You may need to try a different crystal if a variety

of capacitors are not available.  Typical values for C5 range from

10.5 to 12.5 picofarads.  C5 will have the largest effect on the

frequency.  Then adjust C4 to the exact value needed.  If you

cannot reach the desired period even with C4 at maximum, you may

install a 47pf capacitor in parallel with C4.  Rev E boards provide

a place to do this, rev D boards can still do it fairly easily.

Do not exceed a total parallel capacitance of 120pf on C4.  If

you cannot reach the desired frequency with C4 at 120pf, you will

need to reselect C5 and/or X2.

The temperature stability and drift depends on the quality of

components used for C4, C5 and X2.  C5 first and X2 second will

have the largest effect on the stablilty.  Having an accuracy of

one second per month for a unit kept in a room with a 10 degree F

temperature variation can be achieved.  It is beyond the scope

of this document to discuss why someone may want to achieve

high accuracy calibration.

#