Black Box Technical Information

This technical information is for those who wish to write programs that use the data used in the BB. This information need not be understood for the average user; it is for programmers.

The BB consists of several peripheral I/O chips mapped into the $D1xx address. The BB's RAM is split up into 256 byte pages, which are mapped into a window at $D600-$D6FF. The high byte of the RAM address is at location $D1BC, named RAMPAGE. RAMPAGE is normally set to $FF; values $FE through $EE are used for system use, and the rest for the internal RAM printer buffer. On 8k RAM versions, RAMPAGE may only go as low as $E0.

These are the registers of the BB hardware that may be useful:

Address Name Description

$D1BC RAMPAGE This is the high address byte of the BB's internal RAM. Memory is mapped in at page $D6xx. This register is read/write.

$D1BE SWITCH bit 0=dump type switch, bit 1=HD write protect switch, bits 2-7 are dip switches 2-7. This is a read-only register. Each bit is inverted.

$D1C0 SENSEL Bit 5=DSR, bit 6=CTS, bit 7=CD from RS232 Port.  This is a read-only register.

$D1FF PBIBANK Bits 0-3 select which bank of the BB's ROM will be mapped into the $D800-$DFFF memory segment.

Location $0248 is the shadow of this write-only register.

$D6xx BBRAMW This 256 byte window is where the BB's internal RAM is accessed. The high byte of the 64k address range is set in RAMPAGE.

There are two pages of the BB's memory that may be useful. RAM page $FE is where all the miscellaneous working variables are stored. Here are a few locations of interest:

Address Description

$D608-$D610 Floppy high speed table. Each byte corresponds to drives 1-9. If they are configured as a floppy in high speed, these bytes will identify the type of high speed. $80=standard speed, $10=Happy warp, $0F=XF551, $0A=UltraSpeed, $00=not yet determined.

$D653 Partition Index. This byte identifies which entry ($10-$6F) will be used when accessing the partition list read/write routines.

$D656-$D667 Partition data buffer. This is the 18 byte buffer that is used to read/write to the partition list.

Page $F5 holds the main system configuration. Here are the variables used in it:

Address Description

- Drive Configuration -

$D600-$D609 ASCII text of partition name. Only valid when drive is configured as a hard disk.

$D60A Bit 7=Floppy flag. Bits 6-0 are the partition index value. The index value starts at $10, so if this value is less than $10, it is unknown.

$D60B Bit 7=HD write protect flag. Bit 6=Ultra Speed flag (valid if floppy flag=1). Bit 5=HD double density mode (0 means single). Bits 2-0 are the controller index (0-7, which translates to A-H on the menu).

$D60C-$D60E HD starting sector number -1. Low byte first. This is the actual sector number, so when using 512 byte sectors, this number is multiplied by 2 before being displayed on the menu.

$D60F-$D610 HD length +1, bits 0-15. Low byte first. Note that this value does not change for 512 byte sectors.

$D611 Bits 7-4 are the floppy number used if this drive was configured as a floppy. Bits 1-0 are bits 16-17 of the HD length +1 value.

$D600-$D611 The above definition was for logical drive 1. This is repeated 8 more times, for all 9 drives.

$D612-$D623 Configuration for D2:

$D624-$D635 Configuration for D3:

$D636-$D647 Configuration for D4:

$D648-$D659 Configuration for D5:

$D65A-$D66B Configuration for D6:

$D66C-$D67D Configuration for D7:

$D67E-$D68F Configuration for D8:

$D690-$D6A1 Configuration for D9:

- Controller Configuration -

$D6AB Bits 7-5:Logical Unit Number of hard disk. Bit 4 is the SASI flag (0=SCSI). Bit 3 the 512 byte sector flag (0=256 byte sectors). Bits 2-0: SCSI bus controller ID.

$D6AC-$D6AD Number of Cylinders, low byte first.

$D6AE Bits 7-4:number of heads. Bits 3-0:step rate.

$D6AF-$D6B0 Reduce Write Current cylinder (low byte first).

$D6B1-$D6B2 Write Precompensation cylinder (low byte first).

$D6AB-$D6B2 The above definition was for controller 1. This is repeated 7 more times for the 8 controllers possible.

$D6B3-$D6BA Configuration for controller 2

$D6BB-$D6C2 Configuration for controller 3

$D6C3-$D6CA Configuration for controller 4

$D6CB-$D6D2 Configuration for controller 5

$D6D3-$D6DA Configuration for controller 6

$D6DB-$D6E2 Configuration for controller 7

$D6E3-$D6EA Configuration for controller 8

- Port Statuses -

$D6ED Bit 7:I/O sound flag. Bit 6:Enable RS232 port. Bit 5:Enable printer port. Bit 4:Send LF after CR. Bit 3:Make printer respond to all IDS. Bits 2-0: Printer ID+1 to use (if bit 3=0).

$D6EE Bit 7:Spooler enabled. Bit 6: Spooler using XE RAM (0=spooler using internal BB RAM). Bit 5: XE RAM available. Bit 4:Internal RAM available.

$D6EF-$D6F1 Partition list starting sector number, low byte first. This is the actual sector number, so it is multiplied by two if the drive uses 512 byte sectors.

$D6FE-$D6FF Configuration check number. This 16 bit number is calculated based on the information of $D600-$D6FD.  It is checked every time the system is reset. If the calculated number does not match the number here, the configuration from hard disk #0,0 is loaded. If that cannot be done, the config is initialized to all floppy drives.

You may make modifications to the system configuration by changing the parameters in this page, but upon a RESET or a cold start, the BB will reload the saved configuration. If you wish to make a permanent change, you must calculate the new configuration check number, and place it in the last two bytes. This may be accomplished by the following machine language subroutine:

RAMPAGE = $D1BC ;identify hardware registers

BBRAMW = $D600 ;where the window is located

CFG_PAGE = $F5 ;page used for system config

CFGCHKN = $D6FE ;location of the check value

DUMMYPAGE = $FF ;number that should be placed in RAMPAGE after ;we're done so that an inadvertent store to the ;$D6xx page doesn't wipe us out

;

SET_CHECKSUM ;Now, our subroutine

LDA #CFG_PAGE ;tell the BB we want to mess with

STA RAMPAGE ;the configuration page

LDA #0 ;initialize registers to 0

TAX

TAY

CLC

SET_C1 ADC BBRAMW,Y ;add the value of a byte,

PHA ;save for a sec

TXA

ADC #$0E ;this part does some bit

EOR BBRAMW,Y ;manipulations, so that we

TAX ;will detect any bit change,

PLA ;even if a bit was only moved.

INY

CPY #$FE ;do this for all 254 bytes

BCC SET_C1

STX CFGCHKN ;store our new value

STA CFGCHKN+1

LDA #DUMMYPAGE ;now lets set the dummy page value,

STA RAMPAGE ;so that we don't mess up the config

RTS ;and that's it!

This routine could not easily be accomplished using BASIC statements, but it could be embedded within a string and called by a USR statement, since it is relocatable.

The Partition list may be accessed by setting $D653 in the miscellaneous page (RAMPAGE=$FE) to the desired partition number + 15 (The first partition is 16, or $10), then calling the read partition vector. When returning from the read partition subroutine, the carry flag will be set if there was an error. The Partition data buffer at $D656 of the miscellaneous bank will contain the 18 bytes for that partition. The example below will demonstrate how you can incorporate the partition list into your programs. What it will do is take the partition number (1-96) from the A register, and put its information into logical drive 8.

RAMPAGE = $D1BC

CFG_PAGE = $F5

MISC_PAGE = $FE

DUMMYPAGE = $FF ;These were defined in the previous example

PBIBANK = $D1FF ;BB firmware control register

PBISHADOW = $0248 ;RAM shadow register for PBIBANK

HDCODE_BANK = $08 ;bank where hard disk drivers reside

;

SET_PART ;our subroutine. A=(1-96) which partition we

;want to place into drive 8.

CLC

ADC #$0F ;the BB numbers them $10-$6F

LDX #MISC_PAGE ;set ram window for miscellaneous

STX RAMPAGE

STA $D653 ;we needed to set RAMPAGE first, else this

;store might have ended up in the wrong bank

LDA #HDCODE_BANK ;bank for the hd low level code

STA PBISHADOW ;store in shadow register first

STA PBIBANK ;puts the BB's HD code into $D800-$DFFF

JSR $D82C ;** Address for Read Partition subroutine **

LDA #0 ;restore normal FP back

STA PBISHADOW

STA PBIBANK

BCS SET_ERROR ;carry is set if there was an error

LDA $D656 ;if bit 7 (the minus flag) is set, then

BMI SET_ERROR ;this partition is not in use

LDY #0 ;prepare to copy the bytes

SET_1 LDX #MISC_PAGE ;get one byte from the

STX RAMPAGE ;partition data buffer in the

LDA $D656,Y ;miscellaneous page

LDX #CFG_PAGE ;now put the byte in the main

STX RAMPAGE ;configuration, in the config page.

STA $D67E,Y ;$D67E is the first byte for D8:

INY

CPY #18 ;copy 18 bytes this way

BCC SET_1

CLC ;we've completed with success!

BCC SET_2 ;skip the set carry

SET_ERROR SEC ;set the carry upon an error

SET_2 LDA #DUMMYPAGE ;return with the dummy page

STA RAMPAGE

RTS ;and return

This routine will return with the carry flag set if there was an error (no partition list defined or bad sector), or clear if everything was ok.

Random Notes

The BB provides high speed capabilities with some drives. When you format a disk on a modified 1050 or XF551 (not happy drives), it will write out the sectors in a special high speed format. If you read a disk with the high speed format on a standard drive, it will be slower than normal. If you want to format a disk with the standard format, you can either disable the high speed (with the menu, or holding SELECT and OPTION while the drive is being accessed), or hold the SHIFT key down when the format command is given.

There may be a few game programs that will not allow you to enter the BB's config menu. They do so because they load in their own special operating system. Simply press RESET to re-enable the standard OS.

The BB should be used with XL/XE compatible operating systems only. OS chips based on the old 400/800 operating system will function, but will render the BB useless because they have no drivers for parallel bus devices. The same applies for disk based 'translator' programs.

The BB's physical format program provides allows you to change the format interleave factor. CSS has run tests on all controllers listed and determined the optimum interleave. This number should only be changed if you need to slow down the operation of your hard disk.

CSS would like to thank Jeff Williams for his involvement in the BB firmware development, Ed Bachman for his directory format program, and all those who offered suggestions and comments with regards to BB development. It's great to see 8-bit programmers come together to make a product like this possible.

We have set up a support BBS for those of you who have modems, to provide the latest information and make program updates available. There are no connect charges for this service, just the cost of the call. The BBS is online 24 hours a day, running at 300 and 1200 BAUD. ATASCII, ASCII, and VT-52 translations are available. The number is (716) 247-7157. An interesting note: because we are using multiple lines, you will see another of our fine products, THE MULTIPLEXER, in operation!

Appendix A: Compatible Controllers

The BB's SASI/SCSI port should interface to any SASI or SCSI device. However, since most controllers have their own special way of formatting drives, there needs to be separate formatter programs written for each one.

We have combined the format routines for all the controllers we have seen into the HDFMT.COM program. If you have a controller that is not listed, give us a call. We may be able to write a format program for your controller, provided that you have the technical manual for it. Any new format utility programs will be available on our CSS Support BBS.

These are the controllers known to work with the BB:

Adaptec 4000A and 4070 (SCSI) Seagate embedded SCSI drives **

Konan DJ-210 (SASI) SMS 5001 (SASI or SCSI)*

OMTI 3520 and 3527 (SCSI) ** Western Digital 1002-SHD (SASI)

Seagate ST9000 (SASI) (WD clone) Xebec S1410 and S1410A (SASI)

* - The SMS controller needs to be set to SCSI for 256 byte sectors, and to SASI for 512 byte sectors. This is done automatically in the format program; you need only select the sector size.

** - The OMTI controllers can only be used in 512 byte mode.

*** - Some Seagate embedded drives can only be used in 512 byte sector mode. Seagate embedded drives are characterized with a suffix of N.

Appendix B: Hard Drive Data

(The data listed here was so old that it is not relevant.  The BB's format program now auto-detects SCSI drive settings, so no real configuration is necessary unless you are using an older drive with a separate controller.)

Appendix C: Files on the BB DISK

The following is a list of the programs on the BB distribution disk, and a brief description of their purpose:

DOS.SYS and DUP.SYS: These are the DOS files to MYDOS version 4.50. This is a freeware DOS by Charles Marslett of Wordmark Systems that can handle hard disks with ease. It is simple to use, yet powerful in features.

VTOCFIX.COM and VTOCFIX.DOC: The VTOCFIX is a MYDOS utility for fixing up any problems that may occur. The .DOC file is the documentation for this program, and may be printed out by copying to P:.

HDFMT.COM: This is the BB physical format program, used to format hard drives.

FMTDIR.COM: This program is used to set your hard disk partitions up so that DOS can use them. It also maps out any bad sectors that were not able to be mapped out at the controller level.

USPRAM9.COM: This is a utility program for those who use the US+ OS (by CSS). The US+ defaults the RAMdisk to be drive 4. This file will set the US+'s RAMdisk drive number to 9, so that it does not get in the way of the BB (if you wanted to use drive 4 as a hard disk). To install this program, first copy it to your drive 1. Now copy any AUTORUN.SYS file onto this WITH APPEND (use the /A modifier). Rename or remove your old AUTORUN.SYS file, and rename this new file to AUTORUN.SYS.

PARK.COM: This program will position the heads of each connected hard disk to the inner cylinder. This is a precautionary step that should be done prior to powering down your hard disks (if they do not have an auto-park feature). It also reports the last sector available on that drive.