PREFACE

The purpose of this project is to pass on information we have learned about copy protection.  This disk was written for the novice programmer and provides not only useful information, but useful type-in programs, as well.  This software will work on Atari 400's, 800's, 800XL's, 65XE's and 130XE's with 1050 disk drives. There are some special programs which make special use of the Happy 1050 and Duplicator 1050 hybrid drives. For those of you who like to "break" disks, this one will require a more than modest effort.  Also, for all you "back-up file writers" out there, we change our copy protection lock algorithm on every disk (there are 24 different types of copy protection on this disk)!  But, like any other disk, it can be cracked.  So, if you do decide to crack this disk, have fun.  But, "fer cryin' out loud", please don't give copies away!  It has been our effort, at Del Com Services, to provide a top quality disk, unique and useful programs (all files are written to both sides of your disk), and well-written, printed documentation, all at a fair price!  We'll also provide answers to questions, and useful info to registered purchasers (so don't neglect to send back your registration form!).  We'll have more choice goodies comin' your way for Atari 8-bit machines and ST's, if we can make a living at it!

Thank you fellow (and gal!) computer enthusiasts,

the AUTHOR

TABLE OF CONTENTS

I.    THE PROGRAM DISK (an overview)            1

Brief Program Descriptions                2

II.   THE DEVICE CONTROL BLOCK                  3

Device Control Commands                   3

Device Status Indicators                  4

The DAUX and Commands                     4

Happy 1050 DAUX Commands                  5

Duplicator 1050 DAUX Commands             5

Using the SIO WORK FILE                   6

III.  PROGRAM PROTECTION TECHNIQUES             8

Using the Happy & Duplicator

Drives to Protect Your Programs     8

More About Our Disk                       8

Program (Machine Language):

Check For a Happy 1050            9

Program (BASIC): Check For a Happy 1050  10

POKE'ing "Out" the BREAK & RESET Keys    10

Program (BASIC): Prevent a BREAK

During an I/O Function           11

Useful USR Calls                         11

Protecting Against a LIST                12

IV.   THE HAPPY SECTION                        14

The Happy File Maker                     14

The Custom Happy Menu                    14

The SIO Block Introduction               15

Author's Personal Comments               16

V.    THE SIO CALLS & TECHNICAL NOTES          17

A Portion of the SIO Block               17

SIO Block Notes                          19

Program (BASIC with Machine Language

Sub-routine): Check for a

Happy 1050 & Set Fastwrite Mode  20

VI.   TYPE-IN BASIC PROGRAMS                   24

Write "Bad" Sectors with the

Duplicator 1050 Drive              24

Search a Disk for "Bad" Sectors

and Give the Locations            25

Report the Status of all Serial

Devices On Line                   26

VII.  PRODUCT/COMPANY ACKNOWLEDGEMENTS         28

I. THE PROGRAM DISK

This program disk has some files that were intended to be used with DOS's other than ATARI DOS 2.5. You MUST boot this disk with Atari BASIC (Rev A or C) loaded. Also, this software will allow you to do some very interesting things with the Happy 1050 and Duplicator 1050 drives

The following files are in a standard directory and can be transferred to another disk for back-up purposes and to allow space on your work disk to create new files.  You should transfer them to a work disk IMMEDIATELY!

25HAPPY.COM     (uses ATARI DOS 2.0 or 2.5)

25MENU.COM      (uses ATARI DOS 2.0 or 2.5)

FASTWRIT.COM    (needs SPARTA DOS 3.2C or later)

HPYMENU.COM     (needs SPARTA DOS 3.2C or later)

MKECOHPY.COM    (needs SPARTA DOS 3.2C or later)

LABLE.BAS       (needs SPARTA DOS 3.2C or later)

These next files are also on the system disk and were not meant to be backed up (easily). Almost all run under Atari DOS.

HAPPY BASIC MENU FILE

FAST WRITE BASIC FILE

TEST MENU FILE

SIO WORK FILE

LABELER BASIC FILE note: this file cannot be used with

the DOS that is on this disk.  It

only works when used with ICD Sparta

DOS 3.2C or later (you can purchase

it from the good folks at ICD).

1050 DISK DRIVE STATUS FILE

All of the programs on this disk are pretty well user friendly (bet you've never heard that before!) and really don't need any DOC files, but here is a brief overview of what the protected files are all about.

HAPPY BASIC MENU FILE: this is a basic program that

will allow you to program Happy

1050 drives.

FAST WRITE BASIC FILE: this little basic file will set

Happy 1050 drives into a "fast

write" mode.

TEST MENU FILE: same as the HAPPY BASIC MENU except it

will allow you to use the full extent

of the SIO calls, such as changing a

drive # past 8 and so on. It also

checks to see if a Happy 1050 drive is

present.

SIO WORK FILE: this is a refined version of the program                 we used to find all the device control                   block codes that the different disk                      drives on the market would answer to.                    All SIO codes can be excuted from this                   program.

1050 DISK DRIVE STATUS FILE: this is a neat little file

that will tell you just

whose version of the 1050

drive you have on line.

II. THE DEVICE CONTROL BLOCK

The Device Control Block (DCB) is where all device paramater codes are stored to tell disk drives what to do when an SIO action is called.

Memory location numbers that have a $ in front of them are in hexadecimal notation; numbers that have no $ are in decimal notation and will usually follow the hex notation as an alternative: (or ___). Please note that when POKE'ing numbers from a BASIC program, the numbers you POKE will always be in decimal notation, not hexadecimal; BASIC does the conversion, internally, as it is run.

Location $300 (or 768) of the DCB (Device Control Block) is where you POKE the serial device type code; for example: a disk drive = $31 (or 49).  Location $301 (or 769) of the DCB is where you POKE the device (disk drive) unit number; for example: device unit number D1 thru D4 (for an un-modified 1050 drive with Atari DOS 2.0 or 2.5) -or- device unit number D1 thru D8 (for ICD's U.S. Doubler [you can only access D5 thru D8 with a special ICD chip]) -or- D1 thru D79 (THAT'S RIGHT! for the Happy 1050 [no special chips needed]).  Note that the Happy 1050 can be programmed to think it is drive unit number 1 thru 79!  (Now that's a smart drive!)

Location $302 (or 770) of the DCB is where you POKE the device control command in.

call Happy 1050                     72

call Duplicator 1050                98

tell a Duplicator 1050 to write bad sectors           119

format in single density            33

format in 1050 (enhanced) density   34

read a sector                       82

write a sector                      87

give status of device               83

(There are more of these commands in COMPUTE Magazine's informative book, "MAPPING the ATARI".  We highly recommend this very useful book!)

Location $303 (or 771) is where a device status indicator is returned to tell you that the operation you called for is okay, or if there is an error.

1 = everything is okay

128 = break during IO operation

139 = device does not acknowledge

144 = device done error

Note: a 128 must be POKE'd into location $303 (or 771)

in order to tell a Duplicator 1050 to write a bad

sector.

$30A (or 778) and $30B (or 779) are DAUX/1 and DAUX/2, respectively.  These are the locations in the DCB that tell the disk drive what sectors are to be used, written to, read from, status verified, etc.  Here are commands that a standard 1050 disk drive will acknowledge:

get sector               $52 (or 82)

put sector               $50 (or 80)

put sector with verify   $57 (or 87)

status request           $53 (or 83)

format disk              $21 (or 33)

This command information is stored at $302 (or 770) and the sector location information is stored at $30A (or 778) and $30B (or 779), as noted above.

You can refer to the Test Menu option, on the master disk, to see how sector information is passed by these locations.

If you have a Happy 1050, you have a few more calls you can access from the DAUX:

reset controller   POKE $30A,24  POKE $30B,0

forced protect     POKE $30A,24  POKE $30B,16

forced write       POKE $30A,24  POKE $30B,8

initialize drive   POKE $30A,3   POKE $30B,0

un-Happy mode      POKE $30A,224 POKE $30B,224

fast write         POKE $30A,32  POKE $30B,0

There are two more Happy 1050 SIO calls: "change drive number" and "change motor-off delay time".  You can refer to the SIO table (in THE HAPPY SECTION) to see how they work.  Also remember that a 72 must be POKE'd into $302 to tell the Happy 1050 to actually carry out the command you specified.  Also remember that after you have sent the de-program code to the Happy 1050 it will not accept any more Happy SIO code; it has become, for all practical purposes, a standard 1050 disk drive, except it can still read and write true double density.

1050 DUPLICATOR DAUX

As before, you must POKE the proper DCOMAND into location $302 (or 770), in order to tell a Duplicator 1050 what to do.

de-program Duplicator  POKE $30A,3 $30B,0

re-program Duplicator  POKE $30A,0 $30B,0

To write bad sectors, just POKE the desired sector number in the DAUX and POKE $302,118 and $303,128.  Now, POKE your sector number in $30A and $30B, then JSR $E459 and the Duplicator 1050 will write a bad sector in single or 1050 (enhanced) density.  But, please note that there are some sectors that the duplicator cannot write to!  The Duplicator cannot write a bad sector to sector 1 or 720.  It also appears that the last sector in any track cannot be written "bad".  Also note that the 1050 Duplicator cannot be hidden from a status call no matter what kind of code has been sent through the SIO to the drive; it will always tell you that it is a Duplicator.

The program SIO WORK FILE, on the master disk, was written so you could experiment with the DCB and be able to ask your drive (or any other peripherals) "questions" and have them answer without being DOS dependent.

Using this program is easy: just press the letter beside the mnemonic you wish to change and type your number in and press [RETURN].  When you have all your numbers entered in, just press [RETURN] a second time and the program will try to do the command you asked it to do and return information to you.  The SIO Table (found in THE HAPPY SECTION) was included for your use with this work file.  One command which is not shown on the screen is the PRINT command.  If you press the letter P, the program will dump the entire screen to the printer (which is nice if you want a hard copy of some of your work session).  But, you should remove the master disk from the drive, first.  You could make a mistake using this file, with disastrous results to the disk.  It is strongly suggested that you put a blank formatted disk in the drive until you get used to the SIO WORK FILE.

III. PROGRAM PROTECTION TECHNIQUES

In recognition of all the people in this world who just have to pirate other's software, it has become quite necessary to provide maximum protection for authors' programs.  Even programs that are written for the public domain need some kind of protection to prevent people from ripping off other's ideas.  Authors that have Happy 1050's and Duplicator 1050's can use these disk drives to protect their programs with the information from this disk and booklet.  At the present time the best way of protecting a program is in 1050 (enhanced) density.  There is no fast way for anybody to cut a quick backup and if your program has "bad sector calls" in it, it is next to impossible to copy.  The next way of providing maximum protection is to prevent the program from running on hybrid (modified) drives.  You can prevent programs from running on a Duplicator drive by doing an SIO call and "asking" the Duplicator if it is "there" (it always blows the whistle on itself!) and, if it is, then you can write some code to "crash" the program in a creative way.  The same holds true with the Happy 1050 except the Happy can be de-programmed into thinking (and saying) it is a standard disk drive.  However, we have found that changing the drive number in the AUTORUN.SYS file works great.  What happens is: if a person tries to change disks to fool the computer, then the drive automatically changes its number and then the program can be made to crash in some creative way.

By the time some "hacker types" get around to reading this book, they will probably have already found a second disk directory on the master disk and will have tried to copy the files from it.

But, this disk is a little more complex than that!  All the pertinent information is hidden in machine code, even though the programs are written in BASIC.  The way this works is really quite simple: the AUTORUN.SYS file stores information in free RAM and then the BASIC program does a USR call to free ram. If the machine code kernel is not present, then the program crashes!

Here is a machine language program that checks for the presence of a Happy 1050 drive:

LDA#$31

STA$300

LDA#$1   (or the drive number you want)

STA$301

LDA#$48  to call the Happy 1050

STA$302

LDA#$0   to clear STATUS

STA$303

LDA#$0   to clear DAUX/1

STA$30A

LDA#$0   to clear DAUX/2

STA#30B

JSR$E459 to execute an SIO call

LDA$303  to store the call in free RAM

STA$600  STATUS stored in PAGE 6

RTS      to go back to regular program

This program can be located anywhere in memory.  To run the program, do an INIT when (binary) saving the program (this will allow you to append an AUTORUN.SYS file).  Once you have (binary) saved this program, using an assembler/editor or Omnimon XL, you can append your AUTORUN.SYS file to it and, in many parts of your program, you can call to it.  If at any time you get a STATUS 1 from PEEKing into PAGE 6, then you can crash the Happy-copied program.

In BASIC, there are a lot of things you can do to look for a Happy or a Duplicator. Some REM statements in our BASIC programs will be longer than standard line length.

Here is a little program for checking for a Happy:

10 GRAPHICS 0:SETCOLOR 2,9,1

20 POKE 1536,104:POKE 1537,32:POKE 1538,89:POKE

1539,228:POKE 1540,96: REM THIS IS THE PART OF THE

PROGRAM YOUR USR CALL GOES TO TO EXCUTE THE

MACHINE CODE YOU POKE INTO THE DCB

30 RESTORE 30000:FOR V=768 TO 780:READ C:POKE

V,C:NEXT V: REM THIS IS THE PART OF THE PROGRAM

THAT STORES THE INFORMATION IN THE DCB

40 X=USR(1536): REM THIS IS WHERE THE CALL GOES TO

YOUR MACHINE STRING TO EXECUTE THE SIO CALL

50 DRIVE=PEEK(771)

60 IF DRIVE=1 THEN X=USR(58481):REM THIS WILL CRASH

THE PROGRAM IF A HAPPY IS PRESENT, IT WILL GO INTO

SELF TEST

30000 DATA 49,1,72,0,0,0,0,0,0,0,0

Another neat way of protecting your BASIC program is to POKE "out" your [BREAK] and [RESET] keys.  Here are some POKE's that will do just that:

POKE 16,112: REM DISABLES THE BREAK KEY

POKE 53774,112: REM DISABLES THE BREAK KEY

POKE 580,1: REM THIS WILL CAUSE A COLD START IF THE RESET KEY IS PRESSED

POKE 9,255: REM THIS WILL CAUSE A SYSTEM LOCKUP IF THE RESET KEY IS PRESSED

Please note that these POKE's must be re-installed after each GRAPHIC 0.  So, save your program each time you add new POKE's and then run it and see if even you (the author) can break into it!  Also note that you will need some TRAP statements in the program to prevent BREAK's during I/O functions.  Here is a little program that can be used to stop someone from breaking into a program during an I/O function:

10  DIM A$(13)

20  TRAP 100

30  PRINT " ENTER FULL PATHNAME OF BASIC FILE":PRINT

" YOU WISH TO LOAD"

40  INPUT A$: LOAD A$

50  END

100 X=USR(58481):REM THIS WILL CAUSE THE COMPUTER TO

GO INTO SELF TEST IF THERE IS AN ERROR ON LOADING

THE FILE

Here is a list of a few useful USR calls that could be inserted in line number 100:

X=USR(58481) (this will cause the computer to go into

the self-test mode)

X=USR(49808) (this will cause the computer to execute

a warm start)

X=USR(49834) (this will tell the computer to execute

a RESET)

X=USR(49864) (this is the call to do a cold start)

PROTECTING A PROGRAM AGAINST A "LIST"

Here we have two different methods of protecting against a LIST; the first one is:

POKE PEEK(138)+256*PEEK(139)+2,0:SAVE"D:PROGRAM"

After you have de-bugged your new program and checked it out, load it into the editor and type the above line in without a line number, and press [RETURN].  Don't forget to save a copy of your program without this line in it so that your program can be LIST'ed for your own use.  This method is not foolproof, as there are several programs on the market which will restore the file so that it can be viewed.

But, we have another way of scrambling BASIC that will make things more difficult.  Type these two lines in (again, without line numbers) just like they are shown, instead of the previous line:

FOR VARI=PEEK(130)+PEEK(131)*256 TO PEEK (132) + PEEK (133)*256:POKE VARI,155:NEXT VARI "PRESS RETURN"

POKE PEEK(138)+PEEK(139)*256 + 2,0:SAVE"D1:PGMNAME"  "PRESS RETURN"

With this operation done, you can LIST your program, but, all variables will be replaced with the [RETURN] key character.  This makes un-authorized tracing through your LIST'ed program (to make alterations) kinda' like going through a rat's maze!

IV. THE HAPPY SECTION

Well, hello all you fellow Happy 1050 drive owners. Here are a couple of programs written for the Happy and ICD Sparta DOS.  One is a "file maker" and the other is a menu by which you can control your Happy drive.  The "maker" will create a file by which you can direct your Happy 1050 to do what you want from a Sparta DOS STARTUP.BAT file.  This 'MKEHAPPY.COM' program assembles a binary file in memory and then writes it to the file name D1:COLORHPY.COM.  You can "chain in" any desired number of batch commands simply by creating them and then saving them.  MKEHAPPY.COM will automatically append them to your original D1:COLORHPY.COM file (under SPARTA DOS 3.2C or later). Optionally, once this file is written, it can be transferred/re-named for use under any DOS. Additionally, any binary-load file (including compiled run-time BASIC) can be appended so that the Happy options boot first and then auto-load your (appended) program!

For example (a Sparta DOS 3.2C command to append files together):

D1:COPY SUBDIR>FILENAME.EXT D1:COLORHPY.COM/A

(You'll need to UNLOCK the file, before appending, as MKEHAPPY.COM automatically LOCK's the file at creation).

The file HPYMENU.COM will allow you to do most all SPARTA DOS functions and all the Happy 1050 drive options, including changing the drive # beyond 4. This is an option which is included in the DRIVE OPTIONS menu (which is kinda' nice if you have a hard drive which is numbered low!).

If you have Omnimon, you can see what the program is doing by viewing the registers of the Device Control Block ($300 to $30A)!  Both of these programs were written for the use of the Happy 1050 disk drive and SPARTA DOS 3.2C or later.  These programs also use "free" RAM that is un-used by any of the computer's hardware (that we know of), so there should be no conflict.  The onboard BASIC, however, must be dis-abled before you attempt to run these programs or you will get a lovely sound which is advising you that your computer is now totally locked-up and you'll need to re-boot.  There are two files that were written for use in ATARI DOS 2.0 or 2.5; they are: 25MENU.COM and 25HAPPY.COM.  25MENU.COM will do all the Happy 1050 drive options and 25HAPPY.COM will make true binary files that can be appended (using DOS OPTION C: "COPY") to a file with /A on the end.  You can then append DUP.SYS (DUP.SYS must be the last file you append, if you're going this route) and then erase the old DUP.SYS file and RENAME the new file you just created to DUP.SYS.  And there you have it: a DOS DUP that will do any Happy drive option when you boot that disk.  Also, you can append any other binary files to the end of the file produced by 25HAPPY.COM, so that when you boot that 25HAPPY.COM-generated file, it will do your options.  Both programs also use the $300 through $30B location (which is the SIO block) in the computer.  25MENU.COM does the same thing that software houses do to detect the Happy 1050, through the SIO block, and it also does drive option's through the SIO block.  The SIO block ($300 to $30B) is the "workhorse" area between the computer and the disk drive through which goes the FORMAT command as well as all other communications between the computer and the drive.

If you don't have a Happy 1050, these two programs will load and "run", but not actually do anything for you.

A PERSONAL WORD ABOUT 1050 DRIVE ENHANCEMENTS

We have owned the Happy 1050, the ICD U.S. Doubler and the Duplicator 1050, and we personally believe the Happy 1050 is the best performer.  It just doesn't seem to crash and we have found that it allows you to back- up almost any of your precious master disks.  We feel the Happy's are the best drives we have ever used on a BBS.  We have been less than satisfied with the performance of the Duplicator 1050 upgrade.  If you have a Duplicator 1050 you may wish to either remove it from your drive or purchase the Happy "Cheer-up" up-grade, and that will stop the lock-ups that people have experienced with the Duplicator 1050.  Apparently, the Duplicator's "high speed handler" can lead to "crashed" disks.  You may have had this frustrating experience.  We hereby extend our thanks to Rich Adams for the "Cheer-Up" upgrade!  It's nice to have a true "smart" programmable drive.  We have not discussed the ICD U.S. Doubler (in this vein) because it is not one of the drives designed for making back-up copies of master disks, but is simply a true double density 1050 compatible drive.  The foregoing are, of course, the author's personal opinions, but we believe they are shared by other Atari users who have run the gamut of available disk drive upgrades.  That ends the personal "soap-boxing".

THE SIO CALLS & TECHNICAL NOTES

(a small portion of the SIO block)

(read the numbers vertically)

PEEK/POKE locations

_________________________

|7|7|7|7|7|7|7|7|7|7|7|7|

|6|6|7|7|7|7|7|7|7|7|7|7|  (decimal)

|8|9|0|1|2|3|4|5|6|7|8|9|

=========================

|$|$|$|$|$|$|$|$|$|$|$|$|

|3|3|3|3|3|3|3|3|3|3|3|3|

|0|0|0|0|0|0|0|0|0|0|0|0|  (hexadecimal)

|0|1|2|3|4|5|6|7|8|9|A|B|

|=======================|

|   RESET CONTROLLER    |

|_______________________|

|4|D|7|0|0|0|0|0|0|0|2|0|  (See Note for D# column)

|9|#|2| | | | | | | |4| |

|=======================|

|  SET FORCED PROTECT   |

|_______________________|

|4|D|7|0|0|0|0|0|0|0|2|1|

|9|#|2| | | | | | | |4|6|

|=======================|

|   SET FORCED WRITE    |

|_______________________|

|4|D|7|0|0|0|0|0|0|0|2|8|

|9|#|2| | | | | | | |4| |

|=======================|

|   INITIALIZE DRIVE    |

|_______________________|

|4|D|7|0|0|0|0|0|0|0|3|0|

|9|#|2| | | | | | | | | |

|=======================|

PEEK/POKE locations

_________________________

|7|7|7|7|7|7|7|7|7|7|7|7|

|6|6|7|7|7|7|7|7|7|7|7|7|  (decimal)

|8|9|0|1|2|3|4|5|6|7|8|9|

=========================

|$|$|$|$|$|$|$|$|$|$|$|$|

|3|3|3|3|3|3|3|3|3|3|3|3|

|0|0|0|0|0|0|0|0|0|0|0|0|  (hexadecimal)

|0|1|2|3|4|5|6|7|8|9|A|B|

|=======================|

|  SET UN-HAPPY MODE    |

|_______________________|

|4|D|7|0|0|0|0|0|0|0|2|2|  (See Note for D# column)

|9|#|2| | | | | | | |2|2|

| | | | | | | | | | |4|4|

|=======================|

|    SET FAST WRITE     |

|_______________________|

|4|D|7|0|0|0|0|0|0|0|3|0|

|9|#|2| | | | | | | |2| |

|=======================|

|  CHANGE DRIVE NUMBER  |  (See Note X)

|_______________________|

|4|D|7|0|0|0|0|0|0|0|1|X|

|9|#|2| | | | | | | | | |

|=======================|

|CHANGE MOTOR-OFF DELAY |  (See Note Y)

|_______________________|

|4|D|7|0|0|0|0|0|0|0|2|Y|

|9|#|2| | | | | | | | | |

|=======================|

Note: D# is the regular physical drive number

you wish to send your command to.

D1:=1    D5:=5

D2:=2    D6:=6

D3:=3    D7:=7

D4:=4    D8:=8

Note X:   This is where the new drive #

decimal code equals:

D1:=177  D5:=181

D2:=178  D6:=182

D3:=179  D7:=183

D4:=180  D8:=184

Special Note X: You can program the Happy 1050

past D8; the decimal codes are:

D9: =185

D10:=186

D11:=187, etc.

... to D79:=255

Note Y:   This is where the new motor-off

delay time decimal code equals:

255=SHORTEST OFF DELAY

155=4-SECOND OFF DELAY

5=LONGEST OFF DELAY

SPECIAL NOTE ON DRIVE #: Once you have programmed a Happy 1050 past D8: and have opened the drive gate and the change drive command has taken place, all Happy 1050 drive options can be enabled even though there is no DOS around that supports more than eight drives.

Since there is no DOS available (that we know of) to support drives greater than D8:; the best method is to write a handler using the DCB in an 'R' handler mode. Another specifier could be used; however, since R2: isn't used for anything that we know of, writing to the drive through this port on your interface could be possible.  Of course if you're good at writing handlers, you can give it any device specifier you want.

Here is a short program (utilizing a fast machine language sub-routine) that checks all drives to see if any are Happy 1050's, and, if so, sets the FASTWRITE mode:

0 GRAPHICS 0:SETCOLOR 2,9,1

10 POKE 82,0:FOR JKL=1 TO 8

20 RESTORE 30000:POKE 768,49:POKE 769,JKL:FOR V=770 TO      780:READ C:POKE V,C:NEXT V: REM THIS LINE POKES

LOCATIONS 770 TO 780 WITH THE DATA STATEMENT AT LINE

30000

30 RESTORE 30010:FOR M=1536 TO 1540:READ D:POKE

M,D:NEXT M: REM THIS LINE POKES LOCATIONS 1536 TO

1540 WITH THE DATA STATEMENT AT LINE 30010

40 X=USR(1536): REM THIS LINE DOES THE JUMP TO

SUBROUTINE AND RETURNS THE PROGRAM TO THE

DEPARTURE POINT

50 DRIVE=PEEK(771): REM THIS LOCATION CHECKS THE DRIVE

STATUS OF THE DCB AND MAKES DRIVE VARIABLE=PEEK

60 IF DRIVE=1 THEN PRINT "DRIVE #",JKL,"HAPPY FASTWRITE

SET": REM CHECKS TO SEE IF FASTWRITE HAS BEEN

SUCCESSFULLY EXECUTED

70 IF DRIVE=139 THEN PRINT "DRIVE #",JKL,"STANDARD

1050": REM CHECKS TO SEE IF A DRIVE EXISTS BUT

DOESN'T UNDERSTAND FASTWRITE COMMAND

80 IF DRIVE=138 THEN PRINT "DRIVE #",JKL,"NO DRIVE

PRESENT": REM THIS MEANS THERE IS NO DRIVE PRESENT

AT ALL

90 NEXT JKL:END

30000 DATA 72,0,0,0,0,0,32,0,0:REM FASTWRITE DATA

30010 DATA 104,32,89,228,96:REM JSR$E459 AND RTS TO

PROGRAM ACCESS THROUGH X=USR(1536)

Any of the SIO Block codes can be used in line 30000, but remember to drop the first two codes (locations 768 & 769) as the program will automatically POKE these two locations in line 20 (POKE 768,49 POKE 769,JKL).  Only a Happy 1050 will return a status code 1.  All other drives (and de-programmed Happy's) will return a status of 139, and a status of 138 if there is no drive at all.  Also note that the status of a RAMdisk is not returned here (by this program, although it can be determined with the right code) but the status of an ICD MIO will be returned as a standard 1050 disk drive. You can change line 10 to POKE 82,0:JKL= drive # and change the line 60 PRINT statement to post your own message. Then, change line 90 to END.  Other SIO commands (from our listed SIO block or COMPUTE's Mapping the Atari or special codes used by other Atari-compatible drives) can be used.

The Status code for the HAPPY 1050 is: 49,drive #,72,0,0,0,0,0,0,0,0,0 and then JSR$E459 then PEEK 770 and it will return a 1.  If a 138 or 139 are returned then the drive isn't there or it is in UN-HAPPY mode or is another type of disk drive.  Also note that you can turn your controller off or on and change the drive number past 4, and if you set the Happy 1050 in UN-HAPPY mode it will remember its new drive number as well as the controller status.  However, you will no longer be able to do any drive options.  Also a de-programmed Happy 1050 can still read true double density.  But after the UN-HAPPY mode is set, the status will come back as a standard 1050.  The 1050 Duplicator status call is: 49,drive#,98,0,0,0,0,0,

0,0,0,0 and then do a JSR$E459 and then PEEK 771 and if it is a 1050 DUPLICATOR it will return a 1.  The 1 will be returned whether the Duplicator is in de-programmed mode or not!  The Duplicator 1050 will also run all three densities (single, 1050 enhanced, double) in the de-programmed mode.  There is a BASIC program call that does all of these things.  The Duplicator 1050 will format ultra-speed sector skew but it often gets confused after the format.  The Happy 1050 will not format in ultra-speed sector skew, but it is not needed, because the Happy will come up running at top speed and all you need to do is set the FASTWRITE mode on the drive, and you won't get scrambled disks like you sometimes get with the Duplicator.  We are sure that by the time you have read this far, you are assuming that we are master hackers.  We don't classify ourselves as such, but, to find this SIO code in the Device Control Block took a lot of hours of work to gain what was necessary to make these programs run.  No one gave them to us, nor did we dis-assemble any disk or chip for this code.  As you may have guessed, a great deal of time was spent, using Omnimon, viewing the SIO block to bring you the SIO table.

We hope that someday Happy Computers will soon make available to the owners of Happy 1050 drives a good set of technical notes, but, we do understand why they haven't, because software companies would probably write routines in their programs to drive the Happy 1050 "nuts".  We feel that the Happy 1050 enhancement is far ahead of anything any other company has come out with, that we know of, to this day.  We have thoroughly enjoyed working with it and we hope you will, too! (No, we don't get a commission for every Happy that's sold!)

VI. TYPE-IN BASIC PROGRAMS

Here are some type-in BASIC programs for all of you who like to write your own programs.  These programs can be modified for your own use.  You will note that some REM's are longer than standard line length.

Here is a small program that makes the Duplicator write "bad" sectors:

0   GRAPHICS 0

1   BAD=1:DIM Q$(1):POKE 752,1:POKE 82,0

10  SETCOLOR 2,9,0

13  ? :? :? ,"THE BAD SECTOR MAKER":? ,"FOR THE

DUPLICATOR"

15  POKE 1536,104:POKE 1537,32:POKE 1538,89:POKE

1539,228:POKE 1540,96

16  POKE 768,49:POKE 769,2:POKE 770,98:POKE 778,3:POKE

779,0:FOR T=771 TO 777:POKE T,0:NEXT T:X=USR(1536):

REM CHANGE POKE 769,2 TO YOUR CORRECT DUPLICATOR

DRIVE NUMBER

17  IF PEEK(771)>1 THEN ? :PRINT ," RESET DUPLICATOR

1050":CLOSE #1:OPEN #1,4,0,"K:":GET #1,K:CLOSE

#1:GOTO 10

20  POKE 768,49

30  POKE 769,2:REM THIS IS THE POKE FOR DRIVE NUMBER.

IT IS SET TO 2 BUT CAN BE CHANGED TO 1 THROUGH 4.

40  POKE 770,119

50  POKE 771,128:Z=1

60  POKE 772,31

70  POKE 773,187

80  POKE 774,49

90  POKE 775,0

100 POKE 776,128

110 POKE 777,0

120 REM THE NEXT TWO LINES CALCULATE THE SECTOR NUMBER

AND POKE IT INTO THE DCB

130 ? :? :? ,"  SECTOR TO TRASH":INPUT BAD

140 K=BAD-(INT(BAD/256)*256):L=INT(BAD/256):POKE

778,K:POKE 779,L

145 REM THIS LINE DOES THE JUMP TO SUBROUTINE AND

CHECKS THE STATUS CODE

150 X=USR(1536):Z=Z+1:ST=PEEK(771):PRINT BAD,ST

170 BAD=BAD+1:IF BAD<1041 THEN GOTO 20

180 END

This next one searches a disk for "bad" sectors and tells you just where they are (this works on any 1050-based drive):

0   GRAPHICS 0

1   BAD=1:DIM Q$(1):POKE 752,1:POKE 82,0

10  SETCOLOR 2,9,0

12  ? ,"LOOKING FOR BAD SECTOR"

15  POKE 1536,104:POKE 1537,32:POKE 1538,89:POKE

1539,228:POKE 1540,96

20  POKE 768,49

30  POKE 769,1:REM THIS IS WHERE YOU PUT YOUR DRIVE

NUMBER. IT IS SET ON 1 BUT CAN BE CHANGED TO 1

TO 8

40  POKE 770,82

50  POKE 771,0:Z=1

60  POKE 772,31

70  POKE 773,187

80  POKE 774,49

90  POKE 775,0

100 POKE 776,128

110 POKE 777,0

130 ? "      LOOKING AT SECTOR #";BAD;" ON

D";PEEK(769);":"

140 K=BAD-(INT(BAD/256)*256):L=INT(BAD/256):POKE

778,K:POKE 779,L

150 X=USR(1536):Z=Z+1:ST=PEEK(771):IF ST>1 THEN PRINT        BAD,"IS A BAD SECTOR"

170 BAD=BAD+1:IF BAD<721 THEN GOTO 20

180 END

190 REM CHANGE LINE 170 TO "BAD=BAD+1:IF BAD<1041 THEN       GOTO 20" FOR 1050 (ENHANCED) DENSITY

Next is a program to find out the status of all devices on line:

0   GRAPHICS 0:POKE 752,1:POKE 82,2:SETCOLOR 2,9,1:? ""

5    POKE 1536,104:POKE 1537,32:POKE 1538,89:POKE

1539,228:POKE 1540,96

10   POKE 768,64:POKE 769,1:POKE 770,83:FOR S=771 TO          780:POKE S,0:NEXT S

20   X=USR(1536):PRN=PEEK(771)

30   IF PRN=1 THEN PRINT "          PRINTER IS PRESENT"

40   IF PRN=138 THEN PRINT "          NO PRINTER ON

LINE"

50   POKE 768,80:POKE 769,1:POKE 770,83:FOR S=771 TO

780:POKE S,0:NEXT S

60   X=USR(1536):PRN=PEEK(771)

70   IF PRN=1 OR PRN=139 THEN PRINT "          MODEM IS

PRESENT"

80   IF PRN=138 THEN PRINT "          NO MODEM ON LINE"

200  FOR JKL=1 TO 8

241  POKE 768,49:POKE 769,JKL:POKE 770,72:FOR S=771 TO

780:POKE S,0:NEXT S

243  X=USR(1536)

451  DRIVE=PEEK(771)

460  IF DRIVE=1 THEN PRINT "DRIVE #";JKL;"  HAPPY 1050

PRESENT":GOTO 1000

470  IF DRIVE=139 THEN GOTO 500

480  IF DRIVE=138 THEN PRINT "DRIVE #";JKL;"  NO DRIVE

PRESENT"

490  GOTO 1000

500  POKE 768,49:POKE 769,JKL:POKE 770,98:POKE

771,1:FOR S=772 TO 780:POKE S,0:NEXT S

520  X=USR(1536)

530  DRIVE=PEEK(771)

540  IF DRIVE=1 THEN PRINT "DRIVE #";JKL;"  DUPLICATOR

1050 PRESENT":GOTO 1000

550  IF DRIVE=139 THEN GOTO 600

600  POKE 768,49:POKE 769,JKL:POKE 770,79:POKE

771,1:FOR S=772 TO 780:POKE S,0:NEXT S

620  X=USR(1536)

630  DRIVE=PEEK(771)

640  IF DRIVE=1 THEN PRINT "DRIVE #";JKL;"  US DOUBLER

OR MIO":GOTO 1000

650  IF DRIVE=139 THEN GOTO 700

700  POKE 768,49:POKE 769,JKL:POKE 770,83:POKE

771,1:FOR S=772 TO 780:POKE S,0:NEXT S

720  X=USR(1536)

730  DRIVE=PEEK(771)

740  IF DRIVE=1 THEN PRINT "DRIVE #";JKL;"  STANDARD

1050 PRESENT":GOTO 1000

1000 NEXT JKL:POKE 752,0

2000 POKE 12,96:POKE 13,155

2010 END

30000 REM THIS PROGRAM WAS WRITTEN BY MR. HARDWARE

Please note that all of these programs can be changed to do different things by using the decimal SIO Block codes provided (or other peripheral vendors' SIO codes that you may happen to come across).  Have fun!

VII. PRODUCT/COMPANY ACKNOWLEDGEMENTS

U.S. Doubler

Sparta DOS

MIO

are all products of:

ICD, Inc.

1220 Rock Street

Suite 310

Rockford, IL 61101-1437

(815) 229-2999

Happy 1050

Happy Controller Board

Cheerup Upgrade

are all products of:

Happy Computers, Inc.

P.O. Box 1268

Morgan Hill, CA 95037

(408) 779-3830

Omnimon & Omnimon-XL

are products of:

Newell Industries

602 E. Highway 78

Wylie, TX 75098

(214) 442-6612

1050 Duplicator

is a product of:

Duplicating Technologies, Inc.

99 Jericho Turnpike

Suite 302-A

Jericho, NY 11753

Atari DOS 2.0 & 2.5

Atari 1050 Disk Drive

are products of:

Atari Corp.

Sunnyvale, CA 94086

The 1050 Detective

is a product of:

Del Com Services

803 Eisenhower Drive

Augusta, GA 30904

(404) 738-2139

Note: all products mentioned are each copyrighted by and/or registered trademarks of each of the respectively listed companies.

Del Com Services is not affiliated with

any of the other above-listed companies.