HARDWARE DESCRIPTION OF THE EXPANSION BOX

The box consists of two main elements, the Buffer Box and the

Expansion Box Chassis.  This document will concern itself with the

hardware inside the Expansion Box Chassis as the Buffer Box is a simple

signal buffer unit.  The Expansion Chassis contains a backplane, for

option connection, a power supply and I/O channels.

The I/O channels consist of two serial RS232C channels and one

Centronics type parallel output port.  These functions are created by

two 68B50 ACIAs, one 68B21 PIA, and a BR1943 baud rate generator.

Figure 1 shows the locations of the devices in the memory map.  All the

devices respond to Parallel Bus Device #6.

-----------------------------------

| Location    | Device            |

-----------------------------------

| D100H       | 68B50  Channel A  |

| D104H       | 68B50  Channel B  |

| D108H       | 68B21             |

| D110H       | 1943              |

-----------------------------------

Figure 1

Parallel Channel Description

The Parallel channel uses a 68B21 PIA.  There are 11 signals

implemented in the Centronics Interface.  These signals are D0-D7, Data

Strobe, Busy, and Fault.  The data lines use port A in the PIA and are

output only.  Data Strobe is an output line from port B bit 4 which is

used to strobe data to the printer.  This signal should be active after

data is stable and the printer is not busy.  Busy is an input line of

port B bit 5.  This input controls the flow of data to the printer; it

should be checked before next data transmission.  Fault is an input of

port B bit 6.  This input indicates if the printer had a fault (ie.

paper out) and should always be checked.  Figure 2 shows the port

assignments for the PIA.  The register types and control bits are the

same as the standard Atari PIA used in current computers.

---------------------------------------------

| Port  | Direction   | Description         |

---------------------------------------------

| A0    | Output      | Data Bit 0          |

| A1    | Output      | Data Bit 1          |

| A2    | Output      | Data Bit 2          |

| A3    | Output      | Data Bit 3          |

| A4    | Output      | Data Bit 4          |

| A5    | Output      | Data Bit 5          |

| A6    | Output      | Data Bit 6          |

| A7    | Output      | Data Bit 7          |

---------------------------------------------

| B0    | Input       | DSR' Channel A      |

| B1    | Output      | DTR' Channel A      |

| B2    | Output      | DTR' Channel B      |

| B3    | Input       | DSR' Channel B      |

| B4    | Output      | Data Strobe'        |

| B5    | Input       | Busy'               |

| B6    | Input       | Fault'              |

| B7    | Not Used    | Not Used            |

---------------------------------------------

Figure 2

Baud Rate Selection

The baud rate for the two serial channels is created by a BR1943 baud

rate generator.  This device creates two different baud rates using one

write only register.  The upper four bits of this register controls the

baud rate for Channel B, the lower four bits control Channel A.  Figure

3 shows the four bit combinations for the 16 different baud rates.

Sel Value           Baud Rate

-----------------------------

|   0000      |    50       |

|   0001      |    75       |

|   0010      |    110      |

|   0011      |    134.5    |

|   0100      |    150      |

|   0101      |    200      |

|   0110      |    300      |

|   0111      |    600      |

|   1000      |    1200     |

|   1001      |    1800     |

|   1010      |    2400     |

|   1011      |    3600     |

|   1100      |    4800     |

|   1101      |    7200     |

|   1110      |    9600     |

|   1111      |    19200    |

-----------------------------

Figure 3

Serial Channels

The two serial channels are a full RS-232C type.  These channels use

the baud rates from above, handshake internal to the ACIA as well as

extra handshake lines from the PIA.  The ACIA operation is described

below as well as register locations.  The software should provide

correct operation of the extra handshake lines for full modem control.

ACIA REGISTERS DESCRIPTION

The ACIA has internal registers on the chip that are used for the

status, control, receiving, and transmitting of data.  The content of

each of the registers is summarized in Table 1.

-------------------------------------------------------------

|Data|Transmit|Receive |Control           |Status           |

|Bus |Data    |Data    |Register          |Register         |

| #  |Register|Register|                  |                 |

-------------------------------------------------------------

| 0  | Data 0 | Data 0 |C D Sel 1 CRO     |RDRF             |

| 1  | Data 1 | Data 1 |C D Sel 2 CR1     |TDRE             |

| 2  | Data 2 | Data 2 |Word Sel 1 CR2    |DCD'             |

| 3  | Data 3 | Data 3 |Word Sel 2 CR3    |CTS'             |

| 4  | Data 4 | Data 4 |Word Sel 3 CR4    |Frame Error      |

| 5  | Data 5 | Data 5 |Transmit Con 1 CR5|Receiver Overrun |

| 6  | Data 6 | Data 6 |Transmit Con 2 CR6|Parity Error     |

| 7  | Data 7 | Data 7 |Rec Int En CR7    |Interrupt Request|

-------------------------------------------------------------

Table 1

Definition of ACIA Register Contents

TRANSMIT DATA REGISTER (TDR)

Data is written in the Transmit Data Register during the negative

transition of the enable (E) when the ACIA has been addressed and RS .

R/W is selected.  Writing data into the register causes the Transmit

Data Register Empty bit in the Status Register to go low.  Data can

then be transmitted.  If the transmitter is idling and no character is

being transmitted, then the transfer will take place within one bit

time of the trailing edge of the Write command.  If a character is

being transmitted, the new data character will commence as soon as the

previous character is complete.  The transfer of data causes the

Transmit Data Register Empty (TDRE) bit to indicate empty.

RECEIVE DATA REGISTER (RDR)

Data is automatically transferred to the empty Receive Data Register

(RDR) from the receiver deserializer (a shift register) upon receiving

a complete character.  This event causes the Receive Data Register Full

bit (RDRF) in the status buffer to go high (full).  Data may then be

read through the bus by addressing the ACIA and selecting the Receive

Data Register with RS and R/W high when the ACIA is enabled.  The

non-destructive read cycle causes the RDRF bit to be cleared to empty

although the data is retained in the RDR.  The status is maintained by

RDRF as to whether or not the data is current.  When the Receive Data

Register is full, the automatic transfer of data from the Receiver

Shift Register to the Data Register is inhibited and the RDR contents

remain valid with its current status stored in the Status register.

CONTROL REGISTER

The ACIA Control Register consists of eight bits of write-only buffer

that are selected when RS and R/W are low.  This register controls the

function of the receiver, transmitter, interrupt enables, and the

Request-to-Send peripheral/modem control output.

Counter Divide Select Bits (CR0 and CR1) - The counter Divide Select

Bits (CR0 and CR1) determine the divide ratios utilized in both the

transmitter and receiver sections of the ACIA.  Additionally, these

bits are used to provide a master reset for the ACIA which clears the

Status Register (except for external conditions on CTS and DCD) and

initializes both the receiver and transmitter.  Master reset does not

affect other Control Register bits.  Note that after power-on or a

power fail/restart, these bits must be set high to reset the ACIA.

After resetting, the clock divide ratio may be selected.  These counter

select bits provide for the following clock divide ratios:

-----------------------------

| CR1 | CR0 | Function      |

-----------------------------

|  0  |  0  | Divide by 1   |

|  0  |  1  | Divide by 16  |

|  1  |  0  | Divide by 64  |

|  1  |  1  | Master Reset  |

-----------------------------

Word Select Bits (CR2, CR3, and CR4) - The Word Select bits are used

to select word length, parity, and the number of stop bits.  The

encoding format is as follows:

----------------------------------------------

|CR4|CR3|CR2| Function                       |

----------------------------------------------

| 0 | 0 | 0 | 7 Bits Even Parity 2 Stop Bits |

| 0 | 0 | 1 | 7 Bits Odd Parity  2 Stop Bits |

| 0 | 1 | 0 | 7 Bits Even Parity 1 Stop Bit  |

| 0 | 1 | 1 | 7 Bits Odd Parity  1 Stop Bit  |

| 1 | 0 | 0 | 8 Bits 2 Stop Bits             |

| 1 | 0 | 1 | 8 Bits 1 Stop Bit              |

| 1 | 1 | 0 | 8 Bits Even Parity 1 Stop Bit  |

| 1 | 1 | 1 | 8 Bits Odd Parity  1 Stop Bit  |

----------------------------------------------

Word length, Parity Select, and Stop Bit changes are not buffered and

therefore become effective immediately.

Transmitter Control Bits (CR5 and CR6) - Two Transmitter Control bits

provide for the control of the interrupt from the Transmit data

Register Empty condition, the Request-to-Send (RTS) output, and the

transmission of a Break level (space).  The following encoding format

is used:

---------------------------------------

|CR6|CR5|Function                     |

---------------------------------------

| 0 | 0 |RTS'=LOW  Trans INT Disabled |

| 0 | 1 |RTS'=LOW  Trans INT Enabled  |

| 1 | 0 |RTS'=HIGH Trans INT Disabled |

| 1 | 1 |RTS'=LOW  Trans a BREAK CHAR |

---------------------------------------

Receive Interrupt Enable Bit (CR7) - The following interrupts will be

enabled by a high level in bit position 7 of the Control Register

(CR7):  Receive Data Register Full, Overrun, or a low to high

transition on the Data Carrier Detect (DCD) signal line.

STATUS REGISTER

Information on the status of the ACIA is available to the MPU by

reading the ACIA Status Register.  This read-only register is selected

when RS is low and R/W is high.  Information stored in this register

indicates the status of the Transmit Data Register, the Receive Data

Register and error logic, and the peripheral/modem status inputs of the

ACIA.

Receive Data Register Full (RDRF), Bit 0 - Receive Data Register Full

indicates that received data has been transferred to the Receive Data

Register.  RDRF is cleared after an MPU read of the Receive Data

Register or by a master reset.  The cleared or empty state indicates

that the contents of the Receive Data Register are not current.  Data

Carrier Detect being high also causes RDRF to indicate empty.

Transmit Data Register Empty (TDRE), Bit 1 - The Transmit Data

Register Empty bit being set high indicates that the Transmit Data

Register contents have been transferred and that new data may be

entered.  The low state indicates that the register is full and that

transmission of a new character has not begun since the last write data

command.

Data Carrier Detect (DCD), Bit 2 - The Data Carrier Detect bit will

be high when the DCD input from a modem has gone high to indicate that

a carrier is not present.  This bit going high causes an Interrupt

Request to be generated when the Receive Interrupt Enable is set.  It

remains high after the DCD input is returned low until cleared by first

reading the Status Register and then the Data Register or until a

master reset occurs.  If the DCD input remains high after read status

and read data or master reset has occured, the interrupt is cleared,

the DCD status bit remains high and will follow the DCD input.

Clear-to-Send (CTS), Bit 3 - The Clear-to-Send bit indicates the

state of the Clear-to-Send input from a modem.  A low CTS indicates

that there is a Clear-to-Send from the modem.  In the high state, the

Transmit Data Register Empty bit is inhibited and the Clear-to-Send

status bit will be high.  Master reset does not affect the

Clear-to-Send Status bit.

Framing Error (FE), Bit 4 - Framing error indicates that the received

character is improperly framed by a start and a stop bit and is

detected by the absence of the 1st stop bit.  This error indicates a

synchronization error, faulty transmission, or a break condition.  The

framing error flag is set or reset during the receive data transfer

time.  Therefore, this error indicator is present throughout the time

that the associated character is available.

Receiver Overrun (OVRN), Bit 5 - Overrun is an error flag that

indicates that one or more characters in the data stream were lost.

That is, a character or a number of characters were received but not

read from the Receive Data Register (RDR) prior to subsequent

characters being received.  The overrun condition begins at the

midpoint of the last bit of the second character received in succession

without a read of the RDR having occurred.  The Overrun does not occur

in the Status Register until the valid character prior to Overrun has

been read.  The RDRF bit remains set until the Overrun is reset.

Character synchronization is maintained during the Overrun condition.

The Overrun indication is reset after the reading of data from the

Receive Data Register or by a Master Reset.

Parity Error (PE), Bit 6 - The parity error flag indicates that the

number of highs (ones) in the character does not agree with the

preselected odd or even parity.  Odd parity is defined to be when the

total number of ones is odd.  The parity error indication will be

present as long as the data character is in the RDR.  If no parity is

selected, then both the transmitter parity generator output and the

receiver parity check results are inhibited.

Interrupt Request (IRQ), Bit 7 - The IRQ bit indicates the state of

the IRQ output.  Any interrupt condition with its applicable enable

will be indicated in this status bit.  Anytime the IRQ output is low

the IRQ bit will be high to indicate the interrupt or service request

status.  IRQ is cleared by a read operation to the Receive Data

Register or a write operation to the Transmit Data Register.

Expansion Box Specifications

7/27/83

1.0 Introduction

The Expansion Box will be a buffered extension of the Parallel Bus

Interface (P.B.I.) allowing system expansion and enhancement.  The bus

will enable the consumer to connect a variety of different devices to

the Parallel Bus Interface.  Some future devices are listed below.

1) Disk Drive Controller-*

2) Votrax-

3) Modem (300 and/or 1200)-*

4) Touch Tone Decoder-

5) EPROM Programmer-

6) B.S.R. Controller Interface-

7) Relay Switch Card-

8) IEEE 488 Control Card-*

9) Real Time Clock-

10) 80 Column Display-*

11) Printer Spooler/Buffer-

12) APPLE Card-*

13) VCS Adapter Card-*

14) A/D Converter Card-*

15) Infra Red Interface-*

16) Hard Disk Interface-*

17) Music Card-

18) Speech Recognition-*

19) Ram Disk-*

20) Battery Back-up CMOS RAM-

21) Corvus Interface-*

22) Ethernet Interface-*

* indicates a smart device.

1.1 Product Objectives

A. To provide future system expansion.

B. To provide multiple device access to the P.B.I.

C. To provide an interface standard to envoke interest from

peripheral manufacturers in developing P.B.I. Devices.

1.2 References

Sweet 16 Expansion Box Specification, A. Chopra

P.B.I. Specification, S. Miller

SURELY O.S. Rom Specification, S. Scheiman R. Nordin

Software Implementation of Parallel Devices, R. Nordin

2.0 Product Description

The Expansion Box will provide two RS-232C Serial Channels and one

Centronics type Parallel Output Port in addition to eight (8) expansion

slots.  The connection to the XL-Series computer is made through the

Parallel Bus Interface.  Devices that plug into the P.B.I. are called

Parallel Bus Devices (P.B.D.).  The Expansion Box is a P.B.D..  The

Interface in the Expansion Box to the expansion devices is called the

Expansion Box Interface (E.B.I.).  Modules that plug into the expansion

slots will be called Expansion Box Devices (E.B.D.).

2.1 Mechanical

2.1.1 Appearance

The Expansion Box should match the styling of the current XL product

line.  The Box should have all connections from devices, power, and bus

signals in the rear of the box.  The power switch and power indicator

light should be placed on the front panel.

2.2.2 Design Concepts

The Expansion Box should not require more than one square foot of

table space.  It should also be able to support a 13 inch color

television.  The top must be easily removable so the user can access

the option cards, but not be able to access the power supply or come in

contact with the Line Voltage.  The Box will support eight 9 by 5 inch

option cards (see figure 1).

------------------------------  -

|                            |  |

|                            |  |

|                            |  |

|                            |

|                            |  5

|                            |

|                            |  |

|                            |  |

|                            |  |

------------------       -----  -

|       |

---------

|1/2|

| <- 3.15 ->|

| <----------- 9 ----------> |

(all measurements are in inches)

Figure 1  P.B.D. Card Size

2.2 Electrical

2.2.1 Expansion Box Interface

The E.B.I. contains all the signals in the P.B.I. plus power for the

E.B.D..  The description of the E.B.I. in this document is similar to

that of the P.B.I. but note the differences in timing and loading.

2.2.1.1 Connector

The Expansion Box Interface signals will be accessible through a

50-pin connector.  This connector is an AMP type 67987-4.  It will mate

to a standard card edge with gold plated fingers at .100 inch centers.

--------

EXTSEL' |2    1| GND

A1      |4    3| A0

A3      |6    5| A2

A5      |8    7| A4

GND     |10   9| A6

A8      |12  11| A7

A10     |14  13| A9

A12     |16  15| A11

A14     |18  17| A13

A15     |20  19| GND

D1      |22  21| D0

D3      |24  23| D2

D5      |26  25| D4

D7      |28  27| D6

GND     |30  29| GND

GND     |32  31| B02

RST'    |34  33| +5 VDC

RDY     |36  35| IRQ'

EXTENB' |38  37| -12 VDC

REF'    |40  39| +12 VDC

GND     |42  41| CAS'

RAS'    |44  43| MPD'

LR/W'   |46  45| GND

+5 VDC  |48  47| +5 VDC

GND     |50  49| AUDIO

--------

Figure 2  Expansion Socket

2.2.1.2 Signals

Shown in Figure 2 is the pin-out of the connector looking into the

expansion box.  The signal pin numbers and signal names are described

below.

Pin 2   EXTSEL'   External Select (Input) ==> This open collector

line is generated internally by the Expansion Box Device (E.B.D.).

This signal should be active low whenever EXTENB is active and the

E.B.D. is selected and there is a valid E.B.D. address on the bus.

EXTSEL' causes a CAS' inhibit on the main board allowing a remapping

process.  Although an E.B.D. can be mapped in any VALID RAM location,

these devices should follow the ATARI guidelines for E.B.D. locations

so future ATARI devices can be used.  The drive device should be

capable of sinking 10mA.

Pin 3   A0   Address Line 0 (Output) ==> Address line 0 is a buffered

output from the microprocessor.

Pin 4   A1   Address Line 1.  Same as above.

Pin 5   A2   Address Line 2.  Same as above.

Pin 6   A3   Address Line 3.  Same as above.

Pin 7   A4   Address Line 4.  Same as above.

Pin 8   A5   Address Line 5.  Same as above.

Pin 9   A6   Address Line 6.  Same as above.

Pin 11  A7   Address Line 7.  Same as above.

Pin 12  A8   Address Line 8.  Same as above.

Pin 13  A9   Address Line 9.  Same as above.

Pin 14  A10  Address Line 10.  Same as above.

Pin 15  A11  Address Line 11.  Same as above.

Pin 16  A12  Address Line 12.  Same as above.

Pin 17  A13  Address Line 13.  Same as above.

Pin 18  A14  Address Line 14.  Same as above.

Pin 20  A15  Address Line 15.  Same as above.

Pin 21  D0   Data Line 0 (Bi-directional) ==> Data line 0 is a

buffered bi-directional data line.

Pin 22  D1   Data Line 1.  Same as above.

Pin 23  D2   Data Line 2.  Same as above.

Pin 24  D3   Data Line 3.  Same as above.

Pin 25  D4   Data Line 4.  Same as above.

Pin 26  D5   Data Line 5.  Same as above.

Pin 27  D6   Data Line 6.  same as above.

Pin 28  D7   Data Line 7.  Same as above.

Pin 31  B02   Buffered Phase 2 Clock (Output) ==> This clock output

line is a buffered phase 2 clock from the processor.

Pin 34  RST'   Reset (Output) ==> Reset is an active low signal which

occurs either on power-up or by depressing the reset key on the

computer.

Pin 35  IRQ'   Interrupt Request (Input) ==> This open collector line

creates an interrupt on the microprocessor.  The interrupt can then

invoke the handler ROM or other service routines for the E.B.D..  The

driving device should be capable of sinking 10mA.

Pin 36  RDY   Ready (Input) ==> This open collector input signal

allows the E.B.D. to halt the microprocessor ONLY during read cycles.

This will extend the read cycle for slow peripherals.  The driving

device should be capable of sinking 10mA.

Pin 38  EXTENB   External Decoder Enable (Output) ==> This output

goes high when there is a valid ram access.  Any E.B.D. can map during

a valid EXTENB but the E.B.D. should only map in according to ATARI

specified address locations.

Pin 40  REF'   Refresh (Output) ==> This output can be for refresh

timing on volatile memories connected to the E.B.I..

Pin 41  CAS'   Column Address Strobe (Output) ==> This output can be

used for access of DRAM external to the computer.  Due to timing

restrictions caution should be taken when using this signal in the

expansion box.

Pin 43  MPD'   Math Pack Disable (Input) ==> This open collector

input is used to disable the math pack section of the OS ROM

(D800H-DFFFH).  This should be done when the E.B.D. is selected and has

a handler resident.  The driving device should be capable of sinking

10mA.

Pin 44  RAS'   Row Address Strobe (Output) ==> This output can be

used for access of DRAM external to the computer.  Due to timing

restrictions caution should be taken when using this signal in the

expansion box.

Pin 46  LR/W'   Latched Read Write (Output) ==> This output is active

high for a read cycle and active low for a write cycle.

Pin 49  AUDIO   Audio In (Input) ==> This line is tied directly to

the audio summation network of the computer.  The audio signal input is

100 mV peak to peak with 4.7K ohm source impedance.

Pins 1,10,19,29,30,32,42,45,50 are Ground (GND).

Pins 33,47,48 are +5 Volts.

Pin 37 is -12 Volts.

Pin 39 is +12 Volts.

2.2.1.3 Interface Requirements

For peripherals to interface through the E.B.I. to the computer the

following requirements must be followed.

2.2.1.3.1 D.C. Characteristics

All E.B.I. outputs from the Buffer Box have the capability of driving

TTL load.

All E.B.I. open collector input lines must be able to sink 10mA (min)

at .4V (max).

All E.B.I. non-open collector input lines except AUDIO must have the

drive capability of 48mA at logic 0 (Iol) and -15mA at logic 1 (Ioh).

All E.B.I. signals except AUDIO will be at standard TTL logic levels.

The AUDIO input line must drive a 4.7K Ohm source impedance with a

100mV peak to peak signal.

2.2.1.4 Hardware Device Protocol

The E.B.I. is designed to support two types of devices.  The first is

an Expansion Box Peripheral (E.B.P.) and the second is an External

Application Cartridge (E.A.C.).

2.2.1.4.1 Expansion Bus Peripherals (E.B.P.)

The E.B.P.s have the following characteristics:

1.  The interface between the E.B.P. and the CPU is defined through

the handler/OS resident in the OS ROM.  The OS can support 8 devices at

one time with only one enabled during any given interval.

2.  Every E.B.P. has a unique handler that resides in the CPU memory

from D800H-DFFFH.  The ROMS containing the code for these handlers are

physically resident on the respective E.B.P.s.  To access this handler,

the math pack must be disabled with MPD'.  When the math pack is

disabled (this should happen whenever the E.B.P. is selected and has an

external handler) the computer will generate EXTENB for the math pack

area.  The E.B.P. must generate the correct EXTENB/EXTSEL' protocol.

If the device does not generate EXTSEL' the CPU will access (in the 64K

computers) an unused area of ram.  This area should not be used since

all computers of this series do not have that area of ram.

3.  The location D1FFH in the CPU memory map is reserved for passing

control information between the CPU and the E.B.P.s.  The CPU selects

one of the E.B.P. devices by writing a "1" into the desired bit in

location D1FFH.  The device can be deselected by writing a "0" into the

desired bit.

7    6    5    4    3    2    1    0

-----------------------------------------

| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |

-----------------------------------------

Therefore the CPU can then access 8 devices, one at a time.  If the

IRQ' line is pulled "low" the CPU can read the status for location

D1FFH and locate the requesting E.B.P..  A "1" in a bit Ix (where x=0

to 7) corresponds to an interrupt in E.B.P. x.  If the bit is a "0"

then the device has not caused the interrupt.  The E.B.P. must clear

the interrupt flag when the interrupt is being serviced.

7    6    5    4    3    2    1    0

-----------------------------------------

| I7 | I6 | I5 | I4 | I3 | I2 | I1 | I0 |

-----------------------------------------

4.  An E.B.P. should assert MPD' only when it is selected.  An E.B.P.

should assert EXTSEL' only when it is selected and if EXTENB' is

asserted.

5.  An E.B.P. may respond to any selects D0 through D7.  It is

recommended that the E.B.P.s have configuration switches to allow them

to respond to any one of the selects.  Some of the computer systems use

the E.B.I. to support internal devices therefore the user should check

each manual for device locations.  If the system has devices in

specific locations, those are reserved in that computer.

6.  An E.B.P. handler may respond to addresses in the region

D800H-DFFFH only when it is selected.

7.  A peripheral may respond to addresses in the region D100H to

D1FEH only when selected.

8.  The E.B.P.s will have software priority over the SIO peripheral

when they are addressed generically.

9.  Data transfer between the CPU and the E.B.P. is under the control

of the peripheral handler for the E.B.P.s.  The peripheral handler may

or may not be in the E.B.P.  Data transfer is handled by a combination

of code in the O.S. and the E.B.P.

10.  The E.B.P.s shall work if a cartridge is present in the

cartridge slot.

11.  The CPU address space from D600H to D7FFH is reserved for E.B.I.

devices as follows:

Device   Range         Size

------   -----         ----

D0       D600H-D61FH   32 Bytes

D1       D620H-D63FH   32 Bytes

D2       D640H-D65FH   32 Bytes

D3       D660H-D67FH   32 Bytes

D4       D680H-D69FH   32 Bytes

D5       D6A0H-D6BFH   32 Bytes

D6       D6C0H-D6DFH   32 Bytes

D7       D6E0H-D6FFH   32 Bytes

The region from D700H to D7FFH is reserved for use by ATARI.  The CPU

address space from D600H to D7FFH is always mapped to the E.B.I. and

does not require the EXTENB/EXTSEL' protocol.  Some units may have

devices mapped on the E.B.I.; these devices are only active when

selected but care must be taken when using a unit with onboard devices.

2.2.1.4.2 External Application Cartridges

The "External Application Cartridge" (EAC) is a generalization of the

E.B.P..  They have the following characteristics:

1.  An EAC can reside at any or all of the locations in the region

0000H to BFFFH for which the EXTENB is generated.  They may respond to

these addresses only when "opened".

2.  The EACs must be opened by the protocol given for the E.B.P.s.

3.  The EACs must conform to the EXTENB/EXTSEL' protocol.

4.  The EACs must have a handler (resident at location D800H-DFFFH)

that controls their operation.  The EACs must conform to conditions (4)

through (11) of section III. B. above.

5.  The EACs will work if a cartridge is present in the internal

cartridge slot.

2.2.2 I/O Channels

The expansion box contains two RS-232C serial channels and one

Centronics type parallel printer channel.  Both the serial and parallel

channels are preset to device #6.  This device cannot be used in the

Expansion Box.

2.2.2.1 Serial Channel Description

The Expansion Box will have two RS-232C Asynchronous serial channels

capable of simultaneous two way communication.  Both channels should be

addressable through software via CIO using the same format as the

serial channels in the ATARI 850.  The baud rate will be software

selectable from 16 baud rates (see Table 1).  The baud rates of the two

channels are to be independent of each other.

Sel Value     Baud Rate

----------------------------

|    0000    |    50       |

|    0001    |    75       |

|    0010    |    110      |

|    0011    |    134.5    |

|    0100    |    150      |

|    0101    |    200      |

|    0110    |    300      |

|    0111    |    600      |

|    1000    |    1200     |

|    1001    |    1800     |

|    1010    |    2400     |

|    1011    |    3600     |

|    1100    |    4800     |

|    1101    |    7200     |

|    1110    |    9600     |

|    1111    |    19200    |

----------------------------

Table 1  Baud Rate Selection Chart

2.2.2.2 Parallel Channel Description

The basic Parallel Channel is to support a Centronics type parallel

printer.  To do this the following signals are supported.

D0-D7     Data Lines

DS'       Data Strobe

BUSY      Busy Input

FA'       Fault Input

2.2.3 Buffer Box

To allow connection to the XL-series computer, the Expansion Box will

use a buffer box connected to the back of the computer.  This Buffer

Box will be able to drive a 3 foot long cable and the Expansion Box.

The power used by the Buffer Box will be supplied by the Expansion Box

on the Reserved Pins of the cable.  The connectors used will be the

standard 50 pin P.B.I. connectors.

2.2.4 Power Supply

The Expansion Box power supply will have sufficient output to drive

each slot with .75 Amps at +5 VDC, .1 Amps at +12 VDC, and .1 Amps at

-12 VDC.  Table 2 shows max operating current limits of the supply.  A

fan might be necessary to provide cooling in the Expansion Box.

+5 VDC        6.75 Amps

+12 VDC       1.0 Amps

-12 VDC       1.0 Amps

Table 2

2.3 Software

2.3.1 Expansion Bus Protocol

The Software handlers and application programs written for use

with/in the Expansion Box must follow standard Protocol defined by "The

Software Implementation of Parallel Device Handlers and Drivers" by

Rick Nordin.  A Designers guide to interfacing devices to the PBI and

Expansion Box will also contain the required documentation concerning

the protocol.

2.3.2 I/O Channel Handlers

The handler used in the Expansion Box I/O channel will conform to the

above specifications as well as providing support for the printer as a

standard CIO device call.

2.4 Product Performance

This section will specify the projected performance of the Expansion

Box.  The environmental and reliability information that is included is

targeted from the Sweet 16 specification.

2.4.1 Environmental

Operating Environment

Temperature =

Maximum:          45 degrees C

Minimum:          10 degrees C

Humidity =

Maximum:          90% R.H.  (Non Condensing)

Minimum:          15% R.H.

Altitude =

Maximum:          3000 Meters (9840 feet) (720 millibars)

Minimum:          -60 Meters  (-197 feet) below sea level

Non-Operating Environment

Temperature =

Maximum:          60 degrees C

Minimum:          -30 degrees C

Humidity =

Maximum:          90% R.H.

Minimum:          0%  R.H.

Should condensing occur, unit must be dried off before operation.

Altitude =        Same as operating

2.4.2 Endurance Levels

ESD Susceptibility

No product damage or data loss with 10 KV to 20 KV discharge at any

point accessible to the user except the connectors.

Vibration

Operation:         .1g +- 10%  5 to 500Hz

Resonance Search:  Sine scan 5-100 Hz dwell on resonances

1.0g for 10 minutes.

Transportation:    100 to 300 cycles per minute 2 directions 90

degrees apart, 30 minutes; each frequency to be such as to raise

package from the table 0.06 in., acceleration to be 1.0g (min)  No

Damage should result.

Impact Test:       Free fall distance of 24 in. on corner, edge, and

all 6 faces.  No damage should occur to a packed unit.

Random Vibration:  0.04g 2Hz 15 minutes; 10 to 1000Hz 6.3g RMS

Thermal Shock:     10 degrees C for 3 hours power on raise

temperature, 5 degrees per minute to 45 degrees C for 5 hours power off

drop temperature 5 degrees C per minute to 10 degrees C.  Repeat cycle

5 times no damage

2.4.3 MTBF & MTTR

The targeted MTBF for the Expansion Box is 8000 hours continuous

power-on at 25 degrees C.

The targeted MTTR for the unit is 5 minutes.

2.4.4 Compliances

UL 114 & UL 94HB

CSA C22.2 No. 154

FCC Docket 20780, Part 15, Subpart J, Class B

Expansion Box Specifications

10/14/83

Below are the changes to the document of the same name dated 7/27/83.

1.2 Assumptions

1.2.1 Product Reasoning

This product is intended to add a strategic marketing dimension to

the Atari home computer line by providing expansion capability.

1.2.2 Manufacturing

It is assumed that the Expansion Box will be manufactured by some, as

yet unspecified, overseas vendor.

1.2.3 Strategic Fit

This product is intended to broaden Atari's image as a complete

computer supplier.  This is done by offering an expansion path for

computer users to grow with their systems.  A line of option cards will

allow even the most sophisticated users to advance to an advanced

system.

1.2.4 Product History

As of the date of this document the expansion box has been redefined

as a smart device rather than an extension of the Parallel Interface

Bus.  This was done to facilitate third party vendors in the

implementation of option cards.  This will also allow for easy

interfacing of the Expansion Box to the current 400/800 installed base.

This choice should create added sales of this device as well as showing

the public Atari will not leave current users unsupported!  In addition

to the above features, a RFI problem is also reduced creating a faster

path to bring the product to market.

1.3 References

Sweet 16 Expansion Box Specification, A. Chopra

P.B.I. Specification, S. Miller

SURELY O.S. Rom Specification, S. Scheiman, R. Nordin

Software Implementation of Parallel Devices, R. Nordin

Sweetpea Product Specification, D. Lang

JULIE FIFO Specification, S. Miller

2.2.2.4 Hardware Device Protocol

The hardware protocol for the Expansion Box is similar to the

Expansion Bus Interface on the computer in that each card is selected

through a common register.  There will also be device handlers on each

card in a fixed location of memory.  The bus timing is configured so

any type of device can be easily interfaced to the Expansion Box.  For

complete interfacing requirements a third party vendor document will be

created with examples of interfacing methods.

2.2.3 Processor Card

2.2.3.1 CPU

The CPU for the Expansion Box will use a 6502 operating at 1 MHz.

This is done for software compatibility with current developmental

tools.  The placement of the CPU on a card allows the user to swap

other CPU cards for the 6502 card.

2.2.3.2 ROM

There is provision for 8 K Bytes of monitor Rom on the processor

card.  This ROM can also be disabled by asserting ROMDIS' line on the

bus.  The ROM will be located at E000H-FFFFH.

2.2.3.3 RAM

There is provision for 2 K Bytes of general purpose RAM on the

processor card.  The RAM can be disabled by asserting RAMDIS' line.

This will allow for remapping of page 0 and 1 to swap tasks in and out

without moving large areas of memory.  The RAM will be located at the

bottom of the memory space.

2.2.3.4 Serial Channel Description

The processor card will have two RS-232C asynchronous serial

channels, capable of simultaneous two way communication.  Both channels

are addressable through the CIO in the operating system of the Atari

computer.  The baud rates for the channels are found in table 1.  Both

channels have independent transmit and receive baud rates.  One of the

two serial ports will be jumperable to allow operation as a SIO port

for operation with current Atari 400/800 units.

BAUD RATES

50

75

110

134.5

150

200

300

600

1050

1200

1800

2000

2400

4800

7200

9600

19200

38400

Table 1

2.2.3.6 Bus Interface

The Bus Interface for the processor card will support the load

requirements of the Expansion Bus and follow the protocol of the Bus

timing.

2.2.4 Computer Interface

The Expansion Box will interface with the Atari XL product line

through an interface unit connected to the Parallel Expansion Bus.  The

interface unit will contain a JULIE FIFO interface chip designed to

allow communications between the XL computers and the Expansion Box.

In addition the interface unit will have a 2K handler rom for the

P.B.I. and one LS-TTL decoder chip.  Power for the interface unit will

be provided by the expansion box.  The Interface will be connected to

the Expansion Box via a 26 conductor double shielded cable.  The cable

is to be 3 feet in total length.

2.2.5 Power Supply

Shown below are the total power requirements.

+5  VDC       4.5 Amps

+12 VDC       0.5 Amps

-12 VDC       0.5 Amps

+9  VDC       0.2 Amps unregulated

2.2.6 Memory Map

FFFF +----+

|    |

|    | 8K System ROM

|    |

E000 +----+

|    | I/O Space

D000 +----+

|    | Apple Board Space

C000 +----+

|    |

|    |

A000 |    |

|    |

|    |

8000 |    |

|    |

6000 |    | Free Memory Space

|    |

4000 |    |

|    |

2000 |    |

|    |

0800 +----+ 2K RAM

0000 +----+

System Map

DFFF +----+ Device select and INT status

|    |

|    |

DC00 |    |

|    |

|    |

D800 |    | Open Device I/O Area

|    |

|    |

D400 |    |

|    |

|    |

+----+

|    | CPU Interface, UART, Parallel Printer

D000 +----+

I/O Map

3.0 Future Enhancements

The future enhancements include a SIO card to allow operation with

the current Atari 400/800 line as well as the option cards stated in

the introduction.  As of this document no option cards are defined.

4.0 Cost Targets

The targeted standard cost for this unit is $125.  This cost will

include the processor card with 2 serial channels and the interface to

the current XL product line.

5.0 Packaging

5.1 Inner Packing

The inner packing must protect the Expansion Box and accessories from

10 drops from 24 inches on any of six surfaces, three edges radiating

from one corner without any internal damage.  The inner packing must

also protect the Expansion Box and accessories from vibrations of 30

minutes each, 2 directions 90 degrees apart, 100-300 cycles per minute.

Packing must leave the surface 0.06 inches at some time, again leaving

no damage.

5.2 Display Carton

The display carton should be a six sided 24 pt. SBS (white chipboard)

folding carton, printed in 4 colors plus PMS877 silver, with UV or

other abrasion resistance coating.  All graphics will be specified by

Atari.

5.3 Shrink Wrap

The fully packaged product will be shrink wrapped to prevent damage

and pilferage.

5.4 Shipping Carton

The weight and type of corrugate as well as the number of products

per shipping carton are to be specified by Atari.

5.5 Carton Contents

* Expansion Box Unit

* Interface Unit for XL computers

* Interface Cable

* 2 RS-232C Cables

* Processor Card

* Owners Manual and Warranty card

* Atari products catalog

5.6 Product Name

The proposed official name is "Atari Expansion Box".

6.0 Manual Requirements

The unit will include an owners manual to be specified by Atari.

Expansion Box Specifications

5/11/84

Below are the changes to the documents of the same name

dated 7/27/83 and 10/14/83 respectively.

1.0 Introduction

The Expansion Box is an extension device providing support

of expansion cards for the Atari Computer line.  This device

provides Atari systems with expansion and enhancement.  The

bus enables the consumer to connect a variety of different

devices to the Expansion Box Interface (E.B.I.).  Some

devices are listed below.

1) RAM EXPANSION

2) SERIAL/PARALLEL INTERFACE

3) Z-80 PROCESSOR

4) 80-COLUMN VIDEO MONITOR INTERFACE

5) HOBBYIST PROTOTYPING CARD

2.1.1 Appearance

The Expansion Box matches the styling of the current XL

product line.  The Box has all connections to devices and

power in the rear of the box.  A flat cable (connected to the

front of the box) connects the Expansion Box to all

PBI-equipped computers.

2.1.2 Design Concepts

The Expansion Box requires less than one square foot of

table space.  The top is easily removable so the user can

access the option cards.  The Box supports five 9 by 5 inch

option cards.  To remove the top of the box, the AC adaptor

must be unplugged from the box.

2.2.1 Architectural Overview

The expansion box contains a triple-output power supply

providing regulated +12 volts and -12 volts and +10 volts

unregulated.  Also provided is a half-wave rectified AC

waveform (for power-line frequency reference only) and +5

volts (for reference only).

The address bus, data bus and control lines are buffered by

the expansion box and bussed to the five edge connectors

provided.

2.2.2.1 Connector

The Expansion Box Interface signals are accessible through

a 50-pin connector.  This connector is an AMP type

2-530843-9.  It mates to a standard card edge with gold

plated fingers at .100 inch centers.

2.2.2.3.1 D.C. Characteristics

All E.B.I. outputs have the drive capability of 24mA at

logic 0 (Iol) and -6mA at logic 1 (Ioh).

All E.B.I. open collector input lines must be able to sink

10mA (min) at .4V (max).

All E.B.I. non-open collecotr input lines except AUDIO must

have the drive capability of 24mA at logic 0 (Iol) and -6mA

at logic 1 (Ioh).

All E.B.I. signals except AUDIO will be at standard TTL

logic levels.

The AUDIO input line must drive a 4.7KOhm source impedance

with a 100mV peak to peak signal.

Each expansion box card should not load any E.B.I. output

line with more than three standard TTL loads.

2.2.2.3.2 A.C. Characteristics

All specifications are referenced to the standard 2 MHz

6502 specifications.

The signal PHASE2 is the 6502 clock at the processor.

2.2.2.3.3 Materials Requirements

Contact fingers shall be plated with nickel of the

low-stress type, Class II per federal specification QQ-N290A.

Nickel plate thickness shall be 100 +/- 25 microinches.  The

nickel plating shall be overplated with gold type II (23+

karat), 15 microinches minimum (130-200 knoop hardness) or 10

microinches minimum (201 or over knoop hardness).  Impurities

shall not exceed .1%, not including metallic hardeners.

Roughness shall be less than 50 microinches.

2.2.3 Power Supply

The power supply is comprised of an external AC adaptor

capable of providing 40VA at 9.5 vAC to the Expansion Box and

an internal rectifier/filter/regulator system.  This system

provides:

+10 vDC  UNREGULATED (+40% or -15%) at .5A per card slot.

+12 vDC  REGULATED (+ or - 4%) at 30mA per card slot.

-12 vDC  REGULATED (+ or - 4%) at 30mA per card slot.

+5 VREF  REGULATED (+ or - 4%) as a reference only.

AC - a 60 Hz reference only.

Total supply current capacity is five times the per card

slot amounts.

It is recommended that the Expansion Box NOT be operated

with the cover removed for safety reasons and to comply with

radiated emissions standards.

3.0 Notes

3.1 RAM 64KMR

The RAM 64KMR module is designed to be used in the

expansion box.  However, it does not completely adhere to the

E.B.I. protocol since it will allow RAM to be selected in

lieu of the O/S (when used in the "1064 mode".)

3.2 RESERVED MEMORY LOCATIONS

The memory space from D100H to D1AFH is available for use

by most applications.  This space should be used by a device

with the device select bit enabling its use.  If the device

select bit is not used, there is potential bus conflict

between E.B.D.s.  The remainder of the I/O space between

D1B0H and D1FFH is mapped as follows:

D1B0H - D1C7H   Speech/Modem/Disk Registers

D1C8H - D1CEH   Atari Reserved

D1CFH           Alternate Interrupt Register (1450 only)

D1D0H - D1DFH   Audio Registers

D1E0H - D1E7H   Atari Reserved

D1E8H - D1EFH   Parallel/Serial Registers

D1F0H - D1F7H   Alternate CPU Registers

D1F8H - D1FDH   80 Column Video Registers

D1FEH           RAM Bank Select Register

D1FFH           E.B.D. Select/Interrupt Register

PARALLEL BUS INTERFACE SPECIFICATIONS

1.0  Introduction:

The Parallel Bus Interface (PBI) is provided on the S-16 to support

high speed peripherals, communication devices and cartridges.

The objective of the PBI is two-fold:

(1)  To provide for future system improvement and expansion of the

S-16.

(2)  To provide a simplified interface standard that will envoke

interest from peripheral manufacturers in making peripherals for the

S-16.

The physical connection of the peripherals to the S-16 is conceived

of being through a unit called the "Expansion Box".  Roughly, the

"Expansion Box" provides a low cost, easy-to-use multiple access

mechanism to the PBI.  It consists of electronics necessary to

implement the interface protocol between the CPU and the Parallel Bus

peripherals.  Further details of the Expansion box are TBD.

This document gives a logical definition of the pins on the PBI.  It

defines the characteristics of devices that can be supported on the

PBI.

The electrical specifications of the PBI signals and the details of

the PBI protocol are in the S-16 Product Specification.

2.0  THE PARALLEL BUS INTERFACE

The Parallel Bus Interface Connector is a 50 pin connector (See

figure 5.2.9) with the following signals.

PIN 3 through PIN 18 are A0 through A15 respectively.  These are the

CPU address lines.  The CPU addresses the Parallel Bus Interface

devices using these lines.

PIN 19 through PIN 26 are D0 through D7 respectively.  These are the

processor data lines.  The CPU transmits and receives data and control

information from the Parallel Bus Interface devices on these lines.

PINS 35, 36, and 37: are COMM A, COMM B, and COMM C select lines

respectively.  The Parallel Bus Interface devices lie in the CPU memory

map.  Three 256 byte segments of the CPU memory map are reserved for

control of the parallel bus devices.  COMM A selects the segment from

D600 to D6FF.  COMM B selects the segment from D700 to D7FF.  COMM C

selects the segment from D100 to D1FF.  COMM A, COMM B, and COMM C are

low when active.

PIN 31 is for the Buffered Phase 2 Clock (B02) Output to the Parallel

Bus Interface devices.

PINS 2, 44, 45, and 46 are not connected.

PIN 29 is the IRQ (Interrupt Request) input from the Parallel Bus

Interface devices.  A PBI device can pull this input low to invoke the

device handler that services the parallel bus device.  This input is

"open drain".

PIN 32 is the Read/Write (R/W) Output to the Parallel Bus Interface

devices.  This line is "high" for a read cycle.  It is low for a write

cycle.

PIN 34 is the PoWeR ON (PWRON) Input from the Parallel Bus Interface

devices.  This input will allow the implementation of the "auto-wakeup"

feature in future enhancements of the S-16.  This signal is "low" when

active.

PIN 38 is reserved for future expansion.

PIN 33 is Power on Reset (POR) ouput to the Parallel Bus Interface

devices.  This output resets any device so designed on power up and

when the RESET key on the S-16 keyboard is hit.

PIN 49 is audio in and out signal from/to the Parallel Bus devices.

This line is connected directly to the audio summation network of the

S-16.  This audio signal is a 1 volt peak to peak.

PIN 39 is the MODSEN (Module Sense) input from the Parallel Bus

Interface Modules.  This input is pulled low whenever a Parallel Bus

Interface module is connected to the PBI.

PINS 47 and 48 are GROUND.

PIN 28 is External Decoder Enable (EXTENB) Output.  This output goes

high when an address on the CPU bus is an allowed address for a PBI

device.  The S-16 generates the EXTENB signal for all segments of the

CPU memory map except:

(a)  The Active segments of the O.S. ROM.  Any disabled segments of

the O.S. ROM generate the EXTENB signal when addressed.

(b)  The region occupied by the internal cartridge (if present).

PIN 27 is the External Select (EXTSEL) Input.  This input is

generated by the (external) decoder resident in the PBI device.  This

signal should go low whenever the EXTENB is enabled and the PBI device

uses the address generated on the CPU Bus.  This input is used to

disable the S-16 decoder for the duration of the current bus cycle.

PIN 30 is the RDY input to the S-16.  A slow PBI device can extend

the CPU bus cycle by pulling this line low when it is addressed.

PIN 40 is the Refresh (REF) output.  This output may be used for the

refresh timing of volitile memories connected to the PBI.

PIN 1 and PIN 50 are shield grounds.

PIN 41 is Auto-run Present (ARP) pin.  This pin goes high whenever an

auto-run cartridge is present at the PBI.  All auto-run cartridges

should pull this line to the "high" state internally.

PIN 42 is Auto-run Disable (ARD) pin.  This pin goes high whenever a

cartridge is present in the internal cartridge slot.  The Auto-run

cartridge should disable their decoder whenever this line is in the

"high" state.

PIN 43 is Math Pak Disable (MPD) input from the PBI devices.  This

input is pulled to the "low" state whenever a Parallel Bus Peripheral

is selected by the CPU (the enable bit in location D1FDH is set).

----------

SHIELD GND |-1    2-| N/C

A0 |-3    4-| A1

A2 |-5    6-| A3

A4 |-7    8-| A5

A6 |-9   10-| A7

A8 |-11  12-| A9

A10 |-13  14-| A11

A12 |-15  16-| A13

A14 |-17  18-| A15

D0 |-19  20-| D1

D2 |-21  22-| D3

D4 |-23  24-| D5

D6 |-25  26-| D7

EXTSEL |-27  28-| EXTENB

IRQ |-29  30-| RDY

B02 |-31  32-| R/W

POR |-33  34-| PWRON

COMM A |-35  36-| COMM B

COMM C |-37  38-| P0

MODSEN |-39  40-| REF

ARP |-41  42-| ARD

MPD |-43  44-| N/C

N/C |-45  46-| N/C

GND |-47  48-| GND

AUDIO |-49  50-| SHIELD GND

----------

FIGURE 5.2.9  PBI Connector

3.0  Parallel Bus Devices

The Parallel bus will support 3 types of devices:

3.1  Auto-Run Cartridges:

These cartridges are functionally identical to those that plug into

the cartridge slot of the S-16 except that they can be up to 32K Bytes

in size (in 4K Byte increments).  The top address for these cartridges

must be BFFFH and they must assert the "AUTO-RUN PRESENT" line when

inserted into the expansion box.  The auto-run cartridges work under

the following constraints:

(a) Only one such cartridge will work at a time.

(b) If a cartridge is present in the internal cartridge slot, the

auto-run cartridge is ignored by the S-16.

(c) If no cartridge is present in the internal cartridge slot the

auto-run cartridges operate in a manner identical to the cartridge slot

cartridges.  No additional handlers or O.S. changes are needed.

(d) If an auto-run cartridge is present in the expansion slot, any

"External Application" cartridges (See Sec. 3.3) plugged into the

Expansion box will be disabled.

(e) The Auto-run cartridges conform to the EXTSEL/EXTENB protocol

(have their own decoder).

(f) The Auto-run cartridges must disable their decoder if the

"Auto-run Disable" output of the S-16 is active.

3.2  Parallel Bus Peripherals (PBPs)

The PBPs have the following characteristics:

(a) They interface to the S-16 through a well defined handler/O.S.

interface.  The code for this interface is resident in CPU memory

location D600H to D7FFH (COMM A & COMM B areas).  The ROM containing

this code is physically located on the expansion box.

The S-16 O.S. can support up to 8 PBPs.

(b) Each PBP has a unique handler that resides in the CPU memory

space at locations D800H to DFFFH.  The ROM(s) containing the code for

these handlers are physically resident on the respective peripheral

boards (The memory space occupied by the handlers actually contains the

Math-pak within the S-16.  The Math-pak is disabled whenever any PBP is

enabled. (See (c) below).

(c) Locations D1FDH thru D1FFH (in COMM C space) in the CPU memory

map are reserved for passing control information between the S-16 and

the PBPs.

Location D1FDH is Slot-Select location with the following bit

assignments:

7   6   5   4   3   2   1   0

---------------------------------

| X | X | X | X | E | I | I | I |

---------------------------------

E = Enable bit.  The S-16 sets this bit to logic 1 whenever the O.S.

wishes to communicate with a device in the slot whose I.D. is specified

by III.  When the Enable bit is set to logic 1 the S-16 disables the

Math-pak and maps the region from D800H to DFFFH to the expansion box.

The Enable bit is latched in the expansion box and it must be set to

logic "0" to disable the PBPs.

III = SLOT I.D.  The S-16 O.S. can support a maximum of 8 PBPs.  The

slot I.D. is latched in the expansion box and a slot remains selected

till a different slot I.D. is generated or the enable bit is set to

logic 0.

X = Don't Care.

The Slot I.D. decoder is physically located on the expansion box.

location D1FEH is Interrupt vector location with the following bit

assignments:

7   6   5   4   3    2   1   0

----------------------------------

| X | X | X | X | IR | I | I | I |

----------------------------------

IR = Interrupt Request.  This bit, when logic 1, indicates to the

S-16 O.S. that a device in the Expansion box has interrupted.  When

this bit is logic 0, no device in the expansion box has interrupt.

III = is the slot I.D. of the highest priority slot that generates

the interrupt.  The S-16 O.S. assumes that the slots have fixed

priority.  The slot with an I.D. of 7 has the highest priority.

X = Don't Care.

The priority encoder is physically located on the expansion box.

Location D1FFH is un-encoded interrupt location with the following bit

assignments:

7    6    5    4    3    2    1    0

-----------------------------------------

| I7 | I6 | I5 | I4 | I3 | I2 | I1 | I0 |

-----------------------------------------

IX (X = 0 to 7):  This bit is set to logic 1 whenever a device in

slot X generates an interrupt.  This bit is cleared by the interrupt

servicing routine for the device in slot X.

The un-encoded interrupt location allows an application program to

mask the interrupts from the specific slots.  With the masking

arrangement, an application program may ignore the fixed priority by

slots and implement its own priority scheme.

(d) A peripheral handler may respond to address in the region

D800H-DFFFH only when the slot in which it resides is selected.

Location D800H contains the device I.D. of a PBP.  If a slot that

contains no peripheral is selected, address D800H should return FFH as

data.

The peripheral handlers must conform to the EXTSEL/EXTENB protocol as

specified in the S-16 Product Specifications.

(e) A peripheral may respond to address in the region D100H to D1FCH

only when the slot in which it resides is selected.

(f) The PBPs will have priority over the SIO peripheral when they are

addressed generically.

(g) The data between the S-16 and a PBP is passed under control of

the peripheral handler for the PBPs.

(h) The PBPs will work if an auto-run cartridge or an internal

cartridge is present in the system.

3.3  "External Application" Cartridges:

The External Application Cartridge are a generalization of the

auto-run cartridges and the PBPs.  They have the following

characteristics:

(a) An "External Applications" Cartridge (EAC) can reside at any or

all of the address in the region 4000H to BFFFH.  They may respond to

these addresses only when "opened".

(b) The EACs must be "opened" by the S-16 through the slot select

location in the same way as the PBPs.

(c) The EACs must conform to the EXTENB/EXTSEL protocol.

(d) The EACs must have a "handler" (resident in locations D800H to

DFFFH) that controls their operation.  The EACs must conform to

conditions (d) and (e) of section 3.2 above.

(e) The EACs will work only if no auto-run cartridge is enabled in

the expansion box.  The EACs must input the AUTO-RUN DISABLE line to

ascertain that the Auto-run cartridge is disabled before asserting

themselves on the Expansion Bus.

(f) The EACs may work even if a cartridge is present in the internal

cartridge slot.

4.0  assumptions

(a) The above description of the PBI devices assumes that there is a

mechanism in the Expansion Box that prevents more than one auto-run

cartridge from being turned on at a time on the PBI.  This mechanism

may be a daisy chain priority scheme by slots or the auto-run

cartridges may be specified to work in only one of the 8 possible

slots.

(b) The above description also assumes that there is a power shut-off

interlock switch on the internal cartridge door.